3GPP TSG-RAN4 Meeting #116bis	        R4-2514356 Prague, Czech Republic, 13th October – 17th October 2025

Source: 	Qualcomm Inc.

Title: 	6 MHz Channel Bandwidth for UE RF

Agenda Item:	7.1.5.2

Document for:	Discussion

WI Code:	NR_UE_RF_Ph5-Core

# Introduction

RAN has agreed a work item for FR1 NR support to include 6 MHz channel bandwidth for bands n5, n8, n12, n85 and n89 [1]. Part of the work item objectives for RAN4 is to specify generic core and band specific requirements. In this contribution system parameters for the requested bands are discussed and proposal for the spectrum utilization is made.

# Discussion

- Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth

  - Only include 15 kHz SCS

  - Specify the spectrum utilization

  - Specify UE RF requirements for UL and DL

- Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4]

  - UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed

    - Additional Maximum Power Reduction (A-MPR)

NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

The WI objectives which are relevant to RAN4 UE RF work are copied below.

From the WI objectives it can be observed that while the RAN4 requirements are intended to cover 6 MHz spectrum allocation in bands n5, n8, n12, n29 and n85; the support is requested for 15 kHz SCS only.

Observation 1: Only one SCS is in WI scope of addition of a new 6 MHz channel bandwidth.

Proposal 1: Introduction of new 6 MHz channel bandwidth in bands n5, n8, n12, n29 and n85 for SCS = 15 kHz.

## 2.1 : Spectrum Utilization

Maximum transmission bandwidth configuration for channel bandwidths less than 20 MHz with SCS = 15 kHz are analyzed along with the corresponding guard band in Figure 1. 30 RBs are considered for 6 MHz channel bandwidth which translate to 292.5 kHz guard band.

Figure 1: Channel Bandwidth (up to 20 MHz) Characteristics for SCS = 15 kHz

Observation 2: Channel bandwidths less than 10 MHz have 90% spectrum utilization.

Proposal 2: Include 30 RBs for maximum transmission bandwidth configuration for 6 MHz channel bandwidth with SCS = 15 kHz which maintains 90% spectrum utilization and a guard band of 292.5 kHz.

2.2 : Transmitter Characteristics

2.2.1 MPR and A-MPR Evaluation

Proposal 3: Use the same MPR tables for PC1, PC2 and PC3 UEs for 6 MHz channel bandwidth with 30 RBs as maximum transmission bandwidth configuration for SCS = 15 kHz.

It can be observed from the Table 1 below that multiple network signalling (NS) labels apply to the requested bands to support 6 MHz channel bandwidth. Based on the frequency ranges of requested bands for 6 MHz channel bandwidth and the work completed for 7 MHz channel bandwidth [2]; impact on A-MPR impact due to NS_14 can be ignored.

Observation 3: Signalling labels NS_06, NS_14, NS_15, NS_43, NS_43U and NS_100 apply to bands n5, n8, n12 and n85.

Table 1: Mapping of Network Signalling Labels

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n5 | NS_01 | NS_100 | NS_14 (NOTE 3) | NS_15 (NOTE 3) |  |  |  | Reserved |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  | Reserved |
| n12 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n85 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. NOTE 3: Support of the additional requirement indicated by this network signalling value is indicated by modified MPR behaviour for this band (Annex L.1). |  |  |  |  |  |  |  |  |

Proposal 4: There is no A-MPR impact due to NS_14 for 6 MHz channel bandwidth.

Proposal 5: Evaluate the A-MPR tables for all the remaining applicable NS labels and power classes for 6 MHz channel bandwidth.

2.2.2 Output Power Dynamics

The minimum output power table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 2.

Table 2: Minimum output power

| Channel bandwidth | (MHz) | 3,5,6,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -40 | -40+10log10 (BWChannel /20) | -40+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Proposal 6: The minimum output power for 6 MHz channel bandwidth shall be -40 dBm.

The transmit OFF power table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 3.

Table 3: Transmit OFF power

| Channel bandwidth | (MHz) | 3,5,6,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Proposal 7: The transmit OFF power for 6 MHz channel bandwidth shall be -50 dBm.

2.2.3 Output RF Spectrum Emissions

Occupied channel bandwidth table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 4.

Table 4: Occupied channel bandwidth

|  | NR channel bandwidth (MHz) |
| --- | --- |
|  | 3, 5, 6,10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Occupied channel bandwidth (MHz) | Same as NR channel bandwidth |

Proposal 8: 6 MHz channel bandwidth shall be updated in the Occupied channel bandwidth table.

Spectrum emission mask table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 5.

Table 5: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

Proposal 9: 6 MHz channel bandwidth shall be updated in the General NR spectrum emission mask table.

Additional spectrum emission requirements for network signalling label NS_06 shown in table 6 below should be updated with the addition of 6 MHz channel bandwidth.

Table 6: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5, | 10 | 15 |  |
| ± 0 – 0.1 | -13 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 5 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 5 – 6 | -25 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 |  | -25 |  |  |  |
| ± 10 – 15 |  |  | -25 |  |  |
| ± 15 – 20 |  |  |  | -25 |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth

Proposal 10: 6 MHz channel bandwidth shall be updated in the Additional requirements for “NS_06” or “NS_07” table.

NR ACLR measurement bandwidth table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 7.

Table 7: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3,5,6,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Proposal 11: 6 MHz channel bandwidth shall be updated in the NR ACLR measurement bandwidth table.

General spurious emissions limits table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 8.

Table 8: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 6, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 |

Proposal 12: 6 MHz channel bandwidth shall be updated in the General spurious emissions table.

Additional requirements for network signalling label NS_43 and NS_43U should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 9.

Table 9: Additional requirements for "NS_43" and “NS_43U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 6, 10, 15 |  |
| 860 ≤ f ≤ 890 | -40 | 1 MHz |
| NOTE 1: Applicable for 5 MHz and 15 MHz channel BW confined between 900 MHz and 915 MHz and for 10 MHz channel BW confined between 905 MHz and 915 MHz |  |  |

Proposal 13: 6 MHz channel bandwidth shall be updated in the Additional requirements for “NS_43” and “NS_43U” table.

Requirements for network signalling label NS_15 should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 10.

Table 10: Additional requirements "NS_15"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 6 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

Proposal 14: 6 MHz channel bandwidth shall be updated in the Additional requirements for “NS_15” table.

2.3 : Receiver Characteristics

2.3.1 Reference Sensitivity

Proposal 15: The reference sensitivity for band n5 shall be -97.2 dBm for 6 MHz channel bandwidth with SCS = 15 kHz.

Proposal 16: The reference sensitivity for bands n8, n12 and n85 shall be -96.2 dBm for 6 MHz channel bandwidth with SCS = 15 kHz.

Proposal 17: Uplink configuration for reference sensitivity for 6 MHz channel bandwidth with SCS = 15 kHz for requested bands shall be set to 30 RBs.

2.3.2 Maximum Input Level

Maximum input level should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 11.

Table 11: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 6, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. |  |  |  |  |

Proposal 18: Maximum input level for 6 MHz channel bandwidth shall be set to -25 dBm for 64QAM waveform and -27.5 dBm for 256/1024 QAM waveforms.

2.3.3 Adjacent Channel Selectivity

Adjacent channel selectivity should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 12.

Table 12: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 6, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Proposal 19: Adjacent channel selectivity for 6 MHz channel bandwidth shall be 33 dB.

2.3.4 Blocking Characteristics

In-band blocking parameters table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 13.

Table 13: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 6, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Proposal 20: In-band blocking for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

In-band blocking parameters table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 14.

Table 14: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 3 MHz | REFSENS + 6.0 dB |
| 5 MHz | REFSENS + 6.0 dB |
| 6 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

Proposal 21: Out-of-band blocking for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

2.3.5 Spurious Response

Spurious response parameters table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 15.

Table 15: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 6, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |

Proposal 22: Power in transmission bandwidth configuration for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

Wideband intermodulation parameters table should be updated with the addition of 6 MHz channel bandwidth as highlighted below in Table 16.

Table 16: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 6, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Pw in Transmission Bandwidth Configuration, per CC5 | dBm | REFSENS + 8 dB | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
| BWInterferer 2 | MHz | 3 | 5 |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 – 4.5 /+BW/2 + 4.5 | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. NOTE 5: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Proposal 23: Power in transmission bandwidth configuration per CC in Wideband intermodulation parameters table for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

# Conclusions

In this contribution introduction of new channel bandwidth 6 MHz with 15 kHz SCS was discussed. Following observations and proposals were made:

Observation 1: Only one SCS is in WI scope of addition of a new 6 MHz channel bandwidth.

Proposal 1: Introduction of new 6 MHz channel bandwidth in bands n5, n8, n12, n29 and n85 for SCS = 15 kHz.

Observation 2: Channel bandwidths less than 10 MHz have 90% spectrum utilization.

Proposal 2: Include 30 RBs for maximum transmission bandwidth configuration for 6 MHz channel bandwidth with SCS = 15 kHz which maintains 90% spectrum utilization and a guard band of 292.5 kHz.

Proposal 3: Use the same MPR tables for PC1, PC2 and PC3 UEs for 6 MHz channel bandwidth with 30 RBs as maximum transmission bandwidth configuration for SCS = 15 kHz.

Observation 3: Signalling labels NS_06, NS_14, NS_15, NS_43, NS_43U and NS_100 apply to bands n5, n8, n12 and n85.

Proposal 4: There is no A-MPR impact due to NS_14 for 6 MHz channel bandwidth.

Proposal 5: Evaluate the A-MPR tables for all the remaining applicable NS labels and power classes for 6 MHz channel bandwidth.

Proposal 6: The minimum output power for 6 MHz channel bandwidth shall be -40 dBm.

Proposal 7: The transmit OFF power for 6 MHz channel bandwidth shall be -50 dBm.

Proposal 8: 6 MHz channel bandwidth shall be updated in the Occupied channel bandwidth table.

Proposal 9: 6 MHz channel bandwidth shall be updated in the General NR spectrum emission mask table.

Proposal 10 : 6 MHz channel bandwidth shall be updated in Additional requirements for “NS_06” or “NS_07”

table.

Proposal 11: 6 MHz channel bandwidth shall be updated in the NR ACLR measurement bandwidth table.

Proposal 12: 6 MHz channel bandwidth shall be updated in the General spurious emissions table.

Proposal 13 : 6 MHz channel bandwidth shall be updated in Additional requirements for “NS_43” or “NS_43U”

table.

Proposal 14: 6 MHz channel bandwidth shall be updated in the Additional requirements for “NS_15” table.

Proposal 15: The reference sensitivity for band n5 shall be -97.2 dBm for 6 MHz channel bandwidth with SCS = 15 kHz.

Proposal 16: The reference sensitivity for bands n8, n12 and n85 shall be -96.2 dBm for 6 MHz channel bandwidth with SCS = 15 kHz.

Proposal 17: Uplink configuration for reference sensitivity for 6 MHz channel bandwidth with SCS = 15 kHz for requested bands shall be set to 30 RBs.

Proposal 18: Maximum input level for 6 MHz channel bandwidth shall be set to -25 dBm for 64QAM waveform and -27.5 dBm for 256/1024 QAM waveforms.

Proposal 19: Adjacent channel selectivity for 6 MHz channel bandwidth shall be 33 dB.

Proposal 20: In-band blocking for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

Proposal 21: Out-of-band blocking for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

Proposal 22: Power in transmission bandwidth configuration for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

Proposal 23: Power in transmission bandwidth configuration per CC in Wideband intermodulation parameters table for 6 MHz channel bandwidth shall be REFSENS + 6.0 dB.

# References

[1]	RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, vivo, 3GPP TSG-RAN Meeting #109, Beijing, China, 15th – 18th September, 2025.

[2]	R4-2512824, “Big CR on 7 MHz Channel Bandwidth for bands n5 and n26”, Qualcomm Inc. et al., 3GPP TSG-RAN Meeting #116, Bengaluru, India, 25th – 29th August, 2025.