3GPP TSG-RAN WG4 Meeting #116bis	R4-2514378

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Nokia

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR performance part

Agenda item:

Document for:	Discussion

Introduction

This contribution discusses about the performance part of Fast Scell activation with EMR.

Discussion

About test applicability rules

- Issue 2-2-2: Applicability rules of test case

In the previous meetings companies were proposing various applicability rules for the testing. While we understand the intention to limit the amount of test scenarios that UE needs to perform. We should only consider selection where the core functionality is similar.

This means that we would need to have at least one test case to cover scenarios where the carrier configuration or timer / reporting configuration is different.

Consider applicability only if core UE functionality between two test cases / scenarios is the same.

  - measIdleValidityDuration-r18 and measReselectionValidityDuration-r18 use different capabilities and carrier lists

  - MAC CE based activation and RRC based activation is good to test separately

  - Different test cases are needed where the core requirement delays differ

Reporting mechanism and test case design

Issue 2-2-3: Test case design

The Topic summary R4-2509055 outlines the test scenarios quite well and it’s a good starting point for the discussions.

Issue 2-2-4: Report mechanism of EMR results in the test cases

Generally, the EMR principle in the test cases can be followed.

The common part for the EMR test cases is that they can start all from RRCRelease message.

- T331 is configured Idle mode fast CA/DC eEMR measurement

- T331 is not configured for Idle mode fast CA/DC cell reselection measurement

The following figure illustrates the EMR reporting, which is quite important for the SCell activation procedure.

In EMR and eEMR the reporting is either UE information request / response in connected mode, or it is RRCReconfiguration complete. Currently the rel-18 test cases do not seem to support inactive-mode reporting but a test case can be added to support this scenario.

Support UE reporting in RRCReconfigurationComplete message in INACTIVE test case(s)

Support UE reporting in UE information request / response in IDLE-mode tests cases.

Supported scenarios

Issue 2-2-1: Test case list

SCells are used usually for capacity purpose. Therefore, FR2 support is important and adding FR2 also for direct SCell activation should be considered.

Support also Direct SCell activation at SCell addition for FR2

Configuration 3 was included in the core requirements and therefore we should not exclude it from the test cases.

Support configuration 3 as one of the scenarios.

Test cases and configuration combinations

Generally, there are two frameworks that requires combining, one being EMR framework and the other being SCell activation framework. For the EMR framework the following test cases are supported by legacy:

A.6.6.9.1 SA Idle mode CA/DC measurement for FR1

A.6.6.9.2 Idle mode fast CA/DC eEMR measurement for FR1 without valid reporting

A.6.6.9.3 Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting

A.6.6.9.4 Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting

A.7.6.23.1 Test case for Idle mode fast CA/DC eEMR measurement for FR2 without valid reporting

A.7.6.23.2 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting

A.7.6.23.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting

There are many Scell test cases but RAN4 can down select from them to include at least one scenario for FR1 and one for FR2.

A.5.5.3.1 SCell Activation and deactivation of SCell in FR2 intra-band

A.5.5.3.2 SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle

A.5.5.3.5 SCell Activation and deactivation of SCell in FR2

A.5.5.3.7 Direct SCell activation at SCell addition of known SCell in FR2

A.5.5.3.8 Fast SCell Activation of SCell in FR2 intra-band

A.5.5.3.9 PUCCH SCell Activation and deactivation of known SCell in FR2

A.5.5.3.10 PUCCH SCell Activation and deactivation of unknown SCell in FR2

A.6.5.3.3 SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

A.6.5.3.4 Direct SCell activation at SCell addition of known SCell in FR1

A.6.5.3.6 PUCCH SCell Activation and deactivation of known SCell in FR1

A.6.5.3.7 SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

This means that there are quite many combinations of test cases, RAN4 can discuss which ones to select.

Discuss which combinations of EMR configurations are tested with each SCell scenario.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Consider applicability only if core UE functionality between two test cases / scenarios is the same.

-	measIdleValidityDuration-r18 and measReselectionValidityDuration-r18 use different capabilities and carrier lists

-	MAC CE based activation and RRC based activation is good to test separately

-	Different test cases are needed where the core requirement delays differ

Proposal 2: Support UE reporting in RRCReconfigurationComplete message in INACTIVE test case(s)

Proposal 3: Support UE reporting in UE information request / response in IDLE-mode tests cases.

Proposal 4: Support also Direct SCell activation at SCell addition for FR2

Proposal 5: Support configuration 3 as one of the scenarios.

Proposal 6: Discuss which combinations of EMR configurations are tested with each SCell scenario.