3GPP TSG-RAN WG4 Meeting #116-bis	 R4-2514387

Prague, CZ, 13th – 17th October 2025

Source:	Nokia

Title:	Support of 3MHz channel bandwidth for (e)RedCap devices

Agenda item:	7.4.3.1

Document for:	Discussion

Introduction

In the last RAN plenary meeting, a new WID was approved for NR RRM Phase 6, which includes an objective for supporting less than 5MHz channel bandwidth for (e)RedCap devices. In this document, we present our views on what technical changes in the UE RF specifications are required for this work.

Discussion

Additional channel bandwidth

The basic assumption is that 3 MHz will be the additional channel bandwidth supported by the (e)RedCap devices. In the current specification, for bandwidth restricted eRedCap devices, requirements are based on 5MHz channel bandwidth and that shall not change by the addition of 3 MHz channel bandwidth.

3 MHz channel bandwidth shall be introduced as an additional channel bandwidth for (e)RedCap devices.

Current requirements for bandwidth restricted eRedCap devices will still utilize a maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH.

Required UE RF changes

Channel bandwidth for RedCap devices

In Section 5.3I, the specification explicitly states that 3MHz channel bandwidth shall not be supported in Rel18. This has to be modified to support 3 MHz.

One line deletion is required in section 5.3I.

Reference sensitivity for RedCap devices

RedCap UEs with two Rx antenna ports, utilize the REFSENS requirements defined for a normal UEs in section Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands. For RedCap UEs with one Rx antenna port, RedCap specific ΔR1R in Table 7.3I.2-1 for the applicable operating bands. Given both 1Rx and 2Rx along with HD-FDD are in scope of this work, modification is required for Tables 7.3I.2-1, 7.3I.2-2, 7.3I.2-3, and 7.3I.2-4.

Modification is required for Tables 7.3I.2-1, 7.3I.2-2, 7.3I.2-3, and 7.3I.2-4.

In case of eRedCap devices with bandwidth restrictions, current requirements are defined for only channel bandwidth equal to and larger than 5 MHz. In case of 3 MHz, NRB is 15, since NRB_PR3 is defined as 25 RBs for 15 KHz SCS, this will yield RBlow as negative. To fix this, the easiest solution is to just add an additional line stating NRB_PR3 will be 15 in case of 3 MHz channel bandwidth.

Update the value to NRB_PR3 will be 15 in case of bandwidth limited eRedCap device.

Fixed reference channels

Due to bandwidth restrictions for one category of eRedCap devices, new FRCs were defined. For 3 MHz FRCs, the allocated PRBs and the data rate are within limits of what the reduced bandwidth eRedCap devices support, so no new eRedCap specific 3 MHz FRC is required. Further, as all the desired band are FDD band, TDD FRCs are not needed.

No changes to FDD FRCs are required, existing NR 3MHz FRC can be used.

No new FRCs are required as existing NR 3MHz FRC can be used.

Supported power class

Current (e)RedCap specifications support both PC2 and PC3. Currently, RSD for PC2 is not defined for n106 and n100. Further, 1Rx PC2 REFSENS is defined only for n1 and n3. Given the limited time units available to finish this work, it would be best to keep this work item focused only on PC3.

Limited TUs for finishing PC2 RF requirements for all the exemplary bands.

Preclude PC2 for 3 MHz (e)RedCap.

Conclusion

The following Observations and Proposals were made:

Proposal 1: 3 MHz channel bandwidth shall be introduced as an additional channel bandwidth for (e)RedCap devices.

Proposal 2: Current requirements for bandwidth restricted eRedCap devices will still utilize a maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH.

Observation 1: One line deletion is required in section 5.3I.

Observation 2: Modification is required for Tables 7.3I.2-1, 7.3I.2-2, 7.3I.2-3, and 7.3I.2-4.

Observation 3: Update the value to NRB_PR3 will be 15 in case of bandwidth limited eRedCap device.

Observation 4: No new FRCs are required as existing NR 3MHz FRC can be used.

Observation 5: Limited TUs for finishing PC2 RF requirements for all the exemplary bands.

Proposal 3: Preclude PC2 for 3 MHz (e)RedCap.

- References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple, RAN#109, Beijing, China, Sep 15 – Sep 18, 2025.