3GPP TSG-RAN WG4 Meeting #116bis	R4-2514404

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	RRM performance requirements for Beam Management and Positioning accuracy

Agenda item:	6.11.6

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM and Positioning use cases were continued in RAN4#116 in the scope of Rel-19 WI NR_AIML_air [1] based on the objectives agreed in RAN4#107. Agreements achieved during RAN4#116 meeting are summarized in the WF [2].

In this paper, we address RRM performance requirements for AI/ML enabled beam management and positioning use cases. We discuss following main aspects:

- Testing framework for AI/ML based Beam Management

- Simplification of CDL model

- Performance requirements for AI/ML Positioning

Discussion

Testing framework for AI/ML based Beam Management

In RAN4#116 meeting, enhanced IFF (eIFF) test system equipped with 4 test probes was agreed to be the baseline test system for the conformance testing of AI/ML based beam management (BM) use case. This test system was also selected to be used for the FR2 RRM conformance testing. However, the types of spatial channel models that eIFF test system can support is still unclear and needs further investigation. This becomes even more important since RAN4 has already agreed to use CDL based channel model (full/simplified CDL) for the conformance testing of this use case.

Therefore, we below provide our further views on the open issues to steer the discussion towards finalizing the testing framework for AI/ML based BM use case. For convenience, relevant RAN4 agreements for testing framework are provided below.

From RAN4#110 meeting Issue 2-3: Test setup feasibility for FR2 Companies are invited to provide further analysis on what the test setup should enable in terms of test environment From RAN4#113 meeting 2.3.2 Agreements in main session: Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. From RAN4#114 meeting 2.4 Testing for one sided models Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point From RAN4#114-bis meeting Issue 2-6: Test system setup Agreement: Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified. From RAN4#116 meeting Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline (enhanced IFF system with 4 probes used for FR2 RRM conformance testing) - Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup - Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. - Take CDL-C Umi as one example - Other channel models can be further discussed

Test Setup

Before moving forward, first it is important to understand:

- What are the reasons behind choosing the eIFF test system for the FR2 RRM conformance testing?

- Whether eIFF test system of FR2 RRM conformance testing can directly be used for the conformance testing of AI/ML BM use case with CDL based channel model(s) or further enhancements are needed before it can be used?

To answer the above questions, we need to revisit the corresponding sections from the relevant specifications, which are reproduced below.

## [TS 38.133, Annexure A.3.15] ## A.3.15 Angle of Arrival (AoA) for FR2 RRM test cases ### [Text Skipped] ### A.3.15.6 Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes. Out of the three AoAs, one AoA (AoA1) is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs (AoA2 and AoA3) are from the set of AoA pairs, denoted by (AoA2, AoA3) that can support 2 AoA reception for UE declared AoA angular separation and declared orientation in the positioner of the test system according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 [19] for power class 3 supporting simultaneous reception from multiple directions. The UE positioning shall be such that the UE passes both spherical coverage requirements. The angular offset between the directions of the AoA pair (AoA2, AoA3) is declared from table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and shall not be changed for each test iteration. NOTE: The chosen AoA pair (AoA2, AoA3) is up to RAN5. ### A.3.15.7 Setup 7: 3 AoAs There are 3 active probes in the test. The DL signals, and noise if applicable, transmitted from the three active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. [Text Skipped]

The AoA setups for FR2 RRM test cases (eIFF test system) defined in Annexure A.3.15 of TS 38.133 can have a maximum of 3 active probes during the test.

The FR2 RRM testing setup reuses the FR2 RF testing setup to reduce the test system complexity and test cost.

The FR2 RF testing setup is based on the eIFF test system and is used to test 2 AoA spherical coverage requirement for multi-Rx chain DL reception and is reproduced below.

[TS 38.101-2, Clause 7.3K.3] ### 7.3K.3 2AoA spherical coverage of power class 3 The requirements apply to the UE when tested in a test system as described in Annex L. The requirement is verified with the test metric of throughput (Link= 2AoA spherical coverage grid, Meas=Link Angle). The spherical coverage requirement for simultaneous reception from multiple directions is defined in terms of the probability to support simultaneous reception of rank 2 PDSCH defined in sub-clause 7.3K.0. The probability (see Annex L) is defined as the spatial average over the full sphere around the UE of the probability of any one direction to support 2 AoA reception. In the applicable test system (see Annex L), the probability of any one direction of the UE to support 2 AoA reception for any specific AoA separation is the ratio of the number of unique AoA pairs that include that direction and can support 2 AoA reception to the total number of verified unique AoA pairs that include that direction. The requirement applies only for the UE’s declared orientation in the positioner of the test system. The requirement for each AoA separation condition applies only for the UE’s declared orientation in the positioner of the test system for that AoA separation. The minimum required overall probability to support 2 AoA reception for power class 3 UEs for any channel bandwidth is specified by AoA separation in table 7.3K.3-1. The UE is only required to fulfil the requirement at any one of AoA separations declared from Table 7.3K.3-1. Table 7.3K.3-1: Requirement for power class 3 AoA separation (degrees); Probability (%). 30; 18.5. 60; 13.5. 90; 12.5. 120; 20.5. 150; 28.5.

For 2 AoA spherical coverage test, the UE is tested only for one AoA separation as preferred by the UE.

UE RF test setup to test 2 AoA spherical coverage for multi-Rx chain DL reception was selected to cater the need of different implementations of multi-panel UE, i.e., UEs having 2 antenna panels on opposite, adjacent and same sides of the UE.

Existing eIFF test system cannot be directly reused for the conformance testing of AI/ML BM use case with CDL based channel model(s).

A better test system which can be used along with CDL channel model is 3D Multi-Probe Anechoic Chamber (MPAC) test method which is the reference methodology for FR2 NR MIMO OTA testing. This test system is equipped with 6 dual polarized test probes placed on a sector for NR FR2 MIMO OTA testing.

3D MPAC test system is better suited for CDL channel model-based testing.

In RAN4#116, the agreement on “Test system channel model” was to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels using the eIFF test system. For this, FR2 CDL-C Umi channel models (at 28 GHz) is reproduced below.

[TS 38.151 Annexure D.1] # D.1 FR2 Channel models The following channel model is required for FR2 MIMO OTA measurement. The generic model is Table D.1-1 FR2 UMi CDL-C, which does not include base station antenna filtering. Therefore, in addition, the BS beam filtering effect defined in Annex D.2 also apply when emulating the channel models. Table D.1-1: Channel model parameters for UMi CDL-C at 28 GHz Cluster #; Absolute Delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -4.4215; -30.4353; -134.4434; 98.9242; 83.3318. 2; 12.594; -1.25; -20.9269; 129.1633; 99.1915; 72.5229. 3; 13.314; -3.4684; -20.9269; 129.1633; 99.1915; 72.5229. 4; 13.974; -5.2294; -20.9269; 129.1633; 99.1915; 72.5229. 5; 13.056; -2.5215; -28.0782; -152.8206; 99.5732; 71.1282. 6; 38.196; 0; -11.6982; 164.1145; 99.306; 74.7544. 7; 38.688; -2.2185; -11.6982; 164.1145; 99.306; 74.7544. 8; 39.36; -3.9794; -11.6982; 164.1145; 99.306; 74.7544. 9; 39.504; -7.4215; 17.3861; 84.3647; 100.4513; 69.2454. 10; 47.61; -7.1215; -37.5865; 92.0623; 98.5616; 66.7349. 11; 49.278; -10.7215; 20.2226; -97.7585; 100.6231; 72.0348. 12; 56.016; -11.1215; -50.6106; 78.4702; 98.218; 64.4337. 13; 73.71; -5.1215; -33.911; 93.1719; 100.165; 85.4238. 14; 78.498; -6.8215; -37.5066; -112.0441; 100.2604; 64.1548. 15; 130.224; -8.7215; -43.1797; 102.4645; 98.1225; 64.7824. 16; 162.63; -13.2215; 29.2116; 67.2359; 100.2604; 92.467. 17; 255.534; -13.9215; 27.8133; 34.5731; 98.4852; 65.6889. 18; 276.018; -13.9215; 23.6584; 48.5813; 98.1416; 68.7572. 19; 329.412; -15.8215; -52.5282; 36.4455; 97.9698; 59.1339. 20; 336.462; -17.1215; 25.0168; 52.6729; 100.7376; 65.3402. 21; 378.39; -16.0215; 25.4562; 49.8296; 98.1225; 58.4365. 22; 398.244; -15.7215; 30.7697; 46.4316; 98.1034; 65.2705. 23; 422.562; -21.6215; 35.9234; 30.759; 100.4513; 62.6903. 24; 519.138; -22.8215; -61.2775; 69.2469; 100.9476; 61.993. Per-Cluster Parameters. Parameter; CASD in [°]; CASA in [°]; CZSD in [°]; CZSA in [°]; XPR in [dB]. Value; 0.799; 10.4021; 0.5726; 4.8814; 7.

- To emulate UMi CDL-C channel model during the test, BS beam filtering effect defined in Annex D.2 of TS 38.151 is required, which is reproduced below.

[TS 38.151, Annexure D.2] # D.2 FR2 Base Station beam configuration The emulated BS beam configuration to be used for emulation of channel model defined in Annex D.1 is specified in this clause. The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table D.2-1. Table D.2-1: FR2 BS Antenna Parameters Parameter description; Symbol; Parameter value. FR2. Antenna panels in vertical dimension; Mg; 1. Antenna panels in horizontal dimension; Ng; 1. Elements per panel in vertical dimension; Me; 8. Elements per panel in horizontal dimension; Ne; 16. Number of polarizations per panel; P; 2. Element spacing in horizontal dimension (); dH; 0.5. Element spacing in vertical dimension (); dV; 0.5. Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array are as in the example pattern in Table 7.3-1 of TR 38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi. The beamforming characteristic of the FR2 BS pattern is defined as follow: - A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size； - 1 strongest transmitting beam is generated from BS, the direction of this beam towards the strongest cluster of the FR2 channel model. Specifically, the direction in UMi CDL-C model is: AoD: -12o, ZoD: 100.71

- If the FR2 CDL-C Umi channel defined in Annexure D.1 of TS 38.151 is used for the conformance testing of AI/ML BM use case, the BS beam configuration defined in Annexure D.2 of TS 38.151 needs to be emulated. For this, a codebook of 128 fixed beams, based on a grid of eight elevation angles ranging from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size, is defined.

Furthermore, in RAN4#114 meeting, Set A = 32 beams and Set B = 8 beams was agreed as a starting point.

- RAN4 to agree on selecting the 32 Set A beams, to be emulated during the conformance testing of AI/ML BM use case, from the existing codebook of 128 fixed beams (constructed from a grid of eight elevation angles from –25o to +25 o with ~7.1 o step size and 16 azimuth angles from –60 o to +60 o with 8 o step size) along with the other parameters as defined in Table D.2-1 of TS 38.151.

Selection of 32 Set A beams from the codebook of 128 beams can be done is several possible ways. For example,

- By restricting the range of azimuth or elevation or both (azimuth and elevation) angles.

- By increasing the step size in azimuth or elevation or both (azimuth and elevation) directions.

- Test Equipment dynamically selects 32 beams out of 128 in each test iteration. FFS if some information about this needs to be shared with the DUT during the test or not.

- RAN4 to study the impact of selecting different combinations of 32 beams from the 128-beam codebook on the performance with simplified CDL channel model.  How to select the 32 beams from 128 beam codebook is currently FFS.

Moreover, how to emulate 32 Set A beams by using a limited number of test probes during the AI/ML BM conformance testing is a well acknowledged issue in RAN4. We next provide two candidate solutions along with the corresponding test procedures for the same.

      - Emulation of Set A and/or Set B beams without using UE rotation

- This solution provides a method, applicable only for the multiple AoAs based test systems, to emulate the Set A and/or Set B beams without using the UE rotation. Considering a test chamber equipped with ‘M’ test probe antennas where each test probe antenna is contributing to form a definite Tx antenna pattern and/or Tx beam(s) which is also affected by the amount of Tx power corresponding to the signals applied to each test probe antenna. A particular Tx antenna pattern (containing at least a Tx beam) can be formed by selecting and activating any of the ‘N’ test probes out of the available ‘M’ probes, where 1≤N≤M, i.e.,

- MC1 Tx antenna patterns can be formed if only 1 probe is selected/activated.

- …

- MCM Tx antenna patterns can be formed if ‘M’ probes are selected/activated.

- Hence, a test system with ‘M’ test probe antennas in the test chamber can emulate (MC1 + … +MCM) Tx antenna radiation patterns (or beams). For example, for a test system having 6 test probes, simultaneously activating at least 3 test probe antennas will result in the creation of (20+15+6+1) = 42 Tx antenna patterns which is sufficient to emulate 32 Set A beams during the conformance testing.

- RAN4 to discuss and explore the feasibility of generating Set A and Set B beams during the conformance testing of the AI/ML-BM use case by selecting unique combinations of available test probes and decide the transmit power of each selected probe. Tx power of each selected probe is FFS.

This solution may need a special beam codebook for the generation of Set A and Set B beams using unique combinations of test probes during the conformance testing. To ensure that the UE can perform correct inferences using the measurement results of Set B beams generated using this method, a pre-alignment check may be needed before the exact start of the conformance test. The objective of the pre-alignment check is to ensure that the TE provided parameters (e.g., parameters corresponding to the beam codebook) and the DUT’s model/functionality/configuration’s current/preferred parameter (e.g., the parameters using which the training was performed) agree with each other. This is needed to ensure that the test does not fail because of the (AI/ML) parameter(s) mismatch between the DUT (pre)-training and conformance testing.

- RAN4 to discuss and finalize the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

- We next provide a test procedure which utilizes this concept and will call it as “Test Procedure A”.

- Test Procedure A:

- Place the DUT inside the testing chamber in the pre-selected or standardized orientation.

- Perform the pre-alignment check between the TE and the DUT. If the pre-alignment check is successful, TE parameters and DUT training parameters are not mismatched. FFS the content of pre-alignment check. Example: parameters related to beam codebook for the testing.

- If the pre-alignment check is successful, go to Step 4 else end the test and mark it as not feasible/applicable.

- TE selects a unique combination of three or more test probes from the available test probes antennas and decides the Tx power of the corresponding signals applied to the selected test probes to form one Tx antenna pattern comprising one Set A (or Set B) beam.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- TE selects another unique combination of three or more test probes and decides the Tx power of the corresponding signals applied to the selected test probes to form another Tx pattern containing a different Set A (or Set B) beam.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Repeat till all Set A (or Set B) beams are transmitted, and DUT has measured the RSRP of all transmitted beams.

- If Set A beams are transmitted first, DUT reports the ground truth RSRP of the measured Set A beams to the TE.

- TE stores the UE reported Set A measurement results for deriving the test verdict.

- TE transmits each Set B beams to the DUT by forming a unique Tx antenna pattern via selecting a unique combination of three or more test probes selected from the combinations used for transmitting Set A beams.

- DUT measures the RSRP of each Set B beam and perform the AI/ML based inference at the UE side.

- DUT reports the inference results back to the TE (e.g., reporting top-K beams along with their predicted RSRPs).

- TE derives the test verdict based on the UE reported prediction results and UE reported measurement results.

      - Emulation of Set A and/or Set B beams with UE rotation

- This solution provides a method, which is applicable for both single AoA and multiple AoAs based test systems, to emulate the Set A and/or Set B beams by using the concept of UE rotation. It is expected during the test that different Set A/B beams should be generated in a way that these will arrive at the UE antenna array from different directions as similar scenario will be observed in the real deployments. Hence, different UE rotations can be used during the test to ensure that different beams arrive the DUT from different directions. One benefit of using the UE rotation is that it enables the test system to emulate the AoA angles which are outside the angular range of test probes (from UE’s point of view) and hence the test system can generate any possible AoA beam angles.

The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible UE orientations is selected and TE selects the power level (typically different) for each of the available test probes to form different beams towards the UE. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure 2: An example of the proposed test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

- RAN4 to discuss the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

- We next provide a test procedure which utilizes this concept and will call it as “Test Procedure B”.

- Test Procedure B:

- Place the DUT inside the testing chamber in the pre-selected or standardized orientation.

- TE sends the test command to the DUT to configure itself and perform beam sweeping in multiple iterations.

- DUT configures itself to perform beam sweeping in multiple iterations and sends an acknowledgement to the TE for the same.

- TE transmits one of the Set A (or Set B) beams to the DUT using the available test probes in the testing chamber. This means that for single AoA based test systems, the only available test probe will be used to transmit the required beam towards the DUT whereas for multi-AoA based test systems, either one or more available test probes will be selected based on the agreement on channel model to be used during the conformance test.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Rotate the UE to a different orientation. Allow the UE to settle down.

- Transmit next Set A (or Set B) beams to the DUT.

- DUT measures the RSRP of the received Set A (or Set B) beam and stores it for either ground truth reporting or UE side AI/ML based inference purposes.

- Repeat Steps 6-8 till all Set A (or Set B) beams are transmitted, and DUT has measured the RSRP of all transmitted beams.

- If Set A beams are transmitted first, DUT reports the ground truth RSRP of the measured Set A beams to the TE.

- TE stores the UE reported Set A measurement results for deriving the test verdict.

- TE transmits each Set B beams to the DUT using a subset of the UE orientations previously used to transmit Set A beams.

- DUT measures the RSRP of each Set B beam and performs the AI/ML based inference at the UE side.

- DUT reports the inference results back to the TE (e.g., reporting top-K beams along with their predicted RSRPs).

- TE derives the test verdict based on the UE reported prediction results and UE reported measurement results.

Simplification of CDL model

Spatial filter applied to a normal CDL model

The UMi CDL-C channel model has its scattering clusters distributed in all directions, as illustrated in Figure 1. However, due to the limited beamwidth of the SSB beam, it can only interact with a subset of clusters in such a scenario. To address this, a spatial filter needs to be applied according to the SSB beamwidth. This procedure is described in 3GPP TR 38.901, Chapter 7.7.4 “Spatial filter for generating TDL channel model”. The extension of using the spatial filter to generate a CDL model has been developed in SCM WI in Rel-19, R4-2512646 [4].

By applying the spatial filter based on the SSB beamwidth and the UE Rx beam’s beamwidth, only the clusters that interact with the corresponding SSB and UE Rx beam are retained, which resulting in significantly fewer than the 24 clusters defined in the UMi CDL-C model.

Based on the reflection and diffusion of the SSB from the gNB to the UE and its reception by the Rx beam, the power angular spectrum (PAS) represents the “FR2 channel” that needs to be emulated in the anechoic chamber using probes. Since the PAS depends on the number of clusters, methods for generating it with a limited number of probes in the chamber should be further discussed.

Figure 1 Directional channel based on a CDL model

A spatial filter should be applied to the UMi CDL-C channel model based on the SSB and UE Rx beam, so that only the clusters interacting with them are retained. In this case, the number of clusters will be significantly fewer than the 24 clusters defined in the UMi CDL-C model.

Based on the reflection and diffusion of the SSB from gNB to UE and its reception by the Rx beam, the power angular spectrum (PAS) represents the “FR2 channel” that needs to be emulated in the anechoic chamber using probes.

    - Spatial filter applied to a reduced-clusters CDL model

Regarding the simplification of the CDL model discussed in previous meetings, it is straightforward to create a similar cluster map while reducing the number of clusters to six, as illustrated in Figure 2. The same spatial filtering procedure should then be applied to this reduced-cluster CDL model.

Figure 2 Directional channel based on a reduced-cluster CDL model

Each CDL channel model is created based on a specific channel scenario, with defined cluster locations, angles, delays, and powers that preserve the agreed channel characteristics, for example, the agreed UMi CDL-C model reflects the properties of the UMi scenario. Reducing the number of clusters in a CDL model can significantly alter these properties.  potentially shifting the UMi characteristics toward those of UMa, or even resulting in undefined channel properties.

Spatial filter should also be applied to the reduced-cluster channel model according to the beamwidth of SSB and UE Rx beam, so that the interacting clusters can be identified, and the rest of clusters can be ignored.

- Reducing the number of clusters in the CDL model carries a high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

- It is not recommended to reduce the number of clusters in the UMi CDL-C model for simplification. There is high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

    - Procedure of representing clusters interact with clusters in an anechoic chamber

Based on the previous analysis, the spatial filter will inevitably be applied to either the UMi CDL-C model or its reduced version. Therefore, it is recommended to follow the procedure based on the original UMi CDL-C model to avoid the risk of altering the UMi channel properties into those of UMa or an undefined channel property.

1. The CDL-C UMi models, as agreed in the WF, are omnidirectional, meaning the 24 clusters are distributed across the entire channel environment around the gNB and UE. Since we are using beams, the SSB/CSI-RS from the gNB will only interact with clusters in certain directions. Therefore, we propose to retain all 24 clusters and not remove any of them.

2. Next, we can generate or simulate the SSB beams, starting with 32 beams as agreed. Each SSB beam has a beamwidth of only 11.25°, meaning it will interact with only a small subset of the clusters in the CDL-C UMi model, which is significantly fewer than all 24 clusters. The SSB then propagates to the UE and is received according to the UE’s power angular spectrum (Rx-PAS). Based on this, we can determine which clusters contribute for a given SSB or test case in the CDL-C model, using those clusters while ignoring the others.

3. At this step, we can also check whether the power of the clusters contributing to the UE’s Rx-PAS is too weak. If so, these clusters can be ignored as well.

4. When creating the Rx-PAS in the anechoic chamber, we can simulate or emulate only the clusters that interact with the SSB beam, while also applying the condition that the cluster power is not too weak. Consequently, the number of clusters that need to be generated by the probes in the chamber for each test case is much fewer than 24.

Throughout the entire procedure, we do not remove any clusters from the CDL model directly; instead, we simply ignore the clusters that are not used. This ensures that the underlying channel properties remain unchanged.

The above procedure can be applied to any test case. If a test case requires generating a large number of SSBs simultaneously, we can consider developing methods to generate more than 4–8 clusters, which is more than the limited number of probes available in the chamber.

- A spatial filter should be applied to the CDL-C model first, which can significantly reduce the number of clusters. Next, clusters with insufficient power can be ignored, further reducing the number of clusters. Only the remaining clusters need to be represented in the chamber using probes.

- Endorse the above-mentioned 4-step procedure for representing the clusters in the chamber using probes.

    - The threshold of weak clusters that can be ignored

There are reference sensitivity power levels defined in 3GPP TS 38.101-2, Chapter 7.3.2 (“Reference Sensitivity Power Level”). According to the standard:

“The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX beam peak direction, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.”

This means that REFSENS represents a lower bound for the power level at which the UE should be able to successfully receive the signal from the gNB.

Here is an example of REFSENS for PC3 in 3GPP TS 38.101-2, for different signal bandwidths. We can map these reference sensitivity levels to channel scenarios (e.g., UMi CDL-C) when SSB or CSI-RS signals are transmitted from the gNB. In the anechoic chamber, any cluster with a power level below the corresponding REFSENS can be safely ignored.

- The reference sensitivity power levels can be reused to determine whether the power of a cluster is too weak and can therefore be ignored.

Performance requirements for AI/ML Positioning

Requirements for Case 3a

      - Other reported quantities

RAN1#121 agreed the following on mandatory/optional inference output reporting for Case 3a.

Agreement For Case 3a, the FFS in RAN1#118bis agreement is resolved by updating the agreement as follows. Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. - FFS: LOS/NLOS indicator. - (Optional) LOS/NLOS indicator with legacy format Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept..

In our view, no RAN4 requirements for inference need to be defined for this LOS/NLOS indicator agreed by RAN1 for Case 3a.

- No RAN4 requirements for inference need to be defined for the LOS/NLOS indicator agreed by RAN1 for Case 3a.

    - Requirements for Case 3b

RAN4 already agreed to reuse legacy accuracy requirements for gNB Rx-Tx time difference for the UL SRS-TDCT measurements for the first path. It is noted that gNB Rx-Tx time difference accuracy was specified in Rel-16 based on N=4 measurement samples. For reduced number of measurement samples, introduced in Rel-17, no accuracy requirements were defined. Thus, accuracy requirements for UL SRS-TDCT apply for the same number of measurement samples N=4.

The same is valid for UL SRS-TDCP. Rel-16 measurement accuracy requirements for UL SRS RSRP were based on N=4 measurement samples.

- Measurement accuracy requirements for gNB Rx-Tx time difference apply for UL SRS-TDCT first path under the condition that the reported measurement is based on N=4 samples.

- Measurement accuracy requirements for UL SRS RSRP apply for UL SRS-TDCP under the condition that the reported measurement is based on N=4 samples.

      - Requirements on ground truth accuracy in data collection

In RAN1#116-bis meeting, training data collection of AI/ML based positioning has been discussed to identify the collect ed data samples as follows.

Agreement(RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement(RAN1#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. Agreement(RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

Also, an agreement is made in RAN1#120 on the label and quality indicator of label.

Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. · From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

In Part B, data collection with ground-truth dataset is essential for training. In particular, the training data needs to include both the ground-truth UE location and measurements associated with the UE’s location. To enhance the accuracy of AIML positioning, it is important to have accurate ground-truth information for positioning labelling. When collecting ground-truth information from the UE side, a high accuracy positioning information needs to be acquired by the UE. If UE is unable to acquire a high accuracy ground-truth information, the network/testing equipment may assist UE to acquire a high accuracy ground-truth positioning label. For instance, UE may obtain positioning labels from multiple sources e.g., non-NR and/or NR RAT-dependent positioning methods. Therefore, the degree of accuracy on ground-truth positioning label can be defined as a quality indicator. As captured in RAN1’s agreement, if the existing IE is used with a geographic shape in TS 23.032, the uncertainty/confidence on location estimate can be used as a quality indicator of the label. In this case, the legacy positioning does not specify a requirement on the quality indicator. Also, the ground-truth accuracy can be ensured by the definition on labels. Hence, a requirement on training data collection is not required in Case 3a/3b.

RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM performance requirements for AI/ML enabled beam management and positioning use cases. Specifically, we cover following aspects:

- Testing framework for AI/ML based Beam Management

- Simplification of CDL model

- Performance requirements for AI/ML Positioning

In the paper, the following Observations and Proposals were made:

Observation 1: The AoA setups for FR2 RRM test cases (eIFF test system) defined in Annexure A.3.15 of TS 38.133 can have a maximum of 3 active probes during the test.

Observation 2: The FR2 RRM testing setup reuses the FR2 RF testing setup to reduce the test system complexity and test cost.

Observation 3: For 2 AoA spherical coverage test, the UE is tested only for one AoA separation as preferred by the UE.

Observation 4: UE RF test setup to test 2 AoA spherical coverage for multi-Rx chain DL reception was selected to cater the need of different implementations of multi-panel UE, i.e., UEs having 2 antenna panels on opposite, adjacent and same sides of the UE.

Observation 5: Existing eIFF test system cannot be directly reused for the conformance testing of AI/ML BM use case with CDL based channel model(s).

Observation 6: 3D MPAC test system is better suited for CDL channel model-based testing.

Proposal 1: RAN4 to agree on selecting the 32 Set A beams, to be emulated during the conformance testing of AI/ML BM use case, from the existing codebook of 128 fixed beams (constructed from a grid of eight elevation angles from –25o to +25 o with ~7.1 o step size and 16 azimuth angles from –60 o to +60 o with 8 o step size) along with the other parameters as defined in Table D.2-1 of TS 38.151.

Proposal 2: RAN4 to study the impact of selecting different combinations of 32 beams from the 128-beam codebook on the performance with simplified CDL channel model.  How to select the 32 beams from 128 beam codebook is currently FFS.

Proposal 3: RAN4 to discuss and explore the feasibility of generating Set A and Set B beams during the conformance testing of the AI/ML-BM use case by selecting unique combinations of available test probes and decide the transmit power of each selected probe. Tx power of each selected probe is FFS.

Proposal 4: RAN4 to discuss and finalize the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

Proposal 5: RAN4 to discuss the feasibility of performing beam sweeping using UE rotation in multiple iterations during the conformance testing of AI/ML based BM use case.

Observation 7: A spatial filter should be applied to the UMi CDL-C channel model based on the SSB and UE Rx beam, so that only the clusters interacting with them are retained. In this case, the number of clusters will be significantly fewer than the 24 clusters defined in the UMi CDL-C model.

Observation 8: Based on the reflection and diffusion of the SSB from gNB to UE and its reception by the Rx beam, the power angular spectrum (PAS) represents the “FR2 channel” that needs to be emulated in the anechoic chamber using probes.

Observation 9: Spatial filter should also be applied to the reduced-cluster channel model according to the beamwidth of SSB and UE Rx beam, so that the interacting clusters can be identified, and the rest of clusters can be ignored.

Observation 10: Reducing the number of clusters in the CDL model carries a high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

Proposal 6: It is not recommended to reduce the number of clusters in the UMi CDL-C model for simplification. There is high risk of significantly altering the channel properties, potentially shifting the UMi characteristics to those of UMa or to an undefined channel property.

Proposal 7: A spatial filter should be applied to the CDL-C model first, which can significantly reduce the number of clusters. Next, clusters with insufficient power can be ignored, further reducing the number of clusters. Only the remaining clusters need to be represented in the chamber using probes.

Proposal 8: Endorse the above-mentioned 4-step procedure for representing the clusters in the chamber using probes.

Proposal 9: The reference sensitivity power levels can be reused to determine whether the power of a cluster is too weak and can therefore be ignored.

Proposal 10: No RAN4 requirements for inference need to be defined for the LOS/NLOS indicator agreed by RAN1 for Case 3a.

Proposal 11: Measurement accuracy requirements for gNB Rx-Tx time difference apply for UL SRS-TDCT first path under the condition that the reported measurement is based on N=4 samples.

Proposal 12: Measurement accuracy requirements for UL SRS RSRP apply for UL SRS-TDCP under the condition that the reported measurement is based on N=4 samples.

Proposal 13: RAN4 to not specify a requirement on training data collection of Part B in Case 3a/3b.

- References

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- R4-2511890, “WF on NR_AIML_air_part1”, Qualcomm Incorporated, RAN4#116, Bengaluru, India, 25th ‒ 29th August, 2025.

- R1-2503243, Consolidated Rel-19 higher layers parameters list Post RAN1#121.xlsx, RAN1#121 meeting, 19th - 23rd May 2025, Malta.

- R4-2512646	(FS_NR_demod_SCM) CR to 38.753 Annex B and Section 5