3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514412

Prague, Czech Republic, 13th – 17th October 2025

Source:	BT

Title:	Discussion on performance requirements for SU-MIMO cases using Spatial Channel Model

Agenda item:	7.5.3

Document for:	Discussion

Introduction

Following conclusion of Rel-19 Study on Spatial Channel Modelling for Demodulation Performance Requirements [1], in RAN#109 the work item with the following objective [2]:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - Using rCDL-C1 channel model detailed in TR 38.753 as starting point

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

This contribution provides our views on the approach to the WI objective.

Channel Model (rCDL-C1) Updates

Angular Scaling

Adopting the latest RAN1 angular scaling definition from TR 38.901 v19.1.0 would be preferred for maintaining consistency and robustness across 3GPP specifications. This can prevent future discrepancies and facilitate a unified understanding for ongoing and future standardization efforts.

To ensure consistency and accuracy, RAN4 should update the rCDL-C1 angular scaling procedure to align with the latest TR 38.901 (v19.1.0) definitions.

Frequency Generalization

Frequency generalization is an important consideration for channel modeling, as real-world practical differences across frequency bands, such as varying diversity and correlation parameters, significantly impact MIMO system performance. Reflecting these band-specific characteristics in test cases is essential for comprehensive and realistic evaluations.

However, given the focused objectives and timeline of the current Work Item on FR1 SU-MIMO with SCM, we propose to defer extensive effort on frequency generalization for now. This aspect can be more thoroughly addressed either during the upcoming MU-MIMO study item, or as part of the broader evaluations within the 6G Radio Study Item, allowing for a more holistic and future-proof approach.

Defer detailed study of frequency generalization to the MU-MIMO or 6G Radio Study Items.

PMI Bias

PMI bias remains is recognised as a challenge for any channel models, as the fixed angular properties can lead to unrealistic and easily exploitable test scenarios where a single PMI might consistently yield optimal performance. However, we propose to defer a comprehensive review of physically accurate long-term solutions to the 6G Radio Study Item.

Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item.

PDSCH Demodulation Requirements

For PDSCH Demodulation Requirements, particularly in the 8Rx case, it has been observed that there are significant performance differences between codewords. Therefore, to accurately reflect and test UE capabilities under these conditions, we propose that separate SNR requirements for each codeword should be used for the 8Rx case.

For 8Rx PDSCH demodulation, define separate SNR requirements for each codeword to accurately reflect observed performance differences.

CSI / PMI Reporting Requirements

While current simulation results for CSI/PMI Reporting Requirements exhibit observable clusters of alignment, the overall divergence across companies remains a significant hurdle for defining robust requirements. We strongly encourage all participants to undertake a thorough review of their simulation implementations. By meticulously examining methodologies, assumptions, and tool chains, we can identify and rectify the underlying causes of discrepancies, ultimately fostering a much-needed convergence that will enable the establishment of reliable and widely accepted performance requirements for CSI/PMI reporting

companies review simulation implementations to improve alignment of CSI/PMI reporting results, despite existing alignment clusters, to enable robust requirement definition

Conclusion

This contribution provides SNR data points for the CDL channel model alignment cases in [7] and can be found in [10]. These results highlight the importance of PMI strategy and UE capability in achieving high throughput under realistic Doppler conditions.

Full numerical results were collected and provided to the results collection table R4-2509413 during RAN4#116.

References

- TR 38.753, Study on spatial channel model for demodulation performance requirements for NR, v.2.0.0

- RP-252957, NR demodulation performance: Phase 6, Apple