3GPP TSG-RAN WG4 Meeting #116bis	R4- 2514416

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	4.20.1.1

Source: 	Huawei, HiSilicon

Title:	Discussion on test configuration and RMC for AIoT devices

Document for:	Approval

# 1	Introduction

With the completion of core RF/RRM requirements, the TS 38.191 for Ambient IoT device was approved at RAN#109 [1]. There’re some open issues related to test configuration and RMC yet to be concluded. Some of them are captured in the WF [2].

# 2	Discussion

### 2.1 Test configuration for backscatter power

Agreement in RAN4#116:

Define flat SEM requirements, i.e. 10dBc, The RBW is 1SB

- The carrier power includes the two sidebands centred at +/-SFS, but excludes the spectrum around the carrier frequency.

Agreement in RAN4#115:

- Performance metric: EIRP

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

- Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna.

  - FFS for BPSK

- The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself.

As seen from the previous agreements, for the measurement of the D2R signal power, only the 1st lower/upper sideband around Fc+/-fSFS are included. How to capture this requirement into the spec need to be discussed.

Assuming the D2R signal is received and down-converted to baseband. The sub-carrier frequency corresponding to the Small Frequency Shift (SFS), the bandwidth of the sideband, the lower frequency edge of the sideband and the upper frequency edge of the sideband are denoted as fSFS, 1SB, fl and fh, respectively. And the nominal bit duration and the SFS factor are denoted as Tb and RSFS, respectively.

The lower/upper edge frequency of the upper sideband can be obtained as follows:

fl=fSFS-1SB2,  fh=fSFS+1SB2

Without loss of generality, assuming the Sample Frequency Offset (SFO) is 0<= SFO <=0.1. The frequency characteristics of the upper sideband can be analysed as follows for three cases: no frequency error, sample frequency higher/lower than the nominal frequency.

Table 1: Frequency Characteristics for D2R upper sideband

|  | Sub-carrier fSFS | 1SB | Lower edge fl | Upper edge fh |
| --- | --- | --- | --- | --- |
| No Frequency Error | RSFSTb | 2Tb | RSFS-1Tb | RSFS+1Tb |
| Higher Sample Frequency | RSFSTb(1+SFO) | 2Tb(1+SFO) | RSFS-1Tb(1+SFO) | RSFS+1Tb(1+SFO) |
| Lower Sample Frequency | RSFSTb(1-SFO) | 2Tb(1-SFO) | RSFS-1Tb(1-SFO) | RSFS+1Tb(1-SFO) |

In order to cover the worst case for both the lower edge and the upper edge of the sideband (as highlighted in Table 1), the parameters for the measurement filter are recommended as follows.

- Use the test configuration in Table 2 for D2R measurement.

Table 2: Test configuration for D2R measurement

| Measurement parameter | Value |
| --- | --- |
| CW frequency (FCW) | According to TS 38.192 |
| Filter passband range for D2R upper sideband | FCW+RSFS-1Tb*0.9 to FCW+RSFS+1Tb*1.1 |
| Filter passband range for D2R lower sideband | FCW-RSFS+1Tb*1.1 to FCW-RSFS-1Tb*0.9 |

### 2.1 RMC for R2D and D2R

Agreement in RAN4#116:

- EIS at peak antenna gain direction is -34dBm. Additional peak EIS levels can be added in the future if needed

- EIS spherical coverage requirement is defined as sensitivity over partial sphere, i.e. XdBm at Y solid angle range partial sphere . Y is suggested to be +-45 degree with respect to the bore sight direction (or UE declared direction),  X is suggested to be 5.5dB worse than peak EIS.

  - For testing

    - 90% success rate.

    - Use CFRA for REFSENS test procedure

    - Allow set time + SFO for the device response time.

    - Leave number of repeats to reach 90% success rate for the RAN5 to determine.

    - Further discuss if false alarm test can be considered in demod.

- FRC is listed as below (note: it does not mean UE should be tested for all specified FRC)

|  | SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- | --- |
|  | PRB | 1 | 2 | 3 | 4 |
| SIP | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | Depending on the size of the MAC PDU of A-IoT paging message indicating CFA |  |  |  |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | OOK |  |  |  |  |
|  | M | 6 | 6 | 6 | 6 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
| chip number except for SIP, padding |  | 228 | 228 | 228 | 228 |
| Padding |  | 6 | 12 | 2 | 2 |

As agreed in RAN4#116, M=6 is selected for REFSENS test, which is done via CFA procedure. As specified in TS 38.391, the MDC PDU for CFA is shown as below.

Figure 6.2.1.1-2: MAC PDU of A-IoT Paging message indicating CFA [3]

For CFA, the paging ID is the device permanent ID, which is defined in TS 23.003 [4] as follows.

Figure 31.2-1: Structure of AIoT Device Permanent Identifier [4]

Since the domain information in the device ID is optional, the minimum length of the device ID is 104 bits.

Since the length of the device ID and that of the D2R scheduling info are variable, the TBS for the PRDCH is not fixed.

- Use the FRC in Table 3 for REFSENS test.

Table 3: Fixed Reference Channels for REFSENS

| Component | Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| General | PRB | RBs | 1 | 2 | 3 | 4 |
|  | SCS | kHz | 15 | 15 | 15 | 15 |
| SIP | Bit length | Bits | 8 | 8 | 8 | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 | 4 | 4 | 4 |
| CAP | Bit length | Bits | 4 | 4 | 4 | 4 |
|  | M | Chips/Symbol | 6 | 6 | 6 | 6 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |  |  |  |
|  | CRC | Bits | 16 | 16 | 16 | 16 |
|  | M | Chips/Symbol | 6 | 6 | 6 | 6 |
| Postamble | Bit length | Bits | 4 | 4 | 4 | 4 |
|  | M | Chips/Symbol | 6 | 6 | 6 | 6 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |  |  |  |

In NR or LTE, the UE maximum input requirement is verified with 64QAM/256QAM/1024QAM with code rate=3/4. Hence, it’s reasonable to use a larger M value such as 24 or 12 to test the maximum input level for devices.

- Consider to use a larger M value (M=12 or 24) for PRDCH to test the maximum input level for devices. An example configuration is shown in Table 4.

Table 4: Fixed Reference Channels for maximum input level

| Component | Parameter | Unit | Value |  |
| --- | --- | --- | --- | --- |
| General | PRB | RBs | 2 | 3 |
|  | SCS | kHz | 15 | 15 |
| SIP | Bit length | Bits | 8 | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 | 4 |
| CAP | Bit length | Bits | 4 | 4 |
|  | M | Chips/Symbol | 12 | 24 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |  |
|  | CRC | Bits | 16 | 16 |
|  | M | Chips/Symbol | 12 | 24 |
| Postamble | Bit length | Bits | 4 | 4 |
|  | M | Chips/Symbol | 12 | 24 |
| Padding | Padding per OFDM symbol excluding SIP and Postamble | Chips | N/A | 2 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |  |

During the discussion on spurious emission requirements, it was agreed to use the following conditions for testing [2].

Agreement in RAN4#116:

- For testing:

  - Only using limited DSB and SFS combination for testing with -5 dBm CW input power.

    - SFS = 480kHz with 15kHz, 2SB transmission bandwidth need to be tested

As per TS 38.291, the following parameters for PRDCH are provided by the paging message.

Table 5: D2R scheduling information

| Symbol | Definition |
| --- | --- |
| TbitD2R | the duration in microseconds of each D2R bit |
| Rblock | the block repetition number |
| RSFS | the small frequency shift factor to be used |
| Ibit | the interval in bits for D2R midamble insertion |
| Lamble | sequence length indicator for D2R-ambles |
| Iadd | the additional D2R midamble insertion indicator |
| Rcode | the channel coding indicator |

Therefore, it’s proposed to define the D2R RMC as follows.

- Use the D2R RMC in Table 6 to test spurious emission requirement. Whether to use the same RMC for testing backscatter power and SEM is to be confirmed.

Table 6: FRC for [backscatter power, SEM and] spurious emissions

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission BW | kHz | 15 |
| TBS | Bits | Depending on the length of AIoT device ID |
| CRC | Bits | 16 |
| FEC code rate |  | 1/3 |
| Block repetition number |  | 1 |
| Preamble length | Bits | 31 |
| Midamble length | Bits | 31 |
| Interval for midamble insersion | Bits | 48 |
| Additional midamble insertion |  | No |
| Small frequency shift | kHz | 480 |
| Modulation |  | BPSK/OOK (NOTE1) |
| NOTE 1: The modulation scheme used is up to device implementation. |  |  |

# 3	Conclusion

In this paper, we have shared the analysis on the test configuration and RMC channels needed for conformance tests. The following proposals are made:

Proposal 1: Use the test configuration in Table 2 for D2R measurement.

Table 2: Test configuration for D2R measurement

| Measurement parameter | Value |
| --- | --- |
| CW frequency (FCW) | According to TS 38.192 |
| Filter passband range for D2R upper sideband | FCW+RSFS-1Tb*0.9 to FCW+RSFS+1Tb*1.1 |
| Filter passband range for D2R lower sideband | FCW-RSFS+1Tb*1.1 to FCW-RSFS-1Tb*0.9 |

Proposal 2: Use the FRC in Table 3 for REFSENS test.

Table 3: Fixed Reference Channels for REFSENS

| Component | Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| General | PRB | RBs | 1 | 2 | 3 | 4 |
|  | SCS | kHz | 15 | 15 | 15 | 15 |
| SIP | Bit length | Bits | 8 | 8 | 8 | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 | 4 | 4 | 4 |
| CAP | Bit length | Bits | 4 | 4 | 4 | 4 |
|  | M | Chips/Symbol | 6 | 6 | 6 | 6 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |  |  |  |
|  | CRC | Bits | 16 | 16 | 16 | 16 |
|  | M | Chips/Symbol | 6 | 6 | 6 | 6 |
| Postamble | Bit length | Bits | 4 | 4 | 4 | 4 |
|  | M | Chips/Symbol | 6 | 6 | 6 | 6 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |  |  |  |

Proposal 3: Consider to use a larger M value (M=12 or 24) for PRDCH to test the maximum input level for devices. An example configuration is shown in Table 4.

Table 4: Fixed Reference Channels for maximum input level

| Component | Parameter | Unit | Value |  |
| --- | --- | --- | --- | --- |
| General | PRB | RBs | 2 | 3 |
|  | SCS | kHz | 15 | 15 |
| SIP | Bit length | Bits | 8 | 8 |
|  | Mapping to OFDM | Chips/Symbol | 4 | 4 |
| CAP | Bit length | Bits | 4 | 4 |
|  | M | Chips/Symbol | 12 | 24 |
| PRDCH | TBS | Bits | Depending on the size of the MAC PDU of A-IoT CFA paging message |  |
|  | CRC | Bits | 16 | 16 |
|  | M | Chips/Symbol | 12 | 24 |
| Postamble | Bit length | Bits | 4 | 4 |
|  | M | Chips/Symbol | 12 | 24 |
| Padding | Padding per OFDM symbol excluding SIP and Postamble | Chips | N/A | 2 |
| Padding | Padding for last OFDM symbol | Chips | Depending on the TBS |  |

Proposal 4: Use the D2R RMC in Table 5 to test spurious emission requirement. Whether to use the same RMC for testing backscatter power and SEM is to be confirmed.

Table 6: FRC for [backscatter power, SEM and] spurious emissions

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission BW | kHz | 15 |
| TBS | Bits | Depending on the length of AIoT device ID |
| CRC | Bits | 16 |
| FEC code rate |  | 1/3 |
| Block repetition number |  | 1 |
| Preamble length | Bits | 31 |
| Midamble length | Bits | 31 |
| Interval for midamble insersion | Bits | 48 |
| Additional midamble insertion |  | No |
| Small frequency shift | kHz | 480 |
| Modulation |  | BPSK/OOK (NOTE1) |
| NOTE 1: The modulation scheme used is up to device implementation. |  |  |

# References

- RP-252811, “Presentation of Specification: TS 38.191, Version 1.1.0”, RAN4, TSG RAN#109

- R4-2511859 WF on A-IoT, CMCC, RAN4#116

- 3GPP TS 38.391 v19.0.0

- 3GPP TS 23.003