3GPP TSG-RAN WG4 Meeting #116bis	R4-2514451

Prague, Czech Republic, 13th – 18th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.115-2 with improvements for EIRP mask for NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is draft CR to NCR conformance specification TS 38.115-2 with corrections and updates to EEIRP mask to align with BS conformance test specification TS 38.141-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections and text improvements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligment between BS test and NCR test specifications on Expected EIRP mask. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17.4.2, 6.17.5, Annex J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

#### 6.17.4.2	Procedure

When calibrated and operated within the guidance of 3GPP TR 37.941 [25] the measurement methods are applicable and selected depending on availability at the test facility.

The test range shall be calibrated according to calibration method described in TR 37.941, clause 8.3.

1. Place the NCR at the positioner, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the repeater with the coordinate system used by the test system.

3. The measurement device characteristics shall be: Detection mode set to True RMS.

4. Set the NCR to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2. The configured EIRP for each of the test beams shall be in direct relation to the intended use of the NCR.

5. Orient the positioner (and the repeater) to the angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex J, clause J.3.

6. Configure test beam(s) and test equipment for measuring test beam k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRP as a sum of two orthogonal polarizations:

EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denote two orthogonal polarizations.

Follow either the order of steps 8a/9a or 8b/9b

8a. Repeat step 6 and 7 for all K test beams.

8b. Repeat step 5 and 7 for all M and N positioner angles

9a. Repeat step 5, and 8a for all M and N positioner angles.

9b. Repeat step 6 and 8b for all K test beams.

10. Calculate EEIRPi from measured EIRPkθn,φm for bins i=1..7, as described in Annex J.

### 6.17.5	Test requirements

#### 6.17.5.1	Requirement for NCR type 1-H

For NCR type 1-H operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, the power measured EEIRP shall not exceed the limits specified in table 6.17.5.1-1 for mechanical down-tilt angle equal to zero.

NOTE: OTA spatial emissions in table 6.17.5.1-1 are defined with respect to the horizon with angles above the horizon positive and below the horizon negative. The reference NCR coordinate system in sub-clause 4.12 system is defined with respect to the BS enclosure with angles above the boresight negative and below the boresight positive. If the NCR is aligned with the horizon (mechanical tilt is zero) then the coordinate systems θ  sign is inverted.

Table 6.17.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| NOTE 1: The requirement shall apply to all supported mechanical tilts. |  |  |

Figure 6.17.5.1-1: Definitions of θHLi and θHHi angles.

<Next changes>

# Annex J (normative):

# Expected EIRP calculation and spatial sampling grid

# J.1	General

The Expected EIRP calculation is based on two averaging processes:

- Averaging over the test beam directions.

- Averaging over horizontal and vertical angles within specified vertical angles bins.

Clause J.2 describes the details of the calculation of the EEIRP from the measured EIRP samples.

The required sampling resolution for the EIRP measurement per test beam direction is described in Annex J.3

# J.2	Averaging processes

    - J.2.1 Averaging over beam directions

The first averaging process involves averaging over different test beam directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φ=k=1KwkEIRPk(θ,φ)

The power measured is the sum of the EIRP in both polarisations. The EIRPθ,φ is the average EIRPk(θ,φ) over the K number of test beam directions for the angle (θ,φ).

For the case where non-zero mechanical tilt is considered the reference coordinate system in clause 4.12 in this document needs to be converted to the coordinate system used by the test requirement (Figure 6.17.5.1-1). The angles in the reference coordinate system can be translated to the coordinate system with respect to horizon as:

θH=90-arccoscosφsinθ+90sin-θMT+cosθ+90cos-θMT

φH=argcosφsinθ+90cos-θMT-cosθ+90sin-θMT+jsinφsinθ+90 ,

where:

θ 		The angle in the reference coordinate system defined in clause 4.12 between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the direction perpendicular to the y/z plane. The angle is aligned with the down-tilt angle.

φ 	is the angle in the reference coordinate system defined in clause 4.12 between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°.

θH 	is the angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

θMT 	is the down-tilt angle representing the mechanical tilt defined between -90° and +90° (positive towards the ground).

φH         The horizontal angle in the global coordinate system, defined with respect to the horizon.

    - J.2.2 Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This EEIRP for a spherical strip is therefore:

EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn  ,

Where

(qn, jm) represents discrete spatial measurement points in the measured average EIRP samples. The sample resolution is defined in clause J.3.

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi in degrees, e.g. for bin 1, this will be 0⁰ and 5⁰ respectively.

θn	is the corresponding angle of the nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φm	is the corresponding angle of the mth midpoint between the azimuth intervals

nL	is the lowest elevation index of sampling points within the θHLi , θHHi range

nH	is the highest elevation index of sampling points within the θHLi , θHHi range

M	is the number of azimuth points within the (-180°, 180°) range

N	is the number of elevation points within the θHLi , θHHi range

Note: All angles are in degrees.

    - J.3 Spatial sampling grid

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD ,

where  is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=w2+h2 ,

where w is the array antenna width, and h is the array antenna height.

<End of changes>