3GPP TSG-RAN WG4 Meeting #116bis	R4-2514454

Prague, Czech, 13th – 17th October, 2025

Agenda item:		7.2.4

Source:	Nokia

Title:	On SBFD BS to SBFD BS adjacent channel coexistence

Document for:		Approval

# Introduction

A new Rel-20 WID [1] was agreed in RAN#109 regarding NR base station RF requirement evolution for FR1 and testing phase 2. There are two topics introduced in this WID and this contribution further elaborates Nokia’s views on topic 1: SBFD BS to SBFD BS adjacent channel coexistence

# Discussion

As SBFD BS to SBFD BS adjacent channel coexistence is one of the objective in BS RF evolution work item [1] in our companion contribution [2] we provide and propose work plan for this item including objective for SBFD.

The following objectives are given in the WID for core part:

- SBFD BS to SBFD BS adjacent channel coexistence - Focus on FR1 WA. - Consider both scenarios including SBFD capable BS being co-located and non-collocated. - Focus on bands: n104, n79, n41. - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include - TX unwanted emission - RX in-band blocking - Note 1: whether to comply with requirements is declared by BS. - Note 2: The existing TDD requirements apply for non-SBFD BSs

As noted above, the focus shall be on FR1 wide area scenarios including co-located and non-collocated scenarios and different frequency bands. One of the first questions that RAN4 needs to discuss is how the requirements shall be derived: One option is to perform co-existence simulations while a second option is to derive (at least) some of the requirements based on coupling loss (MCL) and desensitization target between SBFD base stations.

Considering the fact that co-located and non-collocated are in the focus of the WI, we think co-existence simulations will be needed at least for the non-colocated case. In this case, similar methodology as the Rel-19 co-existence simulations with 100% grid-shift could be assumed. For the co-located case, some requirements can be defined based on MCL assumption and/or co-existence simulations assuming e.g. 10% grid shift between operators. This shall be discussed and agreed by RAN4 in this first meeting.

Both co-located and non-colocated scenarios are within the scope of the Rel-20 work item.

Methodology for deriving the requirements can be based on coupling loss (MCL) and/or co-existence studies. The selection of one or another option can be dependent on the specific scenario (e.g. co-located vs non-colocated)

If coexistence simulations is to be used for the co-located scenario, the assumed grid-shift shall be no more than 10%.

Last but not least, the WID also mentions different frequency bands: n104, n79, n41. This is different than the Rel-19 study which primarily assumed 4 GHz frequency band. Considering the different propagation and BS array size in those bands, those 3 bands may need to be accounted for during the corresponding analyses and evaluations. One option for simplification is to focus first n104 and one of n41/n79. For n41/n79, we could reuse the Rel-19 simulation assumptions including BS array size as starting point.

Considering the different propagation and BS beamforming characteristics, different simulations specific to each of the frequency bands listed in the WID (n104, n79, n41) may be required.

Proposal: It is proposed to reuse Rel-19 simulation assumptions for band n79 and n41.

RAN4 to discuss simulation assumptions for n104.

As noted above, the WID mentions “Specify necessary (and if feasible) BS TX and RX RF requirement …”. In this regard, part of the study in RAN4 should also be focused on discussing on whether the new unwanted emmisisons and in-band blocking requirements are possible to achieve. The preliminary study from Huawei in R4-2507683 [3] shows that ~36 dB improvement of ACIR (on top of the baseline 42.5) is needed for scenarios with 10% grid shift. This brings the question on whether it is possible to achieve such requirement in practical BS implementations, as this will likely requires operator-specific analog filters with little insertion loss. Also, full coordination between operators will be required to ensure that emissions between operators are minimised.

RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation.

# Conclusion

This document shares Nokia’s views on SBFD BS to SBFD BS adjacent channel coexistence. Following observations and proposals were introduced:

- Both co-located and non-colocated scenarios are within the scope of the Rel-20 work item.

Methodology for deriving the requirements can be based on coupling loss (MCL) and/or co-existence studies. The selection of one or another option can be dependent on the specific scenario (e.g. co-located vs non-colocated)

- If coexistence simulations is to be used for the co-located scenario, the assumed grid-shift shall be no more than 10%.

Considering the different propagation and BS beamforming characteristics, different simulations specific to each of the frequency bands listed in the WID (n104, n79, n41) may be required.

Proposal: It is proposed to reuse Rel-19 simulation assumptions for band n79 and n41.

RAN4 to discuss simulation assumptions for n104.

RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation.

# References

- RP-252905 “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, RAN#109, Beijing, China, 15th-18th September, 2025.

- R4-2514089, On NR BS RF requirement evolution phase 2 general aspects and work plan, Nokia

- R4-2507683, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon