3GPP TSG-RAN4 Meeting #116bis	R4-2514456

# Prague, Czech Republic 13th – 17th October, 2025

Source: 	Qualcomm Incorporated

Title: 	AI-ML based CSI Prediction: Core Requirements

Agenda item:	6.11.3

Document for:	Discussion

# Introduction

In this paper, we primarily focus on performance monitoring related core and accuracy requirements of CSI prediction.

# Prior Agreements

Following agreement happened during the last few meetings [1][2][3][4]

Issue 1-1: Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test.

Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this.

# Discussion of Performance Monitoring

Figure 1: Performance Monitoring in CSI Prediction

## Focus of Discussion

Based on the recent RAN4 agreement regarding performance monitoring [4], RAN4 needs to finalize metrics to define requirement, along with defining corresponding reporting delay, accuracy and mapping table. RAN4 also needs to check if it is feasible to define reporting accuracy of performance monitoring.

Observation 1: RAN4 needs to finalize following aspects of CSI performance monitoring:

- Selection of metric for requirement

- Reporting delay,

- Reporting accuracy, including the feasibility of defining reporting accuracy.

- Mapping table.

## Selection of Metric

RAN1 made the following agreement regarding performance monitoring [5]:

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1

Above RAN1 agreement suggests that UE needs to report two SGCS values in performance monitoring.

- One SGCS is based on predicted CSI for one inference reporting and ground truth CSI. Let’s call it SGCS1.

- Another SGCS is based on ground truth CSI and non-predicted CSI corresponding to the latest CSI-RS occasion not later than the CSI reference resource of the inference reporting instance. Let’s call this SGCS2.

Figure 1 shows how SGCS1 and SGCS2 get calculated in the context of performance monitoring of CSI prediction.

Observation 2: RAN1 agreed to define two SGCS values in performance monitoring. First SGCS is based on predicted CSI for one inference reporting and ground truth CSI. Second SGCS is based on ground truth CSI and non-predicted CSI corresponding to the latest CSI-RS occasion.

First SGCS, i.e., SGCS1 conveys how well CSI prediction is performing. Hence, SGCS1 has to be decided as an evaluation metric for performance metric.

Second SGCS, i.e., SGCS2, on the other hand, tries to convey the performance of “sample-and-hold” to network. The ratio of SGCS1 to SGCS2 conveys the ratio of the performance of CSI prediction to that of sample-and-hold based CSI feedback to the network.

One issue of SGCS1/SGCS2 is that the ratio can become high even if SGCS1 and SGCS2 are both low. This can happen specially when SGCS1 and SGCS2 are quite low; and a sudden fluctuation in one of these two metrics can make SGCS1/SGCS2 quite high.

Hence, RAN4 should select SGCS1 as the performance metric for performance monitoring of CSI prediction. RAN4 needs to discuss further if SGCS1/SGCS2 can be used as the performance metric.

RAN1 defined signalling allows UE to report one instantaneous value of SGCS1 and SGCS2. UE’s pass or fail criteria should not depend on one instantaneous value of SGCS1 and/or SGCS2. RAN4 should collect UE’s report across multiple occasions and UE’s pass or fail criteria should depend on the statistics of selected metric.

Observation 3:  The SGCS based on predicted CSI for one inference reporting and ground truth CSI conveys the performance of CSI prediction.

Observation 4: The SGCS based on ground truth CSI and non-predicted CSI corresponding to the latest CSI-RS occasion conveys how well sample-and-hold based CSI feedback would perform.

Observation 5: The ratio of the first and second SGCS of observation 2 conveys the ratio of the performance of CSI prediction to that of sample-and-hold to the network.

Observation 6: The ratio of the first and second SGCS of observation 2 can become high even if first and second SGCS are low. This can happen specially when first and second SGCS are quite low; and a sudden fluctuation in one of these two metrics can make the ratio quite high.

Proposal 1: RAN4 selects the statistics of SGCS1, defined based on predicted CSI for inference reporting and ground truth CSI, as the metric to evaluate UE’s performance monitoring for CSI prediction. UE would pass a test if its reported SGCS1, averaged across many occasions, exceed a threshold.

## Mapping Table

RAN1 made the following agreement during the last meeting:

Agreement Adopt following TPs for TS38.214 Section 5.2.1.4.6. Reason for change; In current specification, there is no SGCS quantization mapping table.. Summary of change; SGCS quantization mapping table is added.. Consequences if not approved; Incomplete capture of the agreement.. TS38.214 ##### 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in [xx, TS 38.abc]Table 5.2.1.4.6-1. Table 5.2.1.4.6-1: SGCS Quantization Mapping Code Point; Range. 0; [0.0000, 0.3000]. 1; (0.3000, 0.3500]. 2; (0.3500, 0.4000]. 3; (0.4000, 0.4500]. 4; (0.4500, 0.5000]. 5; (0.5000, 0.5500]. 6; (0.5500, 0.6000]. 7; (0.6000, 0.6500]. 8; (0.6500, 0.7000]. 9; (0.7000, 0.7500]. 10; (0.7500, 0.8000]. 11; (0.8000, 0.8500]. 12; (0.8500, 0.9000]. 13; (0.9000, 0.9500]. 14; (0.9500, 1.0000]. 15; Reserved. << Unchanged parts are omitted >>.

Above RAN1 agreement shows that RAN1 has already defined the mapping table for reporting of performance monitoring. The RAN1 defined mapping table explicitly maps SGCS value to codepoints. RAN4 can just reuse the RAN1 defined mapping table for reporting of CSI prediction’s performance monitoring

Observation 7: RAN1 has already defined the mapping table for reporting of performance monitoring. The RAN1 defined mapping table explicitly maps SGCS value, reported to convey the outcome of performance monitoring, to codepoints.

Proposal 2: RAN4 reuses the RAN1 defined mapping table for reporting of CSI prediction’s performance monitoring.

## Reporting Accuracy

Only UE would truly know its observed SGCS during a test. gNB and test equipment vendor must rely on UE’s report to know the SGCS value in the field and conformance test respectively. Test equipment vendor cannot truly estimate the accuracy of UE’ reported SGCS during performance monitoring. Test equipment vendor can only check if the UE’s reported SGCS, averaged across many occasions, exceed a threshold where the threshold is defined based on simulations.

Observation 8: Test equipment vendor cannot truly estimate the accuracy of UE’ reported SGCS during performance monitoring. Test equipment vendor can only check if the UE’s reported SGCS, averaged across many occasions, exceed a threshold where the threshold is defined based on simulations.

Proposal 3: RAN4 considers the following note while discussing to define the requirements for performance monitoring.

- Note: Test equipment vendor cannot truly estimate the accuracy of UE’ reported SGCS during performance monitoring. Test equipment vendor can only check if the UE’s reported SGCS, averaged across many occasions, exceed a threshold where the threshold is defined based on simulations.

## Selection of Channel

RAN4 has selected TDL to define throughput ratio related requirements of AI-ML based CSI prediction. Until Rel-18, RAN4 has only calibrated TDL based channel models for demod testing, including requirements for PMI testing. Hence, RAN4 should use TDL channels to define requirement of performance monitoring of CSI prediction.

One of the main objectives of defining requirements for performance monitoring of CSI prediction is to inform gNB what it can expect in the field. Hence, RAN4 should investigate UE’s reported SGCS1 during performance monitoring for CSI prediction across various channels, e.g., TDL-A, TDL-B and TDL-C, to define the threshold for requirements.

Observation 9: Until Rel-18, RAN4 has primarily used TDL based channel models for demod testing, including requirements for PMI testing.

Observation 10: RAN4 has selected TDL to define throughput ratio related requirements of AI-ML based CSI prediction.

Observation 11: One of the main objectives of defining requirements for performance monitoring of CSI prediction is to inform gNB what it can expect in the field.

Proposal 4: RAN4 investigates UE’s reported SGCS1 during performance monitoring for CSI prediction across various channels, e.g., TDL-A, TDL-B and TDL-C, and selects the worst-case scenario to define the threshold for requirements.

# Conclusions

Observation 1: RAN4 needs to finalize following aspects of CSI performance monitoring:

- Selection of metric for requirement

- Reporting delay,

- Reporting accuracy, including the feasibility of defining reporting accuracy.

- Mapping table.

Observation 2: RAN1 agreed to define two SGCS values in performance monitoring. First SGCS is based on predicted CSI for one inference reporting and ground truth CSI. Second SGCS is based on ground truth CSI and non-predicted CSI corresponding to the latest CSI-RS occasion.

Observation 3:  The SGCS based on predicted CSI for one inference reporting and ground truth CSI conveys the performance of CSI prediction.

Observation 4: The SGCS based on ground truth CSI and non-predicted CSI corresponding to the latest CSI-RS occasion conveys how well sample-and-hold based CSI feedback would perform.

Observation 5: The ratio of the first and second SGCS of observation 2 conveys the ratio of the performance of CSI prediction to that of sample-and-hold to the network.

Observation 6: The ratio of the first and second SGCS of observation 2 can become high even if first and second SGCS are low. This can happen specially when first and second SGCS are quite low; and a sudden fluctuation in one of these two metrics can make the ratio quite high.

Observation 7: RAN1 has already defined the mapping table for reporting of performance monitoring. The RAN1 defined mapping table explicitly maps SGCS value, reported to convey the outcome of performance monitoring, to codepoints.

Observation 8: Test equipment vendor cannot truly estimate the accuracy of UE’ reported SGCS during performance monitoring. Test equipment vendor can only check if the UE’s reported SGCS, averaged across many occasions, exceed a threshold where the threshold is defined based on simulations.

Observation 9: Until Rel-18, RAN4 has primarily used TDL based channel models for demod testing, including requirements for PMI testing.

Observation 10: RAN4 has selected TDL to define throughput ratio related requirements of AI-ML based CSI prediction.

Observation 11: One of the main objectives of defining requirements for performance monitoring of CSI prediction is to inform gNB what it can expect in the field.

Proposal 1: RAN4 selects the statistics of SGCS1, defined based on predicted CSI for inference reporting and ground truth CSI, as the metric to evaluate UE’s performance monitoring for CSI prediction. UE would pass a test if its reported SGCS1, averaged across many occasions, exceed a threshold.

Proposal 2: RAN4 reuses the RAN1 defined mapping table for reporting of CSI prediction’s performance monitoring

Proposal 3: RAN4 considers the following note while discussing to define the requirements for performance monitoring.

- Note: Test equipment vendor cannot truly estimate the accuracy of UE’ reported SGCS during performance monitoring. Test equipment vendor can only check if the UE’s reported SGCS, averaged across many occasions, exceed a threshold where the threshold is defined based on simulations.

Proposal 4: RAN4 investigates UE’s reported SGCS1 during performance monitoring for CSI prediction across various channels, e.g., TDL-A, TDL-B and TDL-C, and selects the worst-case scenario to define the threshold for requirements.

# References

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- R4-2502856, WF on the requirements for AI/ML air interface, 3GPP TSG RAN WG4 #114.

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN WG4 #114bis.

- R4-2511890, WF on AI-ML air interface, 3GPP TSG-RAN WG4 #116

- Chairman’s Notes, 3GPP TSG RAN WG1 121.