3GPP TSG-RAN4 Meeting #116bis	R4-2514458

# Prague, Czech Republic 25th – 29th August, 2025

Source: 	Qualcomm Incorporated

Title: 	Core Requirements of AI-ML based CSF Compression

Agenda item:	7.9.4

Document for:	Discussion

# Introduction

In this paper, we present our initial views regarding core requirements for two-sided CSF compression.

# Discussion and Results

## Preferred Option for Normative Work

During the study item, RAN4 has focused on both fully specified option 3 and partially specified option 4. Recently concluded RAN plenary captured the following texts in the WID of Rel-20 AI-ML NR air interface [1].

Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Above texts of WID clearly show that RAN plenary requires RAN4 to define requirement for the fully specified test decoder, i.e., option 3.

A careful look at the agreements related to option 3 and option 4 reveal that both these options are quite similar in terms of capturing or specifying a reference encoder, a reference decoder and a training dataset. The major difference between the two options lies in the fact that option 4 allows test vendor to implement test decoder with different parameters (i.e., weights and biases) or with a partially different structure. But RAN4 has not yet investigated the interoperability in the case that the structure of the TE decoder is not fully specified.

RAN4 115 agreement: - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

Besides, designing test via option 4 may lead to unintended complications in the tests. Typically, when UE fails a test, UE needs to be debugged because the issue is expected to happen at the UE. In the partially specified option, UE’s failure during a test could stem from UE issue or from a potential TE issue where TE’s selected parameters for the test vendor are not suitable for the reference encoder and decoder. Hence, both UE and TE need to be debugged during a test that focuses on option 4. Hence, RAN4 should just focus on option 3 during the work item.

Observation 1: Recently agreed RAN plenary requires RAN4 to define requirement for the fully specified test decoder, i.e., option 3.

Observation 2: Both option 3 and option 4 are quite similar in terms of capturing or specifying a reference encoder, a reference decoder and a training dataset.

Observation 3: Option 4 allows test vendor to implement test decoder with different parameters or with a partially different structure

Observation 4: In option 4, UE’s failure during a test could stem from UE issue or from a potential TE issue where TE’s selected parameters for the test vendor are not compatible for the reference encoder and decoder

Proposal 1: RAN4 follows the guidance of the WID of Rel-20 AI/ML and focuses only on fully specified decoder option 3 during the WI.

## Simulation Results with Low Complexity Encoder-Decoder

Figure 1: Steps to calculate SGCS of Table 1 (joint training with low complexity encoder and high complexity decoder)

During the last meeting, companies simulated the performance of different low complexity encoders with the decoder that got agreed during RAN4 study item. Companies were requested to report the SGCS, denoted as SGCS-3 in Figure 1 [3]

Table 1 shows the performance of different low complexity encoders with the high complexity decoder. [2]. The models of these low complexity encoders are shown in the appendix section.

Table 1: SGCS Performance of joint training with low complexity encoder and the high complexity decoder

|  |  | Enc-2 (Transformer 3.63 MFLOPs) | Enc-3 (CNN 4.14 MFLOPs) | Enc-4 (MLP 0.22 MFLOPs) | Enc-5 (Transformer 8 MFLOPs) |
| --- | --- | --- | --- | --- | --- |
| Samsung |  | 0.7163 | 0.6854 | 0.6741 |  |
| ZTE |  | 0.7061 | 0.6809 | 0.6797 |  |
| Ericsson |  | 0.7112 | 0.679 | 0.6677 |  |
| OPPO |  | 0.7175 | 0.6988 | 0.6775 |  |
| VIVO |  | 0.709 | 0.691 | 0.671 |  |
| Nokia |  | 0.702 | 0.686 | 0.671 |  |
| CMCC |  | 0.7127 | 0.6739 | 0.664 |  |
| Huawei,Hisilicon |  | 0.6793 | 0.6911 | 0.6782 |  |
| MediaTeK |  | 0.7064 | 0.6744 | 0.6705 |  |
| CATT |  | 0.6999 | 0.6831 | 0.6344 |  |
| Qualcomm |  |  |  |  | 0.7087 |

According to almost all companies’ results of Table 1, low complexity transformer-based encoder performs better, in terms of SGCS, than low complexity CNN and MLP based encoder when it gets jointly trained with high complexity CNN decoder. RAN4 has not investigated the performance of low complexity encoder and decoder for different networks (e.g., transformer, CNN and MLP). However, we internally compared the performance of transformer based low complexity decoder with that of CNN based high complexity decoder (agreed model during the SI).

Table 2: SGCS performance of option 3 track 2 with mixed dataset based training and testing (# of Flops = 8M)

| Training type | Enc-model | Dec-model | SGCS | Ref SGCS of SAM Enc-Dec |
| --- | --- | --- | --- | --- |
| joint training | Low complexity TF | Low complexity TF | 0.7084 | 0.7016 |
| joint training | Low complexity TF | High complexity CNN | 0.7087 |  |
| seq training | Low complexity TF | High complexity CNN (Samsung Decoder) | 0.7028 |  |

Table 2 shows that the SGCS performance of jointly trained low complexity TF encoder -  low complexity TF decoder is same as that of jointly trained low complexity TF encoder – high complexity CNN decoder. . The required number of flops for the considered transformer-based architecture is much smaller compared to the CNN based architecture that RAN4 is currently considering in study item. Both UE and TE vendors have no reason to consider a more complex model if an encoder-decoder with lower complexity can achieve same performance.

RAN1 has extensively studied the feasibility of scalable model structure specification by adopting transformer as backbone structure. RAN1 considered transformer-based encoder structure of Figure 2 as an example in its study of feasibility and scalability. We use this transformer model to evaluate the performance of Option 3 track 2 for both joint and sequential training of Table 2.

RAN1 made the following agreements regarding scalability of AI-ML models of temporal domain case 0. Scalability over payload, along with feature and token dimensions are essential criteria to decide decoder structure of Direction C.

Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters

Figure 2: RAN1’s considered transformer based encoder structure for scalability and feasibility study

If RAN4 adopts transformer as the basic structure for encoder and decoder of option 3, RAN4 can use the outcome of scalability related discussion of RAN1 study as its starting point. RAN4 will have to perform the entire scalability study again if it selects CNN or MLP as the structure of decoder in option 3.

Hence, RAN4 should use transformer as the backbone structure to define the standardized test structure in RAN4 tests, and “Dir C” and “Direction A, sub-option 3a-1” of RAN1 options during the work item.

Observation 5: RAN1 considered transformer-based encoder structure of Figure 2 as an example in its study of feasibility and scalability.

Observation 6: According to the results of almost all companies, the SGCS performance of low complexity transformer-based encoder is better than that of low complexity CNN and MLP based encoders, when the low complexity encoder gets jointly trained and tested with the high complexity CNN based decoder.

Observation 7: The SGCS performance of jointly trained low complexity TF encoder -  low complexity TF decoder is same as that of jointly trained low complexity TF encoder – high complexity CNN decoder.

Observation 8: Both UE and TE vendors have no reason to consider a more complex model if an encoder-decoder with lower complexity can achieve same performance.

Observation 9: Scalability over payload, along with feature and token dimensions, are essential criteria to decide decoder structure of Direction C and 3a-1.

Observation 10: RAN1 has extensively studied the feasibility of scalable model structure specification by adopting transformer as backbone structure.

Observation 11: If RAN4 adopts transformer as the basic structure for encoder and decoder of option 3, RAN4 can use the outcome of scalability related discussion of RAN1 study as its starting point. RAN4 will have to perform the entire scalability study again if it selects CNN or MLP as the structure of decoder in option 3.

Proposal 2: RAN4 uses transformer as the backbone structure to define the standardized test structure in RAN4,  and “Dir C” and “Direction A, sub-option 3a-1” during the work item.

# Conclusions

Observation 1: Recently agreed RAN plenary requires RAN4 to define requirement for the fully specified test decoder, i.e., option 3.

Observation 2: Both option 3 and option 4 are quite similar in terms of capturing or specifying a reference encoder, a reference decoder and a training dataset.

Observation 3: Option 4 allows test vendor to implement test decoder with different parameters or with a partially different structure

Observation 4: In option 4, UE’s failure during a test could stem from UE issue or from a potential TE issue where TE’s selected parameters for the test vendor are not compatible for the reference encoder and decoder

Observation 5: RAN1 considered transformer-based encoder structure of Figure 2 as an example in its study of feasibility and scalability.

Observation 6: According to the results of almost all companies, the SGCS performance of low complexity transformer-based encoder is better than that of low complexity CNN and MLP based encoders, when the low complexity encoder gets jointly trained and tested with the high complexity CNN based decoder.

Observation 7: The SGCS performance of jointly trained low complexity TF encoder -  low complexity TF decoder is same as that of jointly trained low complexity TF encoder – high complexity CNN decoder.

Observation 8: Both UE and TE vendors have no reason to consider a more complex model if an encoder-decoder with lower complexity can achieve same performance.

Observation 9: Scalability over payload, along with feature and token dimensions, are essential criteria to decide decoder structure of Direction C and 3a-1.

Observation 10: RAN1 has extensively studied the feasibility of scalable model structure specification by adopting transformer as backbone structure.

Observation 11: If RAN4 adopts transformer as the basic structure for encoder and decoder of option 3, RAN4 can use the outcome of scalability related discussion of RAN1 study as its starting point. RAN4 will have to perform the entire scalability study again if it selects CNN or MLP as the structure of decoder in option 3.

Proposal 1: RAN4 follows the guidance of the WID of Rel-20 AI/ML and focuses only on fully specified decoder option 3 during the WI.

Proposal 2: RAN4 uses transformer as the backbone structure to define the standardized test structure in RAN4,  and “Dir C” and “Direction A, sub-option 3a-1” during the work item.

# References

[1] RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, 3GPP TSG RAN Meeting #108.

[2] R4-2510880, Collection of simulation results on AIML CSI compression, 3GPP TSG RAN4 Meeting #116.

[3] R4-2508084, Simulation assumptions for CSI compression. RAN4, 3GPP TSG RAN4 Meeting #115.

[4] Chairman’s notes, 3GPP TSG RAN1 Meeting #120.

# Appendix (Models and Descriptions of Different Low Complexity Encoders)

## Encoder-2 (Transformer 3.63MFLOPs)

## Encoder-3 (CNN 4.14 MFLOPs)

## Encoder-4 (MLP 0.22MFLOPs)

## Encoder-5 (Transformer 8M FLOPs)