3GPP TSG-RAN4 Meeting #116bis		R4-2514461

# Prague, Czech Republic 13th – 17th October, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Performance Requirements and Simulation Results of AI-ML Beam Management

Agenda item:	6.12.6

Document for:	Discussion

# Introduction

This paper presents simulation results for AI/ML based beam management. The simulation assumptions were agreed during the last couple of meetings [1][4]. We also discuss how to set simulation to define performance requirements.

# Simulation Setting for Performance Requirements

RAN4 has, so far, focused on dense urban macro channel to simulate and investigate the performance of spatial only beam prediction. System level channel is more realistic for field deployment.

On the other hand, RAN4 has not calibrated system level channel for FR2 OTA testing. RAN4 has calibrated Umi CDL-C channel for FR2 MIMO OTA testing and captured it in Table D.1-1 of 38.151.

During the last meeting, RAN4 agreed to consider multi AoA based RRM testing setup as baseline to test AI-ML based beam prediction.

Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing) Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. Take CDL-C Umi as one example Other channel models can be further discussed

RAN4 must consider the spatial channel model that gets defined for AI-ML BM OTA testing to define performance requirement.

Hence, RAN4 can define performance requirement based on the worst-case performance of dense urban macro and the spatial channel model that gets selected for AI-ML BM OTA testing. This will allow RAN4 requirement to be realistic and achievable in a test chamber.

Observation 1: RAN4 is currently focusing on system level channel, e.g., dense urban macro, to simulate and investigate the performance of spatial only beam prediction.

Observation 2: AI-ML beam prediction requirement defined based on system level channel is more realistic for field deployment.

Observation 3: RAN4 has not calibrated system level channel for FR2 OTA testing

Observation 4: RAN4 has defined multi AoA based RRM testing setup as baseline for AI-ML BM OTA. Companies are invited to bring analysis on what simplified spatial channel model can be emulated in this test setup.

Observation 5: RAN4 has requested companies to bring analysis regarding how CDL channels can be simplified to emulate the channels in the enhanced IFF chamber.

Proposal 1: Accuracy of AI-ML BM-case 1 is defined based on the worst-case performance of two test dataset where datasets come from following two scenarios:

- Dense urban macro

  - Note: This is currently under consideration and captured in the recently agreed simulation assumption of R4-2508081

- The CDL channel that gets defined to test AI-ML BM OTA

# Simulation Results and Error Modeling

## Baseband Error Modelling

RAN4 focused on following two different options to investigate the impact of baseband error distribution on AI-ML beam prediction accuracy.

Option 1: Apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS Option 2: Generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS

We use option 2 in this contribution, i.e., we generate baseband error distribution separately for each individual TX-RX beam pair’s SNRs, model these distributions as Gaussian distributions and then apply the distributions to the corresponding Tx-Rx beam pair measurement in the SLS.

Just for illustration purposes, we show the distribution of baseband error at -3 dB with AWGN and TDL-C in Figure [1] and [2] respectively.

Figure 1: Baseband absolute error distribution of SSB L1-RSRP at AWGN (-3 dB SNR, curve fitted with zero mean normal distribution and sigma = 0.6 dB)

Figure 2: Baseband absolute error distribution of SSB L1-RSRP (TDL-C, -3 dB SNR, curve fitted with normal distribution and mean = -0.12 dB and sigma = 0.85 dB)

Observation 6: Baseband error at different SNRs can be modelled with Gaussian distribution. For example, at -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-C: Gaussian with mean = -0.12 dB and sigma = 0.85 dB

## 2.4.3 Simulation Results

### 2.4.3.1 Simulation Assumptions

Table 1 displays the simulation parameters that we used in this study.

Table 1: Baseline System Level Simulation assumptions for AI/ML in “spatial only” beam prediction evaluations

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 3km/h |
| UE distribution | 100% outdoor |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 downlink Rx beams per UE panel at UE side. |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Inter-panel calibration for UE | Ideal |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200 m |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |
| Scenario | Dense Urban (macro-layer only, TR 38.913) |
| AoDs of Set A and Set B (°) | - Set A: - 4 elevation DFT beams: [106.25, 118.75, 131.25, 143.75] - 8 azimuth DFT beams: [-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5] - Set B [(azimuth, elevation)]: [(-52.5, 143.75), (-37.5, 131.25), (-22.5, 118.75), (-7.5, 106.25), (7.5, 143.75), (22.5, 131.25), (37.5, 118.75), (52.5, 106.25) |
| Beam pattern legend for Set A and Set B |  |
| Baseline for performance evaluation | Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) |

### 2.4.3.2 Definition

Table 2 shows the definition of different metrics for which we generated results in this contribution.

Table 2: Definitions of Metrics for AI-ML based Beam Prediction

| Metric | Definition |
| --- | --- |
| L1-RSRP absolute accuracy | 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1 predicted beam and the ground truth L1-RSRP of the same beam. |
| Worst case L1-RSRP absolute accuracy among top-K predicted beams | 90%-ile of the worst case L1-RSRP difference between the predicted L1-RSRP of the top-K predicted beams and the ground truth L1-RSRP of the same beams. This gets calculated in following steps: - In one sample of inference reporting, assume I = arg max_{k \in K} (predicted RSRP of beam k – ground truth of beam k) - Store (predicted RSRP of beam I – ground truth of beam I) for this sample of inference - Report 90%-ile distribution of above metric |
| Relative L1-RSRP of beam owning the 2nd largest predicted reported value | 90%-ile of following metric: (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. |
| Top K/1 without margin | The percentage of the time when the Top-1 strongest beam (ground truth based) is one of the Top-K predicted beams |
| Top 1/K without margin | The percentage of the time when the Top-1 predicted beam is one of the Top-K strongest beams (ground truth based) |
| Top K/N (m) without margin | The percentage of the time when the Top-K predicted beams contain m of the top-N strongest beams (ground truth based) |
| Top 1/1 with margin of X dB | The successful rate for the correct prediction which is considered as maximum ideal RSRP among top-1 predicted beams is larger than the ideal RSRP of the strongest genie-aided beam – x dB |

### 2.4.4.3. Results

Observation 7: Table 3 contains the results in “narrow” to “narrow” beam prediction scenario.

Table 3: Results of AI-ML based “spatial only” beam prediction (“narrow” to “narrow” scenario)

| Metrics |  | Training and testing without measurement error | Training and testing dataset, along with ground truth for training and testing, with measurement error |
| --- | --- | --- | --- |
| L1-RSRP absolute accuracy (dB) |  | +- 3.72 | +- 4.6 |
| Top K/1 without margin (%) | K = 1 | 78.25 | 71.3 |
|  | K = 3 | 95.6 | 93.6 |
|  | K = 5 | 98 | 97 |
| Top 1/1 with margin of X dB (%) | X = 1 | 87.45 | 80.53 |
|  | X = 3 | 94.93 | 91.43 |
|  | X = 5 | 96.94 | 95.39 |

# Conclusions

Observation 1: RAN4 is currently focusing on system level channel, e.g., dense urban macro, to simulate and investigate the performance of spatial only beam prediction.

Observation 2: AI-ML beam prediction requirement defined based on system level channel is more realistic for field deployment.

Observation 3: RAN4 has not calibrated system level channel for FR2 OTA testing

Observation 4: RAN4 has defined multi AoA based RRM testing setup as baseline for AI-ML BM OTA. Companies are invited to bring analysis on what simplified spatial channel model can be emulated in this test setup.

Observation 5: RAN4 has requested companies to bring analysis regarding how CDL channels can be simplified to emulate the channels in the enhanced IFF chamber.

Observation 6: Baseband error at different SNRs can be modelled with Gaussian distribution. For example, at -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-C: Gaussian with mean = -0.12 dB and sigma = 0.85 dB

Observation 7: Table 3 contains the results in “narrow” to “narrow” beam prediction scenario.

Table 3: Results of AI-ML based “spatial only” beam prediction (“narrow” to “narrow” scenario)

| Metrics |  | Training and testing without measurement error | Training and testing dataset, along with ground truth for training and testing, with measurement error |
| --- | --- | --- | --- |
| L1-RSRP absolute accuracy (dB) |  | +- 3.72 | +- 4.6 |
| Top K/1 without margin (%) | K = 1 | 78.25 | 71.3 |
|  | K = 3 | 95.6 | 93.6 |
|  | K = 5 | 98 | 97 |
| Top 1/1 with margin of X dB (%) | X = 1 | 87.45 | 80.53 |
|  | X = 3 | 94.93 | 91.43 |
|  | X = 5 | 96.94 | 95.39 |

Proposal 1: Accuracy of AI-ML BM-case 1 is defined based on the worst-case performance of two test dataset where datasets come from following two scenarios:

- Dense urban macro

  - Note: This is currently under consideration and captured in the recently agreed simulation assumption of R4-2508081

- The CDL channel that gets defined to test AI-ML BM OTA

# References

- R4-2420480, Updated Simulation Assumptions for Beam Prediction, RAN4 #113.

[2] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114

[3] R4-2505105, “WF on requirements for AI/ML air interface normative work”, 3GPP TSG-RAN WG4 Meeting#114bis

[4] R4-2505152, “Updated simulation assumptions for AI/ML BM”, 3GPP TSG-RAN WG4 Meeting#114bis

[5] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114

[6] R4-2511890, “WF on AI/ML for air interface for [116][131]”. 3GPP TSG-RAN WG4 #116.