3GPP TSG-RAN4 Meeting #116bis	R4-2514462

# Prague, Czech Republic 13th – 17th October, 2025

Source: 	Qualcomm Incorporated

Title: 	Inter-vendor training collaboration and Interoperability of CSF Compression

Agenda item:	7.9.3

Document for:	Discussion

# Introduction

In this paper, we present our views regarding the channel model to design for Direction C and define performance requirement.

We also re-submit simulation results from the last meeting that are based on the assumptions that got agreed during the study item.

- Results with fully specified option 3 based on mixed dataset of track 2 with fully specified decoder

- Results with partially specified option 4 based on mixed dataset of track 2 with the reference encoder

# Past Agreements

## System Parameters

The system parameter assumptions of 38.843 were considered to be baseline for RAN4 performance evaluation.

Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz as baseline, optional for 4GHz (if R16 as baseline) FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz (if R17 as baseline) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R16 as baseline) 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R17 as baseline) |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| Feedback assumption |  | Realistic |

# Discussion and Results

## Simulation Results with Option 3

Table 1: Simulation result with Option 3 Track 2 (resubmission from last meeting)

| Option 3 |  |  |  |
| --- | --- | --- | --- |
|  |  | Joint training (QC encoder with QC decoder) | New QC encoder training with (Samsung’s) frozen decoder |
|  |  | Training and testing on mixed | Training and testing on mixed |
| RAN4 Agreed encoder structure, CNN | Qualcomm | 0.683 | 0.7005 |

Table 1 shows the SGCS results with track 2, Qualcomm encoder-decoder with joint training and Qualcomm encoder based on Samsung’s decoder. Following steps were performed to generate the result of right hand side of Table 1.

- Samsung’s decoder is obtained from [9] (uploaded after RAN4 114 meeting) and considered as a frozen decoder.

- Qualcomm’s encoder is trained with Samsung’s frozen decoder based on mixed dataset of [10] (generated and uploaded after RAN4 114 meeting)

- Trained Qualcomm encoder from previous step and Samsung’s frozen decoder is tested based on mixed dataset of [10]

Table 1 shows that the SGCS obtained with separate encoder decoder training (QC encoder with Samsung’s frozen decoder) is very close to the SGCS obtained with Qualcomm’s joint encoder-decoder training. Hence, fully specified decoder in track 2 with mixed dataset seems to be a feasible approach.

Observation 1: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenario where encoder is trained with the same dataset as the decoder.

## Simulation Results with Option 4

The following steps were performed to evaluate the feasibility of option 4:

- Take Samsung reference encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new “own” nominal decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Test “own” encoder with the nominal decoder of all companies, including own “nominal” decoder

  - Note: Nominal decoders of all companies are generated based on Samsung’s reference encoder

Table 2 shows that the SGCS obtained with all other companies’ nominal decoders are quite aligned and they are very similar to the SGCS obtained with Qualcomm’s own encoder and own decoder/

Table 2: Simulation results with Option 4 reference encoder with mixed dataset (resubmission from last meeting)

| Option 4 Table shows result of ‘own encoder’ with ‘own decoder’ and other companies’ nominal decoders (all developed based on Samsung’s reference encoder) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Dataset1=Dataset2=Dataset3=mixed |  |  |  |  |  |
|  |  | Qualcomm | MediaTek | VIVO | Nokia | Huawei, Hisilicon | Ericsson |
| RAN4 Agreed encoder, CNN | Qualcomm | 0.6972 | 0.6832 | 0.6894 | 0.6826 | 0.701 | 0.6909 |

Observation 2: Option 4 leads to good SGCS between company’s own encoder and nominal decoders of all other companies when all encoders and decoders are generated based on Samsung’s reference encoder and mixed dataset.

The following simulations were performed to evaluate the importance of dataset standardization in option 4b:

- Take Samsung reference encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new “own” nominal decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Test “own” encoder with own “nominal” decoder

Table 3: Simulation results with Option 4 reference encoder with different datasets (new submissions from last meeting)

| Option 4 Table shows result of ‘own encoder’ with ‘own nominal decoder’ with different datasets (Reference encoder comes from Samsung) |  |  |
| --- | --- | --- |
|  | Qualcomm “nominal” decoder Dataset1= Dataset4 = mixed Dataset2=Dataset3=own dataset | Qualcomm “nominal” decoder Dataset1 = Dataset2 = Dataset3 = mixed Dataset4 = own dataset |
| RAN4 Agreed encoder, CNN Qualcomm “own” encoder | 0.4713 | 0.4577 |

Table 3 shows that the performance of option 4b can significantly drop if a different dataset is used during training the nominal decoder or during the tests. That’s why, during the last meeting, RAN4 has made the correct decision to capture the dataset to train the test decoder and frozen “no minal” decoder.

Observation 3: Performance of option 4b can significantly drop if a different dataset is used while training the nominal decoder and the test decoder.

## Channel Selection For Normative Work

RAN plenary has assigned the following tasks for RAN4 in Rel-20’s two-sided CSF compression work item [13]

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Above part shows that RAN4 has, primarily, three tasks in Rel-20:

- Define and specify reference model for Direction C that could be used in the field deployment

  - Note: The structure of model, defined in Direction C, will be used in Direction 3a-1, too.

- Specify the standardized test decoder for performance tests and define corresponding performance requirements

- Define RRM requirements for LCM

RAN plenary guideline does not clarify whether the standardized test decoder to define performance requirement should be same as that of the reference decoder for Direction C and Direction 3a-1. From a RAN4 perspective, there is no clear benefit in designing different decoder structures for performance requirement and Direction C/Direction 3a-1. RAN4 would have to duplicate the effort if it designs different decoder structures for performance requirement and Direction C/Direction 3a-1. RAN4 should design a single decoder structure for Direction C/Direction 3a-1 and that test decoder should be used in defining performance requirement.

Observation 4: RAN4 would have to duplicate its efforts if it designs different decoder structures for performance requirement and Direction C/Direction 3a-1.

Proposal 1: RAN4’s designed decoder for Direction C/Direction 3a-1 also gets used as the standardized test decoder to define performance requirements.

The ‘system parameters’ section shows that RAN4 used a system level channel to generate SGCS results during the Rel-19 study item of CSF compression. Simulation results of previous section suggest that the distribution of eigen vectors, generated from system level channel, of some companies were not aligned with other companies. Besides, system level channel has not been calibrated by RAN4 to define demodulation performance requirements, including PMI related performance requirements.

Until Rel-18, RAN4 has only calibrated TDL based channel models for demod testing. RAN4 had a work item to calibrate CDL channel models for demod testing in Rel-19. At the end of that work item, RAN4 decided to specify PDSCH demod performance requirements with CDL channel model. However, RAN4 decided to further study if performance alignment can be achieved for PMI testing with CDL channel models. This has been captured through following texts in the WID of Rel-20 WI.

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

Performance requirements of two-sided CSF compression are similar to PMI tests. Hence, it can be concluded that CDL channels cannot yet be used for AI-ML based CSF compression tests in Rel-20.

Hence, the actual performance test of AI-ML based two-sided CSF compression can only happen in a link level TDL channel to avoid miscalibration issue.

Typically, RAN4 would select the same calibrated dataset for testing and defining performance requirement. However, system level channel is more realistic for field deployment. Due to field deployment consideration and option C/A’s dependence on RAN4, RAN4 should discuss whether to consider system level channel, in addition to TDL channels, to design the fully defined/specified reference model of “Direction 3” and the standardized encoder model structure of “Direction A, sub-option 3a-1”.

Observation 5: RAN4 has not calibrated system level channel for PMI testing.

Observation 6: RAN4 has calibrated rCDL channel models for PDSCH demodulation performance requirements. RAN4 is still studying if performance alignment in PMI reporting can be achieved with rCDL channels

Observation 7: The dataset used during performance tests should be included in the training dataset to design Dir C and Direction A, sub-option 3a-1.

Proposal 2: RAN4 selects TDL channel to define the performance requirements of two-sided CSI feedback mechanism of Rel-20.

Proposal 3: RAN4 discusses which of following channels to consider in the training data set to design the fully defined/specified reference model of “Direction C”.

- TDL based dataset

- Generalized dataset containing system level channel’s datasets plus TDL datasets.

## Performance Metric Selection for Normative Work

Different metrics can be considered to design the reference encoder/decoder of Dir C and define performance requirements. There metrics can be the following:

- SGCS

- Normalized MSE (NMSE)

- Ratio of throughput (The same metric as for the current PMI reporting requirements in 38.101-4)

RAN4 has used SGCS as the defining metric to derive the reference encoder and decoders during Rel-19. RAN4 can use reuse SGCS. Hence, RAN4 can reuse the steps that it went through during SI to align results across companies if SGCS is used as the performance metric to design the reference encoder/decoder.

RAN4 has always used the ratio of throughput as the metric to define performance requirements of PMI reporting. Although, SGCS can hint potential performance of an encoder-decoder pair, the real performance can only be observed through throughput study. Besides, SGCS based investigation does not allow us to observe the impact of practical issues like channel estimation error in performance.

Observation 8: RAN4 can reuse the steps that it went through during SI to align results across companies if SGCS is used as the performance metric to design the reference encoder/decoder of Dir C and Dir 3a-1.

Observation 9: RAN4 has always used ratio of throughput as the metric to define performance requirements of PMI reporting.

Observation 10: SGCS based investigation does not allow us to observe the impact of practical issues like channel estimation error in performance.

Proposal 4: RAN4 uses SGCS to design the reference encoder/decoder models of Dir C and Dir 3a-1.

Proposal 5: RAN4 uses, at least, the ratio of throughput (same metric as the current PMI reporting requirements in 38.101-4) to define the performance requirement of the standardized test decoder.

# Conclusions

Observation 1: Option 3 (fully specified decoder) leads to good SGCS in, at least, the scenario where encoder is trained with the same dataset as the decoder.

Observation 2: Option 4 leads to good SGCS between company’s own encoder and nominal decoders of all other companies when all encoders and decoders are generated based on Samsung’s reference encoder and mixed dataset.

Observation 3: Performance of option 4b can significantly drop if a different dataset is used while training the nominal decoder and the test decoder.

Observation 4: RAN4 would have to duplicate the effort if it designs different decoder structures for performance requirement and Direction C/Direction 3a-1.

Observation 5: RAN4 has not calibrated system level channel for PMI testing.

Observation 6: RAN4 has calibrated rCDL channel models for PDSCH demodulation performance requirements. RAN4 is still studying if performance alignment in PMI reporting can be achieved with rCDL channels

Observation 7: The dataset used during performance tests should be included in the training dataset to design Dir C and Direction A, sub-option 3a-1.

Observation 8: RAN4 can reuse the steps that it went through during SI to align results across companies if SGCS is used as the performance metric to design the reference encoder/decoder of Dir C and Dir 3a-1.

Observation 9: RAN4 has always used ratio of throughput as the metric to define performance requirements of PMI reporting.

Observation 10: SGCS based investigation does not allow us to observe the impact of practical issues like channel estimation error in performance.

Proposal 1: RAN4’s designed decoder for Direction C/Direction 3a-1 also gets used as the test decoder to define performance requirements.

Proposal 2: RAN4 selects TDL channel to define the performance requirements of two-sided CSI feedback mechanism of Rel-20.

Proposal 3: RAN4 discusses which of following channels to consider in the training data set to design the fully defined/specified reference model of “Direction C”.

- TDL based dataset

- Generalized dataset containing system level channel’s datasets plus TDL datasets.

Proposal 4: RAN4 uses SGCS to design the reference encoder/decoder models of Dir C and Dir 3a-1.

Proposal 5: RAN4 uses, at least, the ratio of throughput (same metric as the current PMI reporting requirements in 38.101-4) to define the performance requirement of the standardized test decoder.

# References

[1] R4-[111][133] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, Nokia,  3GPP TSG-RAN4 Meeting #112.

[3] R4-2413169, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[4] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112

[5] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis.

[6] R4-2415889, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112-bis

[7]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_113/Inbox/Drafts/%5B113%5D%5B129%5D%20NR_AIML_air_part1/CSI_Compression_Results_Summary/%5B113%5D%5B129%5DAIML_Summary_of_CSI_Compression_Sim_Results_v3_Nokia2_QC2.xlsx

[8]https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_113

[9] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Models/R4_114

[10] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression/Datasets/R4_114

[11] R4-2505107, WF on AI-ML for air interface study, 3GPP TSG WG RAN1 #114bis.

[12] R4-2508051, WF on AI/ML part 2, 3GPP TSG WG RAN1 #115.

[13] RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, 3GPP TSG RAN Meeting #108.

[14] RP-252957, WID: NR demodulation performance: Phase 6, 3GPP TSG RAN Meeting #109.