# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514466

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on parameters for AAS BS operating in bands below 1 GHz

Agenda Item:	10.4

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

ECC PT1 has submitted to TSG RAN and RAN4 a LS in R4-2513058 [1] on the topic of parameters for AAS BS operating in bands below 1 GHz. The work will be carried out in ECC PT1 and captured in new ECC Reports that would form the basis for the update of the regulatory technical conditions in ECC Decision (15)01 [2], ECC Decision (09)03 [3], and ECC Decision (06)13 [4].

ECC PT1 informs that it has started work towards authorisation of AAS BS in the following frequency bands:

- 694-791 MHz (700 MHz),

- 791-862 MHz (800 MHz),

- 880-960 MHz (900 MHz).

ECC PT1 expects that AAS BSs in the three bands under study would be deployed in existing sites and hence the deployment parameters would be similar to non-AAS. However, ECC PT1 does not have information about the technical characteristics of AAS BSs in these frequency bands.

ECC PT1 notes that it does not have the technical information required to conduct compatibility studies, and asks 3GPP for guidance on the following items:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration.

In this contribution we provide discussion on the questions received in LS from CEPT ECC PT1 (R4-2513058) on AAS BS operation in low frequency bands below 1 GHz.

# 2	Discussion

# 2.1	Unwanted emissions characteristics of AAS BS

RAN4 considered in Q4 2024 whether a low frequency boundary should be specified for the BS RF requirements specifications. Specifically, RAN4 considered introduction of a clarification note for AAS BS lower operating band frequency limit in the following form:

“Operating bands below [AAS_low_limit] MHz are not considered as supported by this specification. Operating bands below [AAS_low_limit] MHz are still applicable for protection requirements (i.e., Tx spurious co-location and co-existence, Rx blocking co-location).”

The topic was discussed and closed at RAN4#113, there was no agreement on a lower bound for applicability of the specifications of RF requirements for AAS BS and NR BS. Therefore, the RF requirements – in particular the OTA ACLR limits, the OTA operating band unwanted emissions limits, and the OTA spurious emissions limits – in TS 37.105, TS 37.145-2, TS 38.104, TS 38.141-2 are valid for 3GPP bands below 1 GHz.

Proposal 1: Clarify in the LS reply to CEPT ECC PT1, that the RF characteristics of AAS BS in operating bands below 1 GHz, in particular the unwanted emissions limits of AAS BS operating in the 700 MHz, 800 MHz and 900 MHz bands, are already captured since Rel-15 in AAS BS specifications (TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (TS 38.104, TS 38.141-1, TS 38.141-2).

# 2.2	Antenna model and parameters

3GPP has produced technical reports with AAS antenna models and parameters for different bands, for own use and on request from ITU WP5D:

- 3GPP TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects.

This TR contains in section 5.2.3.2.4 a model developed to support sub-array antenna geometries. The model was created to model AAS base station operating within the frequency range 1710 to 4990 MHz. Section 5.2.3.2.4 of this TR also contains a parameter set, to be used with the model, applicable to BSs operating in the 1710 to 4990 MHz range.

- 3GPP TR 38.922 NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz.

This TR was created in March 2025 in response to a request from WP5D and is the most recent report in 3GPP with an AAS model and parameters. The model equations (section 7.1.3), and the parameters for the 1710 to 4990 MHz range (section 4.4), are the same as in TR 38.803. TR38.922 also includes guidance on how to model the array antenna pattern outside the wanted channel.

It is expected that the architecture of AAS BS in operating bands below 1 GHz will be similar to the sub-array antenna geometry considered in the TRs above. Therefore, the model in section 7.1 of TR 38.922 could be used by ECC PT1 in their compatibility studies. Section 7.1.2 in TR 38.922 lists the input parameters required to utilize the model.

It is proposed to consider an antenna array architecture as shown in the figure 1 below and the associated parameters in the table 1 below:

Figure 1: Proposed antenna array model in 700/800/900 MHz operating bands

Table 1: Proposed set of parameters for an array model in 700/800/900 MHz operating bands

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 5.5 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | 2×4 |
| Horizontal/Vertical radiating sub-array or element spacing | 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

Proposal 2: Clarify in the LS reply to CEPT ECC PT1, that the antenna model in section 7.1 of TR 38.922 can be used in 700 MHz, 800 MHz and 900 MHz operating bands, with the following set of parameters to be used in the model:

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 5.5 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | 2×4 |
| Horizontal/Vertical radiating sub-array or element spacing | 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

# 2.3	Out of band characterisation

Section 7 of TR 38.922 provides analysis for modelling the antenna pattern that has support to model the array response outside the wanted carrier bandwidth, within adjacent channels using the correlation factor, r. The radiation pattern includes a correlation factor ρ that, for frequencies offset from the wanted signal, is in the interval 0≤ρ<1. The characterisation of the factor as a function of the frequency offset has been discussed in RAN4, as well as other technical bodies on multiple occasions in the past without conclusion. The out of band antenna gain modelling remains an open issue and is FFS. It is suggested that interested companies provide inputs on the correlation factor for the specific bands under study in CEPT ECC PT1. Beyond this, companies are encouraged to provide views on how to address the topic in 3GPP.

Proposal 3: Companies are encouraged to provide their views on the following items at the RAN4#117:

- Out of band array response for operation in the 700 MHz, 800 MHz and 900 MHz bands,

- Potential technical solution(s) to address the out of band antenna gain modelling for 700/800/900 MHz operating bands. Band agnostic solution are not precluded.

- Possible handling to formally capture the above technical work in RAN4 (e.g., SI/WI approach, RAN task, etc.).

- Envisioned timeline required.

# 2.4	Timeline

Referring to the LS in [1], CEPT ECC PT1 kindly invites 3GPP to provide a response on the above points for the next ECC PT1 meeting scheduled for 12-16 January 2026. It means, that RAN4 shall conclude on this topic by RAN4#117 (November 2025) meeting the latest. Therefore, the following is proposed:

Proposal 4: RAN4 to send LS reply to CEPT ECC PT1 on questions covered in R4-2513058 during RAN4#117 the latest.

# 3	Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: Clarify in the LS reply to CEPT ECC PT1, that the RF characteristics of AAS BS in operating bands below 1 GHz, in particular the unwanted emissions limits of AAS BS operating in the 700 MHz, 800 MHz and 900 MHz bands, are already captured since Rel-15 in AAS BS specifications (TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (TS 38.104, TS 38.141-1, TS 38.141-2).

Proposal 2: Clarify in the LS reply to CEPT ECC PT1, that the antenna model in section 7.1 of TR 38.922 can be used in 700 MHz, 800 MHz and 900 MHz operating bands, with the following set of parameters to be used in the model:

| Parameter | Value |
| --- | --- |
| Antenna pattern model | Extended AAS subarray model |
| Element gain (dBi) | 5.5 (with 2dB Ohmic loss) |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º polarized sub-array |
| Antenna array configuration (Row × Column) | 2×4 |
| Horizontal/Vertical radiating sub-array or element spacing | 0.5 of wavelength for H, 2.8 of wavelength for V |
| Number of element rows in sub-array | 4 |
| Vertical element separation in sub-array | 0.7 of wavelength for V |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Mechanical down-tilt (degrees) | 6 for urban/suburban, 3 for rural |
| Base station horizontal coverage range (degrees) | ±60 |
| Vertical steering range (degree) | 90-100 |

Proposal 3: Companies are encouraged to provide their views on the following items at the RAN4#117:

- Out of band array response for operation in the 700 MHz, 800 MHz and 900 MHz bands,

- Potential technical solution(s) to address the out of band antenna gain modelling for 700/800/900 MHz operating bands. Band agnostic solution are not precluded.

- Possible handling to formally capture the above technical work in RAN4 (e.g., SI/WI approach, RAN task, etc.).

- Envisioned timeline required.

Proposal 4: RAN4 to send LS reply to CEPT ECC PT1 on questions covered in R4-2513058 during RAN4#117 the latest.

Related Draft CR was provided in [5].

# 4	References

[1]	R4-2513058			Parameters for AAS BS operating in bands below 1 GHz, CEPT ECC PT1

[2]	ECC Decision (15)01 	Harmonised technical conditions for mobile/fixed communications networks (MFCN) in the band 694-790 MHz including a paired frequency arrangement (Frequency Division Duplex 2x30 MHz) and an optional unpaired frequency arrangement (Supplemental Downlink)

[3]	ECC Decision (09)03 	ECC/DEC/(09)03 of 30 October 2009 on harmonised conditions for Mobile/Fixed Communications Networks (MFCN) operating in the band 790-862 MHz

[4]	ECC Decision (06)13 	Harmonised technical conditions for mobile/fixed communications networks (MFCN) including terrestrial IMT systems, other than GSM and EC-GSM IoT, in the bands 880-915/925-960 MHz and 1710-1785/1805-1880 MHz

[5]	R4-2514467			Draft LS reply to CEPT ECC PT1 on AAS BS operation in low frequency bands below 1 GHz