# 3GPP TSG RAN WG4 Meeting#116bis	R4-2514480

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for 6Rx devices

Agenda item:	6.1.2

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for 6Rx devices including various simulation assumptions were discussed [1]. In this paper, we present our perspectives on the remaining open issues for defining demodulation and CSI reporting requirements.

2. General

2.1 Test applicability rule for 6Rx test cases

Test applicability rule for 6Rx tests

- Proposals

- Option 1: Introduce the test applicability rules by following core part agreement.

  - 6 MIMO layers performance requirements will be only applicable for FWA devices

  - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices.

- Option 1a: Introduce UE declaration for the proper test applicability design if no explicit capability signaling finally confirmed by RAN2

- Option 2: Postpone the discussion until RAN2 concludes the UE capability signaling discussion

Although the topic wasn’t discussed during the previous meeting, there was a proposal regarding the test applicability rules for the 6Rx test. The way forward is outlined below.

We acknowledge that Option 1 is generally well understood, and RAN4 is actively working on defining the corresponding demodulation perforamcne requirements. Specifically, the simulation assumptions for the 4L test cases remain under discussion. In our view, it is not necessary to differentiate by device type within the demodulation specification, as the relevant signaling has already been defined by RAN2 and subsequently approved by RAN plenary [2][3].

Observation 1: RAN2 has already defined relevant signaling for 6Rx-capable devices, which has subsequently been approved by the RAN Plenary.

Proposal 1: Do not introduce device-specific test applicability rules for 6Rx demodulation performance requirements.

2.2 Test applicability rule for different number of Rx antenna ports

Test applicability rule for different number Rx antenna ports

- Proposals

- Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage.

- Option 2: Postpone the discussion of test applicability rule until the relevant test cases are better defined

Similar to the test applicability rule for the 6Rx test, the topic concerning applicability rules for different numbers of Rx antenna ports was not discussed during the previous meeting. The way forward is outlined below.

In the scope of this WI, RAN4 is focused solely on defining requirements for 6Rx configurations. Since no other Rx configurations are being considered, we see no justification for introducing test applicability rules based on the number of Rx antenna ports for this WI.

Proposal 2: Do not introduce test applicability rules for varying numbers of Rx antenna ports in the context of 6Rx devices.

2.3 Specification structure

Specification structure

- Option 1: Add a new clause for 6Rx after the current 8Rx performance requirements

  - Section 5.2.5 6Rx requirement

- Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

  - Section 5.2.3 4Rx requirements

  - Section 5.2.4 6Rx requirements

  - Section 5.2.5 8Rx requirements

Several proposals were discussed regarding the inclusion of 6Rx demodulation requirements in the specification. The details of these proposals are outlined below.

To properly capture the 6Rx requirements, we think adding a new clause under Section 5.2 of the current RAN4 demodulation specification, for example, Section 5.2.5 for 6Rx Requirements. Therefore, we support option 1. On the other hand, option 2 will lead to many unnecessary changes throughout the specification as it changes the section numbering.

Proposal 3: Support option 1 (Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement)

3. PDSCH Requirements for 6Rx

3.1 Performance requirements for 4 Layers

3.1.1 Test cases and MIMO correlation for 4 MIMO layers

The MIMO correlation for 4 MIMO layers was also discussed during the last meeting. The way forward is outlined below.

MIMO correlation for 4 MIMO layers for Handheld UE testing

- Proposals

  - Option 1: One single test case that is applicable for both FWA and Handheld UE

    - Option 1a: α = 0 and β = 0.07179

    - Option 1b: α = 0 and β = 0

  - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

As previously agreed, RAN4 would consider low correlation for the 6L scenario, which applies specifically to FWA devices. However, we believe it is also feasible to extend this consideration to the 4L case, which is relevant to both handheld and FWA devices. Notably, the specification already includes test cases for 4L with low correlation, supporting this approach. Adopting a single test case for the 4L scenario would also help reduce overall test burden.

Proposal 4: Support option 1b (α = 0 and β = 0)

3.2 Performance requirements for 6 MIMO Layers for FWA

3.2.1 Test Applicability rule for FWA

Test applicability rule for FWA

- Proposals

- Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests

- Option 2: Not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests.

The way forward is outlined below.

We note that the 4L test case is applicable to both handheld and FWA devices. Therefore, we believe it is not feasible to establish a meaningful test applicability rule between the 6L FWA scenario and the 4L test cases.

Proposal 5: Support option 2 (Do not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests)

3.2.2 DMRS configuration for 6 MIMO layers case

DMRS configuration for 6 MIMO layers case

- Proposals

- Option 1: Rel-15 Type 1 DMRS 2+2

  - DMRS ports {1000-1003, 1004, 1006}

- Option 2 : Rel-18 eType1 DMRS 1+1

The DMRS configuration for 6 MIMO layers case was discussed during the last meeting. The way forward is outlined below.

As we have noted in our previous papers, Rel-18 eType1 DMRS is likely to become more prevalent in advanced 5G use cases due to its reduced overhead and better support for higher-order MIMO. Additionally, RAN4 has already introduced requirements for eType1 DMRS during the Rel-18 timeframe [4]. Additionally, based on the simulation results provided in our contribution for RAN4#114 [5], it is evident that Rel-18 eType1 DMRS offers comparable performance with just one frontloaded DMRS symbol compared to the Rel-15 2+0 case. It also provides robustness in high Doppler scenarios due to the additional DMRS symbol. While the Rel-15 2+2 DMRS configuration could be considered for 6Rx requirements, it is important to note that this configuration significantly increases the DMRS overhead, resulting in lower peak throughput. After evaluating all these options and analyzing their pros and cons, we believe that eType1 DMRS should be considered for 6-layer performance requirements for 6Rx devices.

Proposal 6: Support option 2 (Rel-18 eType1 DMRS 1+1)

4. SDR Requirements for 6Rx

4.1 SDR requirements for 1024QAM with 4 layers

SDR requirements for 1024QAM with 4 layers

- Proposals

- Option 1: Not Define SDR requirements for 1024QAM with 4 layers

- Option 2: Consider MCS23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers

During the last meeting, RAN4 briefly discussed whether to introduce SDR requirements for 1024QAM with 4 layers. The outlined way forward is as follows:

We note that the SDR test is intended to verify data rates that can be sustained reliably. However, with 1024QAM modulation, the SNR cap imposed by EVM limitations is too restrictive to support meaningful SDR testing with four layers. Moreover, RAN4 has already agreed to introduce SDR requirements for 1024QAM with two layers, which sufficiently covers the 1024QAM MCS table. In light of these considerations, we believe it is unnecessary to introduce an additional SDR test for 1024QAM with four layers.

Proposal 7: Support option 1 (Not define SDR requirements for 1024QAM with 4 layers).

4.2 SDR requirements

Based on our simulation results for SDR test with 256QAM and 64QAM MCS tables, we propose MCS25 and MCS26 for 6 layers, respectively.

Proposal 8: Consider the following MCS values for 6L SDR tests with 256QAM and 64QAM.

Table 1: MCS values for SDR tests with 6 layers

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 25 |
| 6 | 6 | 1 | 26 |

Based on our simulation results for SDR test with 1024QAM MCS table, we propose MCS23 for 2 layers.

Proposal 9: Consider the following MCS value for 2L SDR test with 1024QAM.

Table 2: MCS value for SDR tests with 2 layers

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 2 | 10 | 1 | 23 |

5. CSI Requirements for 6Rx

5.1 CQI test for 6Rx

CQI test for 6Rx

- Proposals

- Option 1: Not define CQI test

- Option 2: Only define one CQI requirements with rank 4 under static channel

During the last RAN4 meeting, the introduction of CQI reporting requirements for 6Rx was discussed. The way forward is as follows.

As we have previously stated in our papers, we believe that 6Rx can support higher CQI (and hence MCS levels), even at rank 4 when compared to 4Rx alone. This justifies the rationale for introducing CQI reporting requirements.

Proposal 10: Support option 2 (only define one CQI requirement with rank 4 under static channel).

5.2 RI test for 6Rx

RI test for only Rank 6

- Proposals

- Option 1: Not define RI test for 6 layers

- Option 2: Define RI test for 6 layers

During the last meeting, RAN4 discussed whether to introduce RI reporting requirements for 6Rx. However, no agreements were reached. The way forward is as follows.

We note that no RI tests with rank 6 were introduced for 8Rx WI during Rel-18. Additionally, the feasibility study for 6L handheld UE demonstrated that rank 6 is achievable even for handheld UE devices with 6Rx capability. Additionally, the specification currently includes RI test for 4L handheld UE. Therefore, it would be reasonable to consider introducing an RI test with rank 6 for 6Rx FWA UE, as omitting it will result in a coverage gap in CSI reporting requirements.

Observation 2: No RI test with rank 6 was introduced for 8Rx WI during Release 18.

Proposal 11: Support option 2 (Define RI test for 6 layers).

6. Conclusions

In this paper, we share our perspectives on the remaining open issues regarding test scopes, simulation assumptions and results for the 6Rx demodulation and CSI reporting requirements. The following proposals have been made.

Observation 1: RAN2 has already defined relevant signaling for 6Rx-capable devices, which has subsequently been approved by the RAN Plenary.

Proposal 1: Do not introduce device-specific test applicability rules for 6Rx demodulation performance requirements.

Proposal 2: Do not introduce test applicability rules for varying numbers of Rx antenna ports in the context of 6Rx devices.

Proposal 3: Support option 1 (Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement)

Proposal 4: Support option 1b (α = 0 and β = 0)

Proposal 5: Support option 2 (Do not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests)

Proposal 6: Support option 2 (Rel-18 eType1 DMRS 1+1)

Proposal 7: Support option 1 (Not define SDR requirements for 1024QAM with 4 layers).

Proposal 8: Consider the following MCS values for 6L SDR tests with 256QAM and 64QAM.

Table 1: MCS values for SDR tests with 6 layers

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 25 |
| 6 | 6 | 1 | 26 |

Proposal 9: Consider the following MCS value for 2L SDR test with 1024QAM.

Table 2: MCS value for SDR tests with 2 layers

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 2 | 10 | 1 | 23 |

Proposal 10: Support option 2 (only define one CQI requirement with rank 4 under static channel).

Observation 2: No RI test with rank 6 was introduced for 8Rx WI during Release 18.

Proposal 11: Support option 2 (Define RI test for 6 layers).

7. References

[1] R4-2512617, “Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx,” 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.

[2] R2-2506466, “Introduction of UE capability on 6 DL MIMO layers,” 3GPP TSG RAN WG2 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.

[3] R2-2506467, “Introduction of UE capability on 6 DL MIMO layers,” 3GPP TSG RAN WG2 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.

[4] TS 38.101-4, “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)”, v18.6.0, Jan 2025.

[5] R4-2500115, “Discussion on the demodulation performance requirements for 6Rx devices,” 3GPP TSG RAN WG4 Meeting #114, Athens, Greece, Feb 17 – 21, 2025.