# 3GPP TSG RAN WG4 Meeting#116bis	R4-2514485

# Prague, Czech Republic, Oct 13 – 17, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for RedCap and eRedCap UEs over NR-NTN

Agenda item:	6.19.4

Document for:	Discussion

# 1. Introduction

In the last previous meeting, demodulation performance requirements for RedCap and eRedCap devices were discussed [1]. This paper outlines our perspectives on the remaining open issues.

2. Applicability of time-varying Doppler shift and propagation delay model for NGSO Scenarios

Applicability of time-varying Doppler shift and time-varying channel model to NTN RedCap/eRedCap demodulation requirements.

Way forward:

- Postpone the discussion depending on WI NR_IoT_NTN_req_test_enh discussion.

During the last meeting, RAN4 briefly discussed the inclusion of the Rel-19 NGSO channel model in the context of RedCap and eRedCap UEs. The way forward is outlined below.

Compared to the static Rel-17 NTN channel model, the Rel-19 NTN model for NGSO offers a more realistic representation of satellite propagation, enabling UE testing under practical deployment conditions with time-varying Doppler shifts and propagation delays. Utilizing the Rel-19 NTN channel model for RedCap/eRedCap UEs helps prevent these devices from being evaluated under artificial test conditions associated with the Rel-17 model, which is based on fixed Doppler and propagation delay assumptions.

Observation 1: The Rel-19 NTN channel model for NGSO enables realistic UE testing by incorporating dynamic satellite propagation, thereby addressing the limitations of the Rel-17 model, which is constrained by assumptions of fixed Doppler shift and propagation delay.

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenarios.

3. Conclusions

In this paper, we present our perspectives on remaining issues for RedCap and eRedCap UEs over NR-NTN. The following proposals have been made.

Observation 1: The Rel-19 NTN channel model for NGSO enables realistic UE testing by incorporating dynamic satellite propagation, thereby addressing the limitations of the Rel-17 model, which is constrained by assumptions of fixed Doppler shift and propagation delay.

Proposal 1: Define demodulation performance requirements for Redcap/eRedCap UEs with Rel-19 NTN channel model in NGSO scenarios.

4. References

[1] R4-2512644, “Way Forward for [116][331] NR_NTN_Ph3_demod,” 3GPP TSG RAN WG4 Meeting #116, Bengaluru, India, Aug 25 – 29, 2025.