3GPP TSG-RAN WG4 Meeting #116bis	R4-2514488

Prague, Czech Republic, October 13th – 17th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM performance requirements for On-demand SIB1

Agenda item:	6.15.2.3

Document for:	Discussion

- 1. 	Introduction

Performance aspects for Rel-19 network energy saving were discussed in last meeting. In this contribution we continue discussing the test cases for On-demand SIB1 based on latest agreements and recommended WF [1]. Further discussion details can be found in the topic summary [2].

- Discussion

The following progress was achieved at RAN4 #116 on performance requirements for On-demand SIB1 [1].

- Sub-topic 3-2: OD-SIB1 performance part Issue 3-2-1: Test case - Background In RAN4 #115 meeting, Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.. - Proposals - Option 1: RAN4 not to define OD-SIB1 RRM performance test case. - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

At RAN4#115, the following has been agreed on RRM requirements for On-demand SIB1 [3].

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

Thus, a test case for verifying that the paging interruption time is not exceeded in case of OD-SIB1, is desirable to be defined.

In particular, a longer TSI-NR is appropriate since the SI acquisition takes longer in case of OD-SIB1. Additional components for the acquisition time versus legacy SIB1 are:

- Duration of UL WUS transmission to NES cell,

- Duration of RAR reception and message decoding,

- Duration of OD-SIB1 window.

The discussion related to component 3 is still ongoing in RAN1 (5-5120ms, multiple of 160ms, etc. vs. 160ms periodicity of legacy SIB1). Thus, the additional time for OD-SIB1 acquisition allowed in the test should take into account those three components. It can be defined once RAN1 has concluded on component 3.

Proposal 1:  	RAN4 to define a test case for verifying longer interruption in case of OD-SIB1 support in NES cell, taking into account additional delay from following components:

- Duration of UL WUS transmission to NES cell,

- Duration of RAR reception and message decoding,

- Duration of OD-SIB1 window.

However, no paging interruption tests exists in TS 38.133, neither for idle nor inactive mode. The only related test is the cell reselection test in idle mode. In fact, this test verifies the cell reselection delay which is highly correlated to the paging interruption time. Thus, we propose to modify this test for UEs supporting OD-SIB1 and specify a higher value for TSI_NR in all applicable cell reselection tests in idle mode.

Observation 1: No paging interruption test for idle or inactive mode exists in TS 38.133.

These test cases are specified in TS 38.133 Annex A.6.1.1 for FR1 and Annex A.7.1.1 for FR2 to verify the UE’s cell reselection behaviour. In the test cases, the cell reselection delay is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

TS 38.133: A.6.1.1.1.3 Test Requirements The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1, i.e. the 3 components depicted above. Hence the required time for cell reselection in test requirement needs to be extended. A modified test case is needed to verify the cell reselection behaviour to an OD-SIB1 cell.

Observation 2: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1.

Proposal 2: 	RAN4 to specify a new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell, rather than specifying a test case for paging interruption.

Conclusion

In this contribution, we discussed the RRM performance requirements for Rel-19 network energy saving for the feature On-demand SIB1.

The following observations and proposals are made.

Proposal 1: 	RAN4 to define a test case for verifying longer interruption in case of OD-SIB1 support in NES cell, taking into account additional delay from following components:

- Duration of UL WUS transmission to NES cell,

- Duration of RAR reception and message decoding,

- Duration of OD-SIB1 window.

Observation 1: No paging interruption test for idle or inactive mode exists in TS 38.133.

Observation 2: If UE is performing cell reselection to an OD-SIB1 cell, the cell reselection delay must additionally consider the time for requesting OD-SIB1.

Proposal 2: 	RAN4 to specify a new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell, rather than specifying a test case for paging interruption.

- References

[1]	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4 #116

[2]	R4-2509063, Topic summary for [116][220] Netw_Energy_NR_enh_Part1, Ericsson, RAN4 #116

[3]	R4-2508441, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei, RAN4 #115