3GPP TSG-WG4 Meeting #116bis	R4-2514494

# Prague, Czech Republic, 13-17 Oct 2025

# Title:	On Band n71 mandatory support for UL/DL 25MHz CBW

# Agenda Item:	4.2.8

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

In this contribution we present our views on how to introduce the mandatory support of the 25MHz channel bandwidth (CBW) for Band n71 based on RAN4 meeting 116 WF agreements [1].

- Discussion

# The portion of the WF [1] relevant to the Band n71 agreements are reproduced below. This contribution captures our views on the impact of mandating the 25MHz CBW on the Band n71 requirements and on the inter-band CA MSD requirements.

  - Impact on Band n71 requirements

# The agreed WF [1] clarifies that the use of 25MHz CBW is restricted to symmetrical UL/DL operation, meaning that a Rel-19 Band n71 UE supports the following single carrier configurations:

- Mandatory support of symmetrical UL/DL CBW for 5,10,15,20 and now 25MHz CBW;

- 25 MHz CBW is restricted to only symmetrical UL/DL operation, i.e. the UL 25MHz/DL 30MHz and UL 25MHz, DL 35MHz are not supported;

- UL 30MHz and UL 35MHz CBW remain optional, i.e. optional support of symmetrical UL/DL 30MHz and 35MHz CBW.

# There is therefore no need to change the asymmetric UL/DL CBW configuration requirements of Table 5.3.6-1.

# Proposal 1: Remove note 12 from Table 5.3.5-1 for 25MHz CBW as highlighted in yellow in the table below.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 25 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For 25MHz CBW, remove optional CBW REFSENS requirement and Note 10 from the REFSENS Table 7.3.2-1a, and remove Note 6 from the UL configuration requirement Table 7.3.2-3 as highlighted in yellow in the tables below.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -74.8 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -74.9 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Impact on Intra-band CA requirements

# There is no impact to the CA_n71B MSD requirements despite support for BCS4/5 since the agreed MSD test point already covers the worst-case emission scenario of single uplink carrier transmission configured with 30MHz CBW affecting the downlink (DL) 5MHz secondary component carrier (SCC). This is illustrated in Figure 1 which reproduces the Band n71 30MHz UL CBW power amplifier (PA) emissions from [2].

Figure 1: Measured PC3 1UL 30MHz CBW PA emissions at 27dBm output power for CA_n71B 30 MHz + 5 MHz and LCRB= 20RBstart=0 [2]. The 5MHz DL SCC is highlighted in red.

Observation: The MSD requirements for CA_n71B are not impacted by mandating support of symmetrical UL/DL 25MHz CBW.

  - Impact on Inter-band CA requirements

MSD requirements for CA_n12-n71, CA_n20-n71, CA_n26-n71, CA_n28-n71, CA_n29-n71 are not impacted due to BCS requirements restricting the Band n71 UL CBW to a maximum of 20MHz CBW. However, MSD may need to be re-evaluated for:

- CA_n5A-n71A, where BCS4/5 is supported, and the Band n5 PC3 MSD is specified for n71 20MHz UL CBW.

- CA_n71A-n85A: BCS4/5 is supported, and the Band n85 PC3 and PC2 MSD is specified for n71 20MHz UL CBW.

CA_n5A-n71A:

The DL band n5 is affected by “far away” flat noise power spectral density due to the large frequency distance that separates the Band n71 UL carrier from the Band n5 closest DL carrier. It can therefore be assumed that the MSD for n71 20MHz UL CBW also applies for 25MHz UL CBW.

Since 25MHz becomes a new greatest CBW for Band n71, we propose to follow the RAN4 PRD rules, i.e.:

- The test point for Band n71 25MHz UL CBW becomes the so-called “1st mandatory” MSD test point

- The test point for Band n71 20MHz UL CBW may remained specified to become the so-called “2nd optional” MSD test point. However, since the MSD does not change, we do not see to specify this 2nd test point.

Proposal 3: In Table 7.3A.6-1 and for CA_n5-n71, consider replacing the Band n71 20MHz UL CBW MSD test point with the changes highlighted in yellow for the Band n5 PC3 MSD. Due to BCS4/5, attach Note 6 to this requirement.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.06 | >ACLR2 |
| NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

CA_n71A-n85A:

Figure 2 shows the measured Band n71 PA output noise level affecting the DL Band n85 5MHz CBW SCC with the following test conditions:

- The Band n71 PA PC3 linearity calibrated to meet -30dBc ACLR at the PC3 MPR1, using DFT-s-OFDM QPSK 20MHz 100RBstart=0 SCS15kHz waveform,

- The PA output noise is measured at the PC3 MPR0 assuming 4dB post PA loss and DFT-s-OFDM QPSK 25MHz 20RBstart.

Figure 2: Measured PC3 1UL 25MHz CBW PA emissions at 27dBm output power for CA_n71A-n85A and LCRB= 20RBstart=113. The 5MHz DL SCC is highlighted in red.

Table 1 summarizes the Band n85 5MHz CBW MSD and:

- For Band n71 25MHz UL CBW, the MSD is 7.4dB, i.e. 0.8dB smaller than the agreed MSD specified for n71 20MHz UL CBW;

- For Band n71 20MHz UL CBW, the MSD is 6.2dB.

Since the Band n71 25MHz UL CBW measured PA noise levels measurements lead to smaller MSD requirements than those previously agreed for 20MHz UL CBW, we propose to specify the Band n85 5MHz CBW MSD by applying the difference of MSD between 20MHz and 25MHz as reported in Table 1.

Table 1: post MRC calculations for cross band MSD in n85 DL due to 20MHz and 25MHz n71 UL.

| Band n71 UL CBW / LCRB | [MHz]/# | 20 / 20(RBstart= 86) |  | 25 / 20(RBstart= 113) |  |
| --- | --- | --- | --- | --- | --- |
| Meas Tx noise in n85 5MHz SCC | [dBm/CBW] | -47.2 |  | -45.3 |  |
| 5MHz CBW scaled REFSENS | [dBm/CBW] | -97.0 |  | -97.0 |  |
| RX noise floor wo Tx noise: Main/Div | [dBm/CBW] | -93.0 | -93.0 | -93.0 | -93.0 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 42 | 37 | 42 | 37 |
| LNA to antenna insertion loss | [dB] | 4 |  | 4 |  |
| Antenna isolation | [dB] | 0 | 10 | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -85.2 | -94.2 | -83.3 | -92.3 |
| REFSENS degradation after MRC | cor [dB] | 6.2 |  | 7.4 |  |

Based on RAN4 PRD rules, we propose that:

- The test point for Band n71 25MHz UL CBW becomes the so-called “1st mandatory” MSD test point

- The test point for Band n71 35MHz UL CBW remains the so-called “2nd optional” MSD test point.

Proposal 4: In Table 7.3A.6-1 and for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 PC3 MSD due to Band n71 25MHz UL CBW cross-band isolation interference.

| UL band | DL band | UL Fc |  | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) |  | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 |  | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 9.46 | ACLR2 |
| n71 | n85 | 680.5 |  | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |
|  | NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

The PC21Tx and the PC22Tx MSD requirements are calculated using the HPUE MSD simplification Look-Up Tables, i.e.

- PC21Tx MSD = PC3 MSD + 3dB = 12.4dB

- PC22Tx MSD = PC3 MSD + 6dB = 15.4dB.

The 15.4dB PC21Tx MSD for n71 25MHz CBW is smaller than the previously MSD of 15.9dB agreed for n71 20MHz UL CBW. We believe 15.4dB is acceptable considering the small difference in noise rise between n71 UL 20MHz and UL 25MHz CBW.

It should be noted that the difference between the PC22Tx and the PC21Tx MSD for n71 35MHz UL CBW is greater than 3dB and may need to be corrected to account for MSD LUT simplification.

Proposal 5: In Table 7.3A.6-1a-1 and for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 12.44,6 15.44,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

Proposal 6: In Table 7.3A.6-1a-1 and for CA_n71-n85, consider correcting the Band n85 5MHz CBW MSD due to band n71 35MHz UL CBW PC22Tx cross-band isolation interference to 29dB.

- Conclusion

This contribution proposes the following changes to introduce the mandatory support of 25MHz CBW for Band n71.

# Proposal 1: Remove note 12 from Table 5.3.5-1 for 25MHz CBW as highlighted in yellow in the table below.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n71 | 15 |  | 5 | 10 | 15 | 20 | 25 | 3012 | 3512 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 3012 | 3512 |  |  |  |  |  |  |  |  |
| NOTE 12: This UE channel Bandwidth is optional for uplink in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: For 25MHz CBW, remove optional CBW REFSENS requirement and Note 10 from the REFSENS Table 7.3.2-1a, and remove Note 6 from the UL configuration requirement Table 7.3.2-3 as highlighted in yellow in the tables below.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -74.8 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -74.9 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n71 | 15 |  | 25 | 251 | 201 | 201 | 201 | 201,6 | 201,6 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 121 | 101 | 101 | 101 | 101,6 | 101,6 |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 6: UEs supporting the optional symmetrical UL/DL bandwidths shall use this UL configuration. For UEs not supporting this uplink channel bandwidth, the UL configuration of the 20MHz UL channel bandwidth and the nominal Tx-Rx frequency separation specified in Table 5.4.4-1 shall be used, i.e. FTX-RX as defined in clause 5.3.6 does not apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation: The MSD requirements for CA_n71B are not impacted by mandating support of symmetrical UL/DL 25MHz CBW.

Proposal 3: In Table 7.3A.6-1 and for CA_n5-n71, consider replacing the Band n71 20MHz UL CBW MSD test point with the changes highlighted in yellow for the Band n5 PC3 MSD. Due to BCS4/5, attach Note 6 to this requirement.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 685.5 | 25 | 15 | 20 (RBstart=113) | 871.5 | 5 | 2.06 | >ACLR2 |
| NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

Proposal 4: In Table 7.3A.6-1 and for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 PC3 MSD due to Band n71 25MHz UL CBW cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 9.46 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 237 | ACLR1 |
| NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 7: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

Proposal 5: In Table 7.3A.6-1a-1 and for CA_n71-n85, consider adopting the changes highlighted in yellow for the Band n85 MSD due to Band n71 25MHz UL CBW PC2 cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 685.5 | 25 | 15 | 20 (RBstart=113) | 730.5 | 5 | 12.44,6 15.44,7 | ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 265,6 32.35,7 | ACLR1 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

Proposal 6: In Table 7.3A.6-1a-1 and for CA_n71-n85, consider correcting the Band n85 5MHz CBW MSD due to Band n71 35MHz UL CBW PC22Tx cross-band isolation interference to 29dB.

# References

- R4-2511779 WF on optional channel bandwidths, 3GPP TSG-RAN WG4 Meeting # 116, Bangaluru, India, Skyworks Solutions, Inc.