3GPP TSG-WG4 Meeting #116bis	R4-2514495

# Prague, Czech Republic, 13-17 Oct. 2025

# Title:	Missing SUL PC2 MSD and MSD simplification for SUL HPUE

# Agenda Item:	4.6.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution is a companion to the CR [1] as a follow up to implement the corrections proposed in [2,3,4,5].

- Discussion

# At RAN4 meeting 114, it was confirmed in reply to the questions raised in [5] that in the case where HPUE is a valid SUL band power class, one uplink (1UL) HPUE operation is a valid configuration for SUL operation with downlink CA. Hence several HPUE 1UL MSD requirements are missing in TS 38.101. We proposed several corrections in CR [4] based on analyses of [3]. It was also suggested during meeting 114  that the NR-CA HPUE MSD simplification look-up tables (LUT) [6] could be ported to SUL when these changes are agreed upon.

# The CR [1] collects all these inputs.

# For HPUE MSD simplification using LUTs, only the requirements related to 1UL MSD requirements need to be ported. To ensure future proofing, and even though PC1.5 is not yet supported by any SUL band, we propose the following:

- Introduce for SUL the LUTs which enable calculating the PC1.5 MSD from a PC3 MSD requirement

- In the case where:

  - No PC3 MSD requirement is specified, but

  - PC2 MSD is needed or required,

  - And PC1.5 becomes a valid SUL power class,

  - Introduce for UL the LUT requirements which enable calculating the PC1.5 MSD from a PC2 MSD requirement. To enable this special case, it is necessary to introduce new PC2 MSD requirement using the legacy MSD table format.

# Proposal: In clause 7.3C.2

- Introduce missing PC2 MSD requirements for harmonic, cross band isolation and harmonic mixing in new Table 7.3C.2-2a, Table 7.3C.2-4a and Table 7.3C.2-6a respectively based on [2,3,4],

- Introduce 1UL HPUE MSD requirements with LUTs in a new sub-clause 7.3C.2.1 based on [6].

- Conclusion

In this contribution we propose to introduce the missing HPUE MSD requirements for SUL and to port the NRCA HPUE MSD requirements with look-up tables.

# Proposal: In clause 7.3C.2

- Introduce missing PC2 MSD requirements for harmonic, cross band isolation and harmonic mixing in new Table 7.3C.2-2a, Table 7.3C.2-4a and Table 7.3C.2-6a respectively based on [2,3,4];

- Introduce 1UL HPUE MSD requirements with LUTs in a new sub-clause 7.3C.2.1 based on [6].

# References

- R4-2514492 (HPUE_NR_CADC_SUL_R19) Cat B CR on SUL missing MSD and HPUE MSD requirements simplification, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Skyworks Solutions, Inc.

- R4-2511628 SUL REFSENS and missing PC2 MSD, 3GPP TSG-RAN WG4 Meeting # 116, Bengaluru, India, Skyworks Solutions, Inc.

- R4-2502172 HPUE MSD for SUL, 3GPP TSG RAN Meeting #114, Athens, Greece, Skyworks Solutions, Inc.

- R4-2501933 (HPUE_NR_CADC_SUL_R19) Cat F CR for TS 38.101-1-j00 SUL PC2 MSD requirements, 3GPP TSG-RAN WG4 Meeting # 114, Athens, Greece, Skyworks Solutions, Inc

- R4-2419721 Impact of SUL HPUE on MSD, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2511752 [NR_ENDC_RF_Ph4-Core] Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification, RAN4 #116, Bengaluru, India, Skyworks Solutions, Inc., Nokia, T-Mobile USA, ZTE, Oppo, Samsung, Murata Manufacturing Co Ltd., MediaTek, Vivo, Huawei, Apple, Qualcomm France.