3GPP TSG-RAN WG4 Meeting RAN4#116bis      				R4-2514502 Prague, CZ

13th – 17th October 2025

Agenda Item:    5.11.4

Source:	Lenovo, Motorola Mobility

Title:	Handling of Power and PSD Limits for the NR 4940-4990 MHz Band for US Public Safety

Document for:  Approval

- Introduction

In RAN #108, work item [1] was agreed with the objective of defining a new 5G NR TDD operating band in the frequency range of 4940-4990 MHz in the U.S. to allow for 5G public safety operation in line with the framework identified in Amendment of Part 90 of the Commission’s Rules in FCC 24-114 WP Docket No. 07-100 [2].

Included in the rules in [2] are limits on the conducted power as a function of the carrier bandwidth as well as limitations on the power spectral density of the transmission.  These bandwidth dependent power limits and power spectral density limits are not currently addressed in the 3GPP specification.  In this contribution, we propose methods for handling these new requirements.

- PSD Limit and Bandwidth Dependent Power Limits

In Part 90, SubPart Y, § 90.1215, bandwidth dependent power limits are specified in the table below (a) (1) for both high power devices and low power devices.  It can be noted that these are conducted power limits.

§ 90.1215 Power limits

Except as provided in paragraph (f) of this section, the transmitting power of stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this section.

- (1) For base, mobile, and temporary fixed operations, the maximum conducted output power must not exceed:

| Table 1 to Paragraph (a)(1) |  |  |
| --- | --- | --- |
| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

Table 1:  Bandwidth dependent power limits for low power and high power devices.

There are also separate power spectral density limitations for high and low power devices.  For high power devices, the power spectral density limitation is given in Section (a) (2) as follows:

- (2) 	High power devices are also limited to a peak power spectral density of 21 dBm per one MHz. High power devices using channel bandwidths other than those listed above are permitted; however, they are limited to peak power spectral density of 21 dBm/MHz.

The limitation for low power devices is given in Section (b) as follows:

- Low power devices are also limited to a peak power spectral density of 8 dBm per one MHz. Low power devices using channel bandwidths other than those listed above are permitted; however, they are limited to a peak power spectral density of 8 dBm/MHz.

Finally, it can be noted that the power spectral density limits are also conductive as stated in Section (e):

(e) 	The peak power spectral density is measured as conducted emission by direct connection of a calibrated test instrument to the equipment under test.

It can be noted that the bandwidth dependent power classes are not aligned with the 3GPP power classes of 23, 26, 29, and 31 dBm for PC3, PC2, PC1.5 and PC1, respectively, so that the bandwidth dependent power limit may need to be included into the upper and lower bounds for PCMAX,f,c. In general, the bandwidth dependent power limit in Table 1 can be expressed as

FCC4.9BW=20+ 10 log10BW  dBm             	                      	(1)

where FCC4.9BW denotes the FCC bandwidth dependent power limit and the channel bandwidth is expressed in MHz.  Thus, for a PC3 device, the bandwidth dependent power limit is not a concern for any bandwidth greater than 2 MHz.  The bandwidth at which the FCC power limit is equal to PPowerClass is shown in Table 2 below. At bandwidths less than this, the FCC bandwidth dependent power limit is less than the PPowerClass and must be included in the PCMAX,f,c upper bound.  This can be accomplished either by adding the FCC limit as an additional term to PCMAX,f,c or signaling this limit in PEMAX,c.

| Powerclass | Power (dBm) | Bandwidth below which FCC power limit is less than PPowerClass |
| --- | --- | --- |
| PC3 | 23 | 2 MHz |
| PC2 | 26 | 4 MHz |
| PC1.5 | 29 | 8 MHz |
| PC1 | 31 | 12.6 MHz |

Table 2:  Bandwidth below which FCC bandwidth dependent power limit is less than PPowerClass.

For a device with power greater than 21 dBm, the maximum power spectral density can be exceeded if the RB allocation has bandwidth is less than 1 MHz.  In general, the number of RB’s that can be transmitted without an additional power restriction depends on the UE powerclass, the number of RB’s, and the subcarrier spacing. The transmission bandwidth BWT corresponding to a contiguous RB allocation depends on the number of RB’s, LRB, and the subcarrier spacing in kHz (SCS), and is given by

BWT=12*SCS*LRB1000 .                                                                            (2)

For this bandwidth, the maximum power that can be transmitted without violating the 21dBm/MHz PSD limit is given by

PmaxPSD=max10 log1012*SCS*LRB1000, 0+21 			(3)

where the subscript max(PSD) denotes that the power limit PmaxPSD is due to the power spectral density limit.  This equation assumes that the measurement bandwidth is equal to 1 MHz, and that the measurement interval is placed such that

- If BWT≤1MHz, the transmission bandwidth is entirely contained within the measurement interval.

- If BWT>1MHz, the measurement interval is entirely contained within the transmission bandwidth.

The measurement assumption is captured in Figure 1 below.

Figure 1:  Placement of 1 MHz Measurement Bandwidth Relative to Transmission Bandwidth BWT.

From Equation (3), we can determine the threshold number of RB’s, LRB,t at or below which the PSD limit of 21 dBm/MHz is exceeded using

Ppowerclass=max10 log1012*SCS*LRB,t1000, 0+21 ,

from which we have

LRB,t=10Ppowerclass+91012*SCS  .                                                                      (4)

The number of RB’s at or below which the PSD limit of 21 dBm/MHz can be exceeded at maximum power is shown in Table 3 below.

|  |  | Number of RB’s LRB,t at or below which the 21 dBm/MHz PSD limit is exceeded at full power |  |  |
| --- | --- | --- | --- | --- |
| Powerclass | Power (dBm) | 15 kHz | 30 kHz | 60 kHz |
| PC3 | 23 | 8 | 4 | 2 |
| PC2 | 26 | 17 | 8 | 4 |
| PC1.5 | 29 | 35 | 17 | 8 |
| PC1 | 31 | 55 | 27 | 13 |

Table 3:  Number of RB’s at or below which the power spectral density can exceed 21 dBm/MHz as a function of PPowerClass and subcarrier spacing.

- Implications of FCC 4.9 GHz Power Limits on PCMAX,f,c

As discussed above, the FCC regulations for 4940 – 4990 MHz add two new types of power limits.  For high power devices, the limits are:

- A bandwidth dependent maximum power limit given by

FCC4.9BW=20+ 10 log10BW  (dBm).

- A power spectral density limit of 21 dBm/MHz given by

PmaxPSD=max10 log1012*SCS*LRB1000, 0+21 .

To avoid exceeding these power limits, some modifications of the PCMAX,f,c equation is necessary to reflect these requirements.

The equations for the configured maximum power for a single carrier are given in Section 6.2.4 of TS 38.101-1.  For a given RB allocation, the UE is allowed to configure its maximum configured power PCMAX,f,c between a lower bound PCMAX_L,f,c and an upper bound PCMAX_H,f,c where the upper bound and lower bound  are defined below.

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

As noted above, the power spectral density limit of 21 dBm/MHz can be converted to a power limit as

PmaxPSD=max10 log1012*SCS*LRB1000, 0+21

where the subscript maxPSD denotes that the power limit PmaxPSD is due to the power spectral density limit.  This term can be added to both the upper and lower bounds for the configured maximum power as in the following proposal.

Proposal 1: 	For the 4.9 GHz band, modify the upper and lower bounds for PCMAX,f,c to include the 21 dBm/MHz power spectral density limit as in the following:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc),  PmaxPSD}

and

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass,  PmaxPSD}

The bandwidth dependent FCC power limit FCC4.9BW can be incorporated into the PCMAX,f,c upper and lower bounds in one of two ways.  The first method is simply to require that for the 4.9 GHz band that maximum allowed UE output power signalled by higher layers for serving cell c is less than or equal to the bandwidth dependent power limit, so that

PEMAX,c ≤FCC4.9BW .

From this, we have the following proposal.

Proposal 2a: 	For the 4.9 GHz band, require that the maximum allowed UE output power signalled by higher layers for serving cell c is less than the bandwidth dependent power limit, so that

PEMAX,c ≤FCC4.9BW .

The other alternative is to include the bandwidth dependent power limit directly in the upper and lower bounds for the configured maximum power in the same manner as for the power spectral density based limit as in the proposal below.

Proposal 2b: 	For the 4.9 GHz band, modify the upper and lower bounds for PCMAX,f,c to include the FCC bandwidth dependent power limit FCC4.9BW as in the following:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc),  PmaxPSD, FCC4.9BW}

and

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass,  PmaxPSD,   FCC4.9BW}

#### Summary

As discussed above, the FCC regulations for 4940 – 4990 MHz add two new types of power limits.  For high power devices, the limits are:

- A bandwidth dependent maximum power limit given by

FCC4.9BW=20+ 10 log10BW  (dBm).

- A power spectral density limit of 21 dBm/MHz given by

PmaxPSD=max10 log1012*SCS*LRB1000, 0+21 .

To avoid exceeding these power limits, some modifications of the PCMAX,f,c equation is necessary to reflect these requirements.

The power spectral density requirement of 21 dBm/MHz in (ii) can be incorporated into the PCMAX,f,c equation using Proposal 1.

Proposal 1: 	For the 4.9 GHz band, modify the upper and lower bounds for PCMAX,f,c to include the 21 dBm/MHz power spectral density limit as in the following

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc),  PmaxPSD}

and

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass,  PmaxPSD}

It is proposed that the bandwidth dependent power limit be incorporated into the PCMAX,f,c equation using either Proposal 2a or Proposal 2b.

Proposal 2a: 	For the 4.9 GHz band, require that the maximum allowed UE output power signalled by higher layers for serving cell c is less than the bandwidth dependent power limit, so that

PEMAX,c ≤FCC4.9BW .

Proposal 2b: 	For the 4.9 GHz band, modify the upper and lower bounds for PCMAX,f,c to include the FCC bandwidth dependent power limit FCC4.9BW as in the following:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc),  PmaxPSD, FCC4.9BW}

and

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass,  PmaxPSD,   FCC4.9BW}

References

- RP-251887 “New WID on Introduction of NR TDD 4.9GHz Band for US Operation,” 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 09-13, 2025.

- FCC’s Eighth Report and Order, Docket 07-100 adopted October 18, 2024, FCC-24-114A1.

- FCC Universal Licensing Database, Radio Service Codes PF (Fixed Services) and PB (Base/Mobile and Temporary Fixed Operation)  License Search - Advanced License Search.

- FCC’s Seventh Report and Order, Docket 07-100 adopted January 18, 2023, FCC-23-3A1.