3GPP TSG-RAN WG4 Meeting #116-bis								                        R4-2514503

Prague, Czech Republic, October 13 – 17, 2025

Agenda item:			7.6.3

Source:	Ericsson

Title:	On HPUE for NTN UE RF requirements

Document for:	Approval

# Introduction

In RAN#109, a new WID has been approved [1] for “Enhanced requirements for NR NTN and IoT NTN Phase 2” WI. The part on HPUE for NTN (PC1.5) has the following objectives:

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

In the following, we will provide our initial views on the objective above.

# Discussion

As stated in the WID justification part, in Rel-19 the high power classes PC2 and PC1 were introduced for both NR-NTN and IoT-NTN. PC1.5 was also considered but due to a lack of sufficient number of MPR/A-MPR evaluations it was agreed to preclude it from Rel-19.

Initially the intention of this WI was to specify PC1.5 for both NR-NTN and IoT-NTN, but it was later decided to merge the discussion on introducing PC1.5 for NR-NTN with the introduction of PC1.5 for 2 Tx for FDD bands for TN or PC1.5 1Tx for TDD bands for FWA UE.

Hence, in this WI, the only objective is to introduce PC1.5 for NB-IoT based IoT-NTN for the single uplink (UL) scenario.

In TN, no high power classes are defined for NB-IoT, so no requirement could be reused from TN, which is the same case as it was for PC2 and PC1 in Rel-19.

Unlike PC1.5 for NR-NTN, in the Rel-19 WI PC1.5 for IoT-NTN wasn’t even considered since PC1.5 is not specified in LTE in general, so no results could be reused.

We have identified that the following requirements will have an impact by introducing PC1.5 for NB-IoT based IoT-NTN:

- UE maximum output power including tolerances

- MPR for both single-tone and multi-tone

- A-MPR for NS_01, NS_24 and NS_02N

- Configured maximum output power: discuss whether a power boosting capability due to a lack of duplexer and thus the reduced insertion loss could be extended to PC1.5

- Additional Spectrum Emission Mask for category NB1 and NB2 for NS_02N

Regarding the ACLR requirements, no updates are expected since for PC2 and PC1 it was agreed to reuse PC3 requirements (37 dB).

Regarding the reference sensitivity degradation requirements, there is no need to define such requirements since NB-IoT based IoT-NTN devices operate in HD-FDD mode.

Proposal 1: The following requirements will have an impact by introducing PC1.5 for NB-IoT based IoT-NTN:

- UE maximum output power including tolerances

- MPR for both single-tone and multi-tone

- A-MPR for NS_01, NS_24 and NS_02N

- Configured maximum output power: discuss whether a power boosting capability due to a lack of duplexer and thus the reduced insertion loss could be extended to PC1.5

- Additional Spectrum Emission Mask for category NB1 and NB2 for NS_02N

Observation: No updates are expected for the ACLR requirements since for PC2 and PC1 it was agreed to reuse the PC3 requirements (37 dB).

Proposal 2: There is no need to define the RSD requirements since NB-IoT based IoT-NTN devices operate in HD-FDD mode.

# Conclusion

In the previous sections, we made the following observation:

Observation: No updates are expected for the ACLR requirements since for PC2 and PC1 it was agreed to reuse the PC3 requirements (37 dB).

Based on the discussion in the previous sections we propose the following:

Proposal 1: The following requirements will have an impact by introducing PC1.5 for NB-IoT based IoT-NTN:

- UE maximum output power including tolerances

- MPR for both single-tone and multi-tone

- A-MPR for NS_01, NS_24 and NS_02N

- Configured maximum output power: discuss whether a power boosting capability due to a lack of duplexer and thus the reduced insertion loss could be extended to PC1.5

- Additional Spectrum Emission Mask for category NB1 and NB2 for NS_02N

Proposal 2: There is no need to define the RSD requirements since NB-IoT based IoT-NTN devices operate in HD-FDD mode.

# References

- RP-252962, “New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2”, RAN#109.