3GPP TSG-RAN WG4 Meeting #116bis	R4-2514504

Prague, CZ, 13th – 17th October 2025

Source: 	Lenovo, Motorola Mobility

Title: 	RSD Values for PC2 with FDD Band n30

Agenda Item:	5.5.3

Document for:	Discussion

# Introduction

In this contribution, we propose RSD values for band n30 PC2 with and without transmit diversity. For 2 Tx transmit diversity, the proposed values are based on comparisons with bands having similar duplex spacing for which RSD has already been specified. Additionally, PC2 RSD was measured for a single Rx chain and these measurements were used to infer RSD for a dual Rx chain receiver. Based on these methods, we propose PC2 RSD values for band n30 of 0.5 dB for 2 Tx transmit diversity and 0.3 dB for a single Tx transmitter.

# Discussion

The n30 band is comprised of 10 MHz of paired spectrum having a duplex gap of 35 MHz and duplex spacing of 45 MHz. The allowed channel bandwidths are 5 and 10 MHz for 15 kHz SCS and 10 MHz for 30 kHz SCS. The uplink configuration for sensitivity measurements is 20 RBs for both 5 and 10 MHz channels with 15 kHz SCS and 10 RB for the 10 MHz channel with 30kHz SCS. In each case, the uplink RBs are placed as close as possible to the Rx channel. The situation for band n30 is similar to that for band n14 which is comprised of 10 MHz of paired spectrum having a duplex gap of 20 MHz and a duplex spacing of 30 MHz and for which 5 and 10 MHz bandwidth channels are allowed. For band n14, the uplink configuration for reference sensitivity measurements is the same as for n30 with 20 RB’s used for both 5 and 10 MHz channels with 15 kHz SCS and 10 RBs used for the 5 MHz channel with 30 kHz SCS. Similarly, as for band n30, in each case the uplink RBs are placed as close as possible to the Rx channel.

The PC2 RSD values for band n14 have been in TS 38.101-1 since Release 18.5.0 and the values for single Tx and dual Tx can be found in tables 7.3.2-1c and 7.3.2-1d, respectively.  Because the duplex gap and duplex spacing are each 10 MHz larger for band n30 than for n14, the RSD values for band n30 should be less than or equal to the values for n14.

For the 10 MHz channel with 15kHz SCS, the occupied bandwidth of the 20 RB uplink channel allocation used for Refsens is only 3.6 MHz.  The bandwidth of the 7th order IM corresponding to this 20 RB allocation is 25.2 MHz. As a result, the 7th order IM extends only 10.8 MHz outside the uplink channel and is thus still 24.2 MHz from the downlink channel (= 35 – (25.2/2 – 3.6/2)). Since IM’s having order greater than 7 are generally considered to be negligible, they are not expected to be a significant contributor to receiver desense for this band.  Instead, the dominant contributor is expected to be thermal noise from the modulator and driver stages amplified by the PA.

a

A recent measurement of n30 receive sensitivity on one of our devices using the 3GPP defined measurement channels and configuration but with a single primary Rx showed sensitivity degradation of only 0.1 dB at maximum transmit power relative to minimal transmit power (≤ 0 dBm) at nominal temperature.  Additionally, increasing the uplink channel allocation did not result in any additional increase in receiver sensitivity degradation with the increased transmit power. This further validates the assumption that IMD products are insignificant contributors to receive desense for this band.

A reasonable range of desense values for a single Rx chain is in the range of 0.1 dB and 1 dB as the transmitter power increases from minimum to maximum. Using an expected sensitivity of -98.5 dBm for a single Rx 10 MHz channel receiver with minimal transmit power and 0.1 dB desense, a straightforward calculation of Tx noise referred to the input is calculated to be -113.8 dBm. Receiver sensitivity for various combinations of single and dual Tx with both single Rx and dual Rx with maximum ratio combining are evaluated assuming similar radio front end architectures. The results are summarized in Table 1 below.

For a single chain PC2 implementation that has negligible IMD contribution as assumed in the case of n30, the Rx band noise was increased by 3 dB compared to the PC3 case. The secondary receiver is assumed to have negligible additional degradation compared to the PC3 case. In a similar manner, receiver sensitivity was calculated for the dual Rx MRC receiver for a dual chain PC2 transmitter. The results are included in the table.

|  | Results using 0.1 dB sensitivity degradation at max power on primary TRx | Results using 1.0 dB sensitivity degradation at max power on primary TRx |
| --- | --- | --- |
| Single Rx sens, low power (dBm) | -98.5 | -98.5 |
| (a) Dual Rx MRC combined sens, low power (dBm) | -101.51 | -101.51 |
| Single Rx sens, PC3 max power (dBm) | -98.4 | -97.5 |
| Single Rx sens, PC2 max power (dBm) | -98.3 | -96.7 |
| (b) Dual Rx, MRC combined sens, PC3 max power (dBm) | -101.46 | -101.04 |
| (c) Dual Rx, MRC combined sens, PC2 max power TxD (dBm) | -101.41 | -100.51 |
| (d) Dual Rx, MRC combined sens, PC2 max power single chain (dBm) | -101.41 | -100.7 |
| (c) – (b) RSD PC2 TxD sensitivity degradation due to Tx (dB) | 0.05 | 0.53 |
| (d) – (b) RSD PC2 single chain sensitivity degradation due to Tx (dB) | 0.05 | 0.34 |

Table 1: Calculation of RSD for Single and Dual Tx

For the case in which the single Rx sensitivity degradation due to Tx noise is only 0.1 dB, dual Tx and single chain Tx PC2 result in similar RSD. As the single Rx sensitivity degradation due to Tx is increased, differences are noted in the single and dual chain Tx desensitization. In the model, the nominal dual Rx sensitivity is significantly better than the 3GPP sensitivity requirement for 10 MHz case of -95.8dBm. Since the 3GPP sensitivity requirements are more relaxed than what can typically be achieved with modern components, it is recommended that a conservative approach is taken with any additional relaxations.

Based on the above, the proposed RSD values are of 0.5 for PC2 TxD and 0.3 dB for single chain PC2.

# Conclusions

Proposal: Based on the analysis above, we propose the following values for 1Tx PC2 REFSENS degradation in Table 2 and 1Tx PC2 REFSENS degradation in Table 3.

Table 2: Proposed REFSENS degradation for 1Tx PC2 versus PC3

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n30 | [0.3] | [0.3] |  |  |  |  |  |  |  |  |

Table 3: Proposed REFSENS degradation for 2Tx PC2 versus PC3

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n30 | [0.5] | [0.5] |  |  |  |  |  |  |  |  |

# References

- TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.