3GPP TSG-RAN WG4 Meeting #116bis	R4-2514507

# Prague, Czech Republic, October 13-17, 2025

Title:	On US 4.9 GHz UE RF requirements

Source:	Qualcomm Incorporated

Agenda item:	5.11.4

Document for:	Discussion

# Background

A new Rel-20 WI [1] was created to ‘allow for public safety operation in line with the framework identified by the FCC within the frequency range, 4940-4990 MHz, within 5G NR’, for which ‘a new NR TDD operating band needs to be defined’.

In this contribution, we discuss the relevant FCC Part 90 regulations governing the 4.9 GHz band in the United States and propose to establish common understanding of the regulation wording. We also look at impact of this mask on 3GPP framework of power classes, NS, etc and identify the need for a simulation campaign to establish necessary back offs for UEs of existing power classes for compliance.

# Discussion

## Background and Motivation

The applicable regulation for operation in the proposed US4.9 band is below:

- For expected 3GPP use, devices in the US 4.9GHz band is governed by applicable FCC Emission mask M (defined by FCC §90.210 Emission masks [4])

- Additionally, UEs are limited to a maximum conducted output power limit of 20dBm/MHz (defined by FCC §90.1215 Power limits [5], with a peak PSD limit of 21dBm/ MHz).

These regulations will benefit from discussion in RAN4 to ensure a common interpretation, prior to any back-off analysis. Note that the FCC does not specify a maximum power limit for a 25 MHz channel bandwidth, but it is currently included in the WID[1].

## 2.2	Interpretation of FCC requirements

### 2.2.1	Interpretation of the PSD limit requirement

The FCC Power limits as defined by §90.1215 states:

“For high power devices, the maximum conducted output power must not exceed 20dBm/1MHz . . . High power devices are also limited to a peak power spectral density of 21 dBm per one MHz”

Proposal 1: RAN4 interprets the PSD limit in FCC §90.1215 of 20 dBm/ MHz as the maximum allowed UL PSD (within the transmission bandwidth) over all channel bandwidths, RB allocation sizes and UE power classes.

Note that proposal 1 would include 1 dB margin to the regulation limit to accommodate imperfect UL spectral flatness at the UE’s test interface.

### 2.2.2	Interpretation of the Emission mask M requirement

#### 2.2.2.1	Frequency-Offset for Mask

The FCC Emission mask M regions defined by §90.210 states:

“On any frequency removed from the assigned frequency between 0-X% of the authorized bandwidth (BW). . . “

In our view, the intent is for the mask to fit over the intended transmission of the channel, and for the roll-off to apply outside the channel. The Emission mask M is defined in terms of a frequency distance from the ‘assigned’ frequency. To reconcile the intent with the wording, the offset would have to apply to frequencies starting from the center frequency of configured UE channel bandwidth, unlike the 3GPP General SEM that applies to FOOB starting from channel edges.

Proposal 2: RAN4 interprets the FCC §90.210 mask parameter ‘frequency removed from the assigned frequency’ as frequency offset referenced to the center frequency of the configured UE channel bandwidth.

#### 2.2.2.2	Resolution bandwidth for the mask requirement

In §90.210, the requirement for setting resolution bandwidth states:

“The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission. . .”

While a fixed Resolution bandwidth (RBW) of 1MHz is preferred for consistency across channel bandwidths and alignment with 3GPP General SEM, there exist cases where the domain or frequency basis of a single section of the FCC part 90 mask M is narrower than 1MHz (for ex. 45-50% region of a 10MHz wide FCC mask M). In other words, a 1 MHz resolution BW will be too coarse for that ‘steep and narrow’ section of the mask.

To ensure compliance across all valid channel bandwidth and power class combinations, while maintaining analytical consistency, we propose setting the RBW to the minimum value recommended by regulation, i.e. 1% of the configured UE channel bandwidth when adopted as a 3GPP requirement.

Proposal 3: RAN4 to adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth.

#### 2.2.2.3	Zero dB mask reference

The FCC Emission Mask M wording is below:

“The zero dB reference is measured relative to the highest average power of the fundamental emission measured across the designated channel bandwidth using a resolution bandwidth of at least one percent of the occupied bandwidth of the fundamental emission”

The wording was originally written assuming full channel occupancy (e.g., 5–20 MHz wideband systems), and in that context the wording is unambiguous. Unfortunately, LTE/5G-NR brought partial allocations and with it some ambiguity.  It remains clear that the 0 dB reference is set to some notion of PSD. The power aspect of the UE’s PSD is relatively straightforward, but the BW aspect is ambiguous due to the partial allocation problem of LTE/5G. Two interpretations are possible:

- Interpretation A: The 0dB mask-reference is derived from the UE’s maximum UL power spread over a fully allocated spectrum – the BW can be simplified to ‘configured channel BW’

- Interpretation B: The 0dB mask-reference depends on the PSD of the RB allocation scheduled for the UE

Each of the above interpretation will have specific implications as elaborated below:

- Interpretation A: For a given CBW and power class, the mask reference is “fixed” to the PSD of the UL over a full allocation and applies to any RB allocation

- Interpretation B: For a given CBW and power class the mask reference is increased (i.e. mask would “float” up) for narrow allocations, because of their higher PSD. A higher mask means a less stringent mask.

Observation 1: Interpreting FCC part 90 regulation so the zero dB mask-reference depends on the PSD of the RB allocation scheduled for the UE rather than the channel BW is more favorable for the network, but it also risks higher interference to incumbents.

Figure 3. Interpretation B: Emission mask M based on a Full allocation vs. Partial allocation (1RB)

Figure 3 shows a comparison of mask M calculated with a full allocation (red) and mask calculated for a partial allocation of 1RB (blue) for PC3 power class. Their respective zero dB references are also shown as dashed lines in the plot. As expected, the mask reference is significantly lower for the full allocation case compared to the partial allocation case due to the lower PSD of the fully allocated case.

Crucially, both cases still need to obey the FCC PSD limit requirement represented by the green horizontal line within the channel transmission bandwidth. The PSD limit is nominally 20 dBm/MHz but it is recast in terms of resolution BW of the mask, which is 150 kHz for a 15 MHz channel BW, the example case. The PSD limit has the effect of ‘cutting away’ the top part of the mask that ‘floats up’ per interpretation B. In figure 3, the mask whose reference is defined by the PSD for a 1RB transmission (blue) is shown with the part about the green line cut off.

Weighing both interpretations, we propose that 3GPP adopt the more conservative Interpretation A so incumbents are better protected. A secondary benefit is that 3GPP will be insulated from any hypothetical future clarification from the FCC that deems the more aggressive interpretation (‘B’) as counter to intent.

Proposal 4: For RAN4, the zero dB reference for FCC Emission mask M is calculated based on UE max power and the configured UE channel bandwidth rather than RB allocation BW.

Another important detail is whether the 0 dB reference of the mask represents a PSD limit for the intended transmission. The FCC part 90.210 regulation wording is reproduced below:

“..the power spectral density of the emissions must be attenuated below the output power of the transmitter …. On any frequency removed from the assigned frequency between 0-45% of the authorized bandwidth (BW): 0 dB.”

The specification explicitly requires ‘0 dB attenuation’ in the middle 90% of the configured channel bandwidth, but does not indicate if there is restriction against the UL signal exceeding the mask’s 0 dB reference level. In other words:

Observation 2: The zero dB reference for the FCC part 90 mask M does not represent a PSD limit for the UL transmission, rather it merely serves to position the emission mask in the power dimension.

Figure 4 shows two different allocations for 20 MHz PC2 UE that illustrates this interpretation of the wording. Both allocations are shown at their respective maximum power levels while assuming the proposals and observations above.

The narrower allocation is sufficiently shielded from the mask due to its location, but it must still comply with the separate PSD limit of 90.1215 (green dashed line). Note here that the 0 dB reference of the mask (the highest power where the magenta mask line stops) does not constrain the output PSD. In this case a back off is needed (~4 dB) from the nominal of 26 dBm.

In contrast, the peak PSD of the wider spectrum is low enough that it does not approach the +20 dBm/MHz PSD limit, but the waveform needs back off to fit in under the mask (~2.0 dB). The relative proximity of the zero dB reference level of the mask and the PSD of the allocation as shown is of no significance.

Figure 4. Example of narrow vs. wide allocation compliance to mask and PSD limit

Proposal 5: For UE compliance to the FCC part 90 mask in the US4.9 band, the zero dB reference is used merely for mask location and does not represent a PSD limit for the UL

FCC mask M is plotted in Figure 5 below (legends correspond to dBm of max. UL power) while implementing the proposals above. The masks are overlaid with the legacy 3GPP SEM for comparison. Figure 5 graphically illustrates that the FCC mask M is significantly more stringent than 3GPP UE General SEM mask. In this situation it is convenient to retain the legacy 3GPP mask as the ‘general requirement’ and the stricter mask as an additional requirement with new network signalling (NS_XX).

Proposal 6: For RAN4, UE compliance to the FCC part 90 mask in the US4.9 band is enabled via NS.

Figure 5. Example comparison of FCC Mask M with 3GPP General SEM [6]

### 2.2.3	Disallowed power classes and channel BWs

The FCC mask M 0 dB reference (assuming earlier proposals) is calculated per equation below:

where,

PUEmax power (dBm) is the nominal power class value (for PC3 = 23dBm, PC2 = 26dBm, etc.)

designated channel bandwidth is the configured UE channel bandwidth in Hz

For some CBW-power class combinations, the 0 dB mask reference itself exceeds the 20dBm/MHz PSD limit in the FCC part 90 requirement. This means that the UE cannot transmit at its power class rated power, they must be operated with certain minimum back off for all conditions, and that defeats the purpose of the power class.

An example calculation for 10 MHz PC1.5 is shown in the equations below:

The example 10 MHz PC1.5 case must be always run with a minimum backoff of 1 dB. This creates follow-up complications with the mask reference level. For ease of RAN4 specification, we propose that such combinations be ‘disallowed’ in the FCC part 90 context.

In contrast, for 10 MHz PC2, the 0dB mask reference complies with the PSD limit and would not automatically be disallowed. Figure 6 helps visualize the difference between an allowed case and a disallowed case based on compliance with the FCC part 90 PSD limit.

Figure 6: Example of a valid case (10 MHz PC2) vs. an invalid case (10 MHz PC1.5)

Note that the ‘UE max power’ term to calculate the mask 0 dB reference level can be considered nominal power for the power class itself only if there are 0dB backoff waveforms. If no 0dB backoff waveform exists that can meet both the general requirements as well as mask M, the mask reference cannot be derived from the nominal PC power as described above. This condition represents another case where UEs are disallowed based on part 90 rules. This situation shows up as a lack of full power waveforms (any allocation, any modulation type) from backoff simulations. The reason for disallowing such a UE is that it sets off an iterative sequence that underscores that the UE cannot transmit at the level expected of the declared power class: The mask must be lowered corresponding to the lowered power expectation, and that makes the mask tighter yet. The calculated back offs may no longer suffice and must be recalculated. The recalculated back offs may yield an even lower reference for the mask which in turn tightens the mask, and so on.

The table below is a summary of the disallowed combinations of UE power class and channel BW in context of FCC part 90. Details of how these were derived are described in section 3 and section 6.

Observation 3:  For US4.9, FCC part 90 rules preclude certain combinations of power classes and configured channel bandwidths.

Observation 4: All channel BWs can be enabled for PC3 UEs for operation in US4.9

Observation 5: Not all channel BWs can be enabled for HPUE power classes for operation in US4.9

## UE power classes

Owing to the complexities captured in observations 3-5, the most efficient RAN4 outcome is to tackle all HPUE classes in the same WI. While it increases effort in the short term, it is more efficient that a separate WI to introduce HPUEs separately.

The other option is to only enable PC3 in this WI and use the HPUE basket WI to enable HPUEs for US4.9. Unfortunately, as identified in observation 4, the default power class (PC3) is a special case in not having to bar certain bandwidths for certain power classes. The more typical case is that of the HPUE, where one or more channel BWs cannot be enabled while complying with FCC part 90 rules.

Observation 6: If PC3 is used as the sole example power class, the core requirement wording will be incomplete for HPUE, thus precluding enabling the latter through a spectrum WI.

Consequently, if just a single power class is enabled in this WI, one of the HPUE power classes would be more representative in terms of establishing specification wording that is extendable to other power classes. Moreover, at 4.9 GHz, propagation losses make PC3 relatively ineffective and HPUE more attractive.

Proposal 6: If the US4.9 WI is limited to enabling a single UE power class, RAN4 to choose PC2 as the prototype power class.

Proposal 7: FFS how to disable certain channel BWs for certain power classes when the associated NS is signaled.

## UE hardware assumption

The WID sets out this objective: ‘Specify the UE RF requirements for the new bands. Reuse NR band n79 UE hardware assumption if possible’. From a UE perspective, since US4.9 is a subset of n79, reuse of hardware is possible, and a valid assumption. The transmit side requirements may impose new challenges (owing to different SEM) but that can be handled separately, for example via NS (Proposal 5).

Proposal 8: RAN4 to confirm assumption of reuse of n79 hardware for US4.9

## 2.5	CA operation

In the US, n79 is not available for mobile use in its entirety. US4.9 is motivated by a desire to exploit spectrum overlapping with n79 for non-public safety operations. Consequently, there should be no occasion for the UE to support inter-band CA between n79 and US4.9.

Proposal 9: There is no need for UEs to support inter-band CA between n79 and US4.9.

The max. available spectrum in US4.9 is 50 MHz, which can be serviced by a single channel. There is hence no need to define intra-band CA in US4.9

Proposal 10: Intra-band CA in US4.9 is not enabled for Rel-20.

# 3. 	Simulations

## 3.1	Simulation assumptions

- Operating band: UL 4940 MHz – 4990 MHz

- Channel bandwidths 10, 15, 20, 25, 30, 40 ,50 MHz

- Subcarrier spacing: 30 kHz

- Waveform: DFT-s-OFDM QPSK (chosen for presence of 0 dB MPR waveforms)

- Power class PC3, PC2, PC1.5, PC1

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0.

- FCC requirements evaluated:

  - FCC Emission mask M (based on Interpretation A discussed in section 2.2.2.3), and

  - In-channel PSD limit of 20dBm/MHz (fixed limit fixed across all channel bandwidths/power classes/ RB allocation sizes as discussed in section 2.2.1)

## 3.2	Simulation results & discussion

In an appendix (section 6), we provide complete sets of backoff triangle simulation plots for valid cases (as noted in Table 1) and the cases that show no 0dB back-off regions, with Fc at 4965 MHz. The plots show required back-off to meet FCC part 90 regulations.

As a first example, 10 MHz PC3 backoff requirement plot is shown in Figure 7 below. The 0dB back-off region is highlighted as ① in the figure. It is observed that the narrower allocations require significantly higher back-off (highlighted as ② in figure) than the wider allocations to pass the requirements. This is because the narrower allocations have much higher PSD and therefore more exposed to the PSD limit. As the width of RB allocation widens, the PSD spreads across wider band and the failures are dominated by the mask M due to the IMDs.

Figure 7. Backoff plot for channel bandwidth 10MHz & Power class 23dBm

For second example, we use a higher channel bandwidth of 15 MHz as an example, with PC3 as well.  The simulated backoff requirement plot is shown in Figure 8. The 0dB back-off region is highlighted as ① in the figure, and ② shows how the PSD limit dominated RB cases have higher back off requirement compared to wider RBs limited by PSD limit until eventually the mask M starts to dominate, similar mechanism as observed in 10 MHz channel bandwidth simulation.

The notable observation lies in region highlighted as ③, where the narrower edge RBs show significantly higher backoff s (>15dB, and up to 30dB for certain higher power classes cases in appendix). This is because for wider channel bandwidths, the ratio of guard band to total channel bandwidth decreases and the narrower edge RBs are more exposed to (45-50%) and (50-55%) stringent mask M regions.

Figure 8. Backoff plot for channel bandwidth 15MHz & Power class 23dBm

Figure 9 below shows an example of 10 MHz PC2, one of the cases where no 0dB back-off regions were observed and hence deemed non-implementable.

Figure 9. Example of non-implementable case with no 0dB back-off regions

The rest of the simulation results generally follow the same mechanisms as shown in Figures 7-9. The following considerations regarding existing 5G NR deployments in the 4.9 GHz band are noteworthy:

• Current NR bands within this frequency range operate under TDD configurations.

• Implementations typically utilize High Power User Equipment (HPUE), corresponding to Power Class 2 (26 dBm).

This effort highlights the need for a simulation campaign so UEs can be provisioned with enough backoff for compliance with FCC part 90 regulations.

Proposal 11: RAN4 to kick off a simulation campaign to identify backoff requirements for UEs to comply with FCC part 90.

# 4.	Conclusions

Proposal 1: RAN4 interprets the PSD limit in FCC §90.1215 of 20 dBm/ MHz as the maximum allowed UL PSD (within the transmission bandwidth) over all channel bandwidths, RB allocation sizes and UE power classes.

Proposal 2: RAN4 interprets the FCC §90.210 mask parameter ‘frequency removed from the assigned frequency’ as frequency offset referenced to the center frequency of the configured UE channel bandwidth.

Proposal 3: RAN4 to adopt a resolution bandwidth for the FCC part 90 mask to 1% of the configured UE channel bandwidth.

Observation 1: Interpreting FCC part 90 regulation so the zero dB mask-reference depends on the PSD of the RB allocation scheduled for the UE rather than the channel BW is more favorable for the network, but it also risks higher interference to incumbents.

Proposal 4: For RAN4, the zero dB reference for FCC Emission mask M is calculated based on UE max power and the configured UE channel bandwidth rather than RB allocation BW

Observation 2: The zero dB reference for the FCC part 90 mask M does not represent a PSD limit for the UL transmission, rather it merely serves to position the emission mask in the power dimension.

Proposal 5: For UE compliance to the FCC part 90 mask in the US4.9 band, the zero dB reference is used merely for mask location and does not represent a PSD limit for the UL

Proposal 6: For RAN4, UE compliance to the FCC part 90 mask in the US4.9 band is enabled via NS.

Observation 3:  For US4.9, FCC part 90 rules preclude certain combinations of power classes and configured channel bandwidths.

Observation 4: All channel BWs can be enabled for PC3 UEs for operation in US4.9

Observation 5: Not all channel BWs can be enabled for HPUE power classes for operation in US4.9

Observation 6: If PC3 is used as the sole example power class, the core requirement wording will be incomplete for HPUE, thus precluding enabling the latter through a spectrum WI.

Proposal 6: If the US4.9 WI is limited to enabling a single UE power class, RAN4 to choose PC2 as the prototype power class.

Proposal 7: FFS how to disable certain channel BWs for certain power classes when the associated NS is signaled.

Proposal 8: RAN4 to confirm assumption of reuse of n79 hardware for US4.9

Proposal 9: There is no need for UEs to support inter-band CA between n79 and US4.9.

Proposal 10: Intra-band CA in US4.9 is not enabled for Rel-20.

Proposal 11: RAN4 to kick off a simulation campaign to identify backoff requirements for UEs to comply with FCC part 90.

# 5.	Reference

[1] RP-251887, “New WID on Introduction of NR TDD 4.9GHz Band for US Operation”, Fujitsu, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2] R4-2510412, “On adjacent spectrum protection requirements for the 4940-4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Aug. 2025

[3] R4-2510429, “Coexistence study for introduction of US 4.9GHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, 25th – 29th August, 2025

[4] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-I/section-90.210

[5] https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90/subpart-Y/section-90.1215

[6] R4-2510411, “General aspects regarding the 4940–4990 MHz band”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, Aug. 2025

# 6.	Appendix

The documents embedded below contain simulation result plots for projected backoff needs all channel bandwidths and power classes as discussed in section 3.2.