3GPP TSG-WG4 Meeting #116bis	R4-2514578

# Prague, Czech Republic, 13th Oct 2025 - 17th Oct 2025

# Title:	Missing MSD for CA_n25-n41-n71

# Agenda Item:	5.3.4 (NR_CADC_R19_3BDL_xBUL)

# Source:	Skyworks Solutions, Inc., T-Mobile USA

# Document for:	Approval

- Introduction

This contribution proposes MSD requirements for CA_n25-n41-n71.

- Discussion

The following power class (PC) 3 and PC2 UL CA configurations are specified in Table 5.5A.3.2-1b for DL CA_n25A-n41A-n71A:

- CA_n25A-n41A, for which the dual UL IMD2 and IMD5 may affect Band n71

- CA_n25A-n71A, for which the dual UL IMD2 and IMD4 may affect Band n41

- CA_n41A-n71A, for which the dual UL IMD2 may affect Band n25

The corresponding PC3 and PC2 MSD requirements are not specified in Rel 19.2.0.

We propose to adopt the following PC3 MSD requirements:

- Band n25 and n71 IMD2 MSD based on agreements for CA_n25-41-n85

- Band n41 IMD2 MSD based on agreements for CA_n2-n5-n41

- Band n71 IMD5 MSD based on agreement for CA_n8-n39-n40

We reflect the WF [1] agreements to specify the Band n41 MSD for 10 MHz CBW.

Proposal 1: Adopt the following PC3 MSD requirements for CA_n25-n41-n71.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_n25-n41-n71 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2532 | 10 | 50 | 2532 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 28.7 | FDD | IMD2 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2534 | 10 | 50 | 2534 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 4.3 | FDD | IMD5 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | N/A | 10 | N/A | 2578 | 30.0 | TDD | IMD22 |  |
|  | n71 | 678 | 5 | 25 | 632 | N/A | FDD | N/A |  |
|  | n25 | N/A | 5 | N/A | 1980 | 26.0 | FDD | IMD2 |  |
|  | n41 | 2658 | 10 | 50 | 2658 | N/A | TDD | N/A |  |
|  | n71 | 678 | 5 | 50 | 632 | N/A | FDD | N/A |  |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

The PC2 MSD requirements are calculated from the PC3 MSD requirements using the recently agreed-upon MSD simplification look-up tables.

Proposal 2: Adopt the following PC2 MSD requirements for CA_n25-n41-n71.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_n25-n41-n71 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2532 | 10 | 50 | 2532 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 34.7 | FDD | IMD2 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2534 | 10 | 50 | 2534 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 19.3 | FDD | IMD5 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | N/A | 10 | N/A | 2578 | 36.0 | TDD | IMD22 |  |
|  | n71 | 678 | 5 | 25 | 632 | N/A | FDD | N/A |  |
|  | n25 | N/A | 5 | N/A | 1980 | 32.0 | FDD | IMD2 |  |
|  | n41 | 2658 | 10 | 50 | 2658 | N/A | TDD | N/A |  |
|  | n71 | 678 | 5 | 50 | 632 | N/A | FDD | N/A |  |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

- Conclusion

This contribution proposes the following PC3 and PC2 MSD requirements for CA_n25-n41-n71.

Proposal 1: Adopt the following PC3 MSD requirements for CA_n25-n41-n71.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_n25-n41-n71 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2532 | 10 | 50 | 2532 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 28.7 | FDD | IMD2 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2534 | 10 | 50 | 2534 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 4.3 | FDD | IMD5 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | N/A | 10 | N/A | 2578 | 30.0 | TDD | IMD22 |  |
|  | n71 | 678 | 5 | 25 | 632 | N/A | FDD | N/A |  |
|  | n25 | N/A | 5 | N/A | 1980 | 26.0 | FDD | IMD2 |  |
|  | n41 | 2658 | 10 | 50 | 2658 | N/A | TDD | N/A |  |
|  | n71 | 678 | 5 | 50 | 632 | N/A | FDD | N/A |  |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

Proposal 2: Adopt the following PC2 MSD requirements for CA_n25-n41-n71.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_n25-n41-n71 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2532 | 10 | 50 | 2532 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 34.7 | FDD | IMD2 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | 2534 | 10 | 50 | 2534 | N/A | TDD | N/A |  |
|  | n71 | N/A | 5 | N/A | 632 | 19.3 | FDD | IMD5 |  |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |  |
|  | n41 | N/A | 10 | N/A | 2578 | 36.0 | TDD | IMD22 |  |
|  | n71 | 678 | 5 | 25 | 632 | N/A | FDD | N/A |  |
|  | n25 | N/A | 5 | N/A | 1980 | 32.0 | FDD | IMD2 |  |
|  | n41 | 2658 | 10 | 50 | 2658 | N/A | TDD | N/A |  |
|  | n71 | 678 | 5 | 50 | 632 | N/A | FDD | N/A |  |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

# References

- R4-2511779 WF on optional channel bandwidths, 3GPP TSG-RAN WG4 Meeting # 116, Bangaluru, India, Skyworks Solutions, Inc.