3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514587
Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			8.1

Source:	Samsung

Title:	WF on 6G AI

Document for:	Approval

# Topic#1: General scope and timeline

##### Issue 1-1: RAN4 6G AI/ML study scope

Agreement:

- In 6G AI/ML study, the detailed RAN4 scope shall include

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

    - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1.

    - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

  - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

    - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs.

    - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability.

##### Issue 1-2: RAN4 6G AI/ML study timeline

Agreement:

- RAN4 timeline for Rel-20 6G AI/ML study

  - Stage 1: Q4'25 and Q1'26

    - To clarify study scope for RAN4 AI/ML framework

    - To select the top-prioritized RAN4-led AI/ML use cases, with the detailed evaluation plan concluded for Stage-2

  - Stage 2: Q2’26 to Q2’27

    - To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)

    - To study comprehensively the RAN4-led use case(s) selected in Stage 1.

      - Other new use case(s) are not precluded depending on the consensus and the limited RAN4 bandwidth

    - Based on the agreement and/or the request from other WG, RAN4 will study the selected other WG-driven use cases with RAN4 analysis and evaluation. RAN4 related bandwidth limitation should be taken into consideration.

# Topic#2: RAN4 AI/ML framework

##### Issue 2-2: Post-deployment enhancement

Tentative agreement to be discussed:

- For post-deployment enhancement, RAN4 could further study post-deployment testing and validation enhancements in 6G study:

  - The study can be conducted in collaboration with RAN2 from early phases of the 6GR SI.

  - The study will begin no early than Q2 2026.

  - Potential study directions:

    - Solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting

    - Other directions are not precluded

    - Companies are encouraged to provide the relevant specification impacts if any.

| Company | Comments |
| --- | --- |
| MTK | We agree that this is an important issue but not the most urgent one. We prefer to focus on discussing the use cases in the first several meetings. |
| Huawei, Hisilicon | We agree with MTK. Detailed discussion could be trigger by companies’ concrete proposals. |
| Apple | We support further study of post-deployment testing and validation enhancements in 6G, especially considering emerging scenarios such as online learning and continuous model adaptation. We recommend that RAN4 approach this study in a phased manner, starting with post-deployment aspects directly tied to the selected RAN4-driven use cases. We also agree it is not the most urgent one and it should not be the priority in the first few meetings. We should focus on use case finalization for the first few meetings. |
| ZTE | We deem that the post-deployment is the mandated LCM procedure in most use cases, RAN4 could study this and more details from other working groups shall be waited. |
| Qualcomm | Post-deployment testing is an important aspect of AI-ML. However, post-deployment testing related discussion often becomes generic. 5G-A experience suggests that even very basic agreements regarding post-deployment testing may take a long time to be concluded in RAN4. Considering the limited time units for AI-ML session, we prefer RAN4 to select a few specific use cases and prioritize the study of those use cases during the SI, instead of spending time in post-deployment testing related discussion. |
| Samsung | Okay to postpone the discussion into Phase-2, while considering the current wording is just listing some research directions, seems not harmful to be included. Then suggest the following change: - For post-deployment enhancement, RAN4 shall could further study post-deployment testing and validation enhancements in 6G study: - The study can be conducted in collaboration with RAN2 from early phases of the 6GR SI. - The study will begin no early than Q2 2026. - Potential study directions: - Solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting - Other directions are not precluded - Companies are encouraged to provide the relevant specification impacts if any.. |
| CMCC | Post-deployment is an important issue to guarantee the performance in real-world. But we also agree other companies’ views that in this initial phase, we can focus on other fundamental issues.e.g. use cases. |

##### Issue 2-4: Model generalization

Tentative agreement to be discussed:

- For model generalization, RAN4 shall study generalization of UE-based AI/ML functionality for 6G and identify its impact on RAN4 specification.

  - The study will begin no early than Q2 2026.

  - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML models for each use case firstly, and then discuss how to test the generalization for the corresponding use case.

    - For testing, study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models

  - Study the relationship between the model generalization requirement/testing and the LCM relevant requirement/testing.

| Company | Comments |
| --- | --- |
| CATT | In different use cases, AI/ML models may be expected to generalize different aspects. Hence, following our comment in the 1st round discussion, we propose to add one more bullet as follows: - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML models for each use case firstly, and then discuss how to test the generalization. |
| MTK | We agree that this is an important issue but not the most urgent one. We prefer to focus on discussing the use cases in the first several meetings. |
| Ericsson | The way discussion points are formulated seems generalization is going to have an impact on testing only. It seems early to make a conclusion like this. |
| Huawei, Hisilicon | Generalization may also have impact on UE capability. We agree to discuss generalization case by case. |
| Nokia | We agree that generalization is an important issue to cover in RAN4. However, RAN4 shouldn’t indulge on physical AI/ML model level generalization aspects. RAN4 should rather focus on generalization aspects of AI/ML functionalities. |
| Apple | We support RAN4's study of generalization for UE-based AI/ML functionality in 6G, as ensuring robust performance across diverse scenarios is critical for practical deployment. We propose that RAN4 take a use-case-specific approach to identify which channel parameters and environmental conditions should be tested for generalization, including studying dynamic parameter changes during performance tests as a more realistic evaluation method. The relationship between generalization testing and LCM procedures (e.g., model monitoring and adaptation) should be clarified to avoid test coupling while recognizing that practical AI/ML systems may leverage both generalization capability and LCM mechanisms to maintain performance. We believe this study should aim to reduce testing burden while ensuring adequate performance validation across representative operating conditions. It should not be the priority in the first few meetings. We should focus on use case finalization for the first few meetings. |
| OPPO | For generalization, we are OK with the FL proposal. We can agree it as a high level principle. Also we think the comments from MTK and Ericsson are valuable |
| ZTE | For model generalization, RAN4 could discuss how to verify model generalization on testing. For example, multiple test cases with static channel conditions or one test case with dynamic channel conditions. |
| Qualcomm | Generalization is an important aspect of AI/ML. We agree with studying generalization of UE based AI-ML functionality. We think that changing channel parameters during a performance test could be an effective way to check the generalization aspects of an AI-ML model. However, according to the online agreement, RAN4 can study LCM relevant requirement based on agreements/requests by other WGs. Hence, the relationship between the model generalization requirement and LCM related requirement can be studied only based on requests/agreements by other WGs. |
| Samsung | Okay to postpone the discussion into Phase-2, while considering the current wording is just listing some research directions, seems not harmful to be included. Then suggest the following change: - For model generalization, RAN4 shall study generalization of UE-based AI/ML functionality for 6G and identify its impact on RAN4 specification. - The study will begin no early than Q2 2026. - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML models for each use case firstly, and then discuss how to test the generalization for the corresponding use case. - For testing, study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models - Study the relationship between the model generalization requirement/testing and the LCM relevant requirement/testing.. |
| CMCC | Generalization is an important issue, which need to be studied. As for the recommended WF, in general, we are OK. Just one minor issue, the last bullet is not clear. Generalization and LCM target for different things. Generalization test is used to verify whether the performance gain of AI/ML functionality/model which is achieved under the identified scenarios and/or configurations, won’t be significantly degraded in other scenarios and/or configurations. While LCM are about procedures of activation,deactivation,etc. Why we need to discuss the relationship between them? What relationship is expected? If it refer to whether functionality/model switch are allowed during the generalization test, it is better to make it clear. |

##### Issue 2-6: AI/ML UE category

Tentative agreement to be discussed:

- For UE AI category in 6G, Following aspects can be considered in RAN4 6G study, and this study will begin no early than Q2 2026.

- : to differentiate different UE capabilities:

  - UE computing capability (e.g., X TOPS or FLOPS)

  - UE energy consumption (e.g., X TOPS/W)

  - UE memory capacity (e.g., X MByte)

  - Types of Computing Resources supported by UE (e.g., CPU or GPU)

  - Location of Computing Resources supported by UE (e.g., inside or outside baseband)

  - Supported Reference Models (e.g., different backbone)

| Company | Comments |
| --- | --- |
| CATT | Considering that RAN Plenary has been discussing device types, RAN4 is not expected to define ‘RAN4’-level device types before RANP make agreements. Otherwise, there might be misalignment between RAN-level device types and RAN4-level device types. But okay to keep this open for future discussion. In addition, the clarification question raised in the 1st round comment is not answered yet: if RAN4 agree to have different AI/ML relevant capabilities/categories, does it mean RAN4 will define different requirements for different capabilities/categories? |
| Ericsson | Given that the use cases are not clear now it does not seem urgent to conclude on this issue. |
| Huawei, Hislicon | Except the last aspect, the others are all left to UE implementation. In legacy, for non-AI based algorithms, these private information has never been touched. Nothing new is identified for AI-based algorithms. |
| Nokia | Although this issue is important, we suggest to not prioritize it on use-cases related issues. We are also not clear how discussions related to UE computing complexity, UE memory capacity, UE processing unit types could be useful in RAN4. |
| Apple | We support studying AI/ML UE categorization aspects that directly impact RAN4 requirements definition, such as computing capability metrics and supported reference models, but recommend waiting for RAN plenary conclusions on 6G device types before defining detailed AI/ML capability categories to ensure proper alignment. Implementation-specific aspects like memory tiers, types of computing resources, and computing resource locations should remain transparent to the network and not be mandated by specifications. RAN4 should first clarify whether different UE AI categories will necessitate different requirement sets or serve primarily as informative guidance, as this fundamentally shapes the scope of this study. We propose deferring detailed categorization discussions until when selected use cases and their specific capability dependencies are better understood. It should not be the priority in the first few meetings as it depends on other WG groups or RANP progress. We should focus on use case finalization for the first few meetings. |
| OPPO | OK to focus on use case finalization first. After that and with more clear RAN plenary conclusions, we can reconsider this issue again. |
| Qualcomm | RAN plenary is expected to discuss UE device types for 6G. That discussion should settle before RAN4 starts discussing AI-ML capability-based UE categorization in 6G. |
| Samsung | We also realized it is important topic to clarify and wait till the end of release will not help. Is that possible we allow interested companies to contribute on this topic which is intended to be captured in TR as the outcome of study. It should be noted that 6G SI is a study item, and we see no harmful to study implementation related issues even in the end it does not have specification impact. |

##### Issue 2-7: Interoperability for one-sided and two-sided model

Tentative agreement to be discussed:

- Interoperability for one-sided model, refers to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature

  - RAN4 to study how to ensure consistent behavior between UEs supporting AI/ML and UEs not supporting AI/ML for the same functionality, e.g., LCM procedure.

  - Identify interoperability issue in the use case study firstly

- Interoperability for two-sided model,

  - FFS the specified mechanism for CSI compression in Rel-20 5G-A WI can be followed in 6G study.

| Company | Comments |
| --- | --- |
| CATT | Agree with FL that clarification is needed for one-sided model, i.e., ‘Interoperability for one-sided model refers to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature’. And LCM procedure is mentioned as an example, but what is not clear to me is that why LCM procedure is linked to UE not supporting AI/ML. Prefer to firstly clarify the interoperability for one-sided model. |
| Ericsson | We suggest to remove e.g., LCM procedure from the first sub-bullet. There is no LCM for UEs that do not support AI/ML based functionality. |
| Xiaomi | First, we share our understanding about how interoperability is achieved for the Two-Side Model: To ensure consistent behavior across different implementations of the Two-Side Model, two core elements are defined. One is a reference model that serves as a unified benchmark to standardize functional logic. The other is a standard dataset used to verify whether all implementations align with the reference model in actual data processing. Together, these two elements guarantee that different Two-Side Model implementations can achieve consistent interoperability. Then, for interoperability of the One-Side Model, we think it’s not related to AI vs. non-AI. Instead, it applies to scenarios where all parties use AI. Specifically, when UEs (User Equipments) test on the same dataset, even though they may employ different models trained on their respective receiver-side data, we expect their performance to maintain a certain level of consistency. For One-Side Model, I wonder if we need to consider reference model or dataset alignment to achieve performance alignment. By the way, if RAN4 intends to compare the performance between AI and non-AI solutions, it’s better to use alternative wording. The term "interoperability" is likely to cause confusion, as it has consistently been associated with AI-related scenarios when following the Two-Side Model framework. |
| Huawei, Hisilicon | Interoperability for one-sided models was not discussed in R18 and R19. The motivation to introduce and define this terminology in 6G needs to be clarified. |
| Nokia | The definition of interoperability doesn’t look correct for one sided model above. Interoperability refers to ability of two or more systems or components to exchange data and use information. It doesn’t refer to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature. |
| Apple | We request clarification on the scope of "consistent behavior" for one-sided model interoperability, as it could mean either: (1) AI/ML-based solutions must meet or exceed non-AI baseline performance, or (2) AI-capable and non-AI UEs must coexist seamlessly in mixed network deployments. To enable constructive study, we suggest RAN4 first identify concrete interoperability issues, including network scheduling fairness when serving mixed UE populations, fallback mechanisms and reporting procedures when AI functionality becomes inapplicable, and compatibility of LCM procedures across different UE capabilities. We propose addressing these challenges on a use-case-specific basis after RAN4 selects priority use cases, as interoperability requirements will differ based on whether the AI functionality is optional or mandatory. For two-sided models, we support leveraging the Rel-20 5G-A CSI compression framework as a proven starting point. It should not be the priority in the first few meetings. We should focus on use case finalization for the first few meetings |
| OPPO | For “Interoperability for one-sided model”, does it mean, for a given feature, the performance of AI based solution should not be worse than the non-AI based solution? Does it belong to performance tests or have some specific issues related to interoperability? |
| Qualcomm | We agree with Ericsson that the phrase “LCM procedure” should be removed from sub-bullet of the first bullet since LCM procedure does not apply for UEs not supporting AI/ML. Besides, we prefer interoperability to be discussed on a case-by-case basis for the selected use cases, instead of discussing it in a generic manner like this. |
| Samsung | For Interoperability for one-sided model, seems we need more clarification and we can put FFS for the main bullet of “Interoperability for one-sided model” to give more time for people to interpret the implication. |
| CMCC | In our view, interoperability is more about consistent behavior across different implementations. We are not sure whether it is a good way to assume that the interoperability for one-sided model refers to AI vs. non-AI. |

# Topic#3: RAN4-driven AI/ML use case (General Issues)

##### Issue 3-1: Principle for AI/ML use cases selection

Agreement:

- Principles for AI/ML RAN4-driven use cases selection

  - RAN4 shall not duplicate the efforts of RAN1/2/3.

    - RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

    - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

  - Typical RAN4-driven use case shall

    - ensure performance gain or gain from other aspects, by focusing on some RF/Demod/RRM topics

  - Prioritize the use case by taking into account:

    - use cases with gain over non-AI counterpart

      - gain may include performance gain or gain from other aspects, like overhead reduction.

      - trade-off between performance and complexity/overhead should also considered.

    - use cases with potential standard impact.

    - Note: the prioritization principle is the general principle and no intention to have implication on RAN4 6G AI work plan.

##### Issue 3-2: Aspects of RAN4-driven use case to be studied

Agreement:

- Aspects of RAN4-driven use case to be studied

  - By following RAN1 agreement, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

    - Definition of each (sub-)use case, including at least AI model input/output

    - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

    - Assumption on training types, e.g.,

      - offline training, online training, finetuning

      - Label construction (if applicable), including whether/how to obtain label data for model training

    - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

    - Collaboration/interaction between UE and NW, e.g.,

      - no collaboration/interaction

      - UE/Network collaboration targeting at separate or joint ML operation

    - High level potential specification impact

  - Additionally, the following RAN4 aspect(s) are encouraged to study:

    - Feasibility, complexity/cost under 6G assumptions

    - Feasibility issues, including complexity, and other aspects related to implementation

    - Other aspects identified by companies are not precluded.

- Here is the exemplary template for company to study/report the details of one certain use case:

| Sub-use case X |  | Company-A | Example |
| --- | --- | --- | --- |
| AI model input | Input in training |  | e.g., estimated DMRS |
|  | Input in inference |  | e.g., estimated data signal |
| AI model output | Label in training (if applicable) |  | e.g., original DMRS |
|  | Output in inference |  | e.g., corrected signal from non-linearity |
| Assumption on training | Training type |  | e.g., offline training, online training, finetuning, etc. |
|  | Label construction (if applicable) |  | whether/how to obtain label data for model training |
| model location for inference |  |  | e.g., UE-sided model, NW-sided model, and two-sided model |
| Collaboration/interaction between UE and NW |  |  | e.g., no collaboration/interaction, UE/Network collaboration targeting at separate or joint ML operation |
| Evaluation methodology |  |  | e.g., system level simulation, link level simulation, with detailed description |
| Evaluation assumption |  |  | Detailed table can be provided for evaluation assumption |
| Evaluation KPI |  |  | e.g., Tput |
| Evaluation benchmark |  |  | Description of non-AI solution to be compared with |
| preliminary evaluation results |  |  | Detailed results can be provided. |
| High level potential specification impact |  |  | e.g., impacted existing clause, potential new requirement, impacts to other WG’s specifications |
| Feasibility issues, including complexity, and other aspects related to implementation |  |  | e.g., description of model complexity, other implementation complexity etc. |

# Topic#4: AI/ML use case for RF issues (Per-use case discussion)

### Use Case#1: AI-nonlinearity compensation

##### Issue 4-1b: Sub use case definition and selection for AI-DPD/DPoD

Agreement:

- The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include

- UL:

  - UE: AI-DPD

  - NW: AI-DPoD

- DL

  - NW:AI-DPD

  - UE:AI-DPoD

- To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

For information:

- The following information is provided by vivo to clarify the proposed of UE AI-DPD for UL

| AI model input | Input in training | Time domain samples after PA distortion |
| --- | --- | --- |
|  | Input in inference | Time domain samples before pre-distortion |
| AI model output | Label in training (if applicable) | Time domain samples w/o distortion |
|  | Output in inference | Option1：Time domain samples after pre-distortion Option2：LUT coefficients |
| Assumption on training | Training type | Offline training；Online finetune |
|  | Label construction (if applicable) | Time domain samples w/o distortion |
| model location for inference |  | UE-sided |
| Collaboration/interaction between UE and NW |  | As for UE side model |
| Evaluation methodology |  | link level simulation |
| Evaluation assumption |  | Model PA model; Parameter. yn= k∈Kpakx(n)x(n)2k; ak; k∈[0,1,…,7]: [-0.618347-0.785905i; 2.0831-1.69506i; -14.7229+16.8335i; 61.6423-76.9171i; -145.139+184.765i; 190.61-239.371i; -130.184+158.957i; 36.0047-42.5192i]. |
| Evaluation KPI |  | BLER, EVM, MPR |
| Evaluation benchmark |  | No DPD |
| preliminary evaluation results |  | 1-3 dB |
| High level potential specification impact |  | 1. RF requirements 2. Dedicated signal (RS/data) for training or monitoring data collection. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Inference complexity: Option1：38.5 MFlops（per slot）,1K parameters in total Option2: similar as LUT-based DPD |

Ad-Hoc minutes:

Vivo: for high modulation order, even 3dB gain expected, while low modulation order can still expect 1dB gain. Concerns received on model complexity and power consumption, but we still hope to keep it as part of study.

QC: need to check the antenna impact on label

MediaTek: Option 2 of the output is LUT is for non-DPD case.

Vivo: similar situation in RAN1.

Apple: the benchmark to compare with is no DPD, but it is also possible to have non-AI DPD.

NTU: hard to get the baseline performance for non-AI DPD.

QC: related to non-linearity model which has been discussed in different agenda.

OPPO: it is just example from one company. For next meeting, maybe more inputs can be provided.

For information:

- The following information is provided by Samsung to clarify the proposed of NW AI-DPoD for UL.

| AI model input | Input in training | The estimated RS signal time domain sample with 1st, 3rd terms |
| --- | --- | --- |
|  | Input in inference | The estimated data signal time domain sample with 1st, 3rd terms |
| AI model output | Label in training (if applicable) | original/known RS signal |
|  | Output in inference | data signal with non-linear distortion compensated |
| Assumption on training | Training type | Online training/finetune |
|  | Label construction (if applicable) | DMRS with 1st, 3rd terms |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | N/A |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Value. Carrier frequency; 3.5 GHz. # of TX layers/users; 1/2. # of RX antennas; 4/8/64. Waveform; NR based. # of PRB; 50. # of symbol per slot; 12. # of DMRS per slot; 2. # of DMRS re-used for training; 1. DMRS subcarrier position; Odd index (0 for even index). FFT size; 2048. CP length; 144. Modulation; 256 / 1024/ 4096 QAM. Coding rate; LDPC 0.9395. Transport Block size; 45120 for 1Tx/256 QAM 56392 for 1Tx/1024 QAM 67680 for Tx/4096 QAM 90200 for 2Tx/256 QAM. Channel model; TDL-A 30ns. Subcarrier spacing; 15 kHz. Evaluation metric; BLER. Benchmark; Conventional receiver (without non-linearity compensation). |
| Evaluation KPI |  | BLER, MPR, EVM, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD Receiver with non-AI DPoD |
| preliminary evaluation results |  | Detailed results can be provided. |
| High level potential specification impact |  | UE capability report on supported MPR relaxing RAN4 measurement requirement on MPR delta |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Performance benefit: AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values. |

##### Issue 4-1c: Evaluation for AI-DPoD

Agreement:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda), which is intended to discuss the common model as much as possible.

    - Using separate models for AI and non-AI DPoD is not precluded.

- Evaluation Procedures and some detailed discussion points

  - Step-1: Selection of Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity at the base station.

      - FFS suitable AI/ML model, type of training.

      - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

      - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations

      - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

  - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation.

    - The potential MPR reduction enabled by AI-DPoD is evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

- Other design principle to be considered:

  - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

##### Issue 4-1d: Evaluation for AI-DPD

Agreement:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda, which is intended to discuss the common model as much as possible.

# Topic#5: AI/ML use case for Demod (Per-use case discussion)

### Use Case#2: AI-based channel estimation

Agreement:

- Postpone RAN4 discussion on AI-based channel estimation including both sub-use cases (AI-based channel estimation with DMRS reduction and AI-based channel estimation with CSI-RS reduction)

  - If triggered by RAN1, RAN4 will start the study to provide RAN4 analysis and evaluation relevant to implementation and performance.

### Use Case#3: AI-SRS-assisted channel reconstruction

##### Issue 5-2a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI use case of AI-SRS assisted channel reconstruction, will be further discussed in the next meeting.

Ad-Hoc minutes:

QC: If power headroom is available, UE can compensate. This case may only be related to cell edge case. We have question on the specification impact part. Concern on the UE reporting part if NW can do the compensation.

Huawei: SRS with IL imbalance is the identified issue over past releases. It is not corner case. CSI-RS is proposed for training phase, so the reporting on the range of SRS IL is needed.

QC: RAN4 has no consensus on that.

For information:

- The following information is provided by Huawei to clarify the proposed of AI-based SRS IL compensation.

| AI model input | Input in training | UL measured channel matrix from SRS with IL imbalance |
| --- | --- | --- |
|  | Input in inference | UL measured channel matrix from SRS with IL imbalance |
| AI model output | Label in training (if applicable) | UL SRS measurement without IL (assuming it is compensated by UE at certain conditions) or DL CSI-RS measurement |
|  | Output in inference | DL channel matrix with IL compensated |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Channel model: TDL-A XPL medium Training/validation/testing Dataset construction: 100000TTI * 80% (TDD pattern 8：2) * 68 subband (4 RBs per subband) = 5.44M - For training：4.352M（5.44M * 70%） - For validation：0.544M（5.44M * 10%） - For testing：1.088M （5.44M * 20%） |
| model location for inference |  | NW-sided AI |
| Collaboration/interaction between UE and NW |  | 1. Inference: UE reporting on the range of SRS IL for model selection and model monitoring (ensuring generalization/consistency between training and inference) 2. Data collection/monitoring: UE reporting of channel matrix with DL CSI-RS measurement |
| Evaluation methodology |  | Parameters; Values. Carrier frequency/BW; 5GHz/100MHz/272RBs. CSI-RS ports; 32. Tx/Rx number @UE; 1T4R. Channel model; TDL-A XPL medium. Doppler; 1Hz. TDD ULDL pattern; DDDDDDDSUU. Power imbalance distribution; [0, -2.5, -5, -7.5]±1dB. SRS pattern; 4 frequency hopping, 40ms between two SRS hopping. SRS pattern: |
| Evaluation assumption |  | A common AI model is trained which applies to each subband. The model input/output is the channel matrix from one subband. Channel aging issue is not considered in this use case, hence the Doppler valued is set as 1Hz. |
| Evaluation KPI |  | SGCS Steps for SGCS calculation during model inference - Step-1: For each subband, after collecting SRS measurements from each SRS port (i.e., from each Rx antenna), using AI to compensate the IL loss and generate the compensated channel matrix. - Step-2: For each D slot, compare the genie-aided channel matrix at this D slot with the compensated channel matrix based on the nearest SRS measurements. |
| Evaluation benchmark |  | 1) SRS without IL imbalance; 2) non-AI based SRS IL imbalance compensation |
| preliminary evaluation results |  | • AI/ML based compensation outperforms non-AI/ML based compensation across the entire SNR range, with more obvious performance gains observed at medium to low SNRs. • AI/ML is capable of compensating without the exact values – even with random IL variation, only minor performance loss is observed for the generalized model compared with the scenario-specific model trained with exact IL values. Note: Refer to R4-2513310 for more detailed results. |
| High level potential specification impact |  | At least UE reporting on SRS IL range. Potential UE RF related testing. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | CNN-based AI model |

For information:

- The following information is provided by Samsung to clarify the proposed of AI-based SRS-assisted channel reconstruction for both NW-sided model and two-sided model.

|  |  | AI-based SRS assisted channel reconstruction Network-side model | AI-based SRS assisted channel reconstruction Two-sided model |
| --- | --- | --- | --- |
| AI model input | Input in training | CSI feedback and SRS measurement | Explicit channel feedback with spatially sub-sampled CSI measurement SRS measurement for decoder input beside the compression CSI |
|  | Input in inference | CSI feedback and SRS measurement | Explicit channel feedback with spatially sub-sampled CSI measurement SRS measurement for decoder input beside the compression CSI |
| AI model output | Label in training (if applicable) | Ideal CSI with full channel dimension | Ideal CSI with full channel dimension |
|  | Output in inference | The reconstructed CSI with full channel dimension | The reconstructed CSI with full channel dimension |
| Assumption on training | Training type | Offline | Offline |
|  | Label construction (if applicable) | Ideal channel estimation | Ideal channel estimation |
| model location for inference |  | Network-sided | UE and NW sided model |
| Collaboration/interaction between UE and NW |  | Yes | Yes |
| Evaluation methodology |  | Link level simulation | Link level simulation |
| Evaluation assumption |  | Parameter; Value; Parameter; Value. M (measured CSI-RS ports); 64; Channel; UMa Dense-urban. N (NW antenna ports); 256; SRS measurement interval; 30ms. Nr (number of UE receive ports); 4; Mean power imbalance (4 antennas); [0 -3 -6 -9]dB. Carrier frequency/BW; 7 GHz/ 20MHz 13 SBs; Power imbalance distribution; Uniform [-1,1] dB centered on the mean.. |
| Evaluation KPI |  | SGCS, Tput | SGCS, Tput |
| Evaluation benchmark |  | SRS only | CSI feedback only with two sided-model |
| preliminary evaluation results |  |  | The proposed wo-sided model-based approach achieves a significant gain, e.g., close to 100% SGCS gain for layer 1, as compared to the impaired SRS measurement |
| High level potential specification impact |  | Interoperability and testability LCM procedure: including data collection, performance monitoring, and model interference pending the assumption of AI/ML receiver |  |
| Feasibility issues, including complexity, and other aspects related to implementation |  |  |  |

### Use Case#4: AI-enabled PRACH receiver

##### Issue 5-3a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI use case of AI-enabled PRACH will be further discussed in the next meeting.

For information:

- The following information is provided by Samsung to clarify the proposed of AI-PRACH receiver.

| AI model input | Input in training | Detected signal with PDP for each detection window |
| --- | --- | --- |
|  | Input in inference | Detected signal with PDP for each detection window |
| AI model output | Label in training (if applicable) | Known whether there is preamble transmission for each detection window |
|  | Output in inference | Whether there is preamble detected or not |
| Assumption on training | Training type | offline |
|  | Label construction (if applicable) | Known preamble transmission for each detection window |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | N/A |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameter; Values. Channel type; AWGN/TDLC300-100. PRACH format; C0. Ncs; 0/23. Logical index; 0/1. Preamble ID; 0/5. Number of Preamble; 64. Number of UE; 1/2. Doppler Shift; 2334Hz for restrict set B. Number of Tx/Rx; 1/2. Timing offset between 2UE; 20samples. |
| Evaluation KPI |  | Miss detection rate and false detection rate |
| Evaluation benchmark |  | Legacy receiver with threshold-based method |
| preliminary evaluation results |  | For multi-UE scenario, when the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%. For single UE scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches And also, for HST scenario, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region |
| High level potential specification impact |  | 1. RAN 4 requirements including both demodulation requirement and RRM requirement for random access 2. LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver |
| Feasibility, complexity/cost |  | Comparable with conventional threshold-based approaches |

### Other AI-enabled demodulation issues

##### Issue 5-4: RAN4 evaluation assumption and procedures

Tentative agreement to be discussed:

- Other issues for AI-Demod related use cases (only if any AI-Demod use case identified):

  - FFS model driven approach for study of AI receiver for 6G

  - RAN4 to study following aspects when applying CDL models for demodulation requirements definition for AI-based receiver

    - Model variants depending on scenarios covered by AI feature

    - Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

    - Balance between channel model richness and test complexity.

    - How to avoid test signals being used for AI model training.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| Qualcomm | AI/ML usage in demod should be deprioritized and should be discussed in RAN1 first since it depends on many PHY related aspects that need to be agreed in RAN1 first. In our understanding, existing CDL models as defined currently in Rel-19/Rel-20 in RAN4 are deterministic after initialization. Hence, existing CDL models are not suitable models in case of AI/ML based receivers since such a receiver can learn the determinism of the channel easily and results are too optimistic. In our view the applicability of CDL models for AU/ML receivers and potential enhancements of CDL, if possible at all, should be discussed in the demod session first. Regarding the model-driven approach for studying AI receiver for 6G, we believe this topic is more suited for RAN1 and depends on their progresss on the PHY structure. RAN4 should not discuss this topic. |
| Samsung | Considering the suggested WF is just research direction, we can support it. |
| ZTE | Regarding AI based receiver, from our understanding, the receiver assumption is always discussed in RAN4 demod part for phase 1 to 6, such as MMSE, MMSE-IRC R-ML receivers. Thus, why we think RAN4 can study the advanced receiver in 6G. Even if RAN1 currently discussed the AI based DMRS, channel estimation, but we think that still have some potential issues need to be clarified, e.g., only consider AI channel estimation or considering other modules. Therefore, we think that study the model driven approach for AI receiver is helpful to alignment results. |

# Topic#6: AI/ML use case for RRM (Per-use case discussion)

### Use Case#5: AI-based measurement prediction

##### Issue 6-1c: Additional principle/method for AI-RRM use case selection

Tentative agreement to be discussed:

- Additional principle/method for AI-RRM use case selection

  - Whether the proposed AI-RRM use case should be driven by RAN4:

    - Prioritize AI-RRM use cases that may fail to be part of 6G Day 1 feature unless fully studied by RAN4.

    - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study.

    - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| Qualcomm | The following is our understanding of the ultimate goal of this exercise (selecting/studying ‘RAN4-driven’ RRM use case): - RAN4 should study use cases that may fail to make it into Day 1 of 6G unless selected by RAN4. - RAN4 does not need to select use cases that are highly likely to be included in Day 1 of 6G, as other working groups are already discussing them and believe they fit well within their scope. - Note: This is distinct from the debate about whether there is cross-working group impact. - RAN4 should consider use cases, with a higher priority, that are not clearly visible in the RAN1/RAN2 specifications but are covered by the RAN4 RRM specification. With the above in mind, we propose the following update to the FL recommended WF: Additional principle/method for AI-RRM use case selection - Whether the proposed AI-RRM use case should be driven by RAN4: - Select and study use cases that may fail to make it into Day 1 of 6G unless selected by RAN4. - Do not prioritize use cases that are highly likely to be included in Day 1 of 6G, as other working groups are already discussing them and believe they fit well within their scope. - Note: This is distinct from the debate about whether there is cross-working group impact. - Prioritize use cases that are not clearly visible in the RAN1/RAN2 specifications but are covered by the RAN4 RRM specification. |
| CATT | Okay with the principles proposed by FL for AI RRM use case selection. |
| Ericsson | We agreed to avoid overlap with the use cases that are to be driven by other WGs. In this regard the proposed WF asking the proponent companies to bring more analysis specifically focusing on the aspects that RAN4 can study by avoiding the overlap with the use cases that are to be driven by other WG needs to be emphasized. |
| Apple | We support RAN4 should lead AI-based measurement prediction and AI-RRM use cases, as measurement performance, accuracy requirements, measurement gaps, scheduling restrictions, and realistic impairment modeling are core RAN4 responsibilities where RAN4 possesses expertise. AI-based measurement prediction fundamentally addresses measurement quality and prediction accuracy under realistic conditions while potentially reducing measurement gap requirements and associated scheduling restrictions While RAN1/2 may define measurement procedures and reporting mechanisms, RAN4's expertise ensures that AI-based solutions are evaluated against practical deployment scenarios with appropriate requirements on prediction accuracy, gap reduction benefits, and robustness while also tackling generalization issues. We agree that existing 5G-A use cases already in RAN1/2 WIs should be excluded to avoid duplication, but RAN4 should proactively lead new AI-RRM use cases with clear coordination mechanisms with other WGs. |
| ZTE | FL recommended WF is ok for us. |

##### Issue 6-1a: Use case definition and supporting level from companies

Agreement:

- The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

For information:

  - Below table is provided to summarize AI-RRM proposals in RAN4#116bis.

  - For each use case, preference/opposition from companies will be collected in future meeting for down-selection.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((d, e), as RAN1-led study), Apple (c, d, e) |  |  |  |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (c, d, e) |  |  |  |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) |  |  |  |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung |  |  |  |
|  | AI-assisted measurement scheduling | UE-sided model | MediaTek |  |  |  |
|  | Dynamic Adaptation of Measurement Procedure (may or may not transparent to NW) | UE sided model | Qualcomm, Apple |  |  |  |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple |  |  |  |
|  | Spatial-Domain Joint Tx/Rx Beam-Level L3/L1 Prediction | UE sided model (Offline training) | Apple (L1 for scheduling restriction and delay reductions), vivo |  |  |  |
|  | Reduced RX Beam Sweeping Factor in FR2 (L1/L3?) | UE sided model | Qualcomm, Apple, MediaTek, CMCC, Xiaomi |  |  |  |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi |  |  |  |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | OPPO, vivo (AI for RLM) |  |  |  |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO |  |  |  |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | vivo |  |  |  |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE |  |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE |  |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT |  |  |  |

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| Qualcomm | Based on our view of the ultimate goal of this exercise (shared in the previous issue item), the following are our preferred use cases for ‘RAN4-driven’ RRM use case: - (5) and (6): These can be merged, with efforts to identify and leverage common aspects between them. - (7) and (8): These can also be merged, aiming to find shared elements that support a unified approach. |
| CATT | Update the ‘Expected benefits’ of use case 14). Suggest keeping all use cases on table at least until next meeting considering more time may be needed to double check the pros and cons of all proposed use cases. A principle is the selected use cases should provide significant gains (performance gain or gains from other aspects) since these use cases belong to a new generation instead of a further enhancement of NR. |
| MTK | We would like to correct our position on use case #8 (Reduced RX Beam Sweeping Factor in FR2 (L1/L3?)). Now we are a bit conservative considering the testability of UE Rx beam related use cases. We similar view as QC that (5) and (6) can be merged. For the use cases that extends 5G AI mobility use cases from cell-level to beam-level, we think if 5G AI mobility use cases are included in 6G Day-1, it is straight forward to include the beam-level use cases too unless critical issue is found. It seems not necessary to have a study phase for these use cases. |
| Ericsson | We agreed to avoid overlap with the use cases that are to be driven by other WGs. In this regard the proposed WF asking the proponent companies to bring more analysis specifically focusing on the aspects that RAN4 can study by avoiding the overlap with the use cases that are to be driven by other WG needs to be emphasized. |
| Apple | (1) L1 vs L3 beam measurement overlap: We need to clarify the common part between bullet 1) L1 beam measurement and 2) L3 beam measurement; e.g., if 1)-(c)(d)(e) is for inter-cell use case, like LTM, is that quite similar as for 2)-(c)(d)(e)? Do we need to study both use cases in parallel, or we can pick one of them and apply the principles/agreements to the other? For 1)-(c) Freq. and spatial domain (inter-cell): L1 beam measurement is also investigated in RAN1 for predicting beam from FR1 to FR2, shall we avoid the overlap of scope? L1 beam level prediction applies to LTM(L1) and RAN4 could focus on RX beam prediction. For 1)-(d) Spatial domain (inter-cell): Consider together non-collocated with Rx beam domain prediction. For 1)-(e) Time and spatial domain (inter-cell): We believe the new topics are only c,d,e. Also we should exclude topics that are part of WI Re-20. For 3) L3 cell measurement: Why do we have time domain and freq-domain (a),(b) these are already part of Re-19 and Re-20. Shall we only have (c)? (2) Single dimension vs. multiple dimension: We'd better to clarify the timeline for the discussion, i.e., RAN4 needs to figure out the single dimension prediction first (e.g., spatial domain, or frequency domain) and then to further consider the multiple dimension (e.g., spatial + frequency domain). (3) Rx beam prediction prioritization: Since RAN4 driven topics shall be prioritized from the beginning of this 6G SI, we are wondering if 7) "Spatial-Domain Joint Tx/Rx Beam-Level L3/L1 Prediction" and 8) "Reduced RX Beam Sweeping Factor in FR2" can be merged into the discussion in spatial domain related discussion in 1) and 2) because Rx side prediction is also related with spatial prediction in 1) and 2). (4) Application use cases as by-products: For 4), 5), 6), 9)~14), we think they are the by-products after we implementing the AI feature for RRM for possible dimensions, and those RRM enhancement based on AI model (such as RLM/BFD/CBD, SCell, dynamic measurement reductions, measurement scheduling, AGC, DL sync, etc.) can be further discussed after we firstly concluding on the most fundamental part, i.e., measurement prediction. |
| OPPO | For use case 1),2) and 3), we understand the expected benefits can be measurement reduction, gap reduction/throughput improvement, or handover performance improvement based on different prediction approach. But for case d) Spatial domain (inter-cell), more clarifications are needed. It is not clear to us how these gains can be reached if it is a pure spatial domain prediction, since it becomes intra-frequency inter-cell beam prediction. For use case 9) and 10), one of the expected benefits can be delay reduction for the related RRM procedure. For use case 4), we understand it should be multiple domains L3 cell level prediction since the multiple domains L3 beam level prediction is already included in use case 2). It is better to make it clear. |
| ZTE | Update the ‘Expected benefits’ of use case 12) and 13). For use case 12), the output of AGC operation is influenced by multiple factors, such as the Transmission power of NW side at the target cell/frequency, the pathloss of the signal propagation, the setting of LNA at UE side. If partial or all of these factors could be inferred through temporal domain or frequency domain prediction, both the RS overhead and UE operation could be saved. For use case 13), we want to explore whether the DL sync for an intra-frequency or inter-frequency cell/frequency layer could be derived through temporal/frequency domain prediction to reduce the measurement. As to the use cases listed in the table, the important issue is how to avoid duplicated discussions. As QC mentioned in the previous issue, we can first bypass issues that highly likely to be included in Day 1 of 6G, and these might be addressed by other working groups. Based on this goal, maybe we should temporarily skip use cases 1-3. |
| Samsung | For each user case: 1) 2) 3) 9) 11) are required the coordinated discussion with other WGs. For 4), measurement Gap is specific for RAN4. But we also need to consider the coordination with non-AI in RAN4 as well. In non-AI part, the MG has been approved to be studied in 6G SI, there will be enhancements for MG framework not only for Gap pattern ID reduction but also for the whole framework. Currently the proposals are based on 5G MG design. 5) It seems it can be merged to L3 measurement. 5) and 6) can be merged as well. 7) 8) can be RAN4 driven. Even they can be merged, to some extent. 10) 12) 13, it is specific for RAN4 but the mechanism of AI-based compared to non-AI based, it is not clear. 14) not RAN4 case. |

## Reference for RAN4#116bis (Oct. 2025, Prague)

R4-2513031		Discussion on AI/ML-Based 6G RRM Use Cases				Apple

R4-2513042		On RAN4-driven AI/ML aspects of 6GR						Qualcomm Technologies Ireland

R4-2513049		Discussion on AI framework and use cases for 6GR			Samsung

R4-2513060		RAN4 study on 6G AI/ML										Ericsson AB.

R4-2513072		6G Considerations on AI										Nokia

R4-2513079		Views on 6G AI/ML												MediaTek Inc.

R4-2513125		Discussion on AI/ML for 6GR study							CMCC

R4-2513229		Discussion on RAN4 driven AI use cases						CATT

R4-2513283		Overview for 6GR RAN4 centric AI use cases 				Xiaomi

R4-2513286		Views on AI study in 6G										ZTE Corporation, Sanechips

R4-2513293		6G RAN4 Study on AI											NTT DOCOMO, INC.

R4-2513298		Views on 6G AI study RAN4 aspects							China Unicom

R4-2513310		Consideration on RAN4 AI study for 6G						Huawei, HiSilicon

R4-2513318		RAN4 driven AI based use cases								OPPO

R4-2513324		Initial discussion on 6G AI for RAN4 driven cases			vivo