3GPP TSG-RAN WG4 Meeting #116bis	R4-2514788

Prague, Czech, Oct 13th – 17th, 2025

Title: 	Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx

Agenda Item:	6.1.1

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-1: Test applicability rule for 6Rx 4 layers performance requirements

- Agreement:

- Introduce the test applicability rules that 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 = 6.

Issue 1-2: Test applicability rule for 6Rx 6 layers performance requirements

- Agreement:

- Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

- Proposals:

- Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

Issue 1-3: Whether to introduce the test applicability rule that FWA UE can skip 4 layers requirements if it can pass the 6 layers requirements.

- Agreement

- Not introduce such test applicability rule

Issue 1-4: Test applicability rule for support different number Rx antenna ports

- Agreements

- Implement the following proposal into the CR, and further check the CR can be agreeable or not.

  - Corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass.

    - Under the condition that there is a similar test cases for different numbers of UE antennas.

    - Companies to further check which test cases can be skipped.

Issue 1-5: Specification structure

- WF

- Add a new clause for 6Rx after the current 8Rx performance requirements

  - Section 5.2.5 6Rx requirement

2 PDSCH requirements for 6Rx

2.2 Performance requirements for 4 Layers

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE

- Agreement

  - One single test case that is applicable for both FWA and Handheld UE with α = 0 and [β = 0.07179]

2.3 Performance requirements for 6 Layers for FWA

Issue 2-3-1: DMRS configuration for 6 MIMO layers case

- Agreement

- Rel-15 Type 1 DMRS 2+2: DMRS ports {1000-1003, 1004, 1006}

2.4 Summary of test parameters for 6Rx PDSCH tests

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: TDLA30-10 ULA correlation channel with α = 0 and β = 0.07179 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Rel-15 Type 1 DMRS 2+2: {1000-1003, 1004, 1006} |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | 6% |
| Test metric |  |  | SNR@70% max TP |

3 SDR requirements for 6Rx

3.1 6Rx SDR tests

Issue 3-1-1: SDR requirements for 1024QAM with 4 layers

- Agreement

- Not Define SDR requirements for 1024QAM with 4 layers

Issue 3-1-2: MCS value for 256QAM/ 6 layers with scaling factor 1

- Proposals

  - Option 1: 27

  - Option 2: 24

  - Option 3: 25

Issue 3-1-3: MCS value for 64QAM/ 6 layers with scaling factor 1

- Proposals

  - Option 1: 28

  - Option 2: 26

Issue 3-1-4: MCS value for 1024QAM/ 2 layers with scaling factor 1

- Proposals

  - Option 1: 23

  - Option 2: 24

3.2 Summary of test parameters for 6Rx SDR tests

Interested companies can provide the MCS values for scaling factor 1 based on simulation assumptions listed in following tables:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2/4: Single-symbol Rank 6: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000-1003, 1004, 1006} for 6 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| CSI-RS for tracking |  |  | Refer to Table 5.5A-1 in TS 38.101-4 |
| NZP CSI-RS for CSI acquisition |  |  |  |
| ZP CSI-RS for CSI acquisition |  |  |  |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied - For 1T6R (Random precoding) H=111111 - For 2T6R (Random precoding): H=111j-jj111-jj-j - For 4T6R (Random precoding): H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j - For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): - H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 6 1024QAM: 2 |
| Antenna configuration | 2 layers |  | 2x6 |
|  | 4 layers |  | 4x6 |
|  | 6 layers |  | 8x6 |

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| PDSCH configuration | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
| Number of HARQ Processes |  |  | 8 |
| K1 value |  |  | Specific to each UL-DL pattern |
| TDD UL-DL pattern |  |  | 15 kHz SCS: FR1.15-1 30 kHz SCS: FR1.30-1 |
| Note 1: PDSCH is scheduled only on full DL slots, except slot #0, 20 and 21 within 20 ms. |  |  |  |

4 CSI requirements for 6Rx

4.1 6Rx CSI test

Issue 4-1-1: CQI test for 6Rx

- Agreements:

- Define CQI requirements with rank 4 under static channel

  - Propagation channel: AWGN

  - Antenna: 4x6 with static channel specified in Annex B.1

  - RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rules.

Issue 4-1-2: RI test for only Rank 6

- Agreements:

- Not define RI tests for 6Rx

4.2 Summary of test parameters for 6Rx CQI test

Interested companies can provide the simulation results and testing SNR values based on simulation assumptions listed in following table:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 4x6 with static channel H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 5,(9) |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 4,(13) |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration (Note 1) |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1, CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | two-one-TypeI-SinglePanel-Restriction |  | 00010001 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Number of HARQ Processes |  |  |  | 8 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |
| Note 1: The PMI associated to i2 = 0 is always used as the precoder regardless of the reported i2 value. |  |  |  |  |  |  |  |

4 CR split

Companies please provide draft CRs based on following CR split table:

| CR | Content | Companies |
| --- | --- | --- |
| Applicability rules for PDSCH requirements | Applicability of requirements for different number of RX antenna ports in 5.1.1.2 | Nokia |
| PDSCH requirements | Capture PDSCH tests in including 4 and 6 layers in section 5.2.5 | Qualcomm |
| SDR requirements | Capture SDR requirements in section 5.5.1 | Ericsson |
| Applicability rules for CQI requirements | Applicability of requirements for different number of RX antenna ports in 6.1.1.2 | Samsung |
| CQI requirements | Capture CQI requirements in section 6.2.5 | China Telecom |
| FRC for PDSCH requirements | FRC for 6 layer test | ZTE |
| Static channel matrix for 6Rx | Capture new static channel matrix for 6Rx in B.1.14 | MTK |
| MIMO correlation matrix for 6Rx | Capture New beta values and new MIMO correlation matrix in section B.2.3.1 | Apple |
| Big CR introduction of 6Rx demodulation and CSI requirements | Big CR | Huawei |