3GPP TSG-RAN WG4 Meeting #116bis										               R4-2514805

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:			6.12.1

Source:	Moderator (Samsung)

Title:	Ad-hoc minutes for NR_MIMO_demod

Document for:	Approval

- Topic #1: UE demodulation/CSI reporting performance requirements

- Sub-topic 1-1 General scope

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19

- Proposals

  - Option 1: RAN4 should specify PMI reporting requirements for eTypeII-r19 (Samsung, Ericsson, Nokia)

  - Option 2: RAN4 do not specify PMI requirements for eTypeII-r19 (Apple, MediaTek)

- Recommended WF

  - Further online discussion

Discussion:

MTK:check companies’ view first.

QC: prefer not to have requirements but do not have strong view.

Nokia: eTypeII complexity need to be considered.

Huawei: Prefer not to have such requirements but do not have strong view.

Samsung: considering the computional complexity and real deployments with eTypeII for some companies product, we see it’s reasonable to introduce requirements for eTypeII-r19 codebook.

Apple: fine with to define requirements.

Ad-hoc agreements:

RAN4 shall specify PMI reporting requirements for eTypeII-r19.

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

- Proposals

  - Option 1: RAN4 do not specify PMI reporting requirements for typeII-Doppler-r19 (Huawei, Qualcomm, Apple, MediaTek, Ericsson, Nokia)

- Recommended WF

  - Based on majority view, agree on Option 1.

Discussion:

Ad-hoc agreements:

RAN4 do not specify PMI reporting requirements for typeII-Doppler-r19.

- Sub-topic 1-2 Test assumptions for typeI-SinglePanel-r19

Open issues in sub-topic 1-2 are related to test assumptions for typeI-SinglePanel-r19.

Issue 1-2-1: How to solve the very low TP issue with random precoder

- Proposals

  - Option 1: Use the same correlation coefficients as legacy and modify the test to use low correlation with random precoder. (Apple)

  - Option 2: Update the correlation coefficient to be suitable for larger number of TX. (Apple)

- Recommended WF

  - Further online discussion

Discussion:

MTK: Issue 1-2-1 related with Issue 1-2-2. Option 2 is good consideration.

Nokia: for option 1, don’t prefer to use different correlation for following and random precoder. For option 2, ok to consider the simulation results in next meeting, but it may be hard since we only have two meetings left. If trying to go with option 2, different coefficient may be proposed.

QC: with low correlation for random precoder, our results is zero throughput.

HW: prefer to keep the same correlation coefficients. concern about option 2 is hard to reach consensus. Not ok to study option 1 or option 2.

E///: quite high gamma per current simulation results. Ok to study option 1 but have concern about the remaining time.

Apple:since current coefficient for XP-medium comes from legacy, is not aligned with 64 CSI-RS ports. Propose alpha = 0.001.

HW: any technical reason for the new proposed value? How to derive it?

QC: different approach to derive the coefficient.

Options:

Option 1: alpha1 = 0.001 (Apple)

Option 2: new value from calculation like 6 layer way (Apple)

Option 3: keep current value.(HW, QC,E///, Samsung)

Option 4: alpha1=0 (Apple)

Option 5: SNR

Ad-hoc agreements:

Keep current values of correlation coefficients.

Issue 1-2-2: Test metric for 64 CSI-RS ports

- Proposals

  - Option 1: Modify the legacy test metric to following: (Huawei, MediaTek, Ericsson)

- Option 1A: X=5dB (Ericsson)

γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.

  - Option 2: Pick high SNR point (not 90% of maximum throughput point) for stable Gamma (Qualcomm)

  - Option 3: Choose a large Gamma at 90% of maximum throughput SNR point for PMI requirements (Qualcomm)

  - Option 4: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed (MediaTek)

  - Option 5: Use legacy test metric (Samsung)

- Recommended WF

  - Further online discussion

Issue 1-2-3: MCS level for 64 CSI-RS ports

- Proposals

  - Option 1: MCS22 (MediaTek, Samsung)

  - Option 2: MCS20 (Huawei (slightly prefer), Ericsson)

- Recommended WF

  - Further online discussion

Ad-hoc agreements:

Option 1: MCS22

Issue 1-2-4: ZP-CSI-RS and CSI-IM configurations for 64 CSI-RS ports

- Proposals

  - Option 1: Use symbol 7 instead of legacy symbol 9 (MediaTek)

- Recommended WF

  - Further discussion together with Issue 1-2-5 needed.

Issue 1-2-5: NZP-CSI-RS configurations for 64 CSI-RS ports

- Proposals

  - Option 1: Use symbols 2, 3, 5, 6, 9, 10, 12, 13 (MediaTek)

  - Option 2: Use symbols 3, 4, 5, 6, 10, 11, 12, 13 (Samsung)

  - Option 3: RAN4 should avoid scheduling CSI-RS for CSI acquisition in the same slot as TRS scheduling. (Ericsson)

  - Option 4: Use symbols 1, 2, 5, 6, 7, 8, 12, 13 (Nokia)

- Recommended WF

  - Further online discussion. According proposals and observations from all contributions, it seems below configuration is a possible solution, check if below configuration could be agreed.

Ad-hoc agreements:

For the NZP-CSI RS and ZP-CSI configurations, use below configuration

Issue 1-2-6: CQI/RI/PMI delay for FDD 64CSI-RS ports

- Proposals

  - Option 1: Configure CSI report slot offset and PMI delay as 7 slots and 11ms. (Ericsson)

- Recommended WF

  - Background info: Values are as same as last meeting agreement, but 7 has square bracket in the agreement on last meeting.

  - Agree on Option 1.

Ad-hoc agreements:

Configure CSI report slot offset and PMI delay as 7 slots and 11ms.

Issue 1-2-7: CQI/RI/PMI delay for TDD 64CSI-RS ports

- Proposals

  - Option 1: Configure CSI report slot offset and PMI delay as 11 slots and 9ms. (Ericsson)

- Recommended WF

  - Background info: Agreement on last meeting: Apply slot n+6, PMI delay (from the PMI estimation slot to the PMI apply slot) 11.5ms, Report slot offset = 17, for TDD SCS=30kHz. The key difference between option and agreement on last meeting is CSI-RS is transmitted on slot #6 instead of slot #1.

  - Further online discussion

Ad-hoc agreements:

Configure CSI report slot offset and PMI delay as 11 slots and 9ms.

Issue 1-2-8: MIMO correlation matrices

- Proposals

  - Option 1: Not to specify the concrete MIMO correlation matrices for 64Tx in the spec (Huawei)

- Recommended WF

  - Agree on Option 1, just to extend antenna correlation matrix models as agreement of issue 1-2-5 in RAN4#115 meeting, not to specify the detailed correlation matrices for 64Tx since it’s too large.

Issue 1-2-9: round-off operation of MIMO correlation matrices

- Proposals

  - Option 1: Include round-off to four digits precision operation (Huawei)

- Recommended WF

  - Check if Option 1 could be agreed.

Issue 1-2-10: Scaling factors for MIMO correlation matrices

- Proposals

  - Option 1: Adopt the following table of scaling factors for MIMO correlation matrices (Huawei)

| MIMO correlation | 64x2 | 64x4 |
| --- | --- | --- |
| Medium | 0.00041 | 0.00062 |
| High | 0.00032 | 0.00065 |

  - Option 2: The ‘a’ scaling factor should be chosen based on MIMO correlation coefficient and antenna config. (Apple)

  - Option 3: Not to specify or use scaling factors for MIMO correlation matrices (MediaTek)

  - Option 4: Use scaling factor a to guarantee channel correlation matrix as a positive semi-definite matrix, for 64Tx (8,4,2) 4Rx, a=0.00091 and for 64Tx (8,4,2) 2Rx, a=0.0005. (Samsung)

  - Option 5: Adopt the following scaling factors for 64- and 128-port High and Medium correlation level matrices to ensure PSD matrices with a measure of numerical stability: (Nokia)

| Array Type | Antenna Configuration | Correlation Level | Scaling Factor |
| --- | --- | --- | --- |
| Cross-polarized | 64 (8,4,2)×2 | High | 0.00073 |
| Cross-polarized | 64 (8,4,2)×4 | High | 0.00145 |
| Cross-polarized | 64 (8,4,2)×2 | Medium | 0.00065 |
| Cross-polarized | 64 (8,4,2)×4 | Medium | 0.00092 |
| Cross-polarized | 128 (16,4,2)×2 | High | 0.00143 |
| Cross-polarized | 128 (16,4,2)×4 | High | 0.00249 |
| Cross-polarized | 128 (16,4,2)×2 | Medium | 0.00124 |
| Cross-polarized | 128 (16,4,2)×4 | Medium | 0.00196 |

- Recommended WF

  - Further online discussion

Issue 1-2-11: Additional codebook mode for typeI-SinglePanel-r19?

- Proposals

  - Option 1: Specify additional test case for typeI-SinglePanel-r19, with typeI-codebookMode-r19 as Scheme B. (Samsung)

- Recommended WF

  - Further online discussion

Issue 1-2-12: Whether to specify requirements for 128 CSI-RS ports?

- Proposals

  - Option 1: No (MediaTek)

  - Option 2: Yes (Ericsson, Nokia)

- Recommended WF

  - Further online discussion

Issue 1-2-13: Number of slots and density for 128 CSI-RS ports (if introduced)

- Proposals

  - Option 1: one slot with density 1/2 (Nokia)

- Recommended WF

  - Agree on option 1.

Issue 1-2-14: Row configuration of CSI-RS resources for 128 CSI-RS ports (if introduced)

- Proposals

  - Option 1: RAN4 to use row#18 of Tabel 7.4.1.5.3-1 for 128 port CSI-RS transmission. (MediaTek)

  - Option 2: Same row configuration of CSI-RS resources as 64 CSI-RS ports, i.e., row#17 of TS38.211 Table 7.4.1.5.3-1. (Samsung, Nokia)

- Recommended WF

  - Check if Option 2 could be agreed.

Issue 1-2-15: typeI-codebookMode-r19 for 128 CSI-RS ports (if introduced)

- Proposals

  - Option 1: Mode A (MediaTek, Nokia)

  - Option 2: both Scheme A and Scheme B (Samsung)

- Recommended WF

  - Follow the agreement of Issue 1-2-11.

Issue 1-2-16: CQI/RI/PMI delay for TDD 128 CSI-RS ports (if introduced)

- Proposals

  - Option 1: Configure CSI report slot offset and PMI delay as 21 slots and 14ms. (Ericsson)

- Recommended WF

  - Follow the agreement of Issue 1-2-7.

Issue 1-2-17: Test metric for 128 CSI-RS ports (if introduced)

- Proposals

  - Option 1: Use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding with Rel-19 Type I codebook. For example, X=5dB. (Ericsson)

- Recommended WF

  - Follow the agreement of Issue 1-2-2.

- Sub-topic 1-3 Test assumptions for eTypeII-r19 (if introduced)

Issue 1-3-1: MCS level

- Proposals

  - Option 1: MCS22 (Samsung)

- Recommended WF

  - Follow the agreement of Issue 1-2-3.

Issue 1-3-2: MIMO correlation matrixes

- Proposals

  - Option 1: Same as typeI-SinglePanel-r19 (Apple)

- Recommended WF

  - Agree on Option 1.

Issue 1-3-3: Test metric for 64 CSI-RS ports

- Proposals

  - Option 1: Use throughput ratio (γ=tue/trnd) as the test metric, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue+XdB with random precoding with Rel-19 Type I codebook. For example, X=5dB. (Ericsson)

- Recommended WF

  - Follow the agreement of Issue 1-2-2.

Note: For eType II codebook, randomization of the dual-cluster beam directions shall be used as specified in Annex B.2.3.2.3A.

- Sub-topic 1-5 Test assumptions for SRS port grouping (if introduced)

Issue 1-5-1: Test Approaches for SRS port grouping

- Proposals

  - Option 1: Approach 1 (Huawei)

Step 1: For simplicity, map each layer to one Tx with same index, i,e, map layer1 to Tx1, layer2 to Tx2, … , map Layer N to Tx N, which can be done by applying identity matrix precoding.

Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities:

-	For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port.

-	For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports.

-	For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports

TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna.  The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively.

Step 3: In TDD pattern n+1, TE updates the channel matrix to following according to the measurement of TDD pattern n

Hi,j=1,  if i  equal to SRS port index which UE antenna j maps to 0,  else

  - Option 2: Approach 2 (Huawei)

TE use static channel matrix and update precoding per TDD pattern. The concrete test setup is shown as follows:

Step 1: Select static channel H defined in B.1 in TS 38.101-4 as testing propagation channel model

Step 2: In each TDD pattern n, SRS is transmitted in last two UL slots, the SRS configuration shall cover all UE antenna switching capabilities:

-	For 1T8R, configure one SRS set having 8 SRS resources transmitted in different symbols, each SRS resource has a single SRS port.

-	For 2T8R, configure one SRS set having 4 SRS resources transmitted in different symbols, each SRS resource has 2 SRS ports.

-	For 4T8R, configure one SRS set having 2 SRS resources transmitted in different symbols, each SRS resource has 4 SRS ports

TE identifies the mapping between SRS and UE antenna index by measuring the power of each antenna.  The mapping index is denoted as [a0,a1,a2,…,a7], which means UE antenna index a0,a1,a2,…,a7 is related to SRS port index 0,1,2,…,7 respectively.

Step 3: In TDD pattern n+1, TE updates the precoder to following with zero forcing algorithm to cancel the interference between two 4Rx.

W=HHHH-1

Where H=H1,H2, H1 (8x4) is channel matrix from TE to UE antennas with index a0,a1,a2,a4, i,e, SRS port index 0,1,2,3, H2 (8x4) is channel matrix from TE to UE antennas with index a5,a6,a7,a8, i,e, SRS port index 4,5,6,7. The columns of H1 and H2 are columns a0,a1,a2,a4 and a5,a6,a7,a8 of propagation conditions H respectively.

- Recommended WF

  - Further online discussion.

Issue 1-5-2: Applicability rules

- Proposals

  - Option 1: Define the applicability rules as the test is applicable to UE supporting both “Simplified 8Rx receiver” and “SRS grouping”. (Huawei)

- Recommended WF

  - Agree on Option 1.

- Sub-topic 1-6 Test assumptions for CJT calibration (if introduced)

Issue 1-6-1: How to define CJT calibration offset reporting requirements

- Proposals

  - Option 1: Consider a FDD setup for the test, including PMI calculation and offset estimation at the UE, as well as PDSCH precoding and offset compensation at the testing equipment side. Baseline may be obtained from non-compensated/random compensation of the CJT offsets and their respective resulting PMI. (Nokia)

- Recommended WF

  - Further online discussion

Issue 1-6-2: Whether the PMI calculation is necessary for the CJT calibration performance requirements

- Proposals

  - Option 1: Use the non-compensated CJT offset case with resulting non-compensated follow PMI as comparison baseline. (Nokia)

- Recommended WF

  - Agree on Option 1.

Issue 1-6-3: Test metric for CJT

- Proposals

  - Option 1: As a starting point, use follow PMI when defining the new requirements. (Nokia)

- Recommended WF

  - Agree on Option 1.

- Topic #2: BS demodulation performance requirements

- Sub-topic 2-1 General scope

Issue 2-1-1: Clarify if demodulation requirements should be specified for 3-antenna-port codebook-based UL transmissions

- Proposals

  - Option 1: RAN4 should specify PUSCH demodulation requirements for 3-antenna-port codebook-based UL transmissions (Samsung, Ericsson, Nokia)

  - Option 2: RAN4 do not introduce requirements for 3-antenna-port codebook-based UL transmissions (Huawei)

- Recommended WF

  - Check if Option 1 could be agreed.

- Sub-topic 2-2 Test assumptions for PUSCH demodulation requirements

Issue 2-2-1: SCS and CHBW for FDD

- Proposals

  - Option 1: 15kHz SCS, 5MHz, 50MHz (Samsung, Ericsson)

- Recommended WF

  - Based on simulation results, it seems Option 1 could be agreed.

Issue 2-2-2: SCS and CHBW for TDD

- Proposals

  - Option 1: 30kHz SCS, 10MHz, 100MHz (Samsung, Ericsson)

- Recommended WF

  - Based on simulation results, it seems Option 1 could be agreed.