3GPP TSG-RAN WG4 Meeting #116bis	R4-2514814

Prague, Czech Republic, 13 - 17 October 2025

Agenda item:			6.17.1

Source:	Nokia

Title:	Ad-hoc minutes for NR_Mob_Ph4_RRM

Document for:	Information

# Ad-hoc issues

Remaining issues after ad-hoc:

Topic 1: RRM core requirements maintenance:

-	Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM

Topic #2 Event triggered L1 measurement reporting performance part

-	Issue 2-1-4: Test case list for event-triggered L1 reporting

Topic #3 CSI-RS based L1 measurement Performance part

-	Issue 3-1-3: Test case list for CSI-RS based L1 measurement 4

Topic #4: CLTM and Sub-sequent LTM performance part

- Issue 4-1-1: Test case list for CLTM

-	Issue 4-2-1: Test case list for Subsequent CLTM

1	Topic #1 RRM Core requirements

Companies’ contributions summary

Draft CRs to RRM core requirements

1.1	Event triggered L1 measurement reporting

Issue 1-1-1: Whether to introduce/update requirements for reporting criteria per measurement category

Issue 1-1-2: Impact on DL synchronization

1.2	CSI-RS based L1 measurement

Issue 1-2-1: Capturing CSI-acquisition note in cell switch delay

Issue 1-2-2: Clarification on CSI-acquisition frequency location

1.3	Conditional Intra-CU LTM

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM

Issue 1-3-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

Issue 1-3-3: CLTM requirements for candidate cells without active TCI states in FR2

2	Topic #2 Event triggered L1 measurement reporting performance part

Companies’ contributions summary

Issue 2-1-1: Testing approach for SSB-based L1 event triggered measurement reporting

Issue 2-1-2: Testing approach for CSI-RS-based L1 event triggered measurement reporting

Issue 2-1-3: Supported events in test cases

Issue 2-1-4: Test case list for event-triggered L1 reporting

3	Topic #3 CSI-RS based L1 measurement Performance part

Companies’ contributions summary

Issue 3-1-1: Whether to define test cases for FR1 and FR2:

Issue 3-1-2: Test configuration for CSI-RS based L1 measurement test cases

Issue 3-1-3: Test case list for CSI-RS based L1 measurement

Issue 3-1-4: Scenarios to test with and without step 2 (Rel-18 SSB measurements):

Issue 3-1-5: SS/CSI-RSRP measurement L1 report mapping

Issue 3-1-6: Conditions when UE supports event triggered reporting and traditional reporting

Issue 3-1-7: Whether to test CSI-acquisition

Issue 3-1-8: Accuracy requirements for CSI-RS based L1-RSRP candidate cell

4	Topic #4: CLTM and Sub-sequent LTM

Companies’ contributions summary

4-1 CLTM Performance part

Issue 4-1-2: frequency range

Issue 4-1-3: intra/inter-frequency

Issue 4-1-5: RACH-less and RACH-based CLTM

Issue 4-1-6: PCell/PSCell

Issue 4-1-7: early DL sync

Issue 4-1-8: PDCCH ordered RACH

Issue 4-1-4: L1/L3 based CLTM

Issue 4-1-1: Test case list for CLTM

4.2  Subsequent CLTM

Issue 4-2-1: Test case list for Subsequent CLTM

Issue 4-2-2: Others

Relevant topic summaries:

R4-2513517	Topic summary for [116bis][211] NR_Mob_Ph4_RRM_Part1

R4-2513518	Topic summary for [116bis][212] NR_Mob_Ph4_RRM_Part2

# Topic #1 RRM Core requirements

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2513102 | Maintenance for NR mobility enhancements Phase 4 | OPPO | Observation 1: It is quite hard to converge to a number for event triggered L1 reporting criteria. Proposal 1: Compromise to not define any new requirements for event triggered L1 reporting criteria. |
| R4-2513121 | RRM Core requirements on NR mobility enhancements Phase 4 | China Telecom | Proposal 1: Discuss whether to include unknown cell case on SSB triggered CLTM, i.e., whether TPSS/SSS_sync_intra + TSSB_measurement_period_intra are needed. If so, the conditions that TSSB_time_index_inter = 0 maybe need to be updated. |
| R4-2513487 | Discussion on Event triggered L1 measurement reporting | Huawei, HiSilicon | Proposal 1: No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command doesn’t represent that the fine timing tracking by L1 measurement on candidate cell is performed within 160ms. |
| R4-2513647 | Discussion on event triggered L1 measurement reporting | CMCC | Proposal 1: no strong view on whether to define capability for supported number of events triggered L1 measurement reporting in RAN4. But if it is agreed to define, it is proposed to define a separate capability for supported number of events triggered L1 measurement reporting, to avoid the impact on L3 measurement reporting. |
| R4-2513895 | Discussion on Event triggered L1 measurement reporting | ZTE Corporation, Sanechips | Proposal 1: Define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM if the number of reporting criteria for LTM event triggered reporting can be unified. Otherwise, not introduce/update requirements for reporting criteria per measurement category. |
| R4-2514078 | Remaining issues for mobility enh phase 4 | vivo | Proposal 1: RAN4 to decide following alternatives to define reporting criteria requirements for LTM. Alt 1: Reporting criteria for LTM event triggered reporting is 48 in total for all configured L1 measurements for LTM. Alt 2: It is clarified in the spec that UE should perform all the event reporting and periodic reporting as configured by network. |
| R4-2514297 | Discussions on remaining open issue in Mobility Enh. Ph4 | Ericsson | Proposal 1: As long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48, RAN4 to discuss whether there should be any defined maximum or minimum number of parallel L1-triggered event instances that a UE can evaluate and run for each configured event or across all reporting configurations. |
| R4-2514372 | Discussion on remaining issues of Rel-19 Mobility Enhancements core part | Nokia | Proposal 1: Capture the following as a note in the specification: “Note: If fine T/F tracking is included in cell switch delay, UE is not expected to start CSI acquisition before the UE finishes fine T/F tracking of the target cell. Otherwise, UE is not expected to start CSI acquisition before the end of TLTM-processing.” Proposal 2: Discuss whether there is need to define applicability condition for CSI acquisition depending on RS frequency location (serving cell active BWP / initial target cell BWP) Proposal 3: RAN4 to capture the agreement about CLTM requirements for candidate cells without active TCI states in FR2 under the measurement time section (6.3.2.2.1). The following wording can be used: “The requirements in this clause apply under the following conditions: 1> The target cell is an FR1 cell, or 1> The target cell is an FR2 cell, and 2> there is at least one TCI state on the active TCI state list of the target cell, or 2> there are no TCI states on the active TCI state list of any of the configured candidate cells.” Proposal 4: RAN4 to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event. Observation 1: When L1 measurements are configured, UE is required to keep track of DL sync for 160 ms after TCI state activation, unless the measurement period is up to 160 ms. Observation 2: When L1 measurements are not configured (LTM is L3-based), UE is required to keep track of DL sync for 480 ms after TCI state activation. Observation 3: Values for TA timer are: 500 ms, 750 ms, 1280 ms, 1920 ms, 2560 ms, 5120 ms, 10240 ms, infinity. Proposal 5: Tfirst-RS = 0 and TRS-proc = 0 if the UE, supporting early DL synchronization through ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18, has received a TA MAC-CE for the target cell based on PDCCH-ordered RACH preamble transmission, and the associated TA timer is running. |

Draft CRs to RRM core requirements

| TDoc | Title | Source | Sections impacted |
| --- | --- | --- | --- |
| R4-2513187 | Draft CR to TS 38.133 on core requirements for Rel-19 mobility enhancements | CATT | 6.3.2.2.1, 9.14a.3.4, 9.14a.3.5, 9.14a.7 |
| R4-2513488 | Corrections on R19 mobility RRM | Huawei, HiSilicon | 9.14a.5 |
| R4-2513947 | DraftCR for Conditional Intra-CU LTM | Nokia | 6.3.2.2, 6.3.2.2.1, 6.3.2.2.2, 6.3.2.2.3, 6.3.2.3 |
| R4-2514052 | DraftCR for LTM event-triggered L1 reporting | Nokia | 6.3.1.1, 8.20.1, 8.25.1, 9.14.3.4, 9.14.3.5, 9.15.3.4, 9.15.3.5 |
| R4-2514373 | DraftCR for LTM CSI-RS applicability | Nokia | 9.14a.2 |
| R4-2514384 | Correction to reporting criteria requirements for LTM event triggered reporting | vivo | 9.1.4 |

## Event triggered L1 measurement reporting

Issue 1-1-1: Whether to introduce/update requirements for reporting criteria per measurement category

Technical background: In the previous meeting, the following has been discussed and agreed:

Candidate options: - Option 1: define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM (CATT, Nokia, CTC, OPPO, Xiaomi, MTK, CMCC, vivo, ZTE, E///) - Option 1a: Introduce reported UE capabilities for supported number of events triggered L1 measurement reporting. The candidate values are {4, 8, 12, 16}. (MTK) - Option 1b: vivo - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1c: E/// - RAN4 to agree 72 as the L1 measurement reporting criteria for a UE supporting event triggered measurement reporting - Option 2: extend existing Ecat to support LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4. (OPPO) Agreement: - No consensus on the necessity to introduce/update requirements for reporting criteria per measurement category. - The issue does not impact the WI core part completion, and interested companies can further discuss this issue in maintenance phase based on contribution driven.

Proposals:

- Proposal 1: (Oppo): Compromise to not define any new requirements for event triggered L1 reporting criteria.

- Proposal 2: (CMCC): no strong view on whether to define capability for supported number of events triggered L1 measurement reporting in RAN4. But if it is agreed to define, it is proposed to define a separate capability for supported number of events triggered L1 measurement reporting, to avoid the impact on L3 measurement reporting.

- Proposal 3 (ZTE): Define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM if the number of reporting criteria for LTM event triggered reporting can be unified. Otherwise, not introduce/update requirements for reporting criteria per measurement category.

- Proposal 4 (Vivo): RAN4 to decide following alternatives to define reporting criteria requirements for LTM.

  - Alt 1: Reporting criteria for LTM event triggered reporting is 48 in total for all configured L1 measurements for LTM.

  - Alt 2: It is clarified in the spec that UE should perform all the event reporting and periodic reporting as configured by network.

- Proposal 5 (Ericsson): As long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48, RAN4 to discuss whether there should be any defined maximum or minimum number of parallel L1-triggered event instances that a UE can evaluate and run for each configured event or across all reporting configurations.

Recommended WF:

- Discuss the following:

- Option 1: Introduce capability

  - Option 1.1: Define a separate capability for supported number of events triggered L1 measurement reporting, avoid the impact on L3 measurement reporting

  - Option 1.2: Reporting criteria for LTM event triggered reporting is 48 in total for all configured L1 measurements for LTM.

  - Option 1.3: As long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48.

    - RAN4 to discuss whether there should be any defined maximum or minimum number of parallel L1-triggered event instances that a UE can evaluate and run for each configured event or across all reporting configurations.

- Option 2: Not to introduce capability

  - Option 2.1: Expected UE behaviour is that the UE shall perform the event reporting and periodic reporting as configured by network and based on RAN1/2 defined signalling and capabilities.

Discussion:

Ericsson: If network configures 48 events, multiple beams satisfy the event, what is the UE behaviour? Does UE report all the beams satisfying the events.

Vivo: Events are configured per cell, not per beam. If network configures 48, UE reports 48.

Ericsson: Maximum allowed configuration number in RAN2 is 48

Tentative Agreement:

- Not to introduce additional UE capability.

- Common understanding is that as long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48, there are no limitations on maximum number of events instances UE can run in parallel. No specification impact is expected.

Issue 1-1-2: Impact on DL synchronization

Technical background: In the previous meetings, the following issue has been discussed:

#### R4-2509066 Issue 2-4: Impact on DL synchronization Candidate options: - Option 1: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command. (Nokia) Recommended WF: - Discuss the option.

Proposals:

- Proposal (Huawei): No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command doesn’t represent that the fine timing tracking by L1 measurement on candidate cell is performed within 160ms.

Recommended WF:

- Discuss the following options:

  - Option 1: No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command doesn’t represent that the fine timing tracking by L1 measurement on candidate cell is performed within 160ms.

  - Option 2: Add new condition.

Discussion:

- HW: 160ms before cell switch command doesn’t represent that the fine timing tracking, no need to introduce the condition. The measurement result may also be coming from neighbouring cell, hence no fine T/F tracking is expected

- Ericsson: We would also need to capture measurement periodicity for candidate cell

- HW: Measurement periodicity is already captured in the spec.

- Nokia: If TCI state is active, and event triggered report is transmitted.

- HW: UE reports whether the TCI state is active or not.

Tentative agreement:

- No need to add new condition for Tfirst-RS and TRS-proc = 0 in cell switch delay, because the event triggered report within 160ms before cell switch command

## CSI-RS based L1 measurement

Issue 1-2-1: Capturing CSI-acquisition note in cell switch delay

Technical background:

Agreement: - No consensus on defining new RAN4 requirements for early CSI acquisition. - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached. - It is RAN4 common understanding that for baseline early CSI acquisition, - If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell. - Otherwise, UE shall start CSI acquisition at least after Tprocessing. - No need to capture above agreement in RAN4 spec.

Proposals:

- Proposal 1 (Nokia): Capture the following as a note in the specification: “Note: If fine T/F tracking is included in cell switch delay, UE is not expected to start CSI acquisition before the UE finishes fine T/F tracking of the target cell. Otherwise, UE is not expected to start CSI acquisition before the end of TLTM-processing.”

Recommended WF:

- Discuss the above.

Discussion:

- Vivo: No need for note

- Oppo: Is legacy UE allowed to do CSI-acquisition.

- No consensus to add a note.

Tentative agreement:

- Common understanding is clear in RAN4 related to the timeline of CSI-acquisition. No need to add note to the specification and we can close the issue.

Issue 1-2-2: Clarification on CSI-acquisition frequency location

Proposals:

- Proposal 1: (Nokia): Discuss whether there is need to define applicability condition for CSI acquisition depending on RS frequency location (serving cell active BWP / initial target cell BWP)

Recommended WF:

- Discuss the above.

Discussion:

- Ericsson: no requirements defined so no need to capture anything.

Tentative agreement:

- No need to further discuss about the frequency location of the CSI-acquisition RS in this WI.

## Conditional Intra-CU LTM

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM

Technical background: RAN4 previously agreed that specifications only cover the known cell case for CSI-RS L1 triggered CLTM. But for SSB L1 triggered CLTM, no agreement had been reached.

Way forward in RAN4#115 For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. Agreement in RAN4#116 Issue 1-1-4: Whether to include unknown cell case < Agreement > - Only cover the known cell case.

Moderator comment: The following is included in the CR:

1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

Proposals:

- Proposal 1 (CTC): Discuss whether to include unknown cell case on SSB triggered CLTM, i.e., whether TPSS/SSS_sync_intra + TSSB_measurement_period_intra are needed. If so, the conditions that TSSB_time_index_inter = 0 maybe need to be updated.

- Proposal 2 (Nokia): RAN4 to distinguish between known and unknown target cell cases in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

Recommended WF:

Discuss the following:

- For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported:

  - Option 1: Support only known case (specification change needed)

  - Option 2: Support both unknown and known case (specification change needed)

  - Option 3: Support only unknown case (no specification change).

Discussion:

- CTC: We prefer to define only known case, support option 1. Motivation to support CLTM is to reduce latency for cell switch.

- Oppo: Support option 1 also.

- Nokia: In CLTM we support known target cell. UE evaluating the condition may have known or unknown cell. Hence, we support option 2.

- Ericsson: We support option 3, case.

- MTK: we would like to have a clarification on why to have option 2. It seems like option 2 can cover unknown case in the spec.

- Vivo: We may not need known / unknown terminology for CLTM.

- Ericsson: No need to change existing specification.

- MTK: We would like to check legacy understanding from the companies for conditional handover.

- QC: Only issue is whether there was measurement report 5s ago or not.

Tentative agreement:

- Further check this issue.

Issue 1-3-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

Technical background:

- Legacy LTM defines how long UE is required to maintain DL sync after the TCI state activation: - When L1 measurements are configured, UE is required to keep track of DL sync for 160 ms after TCI state activation, unless the measurement period is up to 160 ms. - When L1 measurements are not configured (LTM is L3-based), UE is required to keep track of DL sync for 480 ms after TCI state activation.

Proposals:

- Proposal 1 (Nokia): Tfirst-RS = 0 and TRS-proc = 0 if the UE, supporting early DL synchronization through ltm-MAC-CE-JointTCI-r18 or ltm-MAC-CE-SeparateTCI-r18, has received a TA MAC-CE for the target cell based on PDCCH-ordered RACH preamble transmission, and the associated TA timer is running.

Recommended WF:

Discussion:

- Vivo: how this is related to TA timer?

- Nokia: the discussion is about when the TCI state is active, and UE receives PDCCH order and receives TA, how long will UE maintain the TA.

- Ericsson: TAT timer can be infinite also. This would mean long time tracking.

- MTK: We would like to check if PDCCH ordered RACH is rel-18 version or not?

- QC: It’s different to R18 PDCCH order because there is no Cell switch command

- MTK: We may need to further update capaiblities based on this.

- Apple: TA timer will only tell validity of the TA, no relation to DL sync.

- QC: Is this L3 based only? – We don’t support the proposal.

Tentative agreement:

- No need to add additional condition for Tfirst-RS = 0 and TRS-proc = 0 in R19 CLTM.

Issue 1-3-3: CLTM requirements for candidate cells without active TCI states in FR2

Technical background: Currently it is still open whether and how to capture the agreement on CLTM requirements for candidate cells without active TCI states in FR2 in the CR.

Issue 1-2-3: CLTM requirements for candidate cells without active TCI states in FR2 < Agreement > - Capture the following in the WF. - As starting point: L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply. - FFS whether and how to capture in the CR

Proposals:

- Proposal 1 (Nokia): RAN4 to capture the agreement about CLTM requirements for candidate cells without active TCI states in FR2 under the measurement time section (6.3.2.2.1). The following wording can be used:

“The requirements in this clause apply under the following conditions:

1> The target cell is an FR1 cell, or

1> The target cell is an FR2 cell, and

2> there is at least one TCI state on the active TCI state list of the target cell, or

2> there are no TCI states on the active TCI state list of any of the configured candidate cells.”

Tentative agreement:

- Capture agreement, discuss further in CR.

# Topic #2 Event triggered L1 measurement reporting performance part

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2513084 | Discussion on test cases for event triggered L1 measurement reporting | MediaTek Inc. | Observation 1: For L3 measurement, RAN4 only defined test cases for event-triggered reporting. Proposal 1: For event-triggered L1 reporting, define the following two test cases: - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1 - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2 |
| R4-2513108 | On RRM performance requirements of event triggered L1 measurement reporting | OPPO | Observation 1: For L3 measurement, RAN4 only defined test cases for event-triggered reporting. Proposal 1: For event-triggered L1 reporting, define the following two test cases: - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1 - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2 |
| R4-2513118 | RRM Perf requirements on event triggered L1 measurement reporting | China Telecom | Proposal 1: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1. - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2. - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1. Proposal 2: Event LTM3 is best to verify event-triggered L1 reporting, For Event LTM5, discuss that if it is necessary. |
| R4-2513196 | Discussion on performance requirements for event triggered L1 measurement reporting in Rel-19 LTM | CATT | Proposal 1: Test cases for event triggered L1 measurement reporting need to cover SSB based and CSI-RS based measurements. Proposal 2: The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM. Proposal 3: Regarding the direction of defining test case for SSB based event triggered L1 measurement reporting: - Keep R18 test cases and develop necessary new R19 test to verify reporting behaviour. Proposal 4: Regarding the direction of defining test case for CSI-RS based measurements: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1) Proposal 5: For event triggered SSB based L1 measurement reporting, the folloeing new test cases can be defined: - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX - TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading Proposal 6: For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined. |
| R4-2513489 | Discussion on performance requirements of Event triggered L1 measurement reporting | Huawei, HiSilicon | Proposal 1: The following test cases are specified for SSB based L1 measurement event trigger reporting: - FR1 and FR2 SSB based LTM Intra-frequency event triggered reporting tests without gap - FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests with gap - FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests without gap Proposal 2: UE which can pass the above R19 test cases can skip corresponding R18 SSB based L1 measurement test cases. Proposal 3: The following test cases are specified for CSI-RS based L1 measurement event trigger reporting: - FR1 CSI-RS based LTM Intra-frequency event triggered reporting tests without gap - FR2 CSI-RS based LTM Intra-frequency event triggered reporting tests without gap Proposal 4: For event triggered reporting tests for CSI-RS L1 RSRP measurement, “3-step” CSI-RS measurement procedure is to be verified. Proposal 5: UE which can pass CSI-RS based L1 measurement event trigger test cases can skip the test sets of CSI-RS based L1 measurement without reporting. |
| R4-2513622 | Discussion on RRM performance requirements for Event triggered L1 measurement reporting | Apple | Proposal 1: For SSB based L1 RSRP measurement reporting, the following approach 1 is slightly preferred: - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. Proposal 2: For CSI-RS based L1 RSRP measurement reporting: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1) Proposal 3: testing LTM event 3 is enough. However, we are also fine with using other event types in different test cases as long as it won’t result in extra testing time. |
| R4-2513644 | Discussion on RRM performance requirements for event triggered L1 measurement reporting | CMCC | Proposal 1: for event triggered L1 measurement reporting, it is proposed to define test cases for SSB based measurement and CSI-RS based measurements. - Both FR1 and FR2-1 are considered. - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered. - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered. Proposal 2: it is proposed to define test cases for LTM event 2/3/4/5 . |
| R4-2513898 | Discussion on performance requirements for event triggered L1 measurement reporting | ZTE Corporation, Sanechips | Proposal 1: For SSB based L1 RSRP measurement reporting, prefer to duplicate all R18 test cases and add event triggered reporting in R19, and UE which can pass R19 test cases can skip R18 test cases. Observation 1: For CSI-RS based L1 RSRP measurement reporting, reporting behavior can be verified by adding event triggered reporting to all test cases that introduced for CSI-RS based L1-RSRP measurement in R19. Proposal 2: For CSI-RS based L1 RSRP measurement reporting, test cases are introduced to jointly verify CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting. |
| R4-2514055 | Discussion on LTM event-triggered reporting test cases | Nokia | Proposal 1: Introduce the following test cases for event-triggered LTM L1 reporting: - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps) - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps) - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps) - At least 1 test case should test event-triggered periodic reporting (e.g. FR1 Intra-frequency SSB-based event-triggered L1 reporting) Proposal 2: For SSB based measurement reporting, if the UE passes the Rel-19 event-triggered L1 reporting test case, the UE can skip the corresponding Rel-18 test case. Proposal 3: If the UE supports both Rel-19 CSI-RS measurement capability and Rel-19 event-triggered reporting capability for CSI-RS measurement, UE only needs to pass the event-triggered CSI-RS reporting test case. Proposal 4: Use LTM3 as the reporting event in the test cases. |
| R4-2514079 | On performance requirements for LTM event triggered reporting | vivo | Proposal 1: Define following test cases to verify whether UE can meet event triggered reporting requirements, covering SSB-based L1 measurement and CSI-RS based L1 measurement in FR1 and in FR2, respectively. For SSB-based measurement, Intra-frequency measurement without gap in FR1 for event LTM3 Inter-frequency measurement with gap in FR1 for event LTM4 Intra-frequency measurement without gap in FR2 for event LTM3 Inter-frequency measurement with gap in FR2 for event LTM4 For CSI-RS-based measurement, Intra-frequency measurement without gap in FR1 for event LTM3 Intra-frequency measurement without gap in FR1 for event LTM3 |
| R4-2514300 | Discussion on performance requirements for event-triggered L1 measurement reporting | Ericsson | Proposal 1: At least Event LTM3 and Event LTM4 be included in the RRM testcase. Some of the agreed test cases can be tested with Event LTM3 and some of the test cases with Event LTM4. Proposal 2: If the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, it is sufficient to test the event-triggered reporting and skip the Rel-18 LTM L1-RSRP test cases. Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting. Intra-f L1-RSRP measurement for LTM; - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated. Inter-f L1-RSRP measurement with MG; - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells. - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells. Inter-f L1-RSRP measurement without gap for LTM; - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells. - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated. |

Issue 2-1-1: Testing approach for SSB-based L1 event triggered measurement reporting

Technical background: The objective does not introduce any new measurement behaviour. Only a new event triggered reporting mechanism is introduced. For measurement behaviour, RAN4 has already defined core requirements and corresponding test cases introduced in R18, including intra-frequency/inter-frequency, measurement without gap and within gap. Therefore, in this objective RAN4 only needs to introduce some essential test cases to test the correct reporting behaviour.

Proposals:

Proposal 1 (Apple): For SSB based L1 RSRP measurement reporting, the following approach 1 is slightly preferred:

- Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases

- Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

Proposal 2 (HW):

- UE which can pass the following R19 test cases can skip corresponding R18 SSB based L1 measurement test cases.

- FR1 and FR2 SSB based LTM Intra-frequency event triggered reporting tests without gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests with gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests without gap

Proposal 3 (ZTE):

- For SSB based L1 RSRP measurement reporting, prefer to duplicate all R18 test cases and add event triggered reporting in R19, and UE which can pass R19 test cases can skip R18 test cases.

Proposal 4 (Ericsson):

- If the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, it is sufficient to test the event-triggered reporting and skip the Rel-18 LTM L1-RSRP test cases.

Recommended WF:

Further discuss based on the following directions

- Testing approach for SSB based L1 RSRP measurement reporting:

  - Approach 1: duplicate all R18 LTM L1 SSB test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases.

  - Approach 2: keep R18 test cases, develop necessary new R19 test cases for SSB based L1 measurements with event triggered reporting to verify reporting behaviour.

- Rules for skipping SSB-based cases:

  - Option 1: UE which can pass R19 SSB-based event triggered reporting test cases can skip R18 SSB-based test cases.

  - Option 2: UE which can pass the following R19 test cases can skip corresponding R18 SSB based L1 measurement test cases.

    - FR1 and FR2 SSB based LTM Intra-frequency event triggered reporting tests without gap

    - FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests with gap

    - FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests without gap

  - Option 3: If the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, it is sufficient to test the event-triggered reporting and skip the Rel-18 LTM L1-RSRP test cases.

Discussion:

- Ericsson: Support Approach 1. The only new thing is reporting compared to rel-18.

- Vivo: Further study on skipping is needed.

Tentative agreement:

  - Testing approach for SSB based L1 RSRP measurement reporting:

    - Duplicate all R18 LTM L1 SSB test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases.

Issue 2-1-2: Testing approach for CSI-RS-based L1 event triggered measurement reporting

Proposals

Proposal 1 (CATT): Regarding the direction of defining test case for CSI-RS based measurements:

- Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

  - For UE supporting event triggered reporting, UE only needs to pass 2)

  - For UE not supporting event triggered reporting, UE only needs to pass 1)

Proposal 2 (ZTE):

- For CSI-RS based L1 RSRP measurement reporting, test cases are introduced to jointly verify CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting.

Recommended WF:

Further discuss the following:

- Testing approach for CSI-RS based L1 RSRP measurement reporting:

  - For CSI-RS based L1 RSRP measurement reporting, test cases are introduced to jointly verify CSI-RS based L1-RSRP measurement and CSI-RS based L1-RSRP measurement event triggered reporting.

- Rules for skipping test cases:

  - Option 1: Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. For UE supporting event triggered reporting, UE only needs to pass 2). For UE not supporting event triggered reporting, UE only needs to pass 1)

  - Option 2: If the UE supports both periodic, semi-persistent L1-RSRP for the LTM and event-triggered L1-RSRP for the LTM, it is sufficient to test the event-triggered reporting and skip the Rel-18 LTM L1-RSRP test cases.

Discussion:

- Ericsson: UE supporting event triggered can skip periodic and vice versa.

- Vivo: Rel-19 should design these together to avoid overlap.

- HW: It’s not clear how to verify measurements and reporting. Follow the SSB approach if TCs are defined separately per reporting.

- Ericsson:  Same test case can support at least two reporting configurations.

- Apple: Agree that it’s possible to config two reporting configurations. For this scenario, event triggered is optional feature.

- HW, MTK: Let’s have separate test cases.

Tentative agreement:

Regarding the direction of defining test case for CSI-RS based measurements:

- Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

  - For UE supporting event triggered reporting, UE only needs to pass 2)

  - For UE not supporting event triggered reporting, UE only needs to pass 1)

Issue 2-1-3: Supported events in test cases

Technical background:

Agreement from RAN4#116 On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

Proposals:

- Proposal 1 (Apple): testing LTM event 3 is enough. However, we are also fine with using other event types in different test cases as long as it won’t result in extra testing time.

- Proposal 2 (CMCC): it is proposed to define test cases for LTM event 2/3/4/5.

- Proposal 3 (Nokia): Use LTM3 as the reporting event in the test cases.

- Proposal 4: it is proposed to define test cases for LTM event 2/3/4/5. (CMCC)

- Proposal 5: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW)

- Proposal 6: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

- Proposal 7 (CTC): Event LTM3 is best to verify event-triggered L1 reporting, For Event LTM5, discuss that if it is necessary.

- Proposal 8 (Ericsson): At least Event LTM3 and Event LTM4 be included in the RRM testcase. Some of the agreed test cases can be tested with Event LTM3 and some of the test cases with Event LTM4.

Recommended WF:

- Discuss the following options:

  - Define test case(s) at least for Event LTM3: “Beam of candidate cell becomes amount of offset better than beam of serving cell”. Further discuss whether to define test cases for the following events:

    - Event LTM2: Beam of serving cell becomes worse than absolute threshold

    - Event LTM4: Beam of candidate cell becomes better than absolute threshold

    - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Issue 2-1-4: Test case list for event-triggered L1 reporting

Technical background: The objective does not introduce any new measurement behaviour. Only a new event triggered reporting mechanism is introduced. For measurement behaviour, RAN4 has already defined core requirements and corresponding test cases introduced in R18, including intra-frequency/inter-frequency, measurement without gap and within gap. Therefore, in this objective RAN4 only needs to introduce some essential test cases to test the correct reporting behaviour.

Candidate options:

- Option 1: Nokia

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case for event-triggered periodic reporting.

- Option 2: CATT

  - SSB based test cases:

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

    - TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

  - CSI-RS based test case:

    - Only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

- Option 3: Apple, [vivo?]

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

- Option 4 (CTC): keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1.

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2.

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1.

- Option 5: OPPO

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

- Option 6: MTK

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Option 7: CMCC

  - Both FR1 and FR2-1 are considered.

  - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

  - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

- Option 8: ZTE

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

- Option 9: HW

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- Option 10:  E/// Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

Recommended WF:

  - Discuss a list of test cases based on the testing approach and events agreed.

# Topic #3 CSI-RS based L1 measurement Performance part

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| TDoc | Title | Source | Proposals |
| --- | --- | --- | --- |
| R4-2513085 | Discussion on test cases for CSI-RS based L1 measurement | MediaTek Inc. | Proposal 1: For CSI-RS based L1 measurement, define the following 5 test cases: - CSI-RS based L1 RSRP measurement for neighbour cell in FR1 - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 |
| R4-2513109 | On RRM performance requirements of CSI-RS based L1 measurements | OPPO | Proposal 1: Support to define the following test cases: - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 Proposal 2: Not define any Define test case(s) to verify early CSI acquisition if core requirements are specified. |
| R4-2513119 | RRM Perf requirements on CSI-RS based L1 measurement | China Telecom | Proposal 1: - Intra-frequency CSI-RS based L1 RSRP measurement for neighbour cell in FR1. - Intra-frequency CSI-RS based L1 RSRP measurement for neighbour cell in FR2. - Discuss whether include 2-step measurement, i.e., without SSB based L1-RSRP measurement. - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1. - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2. Proposal 2: Consider Periodic CSI-RS L1-RSRP measurements and periodic reporting. |
| R4-2513197 | Discussion on performance requirements for CSI-RS based L1 measurement for Rel-19 LTM | CATT | Proposal 1: RAN4 to define the following test cases for CSI-RS based L1 measurement: - CSI-RS based L1 RSRP measurement for neighbour cell in FR1 - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement Proposal 2: Test configuration is set to have 48 PRBs within the active BWP. Proposal 3: RAN4 not to define the test case to verify early CSI acquisition. |
| R4-2513490 | Discussion on performance requirements of CSI-RS based L1 measurement | Huawei, HiSilicon | Proposal 1: The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are: - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. Proposal 2: The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified: - FR1 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement - FR2 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement Proposal 3: Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell. Proposal 4: The following test cases are specified for CSI-RS based L1 measurement: - FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap - FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap Proposal 5: The following accuracy test cases are specified for CSI-RS L1-RSRP neighbour cell measurement: - FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases - FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases Proposal 6: For UE which supports both capabilities of CSI-RS based L1 measurement and LTM event trigger reporting, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting. |
| R4-2513646 | Discussion on RRM performance requirements for CSI-RS based L1 measurement | CMCC | Proposal 1: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for both FR1 and FR2. Proposal 2: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. |
| R4-2513897 | Discussion on performance for CSI-RS based L1 measurement | ZTE Corporation, Sanechips | Proposal 1: Verify the procedure of intra-frequency CSI-RS based L1-RSRP measurement in FR1 and FR2. For FR2, both the case with step 2 and the case without step 2 should be tested. Observation 1: No core requirements are specified on CSI acquisition. And there is no need to introduce test cases. Proposal 1: Verify the procedure of intra-frequency CSI-RS based L1-RSRP measurement in FR1 and FR2. For FR2, both the case with step 2 and the case without step 2 should be tested. Observation 1: No core requirements are specified on CSI acquisition. And there is no need to introduce test cases. Proposal 1: Verify the procedure of intra-frequency CSI-RS based L1-RSRP measurement in FR1 and FR2. For FR2, both the case with step 2 and the case without step 2 should be tested. Observation 1: No core requirements are specified on CSI acquisition. And there is no need to introduce test cases. |
| R4-2514080 | On performance requirements for CSI-RS based LTM | vivo | Proposal 1: For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3. Proposal 2: For CSI-RS based L1 measurement for LTM, test cases are introduced to verify intra-frequency measurement delay and accuracy requirements for both FR1 and FR2. Proposal 3: RAN4 is to define one test to verify both delay and accuracy requirements. Proposal 4: In FR2, tests are defined for not skipping SSB-based L1 measurement and skipping SSB-based L1 measurement for neighbour cell only, respectively. Proposal 5: No test case(s) to verify early CSI acquisition functionality. Proposal 1: For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3. Proposal 2: For CSI-RS based L1 measurement for LTM, test cases are introduced to verify intra-frequency measurement delay and accuracy requirements for both FR1 and FR2. Proposal 3: RAN4 is to define one test to verify both delay and accuracy requirements. Proposal 4: In FR2, tests are defined for not skipping SSB-based L1 measurement and skipping SSB-based L1 measurement for neighbour cell only, respectively. Proposal 5: No test case(s) to verify early CSI acquisition functionality. |
| R4-2514301 | Discussion on performance requirements for CSI-RS based L1 measurement | Ericsson | - We recommend that RAN4 agree on the following list of test cases for event triggered CSI-RS-based L1-RSRP. Intra-f L1-RSRP measurement for LTM; - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells. - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells,. Inter-f L1-RSRP measurement without gap for LTM; - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells. - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated. - To reduce the number of test cases and the testing burden on the UE, we suggest: - Testing only the event-triggered L1-RSRP test cases for CSI-RS. - If the UE does not support event-triggered reporting for CSI-RS, then it must pass the periodic CSI-RS-based L1-RSRP test cases.. |
| R4-2514374 | Discussion on performance part for LTM L1 CSI-RS measurements | Nokia | Proposal 1: Intra-frequency CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbouring cell. I.e., reuse CSI-RS L1-RSRP accuracy requirements with 48 PRB assumption. Proposal 2: Define CSI-RS accuracy requirements in a new clause (similar approach to Rel-18 LTM). Proposal 3: Test configuration is set to have 48 PRBs within the active BWP. Proposal 4: Define CSI-RS intra-frequency test cases without gaps in rel-19 Proposal 5: Define at least the following test cases and align with event triggered test cases to avoid overlap - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1 - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1 - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping |

Issue 3-1-1: Whether to define test cases for FR1 and FR2:

Technical background: Agreement in RAN4#112bis: Both FR1 and FR2-1 are in scope of RRM requirements for CSI-RS based L1-RSRP measurement on LTM candidate cell(s).

Proposals:

- Proposal 1 (CMCC): for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for both FR1 and FR2.

- Proposal 2 (Vivo): For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

- Recommended WF:

  - Agree the following:

    - Define test cases for CSI-RS based L1 measurements for both FR1 and FR2.

Issue 3-1-2: Test configuration for CSI-RS based L1 measurement test cases

Technical background:

Agreement in RAN4#116:

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and

  - n It is applied for SCS = 60KHz

- at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP.

- Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement.

RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19.

Proposals:

- Proposal 1(CATT): Test configuration is set to have 48 PRBs within the active BWP.

- Proposal 2 (Huawei): The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Proposal 1(Vivo): For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

- Proposal 3 (Nokia):

  - Test configuration is set to have 48 PRBs within the active BWP.

  - Define CSI-RS intra-frequency test cases without gaps in rel-19

Recommended WF:

- Discuss the above proposals

Issue 3-1-3: Test case list for CSI-RS based L1 measurement

Proposals:

- Proposal 1 (MTK, Oppo): For CSI-RS based L1 measurement, define the following 5 test cases:

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR1

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Proposal 2 (CTC):

  - Intra-frequency CSI-RS based L1 RSRP measurement for neighbour cell in FR1.

  - Intra-frequency CSI-RS based L1 RSRP measurement for neighbour cell in FR2.

    - Discuss whether include 2-step measurement, i.e., without SSB based L1-RSRP measurement.

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1.

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2.

- Proposal 3 (CATT): RAN4 to define the following test cases for CSI-RS based L1 measurement:

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR1

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

- Proposal 4 (Huawei): The following test cases are specified for CSI-RS based L1 measurement:

  - FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap

  - FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP measurement test cases without gap

- Proposal 5 (Huawei): The following accuracy test cases are specified for CSI-RS L1-RSRP neighbour cell measurement:

  - FR1 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases

  - FR2 CSI-RS based LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases

- Proposal 6 (Ericsson) We recommend that RAN4 agree on the following list of test cases for CSI-RS-based L1-RSRP.

| Intra-f L1-RSRP measurement for LTM | Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- Proposal 7(Nokia): Define at least the following test cases and align with event triggered test cases to avoid overlap

- Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

- Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

- Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

- Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

- Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

- Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping

Recommended WF:

- Discuss the proposals

Issue 3-1-4: Scenarios to test with and without step 2 (Rel-18 SSB measurements):

Proposals

- Proposal 1 (CMCC): for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Proposal 2 (ZTE): for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Proposal 3 (Vivo): In FR2, tests are defined for not skipping SSB-based L1 measurement and skipping SSB-based L1 measurement for neighbour cell only, respectively.

Recommended WF:

- Discuss the following options:

  - Option 1: Introduce test cases for UE supporting FG 52-3 and UE not supporting FG-52-3 (Skip SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement).

    - Option 1.1: For FR1 introduce test cases both scenarios.

    - Option 1.2: For FR2 introduce test cases both scenarios.

  - Option 2: Not to define test cases for FG 52-3 (Skip SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement)

Issue 3-1-5: SS/CSI-RSRP measurement L1 report mapping

Proposals:

- Proposal 1 (Huawei): Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

Recommended WF:

  - Discuss the following option:

    - Option 1: Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell

Issue 3-1-6: Conditions when UE supports event triggered reporting and traditional reporting

- Proposal 1(Huawei): For UE which supports both capabilities of CSI-RS based L1 measurement and LTM event trigger reporting, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

- Proposal 2 (Ericsson): To reduce the number of test cases and the testing burden on the UE, we suggest:

    - Testing only the event-triggered L1-RSRP test cases for CSI-RS.

    - If the UE does not support event-triggered reporting for CSI-RS, then it must pass the periodic CSI-RS-based L1-RSRP test cases.

Recommended WF:

- Discuss the following:

  - Option 1: Define conditions when UE is allowed to skip test cases:

    - Option 1.1: For UE which supports both capabilities of CSI-RS based L1 measurement and LTM event trigger reporting, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

    - Option 1.2: Test only the event-triggered L1-RSRP test cases for CSI-RS. If the UE does not support event-triggered reporting for CSI-RS, then UE shall pass the periodic CSI-RS-based L1-RSRP test cases.

Issue 3-1-7: Whether to test CSI-acquisition

Proposals:

- Proposal 1 (Oppo): Not define any Define test case(s) to verify early CSI acquisition if core requirements are specified.

- Proposal 2 (CATT): RAN4 not to define the test case to verify early CSI acquisition.

- Proposal 3 / Observation (ZTE): for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Proposal 4 (Vivo): No test case(s) to verify early CSI acquisition functionality

Recommended WF:

- To agree the following:

  - No test cases defined in rel-19 to test early / baseline CSI acquisition if no core requirements are defined.

Issue 3-1-8: Accuracy requirements for CSI-RS based L1-RSRP candidate cell

Technical background from R4-2512141:

| #### Issue 4-2-1: CSI-RS based L1 measurement accuracy Agreement: - CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell. - New requirements are defined in the following sections |  |
| --- | --- |
| Accuracy requirements | Detail |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy |

Proposals

Proposal 1 (Huawei): The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are:

- The bandwidth of CSI-RS is 48 PRBs and the density is 3.

- The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

- The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Proposal 2(Huawei): The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified:

- FR1 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement

- FR2 Absolute and relative CSI-RS based intra-frequency L1-RSRP measurement

Proposal 3 (Vivo): For CSI-RS based L1 measurement for LTM, reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3.

Proposal 4: (Vivo): RAN4 is to define one test to verify both delay and accuracy requirements.

Proposal 5: (Nokia): Intra-frequency CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbouring cell. I.e., reuse CSI-RS L1-RSRP accuracy requirements with 48 PRB assumption.

Proposal 6 (Nokia): Define CSI-RS accuracy requirements in a new clause (similar approach to Rel-18 LTM).

Recommended WF:

- Further discuss the proposal above.

# Topic #4: CLTM and Sub-sequent LTM performance part

Issues are based on:

R4-2513518	Topic summary for [116bis][212] NR_Mob_Ph4_RRM_Part2

Companies’ contributions summary

| TDoc | Source | Proposals |
| --- | --- | --- |
| R4-2513120 | China Telecom | Proposal 1: Introduce the following test cases for CLTM and subsequent CLTM, - RACH-less intra-frequency PCell switch from FR1 to FR1 - RACH-based inter-frequency LTM PCell switch from FR2 to FR2 - RACH-less intra-frequency subsequent CLTM from FR1 to FR1 - RACH-based inter-frequency subsequent CLTM from FR2 to FR2 Proposal 2: Exclude PSCell switch and PDCCH-order Random Access test cases. |
| R4-2513623 | Apple | Proposal 1: introduce the following test cases for CLTM and subsequent CLTM. - Inter-frequency CLTM from FR1 to FR1 - Intra-frequency CLTM from FR2 to FR2 - Intra-frequency subsequent CLTM from FR1 to FR1 - Inter-frequency subsequent CLTM from FR2 to FR2 Proposal 2: for UE which can pass subsequent CLTM, test case for CLTM can be skipped. |
| R4-2513911 | vivo | Proposal 1: RAN4 to define test cases for CLTM Cell switch delay and Subsequent CLTM Cell switch delay by from the following dimensions. And for test case configurations, existing conditional handover test cases and subsequent CPC test cases can serve as the baseline, respectively. - RACH-based and RACH-less based - FR1 and FR2 - Intra-frequency and Inter-frequency |
| R4-2513110 | OPPO | Proposal 1: For Conditional LTM tests, only L1 based CLTM with early DL sync will be considered. Proposal 2: Support to define the following tests for conditional LTM - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1 - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2 - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2 - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1 |
| R4-2513198 | CATT | Proposal 1: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases. Proposal 2: RAN4 to define new test cases for conditional LTM PCell Switch. Proposal 3: the test cases for CLTM should cover both intra and inter-frequency CLTM. Proposal 4: RAN4 to define the test case for conditional LTM PCell switch without L1-RSRP measurement should be defined. Proposal 5: The test cases for conditional LTM need to cover RACH-based and RACH-less cases. Proposal 6: There is no need to define the new test cases for PDCCH ordered RACH for conditional LTM. Proposal 7: For subsequent CLTM test scope, RAN4 to define the following test cases: - Intra-frequency subsequent CLTM from FR1 to FR1 - Inter-frequency subsequent CLTM from FR2 to FR2 |
| R4-2513645 | CMCC | Proposal 1: it is proposed to define test cases for CLTM cell switch delay and subsequent CLTM cell switch delay. Proposal 2: for CLTM and subsequent CLTM, it is proposed to define the test cases for following scenarios: - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell Proposal 2: it is proposed to define test cases for both intra-frequency LTM and inter-frequency LTM. Proposal 3: it is proposed to define test cases to cover both RACH-less and RACH based CLTM. Proposal 4: it is proposed to define test cases to cover both L1 and L3 based CLTM |
| R4-2513086 | MediaTek Inc. | Proposal 1: introduce the following test cases for CLTM and subsequent CLTM. - CLTM from FR1 to FR1 with early TCI state activation - CLTM from FR2 to FR2 with early TCI state activation - subsequent CLTM from FR1 to FR1 - subsequent CLTM from FR2 to FR2 Proposal 2: For UE which passed subsequent CLTM, test cases for CLTM can be skipped. |
| R4-2513491 | Huawei, HiSilicon | Proposal 1: Define test cases in non-DC and don’t define test cases for conditional PSCell switch. Proposal 2: For conditional LTM, the following test case category are to be specified for FR1-FR1 and FR2-FR2: - RACH based Intra-frequency conditional PCell switch - RACH based Inter-frequency conditional PCell switch - RACH less Intra-frequency conditional PCell switch - RACH less Inter-frequency conditional PCell switch Proposal 3: For subsequent conditional LTM, the following test case category are to be specified: - RACH based Intra-frequency subsequent conditional PCell switch - RACH based Inter-frequency subsequent conditional PCell switch - RACH less Intra-frequency subsequent conditional PCell switch - RACH less Inter-frequency subsequent conditional PCell switch Proposal 4: UE who pass subsequent CLTM tests can skip CLTM tests. |
| R4-2513946 | Nokia | Observation 1: The FR scope for CLTM is the same as the FR scope for Rel-18 LTM. Proposal 1: Define test cases for CLTM cell switch to both intra-frequency and inter-frequency target cell. Proposal 2: Define test cases for both L1 and L3 triggered CLTM cell switch in FR1. Define test cases only for L1 triggered CLTM cell switch in FR2. Proposal 3: Define test cases for both RACH-less and RACH-based CLTM cell switch. Proposal 4: In RACH-less CLTM cell switch test case, ensure that the TA timer is running at the time of triggering the cell switch. Proposal 5: Cover both with and without early DL sync in the CLTM cell switch test cases. These can be defined so that, depending on UE capability, early TCI state activation is included or not. Proposal 6: Ensure that UE has DL synchronization at the time of cell switch, if UE supports early TCI state activation capability in the test case. Proposal 7: Apply same test coverage for initial and subsequent CLTM cell switch delay test cases. Proposal 8: For UE which can pass subsequent CLTM test case, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure. Proposal 9: PSCell conditional cell switch is not supported, so define test cases for PCell conditional cell switch only. |
| R4-2514189 | Ericsson | Proposal 1: RAN4 test scope of conditional LTM shall cover both intra and inter-frequency measurement triggered CLTM. Proposal 2: RAN4 test scope shall verify both L1 and L3 measurement based candidate evaluation for CLTM and further discuss how to verify such as whether to reuse partial LTM test procedure. Proposal 3: RAN4 test scope shall cover both RACH based and RACH-less CLTM. Proposal 4: RAN4 test scope shall cover with and without early DL sync for CLTM Proposal 5: The Pscell switch is needed to be covered in the test scope of CLTM. Proposal 6: RAN4 test scope for subsequent CLTM shall cover both Intra and Inter frequency, RACH based and RACH-less scenarios. Proposal 7 : RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. Proposal 8: CLMT and subsequent CLTM can be combined together to have full coverage of the test cases. |
| R4-2513896 | ZTE Corporation, Sanechips | Observation 1: Conditional PScell switch is not considered in R19 CLTM. Proposal 1: Define test cases for CLTM and test cases defined for LTM cell switch can be used as baseline. Follow aspects should be considered: - RACH-based & RACH-less - Intra-f & inter-f - With & without L1-RSRP measurement for FR1 Proposal 2: Define test cases for Subsequent CLTM and test cases defined for CLTM can be used as baseline. Follow aspects should be considered: - FR1 to FR1& FR2 to FR2 - RACH-based & RACH-less - Intra-f & inter-f - With & without L1-RSRP measurement for FR1 Proposal 3: For UE which can pass subsequent CLTM, test case for CLTM can be skipped. |

4-1 CLTM Performance part

Issue 4-1-2: frequency range

- Candidate solutions:

- Proposal 1: FR1 and FR2. (vivo)

Issue 4-1-3: intra/inter-frequency

- Candidate solutions:

- Proposal 1: Intra-frequency and Inter-frequency. (vivo, CATT, CMCC, Nokia, E///, ZTE)

- Proposal 2: (CMCC)

    - PCell switch to a neighboring LTM candidate cell

      - FR1 cell to FR1 cell

      - FR1 cell to FR2 cell

      - FR2 cell to FR2 cell

      - FR2 cell to FR1 cell

    - PCell switch to an LTM candidate cell that is a serving SCell in MCG

      - FR1 cell to FR1 cell

      - FR2 cell to FR2 cell

Issue 4-1-5: RACH-less and RACH-based CLTM

- Candidate solutions:

- Proposal 1: RACH-based and RACH-less. (vivo, CATT, CMCC, Nokia, E///, ZTE)

    - Proposal 1a: In RACH-less CLTM cell switch test case, ensure that the TA timer is running at the time of triggering the cell switch. (Nokia)

    - Proposal 1b: How to verify such as re-use partial LTM test procedure. (E///)

- Recommended WF

- Cover RACH-based and RACH-less. Proposal 1a and 1b needs discussion.

Issue 4-1-6: PCell/PSCell

- Candidate solutions:

- Proposal 1: Only PCell. (CTC, CATT, HW, Nokia)

- Proposal 2: PCell and PSCell. (OPPO, E///)

- Recommended WF

- WID has excluded DC scenario, so only PCell.

Issue 4-1-7: early DL sync

- Candidate solutions:

- Proposal 1: With early DL sync. (OPPO)

- Proposal 2: With and without early DL sync. (Nokia, E///)

    - Proposal 2a: depending on UE capability, early TCI state activation is included or not. (Nokia)

    - Proposal 2b: Ensure that UE has DL synchronization at the time of cell switch, if UE supports early TCI state activation capability in the test case. (Nokia)

- Recommended WF

- Further discussion is needed.

Issue 4-1-8: PDCCH ordered RACH

- Candidate solutions:

- Proposal 1: Do not cover PDCCH ordered RACH. (CTC, CATT)

- Recommended WF

- Do not cover PDCCH ordered RACH.

Issue 4-1-4: L1/L3 based CLTM

- Candidate solutions:

- Proposal 1: L1 and L3 based CLTM. (CATT, CMCC, Nokia, E///, ZTE)

    - Proposal 1a: Define test cases only for L3 triggered CLTM cell switch in FR1. (Nokia, ZTE)

    - Proposal 1b: How to verify such as whether to reuse partial LTM test procedure. (E///)

- Proposal 2: only L1 based CLTM. (OPPO)

- Recommended WF

- Cover L1 and L3 based CLTM, and L3 triggered CLTM cell switch only in FR1

Issue 4-1-1: Test case list for CLTM

- Candidate solutions:

- Proposal 1: (CTC)

    - RACH-less intra-frequency PCell switch from FR1 to FR1.

    - RACH-based inter-frequency LTM PCell switch from FR2 to FR2.

- Proposal 2: (Apple)

    - Inter-frequency CLTM from FR1 to FR1.

    - Intra-frequency CLTM from FR2 to FR2.

- Proposal 3: (MTK)

    - CLTM from FR1 to FR1 with early TCI state activation.

    - CLTM from FR2 to FR2 with early TCI state activation.

- Proposal 4: FR1-FR1 and FR2-FR2 (HW)

    - RACH based Intra-frequency conditional PCell switch.

    - RACH based Inter-frequency conditional PCell switch.

    - RACH less Intra-frequency conditional PCell switch.

    - RACH less Inter-frequency conditional PCell switch.

- Recommended WF

- Further discussion is needed.

Discussion:

    - MTK: How many test cases do we want to cover in total? We prefer to discuss directly test case list. We would like a sensible list of test cases.  L3 CHO is quite similar from functionality point of view to CLTM.  Inter-frequency is covering CHO test cases already.

    - QC: We should also consider RAC-based and RACH-less.

    - HW: We would like to cover intra- and inter- and RACH-based and RACH-less

4.2  Subsequent CLTM

Issue 4-2-1: Test case list for Subsequent CLTM

- Candidate solutions:

- Proposal 1: (CTC)

    - RACH-less intra-frequency subsequent CLTM from FR1 to FR1.

    - RACH-based inter-frequency subsequent CLTM from FR2 to FR2.

- Proposal 2: (Apple, CATT)

    - Intra-frequency subsequent CLTM from FR1 to FR1.

    - Inter-frequency subsequent CLTM from FR2 to FR2.

- Proposal 3: (MTK)

    - subsequent CLTM from FR1 to FR1.

    - subsequent CLTM from FR2 to FR2.

- Proposal 4: (HW)

    - RACH based Intra-frequency subsequent conditional PCell switch.

    - RACH based Inter-frequency subsequent conditional PCell switch.

    - RACH less Intra-frequency subsequent conditional PCell switch.

    - RACH less Inter-frequency subsequent conditional PCell switch.

- Proposal 5: (OPPO)

    - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1.

    - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2.

    - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2.

    - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1.

- Proposal 6: (Nokia)

    - Apply same test coverage for initial and subsequent CLTM cell switch delay test cases.

- Proposal 7: (E///)

    - Cover both Intra and Inter frequency, RACH based and RACH-less scenarios.

    - CLMT and subsequent CLTM can be combined together to have full coverage of the test cases.

- Proposal 8: (ZTE)

    - FR1 to FR1& FR2 to FR2.

    - RACH-based & RACH-less.

    - Intra-f & inter-f.

    - With & without L1-RSRP measurement for FR1.

- Recommended WF

- Further discussion is needed.

Issue 4-2-2: Others

- Candidate solutions:

- Proposal 1: For UE which can pass subsequent CLTM, test case for CLTM can be skipped. (Apple, MTK, HW, Nokia, ZTE)

- Recommended WF

- For UE which can pass subsequent CLTM, test case for CLTM can be skipped.