3GPP TSG-RAN4 Meeting # 116bis	R4-2514836

# Prague, Czech Republic, Oct 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on OD-SSB of R19 NES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change 1: The original “which” is not clear, whether it means the OD-SSB or the first SSB? We make it more clear. Change 2: Add the spec impact when the OD-SSB colliding with measurement gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: The original “which” is not clear, whether it means the OD-SSB or the first SSB? We make it more clear. Change 2: Add the spec impact when the OD-SSB colliding with measurement gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements around OD-SSB are not accurate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x1, 9.x.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

### 8.3.x1	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE supporting On-demand SSB operation and configured with at least one downlink OD-SSB SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one OD-SSB SCell is being activated.

The requirements in the clause 8.3.x1 apply, provided the following conditions are met:

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is not configured, or

-	an OD-SSB transmission is activated in a configured DL BWP where a first SSB transmission is configured and the OD-SSB cannot be used to obtain SIB1.

Where the first SSB transmission is specified as in TS 38.213 and have the following relations with the OD-SSB transmission:

-	are with the same physical cell identity, and

-	are in the same frequency location, and

-	are with same sub-carrier spacing, and

-	are with the same downlink transmit power, and

-	are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SSB index.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall perform SCell activation procedure based on the transmitted SSBs, where the transmitted SSBs are

-	OD-SSBs being transmitted in the configured DL BWP if no first SSB transmission is configured, or

-	the union of OD-SSBs and first SSBs if first SSBs are transmitted and the time instance when receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command, or

-	first SSBs only if first SSBs are transmitted but no OD-SSB transmission during the SCell activation.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot lengtℎ , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in milliseconds.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirst, OD-SSB + T∆ + 5 ms, if the measurement period of the SCell being activated is equal to or smaller than 2400 ms.

-	TFirst, OD-SSB + Trs, OD-SSB + T∆ + 5 ms, if the measurement period of the SCell being activated is larger than 2400 ms.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	  only one SSB is indicated in ‘od-ssb-PositionInBurst’ if provided; otherwise, in ssb-PositionsInBurst, or

-	 multiple SSBs are indicated in ‘od-ssb-PositionInBurst’ if provided; otherwise, in ssb-PositionsInBurst and TCI indication of SSBs is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	TFirst, OD-SSB + 2* Trs,OD-SSB + 5 ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its od-ssb-PositionInBurst if provided; otherwise, ssb-PositionsInBurst provided is the same as the one of contiguous FR1 active serving cell, and

-	its SFN offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB;

-	TFirst, OD-SSB + 3*Trs, OD-SSB + 5 ms, otherwise.

Otherwise, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	6 ms + TFirst, OD-SSB + 2*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3 ms + TFirst, OD-SSB + 2*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its od-ssb-PositionInBurst if provided; otherwise, ssb-PositionsInBurst provided is same as the one of contiguous FR1 active serving cell, and

-	its SSB offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is larger than 260 ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6 dB;

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirst OD_SSB + 5ms provided:

-	the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

-	the parameter od-ssb-PositionsInBurst if provided; otherwise, ssb-PositionsInBurst provided if the SCell being activated is same for the serving cell(s), and

-	the SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3 ms + max(Tuncertainty_MAC + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0, if

-	UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	TCI-ActivatedConfig is configured for the SCell, and UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0, if

-	UE receives the SCell activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	6 ms + TFirst, OD-SSB + 23*Trs, OD-SSB  + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming, OD-SSB  + 2 ms, Tuncertainty_SP), or

If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, then Tactivation_time is:

-	3 ms + TFirst OD-SSB + 23*Trs, OD-SSB + TL1-RSRP, measure, OD-SSB + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, OD-SSB), (Tuncertainty_RRC + TRRC_delay)}.

where,

Trs, OD-SSB is the periodicity of the transmitted SSB.

T∆ is the additional time for fine timing tracking and acquiring full timing information of the SCell. T∆ = 2*Trs, OD-SSB ms for SCell operating with 12 PRB SSB BW. Otherwise, T∆ = 0 ms.

TFirst, OD-SSB is the time to the end of the first complete transmitted SSB burst after slot n + THARQ+3msNR slot lengtℎ.

-	OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or

-	the union of OD-SSBs and first SSBs if first SSBs are transmitted and the time instance when receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command, or

-	first SSBs only if first SSBs are transmitted but no OD-SSB transmission during the SCell activation

TFineTiming, OD-SSB is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete transmited SSB burst corresponding to the TCI state.

TL1-RSRP, measure, OD-SSB is the transmited SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB based on applicability as defined in clause 9.5.4 assuming M=1 and TReport=0

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) or the SSB used for SCell activation which has the same SSB index of the SSB measured also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with the indexes or the SSBs used for SCell activation with the same indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on SpCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot lengtℎ  and not occur after slot slot n+1+THARQ+3ms+TXNR slot lengtℎ, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3 ms;

-	TFirst, OD-SSB, for any scenario where Tactivation_time includes TFirst, OD-SSB;

-	Tuncertainty_MAC + TFineTiming, OD-SSB, for any scenario where Tactivation_time includes only TFineTiming, OD-SSB and no TFirst, OD-SSB.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requirements on interruption due to SCell activation in clause 8.2 apply provided that the first SSB and the OD-SSB of the to-be-activated SCell is within the first active DL BWP of the SCell.

No OD-SSB based SCell activation requirement applies if OD-SSB is transmitting upon UE receiving the SCell activation command but NW deactivates the OD-SSB earlier than the slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot lengtℎ.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

<End of Change #1>

<Start of Change #2>

9.x.5.3	Intra-frequency cell identification for deactivated SCell

When UE is indicated the activation of OD-SSB transmission in slot n in a configured DL BWP of an SCell without a first SSB transmission before SCell activation command,

-	After slot n+THARQ+3ms+TprocessingNR slot lengtℎ, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-1, 9.x.5.3-2 and 9.x.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot lengtℎ, where:

-	TFMW equals min(Treport, TFirst, OD−SSB+ Tidentify_intra), which is the duration of the fast measurement window(FMW).

-	TFirst, OD−SSB is the time to the end of the first complete transmitted SSB burst after slot n + THARQ+3ms+TprocessingNR slot lengtℎ.

-	Tidentify_intra equals Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB if the cell is a newly detectable cell or TSSB_measurement_period_intra_ODSSB if the cell has been detected for a period less than 5 seconds.

-	Tprocessing is the additional processing time as 2ms or 5ms depending on OD-SSB post processing time.

-	Treport is the duration after slot n+THARQ+3ms+TprocessingNR slot lengtℎ to the earliest OD-SSB based L3 measurement reporting.

-	CSSFintra: it is a carrier specific scaling factor and is determined	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps. When single OD-SSB carrier is configured with multiple SCCs without OD-SSB, CSSFintra equals 1 for OD-SSB carrier during the FMW.

-	Kp_OD-SSB is the scaling factor for an OD-SSB based deactivated SCell to be measured. Kp_OD-SSB = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the OD-SSB frequency layer, and starting from the beginning of any OD-SSB occasion:

-	Ntotal is the total number of OD-SSB occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of OD-SSB occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp_OD-SSB = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Otherwise, when the UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When OD-SSB is fully non overlapping with measurement gaps or NCSG, Kp_OD-SSB =1;

When OD-SSB is partially overlapping with measurement gaps, Kp_OD-SSB = 1/(1- (OD-SSB period /MGRP)), where OD-SSB period < MGRP. When OD-SSB is partially overlapping with NCSG, Kp_OD-SSB = 1/(1- (OD-SSB period /VIRP)), where OD-SSB period < VIRP.

- When UE is indicated the activation of another OD-SSB transmission in a configured DL BWP of an SCell within the T2 after the previous OD-SSB deactivation and before the SCell activation command, the UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.4-2 and 9.x.5.4-3 if OD-SSB is transmitted, where T2 is 5*max (measCycleSCell, DRX cycles) in FR1 and 4s for power class 1/5 and 3s for power class 2/3/4 in FR2.

- When UE is indicated the activation of another OD-SSB transmission in a configured DL BWP of an SCell after previous OD-SSB deactivation plus T2 and before the SCell activation command, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-1, 9.x.5.3-2 and 9.x.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot lengtℎ.

-	After the FMW, UE shall follow the legacy deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5 9.x.5.4-2 and 9.x.5.4-3 if the OD-SSB transmission is not deactivated.

No OD-SSB based deactivated SCell requirement applies if NW deactivates the OD-SSB earlier than the time slot n+THARQ+3ms+Tprocessing+min(Treport, Tidentify_intra)NR slot lengtℎ.

When UE is indicated the activation of OD-SSB transmission in slot n in a configured DL BWP of an SCell with a first SSB transmission before SCell activation command, after slot n+THARQ+3ms+TprocessingNR slot lengtℎ,

- UE shall perform the deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.4-2 and 9.x.5.4-3 if UE has sent a valid measurement report before receiving the activation of OD-SSB transmission during the period equal to max(5*measCycleSCell,  5*DRX cycles) in FR1 or 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

- Otherwise, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-1, 9.x.5.3-2 and 9.x.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot lengtℎ..

- Before UE receiving the activation of OD-SSB transmission or after OD-SSB deactivation in a deactivated SCell, UE shall perform deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.4-2 and 9.x.5.4-3 if OD-SSB is transmitted.

- Before UE receiving the activation of OD-SSB transmission or after OD-SSB deactivation in a deactivated SCell, for UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to perform measurements on a deactivated SCell with 12PRB SSB in FR1, TSSB_time_index_intra_less_than_5MHz_ODSSB is given in table 9.x.5.3-6.

UE shall perform the deactivated SCC neighbour cell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.3-6, 9.x.5.4-2 and 9.x.5.4-3 regardless of OD-SSB activation/deactivation.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.x.5.3-2, Table 9.x.5.3-3, Table 9.x.5.3-4, 9.x.5.3-5 and Table 9.x.5.3-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.x.5.3-2, Table 9.x.5.3-3, Table 9.x.5.3-4, 9.x.5.3-5 and Table 9.x.5.3-6 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

When NW indicates OD-SSB transmission in one of the deactivated SCell, the longer cell identification or measurement delay for other frequency layers being measured would be expected during the TFMW.

When the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping, the requirement of deactivated SCells measurement with multiple OD-SSBs SCell shall be the same as the deactivated SCells measurement requirement for single OD-SSB measurement requirement.

Table 9.x.5.3-1: Time period for OD-SSB based PSS/SSS detection in FMW, deactivated SCell

| Frequency range | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| FR1 | ceil(M Note 2 x Kp_OD-SSB) x max(Tlower Note 1, EMP) x CSSFintra |
| FR2 | ceil(M Note 2 x KFR x Kp_OD-SSB x Klayer1_measurement) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. NOTE 2: When multiple frequency layers are configured, M equals 6 in FR1 and Mpss/sss_sync_w/o_gaps+1 in FR2. Otherwise, M equals 5 in FR1 and Mpss/sss_sync_w/o_gaps in FR2. |  |

Table 9.x.5.3-2: Time period for OD-SSB based time index detection in FMW, deactivated SCell (FR1)

| FR | TSSB_time_index_intra_ODSSB |
| --- | --- |
| FR1 | ceil( 3 x Kp_OD-SSB ) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. |  |

Table 9.x.5.3-3: Time period for OD-SSB based PSS/SSS detection, deactivated SCC (FR1)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(5 x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.x.5.3-4: Time period for OD-SSB based PSS/SSS detection, deactivated SCC (FR2)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.x.5.3-5: Time period for OD-SSB based time index detection, deactivated SCC (FR1)

| DRX cycle | TSSB_time_index_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(3 x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.x.5.3-6: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCC (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5MHz |
| --- | --- |
| No DRX | Ceil(7 x Kp_OD-SSB) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp_OD-SSB) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp_OD-SSB) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements apply to CC with deactivated SCG SCell. |  |

<Start of Change #2>