3GPP TSG-RAN4 Meeting #116bis	R4-2514839

# Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			6.9.1

Source:	vivo

Title:	Ad-hoc minutes #2 for TRP_TRS_MIMO_OTA_Ph3 & NR_UE_OTA_Enh

Document for:	Approval

Introduction

This is 2nd ad-hoc meeting minutes cover the discussions of Rel-19 OTA WI (TRP_TRS_MIMO_OTA_Ph3) and Rel-20 OTA WI (NR_UE_OTA_Enh), chaired by Ruixin Wang (vivo).

Rel-19 Topic #3: Rel-19 OTA requirements (agenda 6.9)

Issue 3-1-6: Thresholds value for Rel-19 measurement campaign

- Proposals

  - Proposal 1: RAN4 should consider the following thresholds as reasonable value for Rel-19 measurement campaign. (vivo)

    - Minimum number of devices for each band, each device size, each power class: 40~45

    - Minimum number of device models: 5

    - Minimum number of devices' vendors: 6

    - Percentage of devices from second-half 2022 to first-half 2025: 100%

    - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

    - Define the Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: No need to define this value.

  - Proposal 2: RAN4 should consider the following thresholds as reasonable value for Rel-19 measurement campaign. (Telecom Italia S.p.A., Vodafone, Deutsch Telekom, Orange)

    - Minimum number of devices for each band, each device size, each power class: 60

    - Minimum number of device models: 10 less than above

    - Minimum number of devices' vendors: 6

    - Percentage of devices from second-half 2022 to first-half 2025: 100%

    - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

    - Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: between 80% and 85%

      - If for any band with the number of measured data points < 60 this threshold is not met, RAN4 can request the test labs to measure additional devices to shift the ratio closer to the target value. The number of additional devices is determined based on the delta between the current percentage and the threshold.

  - Proposal 3: remove step 5-a-iv-6 in the current working procedure as imposing a ratio between numbers of size 1 and size 2 devices is against the previous agreement in R4-2508782. (Huawei)

Online Agreement:

- Minimum number of devices for each band, each power class: FFS

  - Decide the number no later than the next meeting.

The target is to collect full set of UE information and measurement results before end of next RAN4 meeting.

2nd ad-hoc discussion:

Apple: The test labs resources is the key limitation of this value. Not sure further increase this value is needed.

TIM: based on the initial plan from test labs, the value might be higher. In rel-18, min is 47 for a band TRS. 45 is compromise value. More is better.

Keysight: what will happen if measured devices has smaller number than this value next meeting?

TIM: if that happens, then we may consider a longer time for collecting results until Feb 2026.

WF for final checking:

    - Minimum number of devices for each band, each power class: 45

    - Minimum number of device models: 5 less than above

    - Minimum number of devices' vendors: 6

    - Percentage of devices from second-half 2022 to first-half 2025: 100%

    - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: 100%

    - Percentage of Size 1 (width >72mm and ≤92mm) devices versus Size 2 (width ≤72mm) devices: size 2 devices can be 0 to 20% of total device number

Issue 3-1-7: Processing of measurement data pool for Rel-19 requirements discussion

- Proposals

  - Proposal 1: Same as Rel-18, keep the data pool as it is without removing data point, and focus on the requirement discussion. Remove 1 best and 1 worst-performing data point approach could only be considered if the total number for that band is more than 60.

  - Proposal 2: remove option 2 under step 6-g from the working procedure as measurement data should not be removed unless proven faulty.

  - Proposal 3: adopt the following procedure:

  - The data pool will be refined based on the following criteria:

    - Total number of devices = 60: remove 1 best and 1 worst-performing data

    - Total number of devices between 61 to 65: remove 2 best and 2 worst-performing data

    - Total number of devices = 66 to 70: remove 3 best and 3 worst-performing data

    - Total number of devices = 71 to 75: remove 4 best and 4 worst-performing data

    - Total number of devices = 76 to 80: remove 5 best and 5 worst-performing data

    - Total number of devices > 81: remove 6 best and 6 worst-performing data

FFS until next meeting.

Issue 3-1-8: On number of Rx for Rel-19 requirements

- Proposals

  - Proposal 1: RAN4 to discuss how to specify the minimum performance requirement for TRS for low bands and high bands such that it remains aligned with the mandatory requirements on the number of receive ports. (Apple)

  - Proposal 2: To allow the test lab to resolve ambiguities related to the number of supported Rx ports, test equipment vendors are requested to read the UE capability report during connection setup and to indicate to the test operator the reported values (per band) of the maximum number of PDSCH MIMO layers and the supported SRS antenna switching capabilities. (Apple)

  - Proposal 3: For bands in the 800 ~ 2300 MHz frequency range RAN4 shall specify TRS requirements based on the performance of 2Rx devices only. (Apple)

  - Proposal 4: For bands in the > 3300 MHz frequency range RAN4 shall specify TRS requirements based on the performance of 4Rx devices only. (Apple)

- Recommended WF

  - In Rel-19, all the 4 bands are PC3 and 2Rx. This can be reflected in the updated working procedure and measurement template.

  - For P2, this can be suggestion to test labs.

  - For providing Device information to MCC, discus whether there is a need for test labs to report the number of Rx.

WF:

- All the 4 bands in Rel-19 are 2Rx and PC3.

  - In case there is ambiguities related to the number of supported Rx ports of a specific device (more than 2Rx), test labs should report that information.

Rel-20 Topic #2: Enhanced test methodologies to cover NTN Ka and Ku bands (agenda 7.3.3)

Sub-topic 2-6: Polarizaion

- Proposals:

- Proposal 4 from Vivo in R4-2513572

  - Proposal 4: RAN4 should target to specify NTN OTA test methodologies supporting circular polarization measurements.

- Proposal 7 from Huawei, Hisilicon In R4-2513580

  - Proposal 7: NTN OTA tests should use circular polarisation.

- Proposal 8 from Huawei, Hisilicon In R4-2513580

  - Proposal 8: If the use of circular polarisation is not agreeable for VSAT measurement, adopt the method in reference [3] in R4-2513580 to derive circular polarisation performance metrics and axial ratio using 3 linear polarisation (LP) measurements with two out of three LP measurements orthogonal to each other to align with E and E

- Proposal 5 from Samsung in R4-2513876

  - Proposal 5:	keep linear polarization as the baseline. RAN4 should carefully examine the testability challenges for circular polarization before moving to next step.

- Proposal 8 from Eutelsat Group in R4-2514266

  - Proposal 8: Circular polarization is essential for the main use cases and should be the priority.

- Proposal 15 from Keysight Technologies in R4-2514328

  - Proposal 15: It is proposed to clarify the applicability of the conformance test requirements with respect to polarization.

- Proposal 16 from Keysight Technologies in R4-2514328

  - Proposal 16: Study whether select conformance metrics, e.g., peak EIRP/EIS, should be assessed using CP before formalizing that NTN VSAT OTA conformance testing (signalling testing) shall leverage linear polarization exclusively

- Moderator Recommendation:

- The majority view is that CP is essential

- Adopt CP using existing LP systems as a starting point

First Ad-hoc discussions:

Keysight: the CP is not captured in the TS under requirements clauses. More polarization supported, more complicated the system will be. We need clear guidance from requirements perspective how to move forward.

Anritsu: it is challenging to cover whole frequency ranges with CP and LP.

Samsung: in the WID, LP is the baseline. CP was discussed in 5G beginning, but not considered.

Huawei: no LP for satellite communication, why use LP for testing?

Keysight: now we are developing new system, but not just upgrade of existing systems, so CP can be considered in the test setup.

Eutelsat: we can update the NTN TS next meeting, with clarification of CP in core requirements. Maybe WID update is needed to remove LP is the baseline.

Jsat: LP is supported in GEO.

WF for further discussion after 1st ad-hoc:

RAN4 start to work on support of CP testing in IFF system. Further discuss Linear polarization is not precluded or supported as basis.

Online discussion:

Keysight: in principle we are ok to looking into CP system. However, the current Linear is still the baseline.

EutelSat: We will modify the WID to say the CP is the baseline.

R&S: We can look into the CP system. But at this moment, we cannot agree it is the baseline.

HW: Subject to the change of the WID. We support to change the baseline to CP.

Anristu: cannot say which one is the baseline.

WID:

  - Study whether and how to support circular polarization measurements

    - Linear polarization is the baseline

Eutelsat: CP is typical for Ku band.

Samsung: We understand the industry demand. We also need to consider the technical challenge.

Keysight: Not repeat the discussion in RAN plenary.

Eutelsat: both CP and LP are supported.

2nd ad-hoc discussion:

Keysight: MU will not be impacted by CP or LP.

Agreements after 2nd ad-hoc:

RAN4 start to work on whether and how to support of CP testing in IFF system.

Linear polarization is supported as basis based on the WID scope.

The core requirement of NTN will be investigated to define the applicability of polarization on the requirements in next RAN4 meeting.

Sub-topic 2-3: Quiet zone size

- Proposals:

- Proposal 3 from Vivo in R4-2513572

  - Proposal 1: RAN4 should specify a reasonable QZ for NTN VSAT testing to minimize the burden of test system upgrade.

- Proposal 2 from Samsung in R4-2513876

  - Proposal 2: RAN4 needs to discuss the radiating aperture diameter assumption and QZ size for the evaluation of minimum range length and SNR range.

- Proposal 1 from Eutelsat Group in R4-2514266

  - Proposal 1: To facilitate applicability to the most common small VSAT type it is proposed to use a 45 cm x 45 cm antenna aperture requiring a 63.6 cm QZ.

- Proposal 4 from Keysight Technologies in R4-2514328

  - Proposal 4: VSAT vendors/NTN operators to provide maximum antenna aperture for each NTN VSAT type

- Moderator Recommendation:

- The 63 cm QZ for a 45 x 45 antenna appears to be the smallest viable size

- For commercial reasons, a larger QZ should be considered

- Discussion required

2nd Ad-hoc Discussion:

FL: more information of VSAT is needed, the current size value can be a starting point.

Agreements after 2nd ad-hoc:

- The [63] cm QZ for a [45 x 45] antenna appears to be the smallest viable size

- For commercial reasons, a larger QZ would be considered

Sub-topic 2-4: DUT size and weight

- Proposals:

- Proposal 3 from Keysight Technologies in R4-2514328

  - Proposal 3: VSAT vendors/NTN operators to provide the maximum VSAT device size and weight limits for each NTN VSAT type

- Moderator Recommendation:

- Inputs required

Agreements after 2nd ad-hoc:

VSAT vendors/NTN operators to provide the maximum VSAT device size, antenna size and relative position, and weight limits for each NTN VSAT type.

Sub-topic 2-5: White box vs black box

- Proposals:

- Proposal 3 from Samsung in R4-2513876

  - Proposal 3:	RAN4 needs to discuss if the centre of radiation aperture to be aligned with QZ centre (like white-box approach)

- Moderator Recommendation:

2nd ad-hoc discussion:

Keysight: this is also related to DUT size and antenna size.

Ad-hoc chair: further discuss next meeting.

Sub-topic 2-9: Beam peak search

- Proposals:

- Proposal 4 from Samsung in R4-2513876

  - Proposal 4:	RAN4 needs to discuss if beam peak search for VSAT testing can be skipped and only focus on the single set of measurement grid.

- Proposal 2 from Eutelsat Group in R4-2514266

  - Proposal 2: The extent of beam pointing performance testing requires further discussion as this is linked to ephemeris data and VSAT propagator performance.

- Proposal 5 from Keysight Technologies in R4-2514328

  - Proposal 5: VSAT vendors to provide the worst-case antenna assumptions and beam steering assumptions for each of the NTN VSAT Types in Table 9.2.1.0-1 of [2] to assess the minimum required measurement angular grid spacing as well as the measurement uncertainty for BP searches, TRP, and off-axis EIRP emission density limits/gain measurements.

- Moderator Recommendation:

- Discussion required

2nd ad-hoc Discussion:

Keysight: the range of beam peak searching is defined in the spec.

Qualcomm: this depends on which requirement will be test, would be different for min EIS or regulatory requirements.

Samsung: we do not think this is required in core requirements. The range is declared by vendor, this is different from FR2 which has peak requirements.

Huawei: we think the peak search is necessary.

Sub-topic 2-10: Beamlock

- Proposals:

- Proposal 3 from Eutelsat Group in R4-2514266

  - Proposal 3: For test purposes, the VSAT shall be locked on boresight and manually positioned to point towards the SAN probe antenna.

- Proposal 7 from Keysight Technologies in R4-2514328

  - Proposal 7: VSAT vendors/NTN operators to clarify whether VSATs automatically steer the UL&DL beams towards the DL direction inside the OTA test system where the DL direction corresponds to the IFF probe (reflector direction) connected to the DL port of the communication tester.

- Proposal 8 from Keysight Technologies in R4-2514328

  - Proposal 8: If VSATs support automatic beam steering towards the DL probe in the OTA test system, a beam lock test function required for TRP measurements must be defined and supported by VSAT vendors.

- Moderator Recommendation:

- Discussion required

2nd ad-hoc Discussion:

Qualcomm: is this VSAT same as FR2 UE?

Huawei: can we locate the link antenna to the top of VSAT

Samsung: this is also related to VSAT type. Maybe different for different VSAT types

Keysight: RAN4 should further study this aspect.

Agreements after 2nd ad-hoc:

RAN4 further check whether beam steering is needed for VSAT conformance testing.

Sub-topic 2-12: Antenna gain measurement

- Proposals:

- Proposal 5 from Eutelsat Group in in R4-2514266

  - Proposal 5: Antenna pattern is required to be measured in the UL and DL for regulatory purposes.

- Proposal 5 from Eutelsat Group in in R4-2514266

  - Proposal 6: Due to lack of RF connectors, the antenna gain can only be measured via the L or S band modem port.

- Proposal 10 from Keysight Technologies in R4-2514328

  - Proposal 10: NTN Operators/VSAT vendors to clarify whether the antenna gain requirements in Clause 9.6.2 are merely system level requirements or whether they shall be verified as part of the conformance assessment of NTN VSAT devices.

- .Proposal 11 from Keysight Technologies in R4-2514328

  - Proposal 11: Clarify the test frequencies for the antenna gain measurements and whether they are applicable only to the UL or UL&DL

- Proposal 12 from Keysight Technologies in R4-2514328

  - Proposal 12: VSAT vendors/NTN operators to confirm that the intention of the antenna performance gain metric is to assess the isolated antenna performance with a non-signalling test

- Proposal 13 from Keysight Technologies in R4-2514328

  - Proposal 13: VSAT vendors/NTN operators to confirm that the conducted port(s) of the antenna system can and will be made available with the beam steering states controllable using a test interface for all NTN VSAT types

- Proposal 17 from Keysight Technologies in R4-2514328

  - Proposal 17: Perform the antenna gain test cases (non-signalling testing) using CP measurements.

- Moderator Recommendation:

- Antenna gain cannot be directly measured due to lack of antenna ports (except for mechanical VSAT). Therefore, as currently defined, this requirement is not testable.

- Discussion required

Online discussion:

WF for further checking: clarification of core spec for this antenna gain issue should be done in NTN WIs.

Eutelsat: in the next meeting, we will bring maintenance CR to 101-5. In this meeting, we need to clarify the antenna gain not testable.

Agreement after online:

The antenna gain is not testable from conformance perspective.

2nd ad-hoc discussion:

Keysight: Wording update is needed

Updated Agreements after 2nd ad-hoc:

Agreement:

The antenna gain is not testable from conformance perspective and should be considered as a system level requirement.

Clarification of core spec for this antenna gain topic should be done from Rel-18/19 NTN WIs.