3GPP TSG-RAN4 Meeting # 116-bis	R4-2514857

# Prague, CZ, October 13th – October 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for Conditional Intra-CU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Capturing latest agreements and performing editorial changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the agreement about CLTM measurement time requirements for target cells without active TCI states in FR2. - Do some editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete CLTM requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.2, 6.3.2.2.1, 6.3.2.2.2, 6.3.2.2.3, 6.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

## 6.3.2	Conditional L1/L2-Triggered Mobility

#### 6.3.2.1	Introduction

The purpose of CLTM cell switch is to switch PCell to a target cell based on configured CLTM execution condition.

The requirements in this clause are applicable to both intra-frequency and inter-frequency CLTM cell switch and subsequent CLTM cell switch.

The requirements in this clause are applicable only to SA, including the following scenarios:

PCell switch to a neighbouring CLTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to a CLTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

#### 6.3.2.2	CLTM Cell Switch delay

Procedure delays for all procedures that can command a CLTM are specified in TS 38.331 [2].

When the UE receives CLTM candidate cell configuration implying CLTM, the UE shall start to transmit the new uplink transmission to the target cell fulfilling the configured CLTM condition within DCLTM seconds from the end of the last TTI containing the CLTM candidate cell configuration.

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt

Where:

- TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CLTM configuration until a condition exists at the measurement reference point which will trigger the CLTM cell switch.

- Tmeasure is the time for the UE to measure and realize the condition for CLTM cell switch is fulfilled. Tmeasure equals to the measurement time stated in clause 6.3.2.2.1.

- TCLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the CLTM target cell stated in clause 6.3.2.2.2.

- TCLTM-interrupt is the interruption time stated in clause 6.3.2.2.3.

##### 6.3.2.2.1	Measurement time

The requirements in this clause apply under the following conditions:

1> The target cell is an FR1 cell, or

1> The target cell is an FR2 cell, and

2> there is at least one TCI state on the active TCI state list of the target cell, or

2> there are no TCI states on the active TCI state list of any of the configured candidate cells.

The measurement delay is defined from the end of TEvent_DU until UE starts conditional cell switch execution to a target cell.

1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell.

2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and

2> if the target cell is an intra-frequency cell:

3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, otherwise.

2> if the target cell is an inter-frequency cell:

3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.3.4 and clause 9.15.5 or 9.15.6 assuming TReport = 0, if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter + TSSB_time_index_inter as defined clause 9.3.4 and in clause 9.15.5 or 9.15.6 assuming TReport = 0, otherwise.

1> If SSB-based L3-RSRP measurement is used in the event and target cell is in FR1, Tmeasure in CHO stated in clause 6.1.4.2.2 is applicable.

1> If CSI-RS-based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of CSI-RS-based L1-RSRP measurement periods of the serving cell and the target cell.

2> serving cell measurement period TL1-RSRP_Measurement_Period_CSI-RS is defined in clause 9.5.4.2 assuming TReport = 0, and

2> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra defined in clause 9.14a.5 assuming TReport = 0

##### 6.3.2.2.2	CLTM RRC processing time

TCLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the CLTM target cell.

1> if the UE supports ltm-FastProcessingConfig-r18 capability, and

2> if the number of LTM and CLTM candidate cell configurations does not exceed maxNumberConfigs-r18, and

2> if the number of the configured serving cells and the cells in the LTM and CLTM configuration does not exceed maxNumberStoredConfigCells-r18

3> TCLTM-RRC-processing = 0

2> else

3> if UE has received CLTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before UE starts cell switch execution to the target cell, and/or

3> if UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before UE starts cell switch execution to the target cell, where NT,2 is defined in section 8.1 of TS 38.213 [3], and

3> if the total number of LTM and CLTM candidate cells for which TCI state(s) were activated or PDCCH order was received before UE starts cell switch execution to target cell does not exceed maxNumberConfigs-r18, and

3> if the total number of serving cells and the cells in the LTM and CLTM configuration for which TCI state(s) were activated and/or PDCCH order was received before UE starts cell switch execution to the target cell does not exceed maxNumberStoredConfigCells-r18

4> TCLTM-RRC-processing = 0

1> else

2> TCLTM-RRC-processing = 10 ms

##### 6.3.2.2.3	Interruption time

The interruption time TCLTM-interrupt is between the time the UE starts to execute cell switch towards the target cell until the time the UE starts to transmit the new uplink transmission to the target cell, excluding TCLTM-RRC-processing.

TCLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

-	TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change, stated in clause 6.3.1.3.

-	Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

-	TRS-proc is the time for SSB processing.

1> if the target TCI state is in the serving cell active TCI state list or is associated to a TCI state in the serving cell active TCI state list

2> Tfirst-RS = 0 and TRS-proc= 0

1> else if the UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list or is associated to a TCI state in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

3> if the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the CLTM candidate cell TCI state activation MAC-CE is received

4> Tfirst-RS = 0 and TRS-proc= 0

1> else if the target cell is an FR1 cell, and the UE is not configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list or is associated to a TCI state in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if not more than TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

3> if not more than TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell

4> Tfirst-RS = 0 and TRS-proc= 0

2> else if the time between the latest PDCCH ordered RACH preamble transmission on the target cell and starting to execute cell switch towards the target cell is not more than 160 ms

3> Tfirst-RS = 0 and TRS-proc= 0

1> else

2> Tfirst-RS is the time to the first SSB transmission on the target cell after TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + TLTM-processing.

2> TRS-proc = 2 ms.

-	TLTM-IU is the interruption uncertainty during CLTM cell switch.

-	For RACH-based CLTM cell switch, TCLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TCLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

-	For RACH-less CLTM cell switch, TCLTM-IU is the uncertainty on transmitting the new uplink transmission on the target cell.

#### 6.3.2.3	Subsequent CLTM Cell Switch delay

When the UE sends a RRCReconfigurationComplete message indicating completion of the previous CLTM cell switch, the UE shall start to transmit the new uplink transmission to the target cell fulfilling the configured CLTM condition within DSubsequent-CLTM seconds from the end of the last TTI containing the RRCReconfigurationComplete message.

DSubsequent-CLTM = TEvent_DU + Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt

Where:

-	Tmeasure, TCLTM-RRC-processing and TCLTM-interrupt are defined in clause 6.3.2.2.

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully sends a RRCReconfigurationComplete message indicating completion of the previous CLTM cell switch until a condition exists at the measurement reference point which will trigger the subsequent CLTM cell switch.

<< End of change >>