3GPP TSG-RAN WG4 Meeting #116bis	R4-2515087

Prague, Czech Republic, 13rd – 17st Oct, 2025

Title: 	           Way Forward for [116bis][316] NR_UE_RF_Ph5

Agenda Item:	7.1.1

Source: 	 Nokia, Murata

Document for:	Approval

# 1. 4Tx with up to 32 dBm MOP

## 1.1 MPR/A-MPR assumptions

Agreement:

- Evaluate assuming following antenna isolation assumptions

  - For CPE, assuming both 10dB and 20dB,

  - For FWA, assuming 20dB.

Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results.

## 1.2 NS to be evaluated

Agreement:

- Evaluate A-MPR for NS_04 and NS_47 for n41.

# 2. PC1.5 with 2Tx for FDD bands

## Maximum output power

Agreement:

- 29 dBm is the maximum output power.

## ACLR assumption

Agreement:

- 31 dB is the agreed ACLR value.

## NS to be evaluated

Agreement:

- Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49.

- Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed.

  - If needed, how to define UTRA ACLR for PC1.5

## REFSENS degradation

Agreement:

Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.