3GPP TSG-RAN WG4 Meeting #116bis		    								               R4-2515156

Prague, Czech Republic, October 13-17, 2025

Agenda item:			7.8.1

Source:	Moderator (Apple)

Title:	Way Forward for [116bis][320] NR_low_band_CA_switching

Document for:	Approval

# Way Forward

Issue 1: Work scope clarification

-	Do not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19

-	Restrict the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier

Issue 2: Common understanding of the UE reference architecture

RAN4 has reached the common understanding of the following aspects related to the UE reference architecture:

-	The main path antenna is shared across Case 1 and Case 3 switching states

-	The diversity path antenna is shared across Case 1 and Case 3 switching states-	UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states

-	An example of the front end filter architecture for the case of adjacent bands is provided below for information

Issue 3: Open issues related to the UE reference architecture

-	Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation

-	Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination

-	Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above

-	Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated

Issue 4: Switching period for switching between Case 1 and Case 3

-	Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations

-	Switching period capability values {35us, 70us, 140us}

-	The capability shall be per band pair per band combination

Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3

-	Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching

Issue 6: Open issues related to the switching time mask for switching between Case 1 and Case 3

-	Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3

-	Whether confirmation by RAN1 is needed on any assumptions made by RAN4

-	How to verify for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests

Issue 7: Possible co-existence issues

-	The preliminary analysis of possible co-existence issues is summarized in the table below for information only (the intention is to give companies a starting point for their analysis)

-	Companies are encouraged to provide their analysis of the issue in the future meetings to allow RAN4 to confirm the co-existence scenarios and to make progress toward identifying possible mitigation

-	It is recommended to arrange future discussion of this issue in the Rel-19 maintenance agenda for the NR_LBCA_Sw work item

| Scenario | Type of interference | CA_n12-n29 | CA_n28-n67 |
| --- | --- | --- | --- |
| UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation) | Co-channel | UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, it may be feasible to mitigate the situation thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)) |
|  | Blocking | Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out |
| BS to UE / UE to BS | Given that co-location is assumed, all those wouldn’t have an issue. |  |  |
| BS to BS | Co-channel interference and blocking | Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out. |

Issue 8: Impact on RX requirements

-	Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking

Issue 9: Related to configuration

-	Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching