3GPP TSG-RAN WG4 Meeting #116bis	R4-2515158

Prague, Czech Republic, 13th – 17th October 2025

Title: 	Way Forward for SBFD BS conformance

Agenda item:	6.13

Source:	Ericsson

Document for:	Approval

# Introduction

The agreements in this WF follow the ad-hoc discussions, online discussion and offline discussions. It also provides the work-split for drafting the CRs for existing requirements.

# Agreements and Way Forward

Issue 1-1-1: References and Definitions related to SBFD capable BS

Agreement:

- Discuss this when drafting CRs based on the work-split

Issue 1-1-2: Manufacture declaration related to SBFD capable BSs

Agreement:

- Introduce new manufacture declarations related to SBFD capable BSs in TS 38.141-1 and TS 38.141-2 based on the work-split.

- Remove multi-band operation from this release and align to Core specs.

- For configuration-limited SBFD-capable BS, all RF channels that implement analogue filters shall be declared together with subband configuration declaration.

- Add the declarations in existing table in clause 4.6

Issue 1-1-3: Testing applicability for SBFD operation

Agreement:

- If both DU/UD and DUD subband configurations are declared to be supported, both DU/UD and DUD subband configurations shall be tested.

Issue 1-1-4: Test signal used to build Test Configurations when testing existing Tx requirements

Agreement:

- For configuration-limited SBFD-capable BS, Table 1 shall be used. FFS SCS in the table.

- Table 1: Signal to be used to build NR SBFD TCs

| SBFD BS type |  | Configuration-limited |
| --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |
|  | Subcarrier spacing | [30] kHz for FR1 [120] kHz for FR2-1 |
|  | Subband configuration | All declared subband configurations |

- For configuration-flexible SBFD-capable BS, use Table 2 as a starting point. FFS SCS in the table.

- Table 2: Signal to be used to build NR SBFD TCs

| SBFD BS type |  | Configuration-flexible |  |
| --- | --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |  |
|  | Subcarrier spacing | [30] kHz for FR1 [120] kHz for FR2-1 |  |
|  | Subband configuration | DUD | DU/UD |
|  |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2. | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL. |

Issue 1-1-5: Test configuration

Agreement:

- Consider single carrier

Issue 1-1-7: RF channel to be tested for SBFD operation

Agreement:

- For configuration-limited SBFD-capable BS, all declared RF channels that implement analogue filters shall be tested.

- FFS RF channels to be tested for configuration-flexible SBFD-capable BS.

Issue 1-1-9: MU and Test tolerance when testing existing Tx requirements

Agreement:

- Re-use existing MU for SBFD symbols testing requirements as a starting point.

- FFS re-use existing TT for SBFD symbols testing requirements.

Issue 1-2-5: Status of Tx when testing Rx requirements

Agreement:

- When testing both conducted and OTA receiver requirements, transmitter shall be ON and transmitting.

# Work-split for CRs drafting

The current work-split is for existing requirements. For the new requirements, work-split to be shared in next meeting.

| Content for both 38.141-1 and 38.141-2 | Current/ New clauses | Responsible company |
| --- | --- | --- |
| References Defintions, symbols and abbreviations Operating bands and channel arrangement | 2, 3, 5 | CATT |
| General conducted and radiated test conditions and declarations | 4 | Ericsson |
| Conducted and Radiated Transmitter characteristics - General - Base station output power - Output power dynamics - Transmit ON/ OFF power | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Samsung |
| Conducted and Radiated Transmitter characteristics - Transmitted signal quality - Unwanted emissions - Transmitted intermodulation | Old clauses: 6.5 to 6.7 New potential clauses to align how it was done with Core specs: 9.5 to 9.7 | Nokia |
| Conducted and Radiated Receiver characteristics - General - Reference sensitivity level - Dynamic range - In-band selectivity and blocking | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Huawei |
| Conducted and Radiated Receiver characteristics - Out of band blocking - Receiver spurious emissions - Receiver intermodulation - In-channel selectivity | Old clauses: 6.5 to 6.8 New potential clauses to align how it was done with Core specs: 9.5 to 9.8 | ZTE |
| Big CR 38.141-1 and 38.141-2 |  | Moderator (Ericsson) |