# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514113

# Prague, Czech Republic, Oct. 13-17, 2025

Source:		Ericsson, Thales

Title:			NTN Ku bands – SAN conformance testing aspects

Agenda item:		5.10.4

Document for:	Discussion

# Introduction

In last RAN4#116 meeting, RAN4 finalized the core aspects of the NTN Ku-bands WI ([1]).

In this contribution we present several aspects related to the SAN conformance testing

# Discussion

## General

With introduction of Ku-bands we have mixed the FR1-NTN and FR2-NTN frequency ranges ranges, with impact on the formulation of several requirements.

For SAN downlink both FR1-NTN and FR2-NTN have been extended with the same 10,7 – 12,75 GHz frequency range:

Table 5.2-1: Satellite operating bands in FR1-NTN

| Satellite operating band1 | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| n252 | 2000 MHz – 2020 MHz | 2180 MHz – 2200 MHz | FDD |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127502,3 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127502,3 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from n256. NOTE 2: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 3: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT |  |  |  |

Table 5.2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n5121 | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n5112 | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n5103 | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 12750 MHz4,5 | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz4,5 | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

As a consequence the definitions of frequency ranges has been modified accordingly:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Due to the overlapping part in FR1-NTN and FR2-NTN the correspondence to traditional Type 1-O and 2-O is affected, one of the implications being that a number of requirements were defined for a new category: SAN Type 1-O operating above 10 GHz.

Observation 1: SAN Type 1-O and SAN Type 2-O are not anymore directly related to the frequency ranges FR1-NTN and FR2-NTN, now sharing a common frequency range. This can be confusing, compared to the BS types, and impact need to be carefully assessed.

The foreseen impact on the conformance testing specifications in TS 38.181 is discussed in the following.

In general, there is no foreseen significant impact on manufacturer declarations, test models or test configurations.

Obviously, support for transmitter channel bandwidth of 25, 35, 50, 70, 100 MHz for SAN Type 1-O need to be introduced, as specified in the recent changes for core specifications TS 38.104.

## Impact on test requirements

Some of the test requirements need modifications, see a few examples:

For output power the new type “SAN type 1-O operating above 10 GHz” can have similar requirements with the Type 2-O. Modification can be as follows:

Table 9.2.5-1: Test requirement for radiated transmit power

|  | Normal test Environment |
| --- | --- |
| SAN type 1-H, SAN type 1-O operating below 10 GHz | f ≤ 3 GHz: ± 3.3 dB |
| SAN type 2-O and SAN type 1-O operating above 10 GHz | 10,7 < f ≤ 12.75GHz and 17.3 GHz < f ≤ 20.2 GHz: ± 5.1 dB |

#### 9.3.5.1	SAN type 1-O operating below 10 GHz

The final TRP measurement result in clause 9.3.4.2 shall remain:

-	within +3.4 dB and -3.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency f ≤ 3.0 GHz;

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the total radiated power is shared between NR and NB-IoT.

#### 9.3.5.2	SAN type 2-O and SAN type 1-O operating above 10 GHz

The final TRP measurement result in clause 9.3.4.2 shall remain:

-	within +5.1 dB and -5.1 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency 10,7 < f ≤ 12.75GHz and 17.3 GHz < f ≤ 20.2 GHz;

## Impact on measurement uncertainty

New measurement uncertainties need to be specified for SAN Type 1-O operating above 10 GHz and eventually for SAN Type 2-O operating below 15 GHz.

Observation 2: For SAN Type 1-O the MU above 10 GHz is not a straightforward choice, as the 10-15 GHz interval lies in the middle of previously defined frequency ranges. Reusing either of the MUs does not seem to be the best option.

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR1-NTN OTA transmitter tests

| Clause |  | Maximum OTA Test System uncertainty |  |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power |  | Normal condition: ±1.1 dB, f ≤ 3 GHz ±x dB 10 GHz < f≤ 15 GHz |  |
| 9.3 OTA base station output power |  | ±1.4 dB, f ≤ 3.0 GHz ±x dB 10 GHz < f≤ 15 GHz |  |
| 9.4.3 OTA total power dynamic range |  | ±0.4 dB |  |
| 9.6.2 OTA frequency error |  | ±12 Hz |  |
| 9.6.3 OTA modulation quality |  | ±1 % |  |
| 9.7.2 OTA occupied bandwidth |  | ±30 kHz, BWChannel 3 MHz ±100 kHz, BWChannel 5 MHz, 10 MHz ±300 kHz, BWChannel 15 MHz, 20 MHz 25, 35, 50, 70, 100 MHz ? |  |
| 9.7.3 OTA ACLR |  | f ≤ 3.0 GHz ±1 dB, BW ≤ 20 MHz ±x dB for 25, 35, 50, 70, 100 MHz |  |
| 9.7.4 OTA out-of-band emissions |  | Absolute power ±1.8 dB, f ≤ 3.0 GHz ±x dB 10 GHz < f≤ 15 GHz |  |
| 9.7.5.2 OTA transmitter spurious emissions, mandatory requirements |  | ±2.3 dB, 30 MHz < f ≤ 6 GHz ±4.2 dB, 6 GHz < f ≤ 15 GHz |  |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |  |  |

Table 4.1.2.2-3: Maximum OTA Test System uncertainty for FR2-NTN OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 9.2 Radiated transmit power | Normal condition: [±1.7] dB, 17.3 GHz ≤ f ≤ 20.2 GHz ±x dB 10 GHz < f≤ 15 GHz |
| 9.3 OTA base station output power | [±2.1] dB, 17.3 GHz ≤ f ≤ 20.2 GHz ±x dB 10 GHz < f≤ 15 GHz |
| 9.4.3 OTA total power dynamic range | [±0.4] dB |
| 9.6.2 OTA frequency error | [±12] Hz |
| 9.6.3 OTA modulation quality | [±1 %] |
| 9.7.2 OTA occupied bandwidth | 600 kHz |
| 9.7.3 OTA ACLR | [±2.3] dB, 17.3 GHz ≤ f ≤ 20.2 GHz ±x dB 10 GHz < f≤ 15 GHz |
| 9.7.4 OTA out-of-band emissions | Absolute power [±2.7] dB, 17.3 GHz ≤ f ≤ 20.2 GHz ±x dB 10 GHz < f≤ 15 GHz |
| 9.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

# Conclusion

Observation 1: SAN Type 1-O and SAN Type 2-O are not anymore directly related to the frequency ranges FR1-NTN and FR2-NTN, now sharing a common frequency range. This can be confusing and need to be carefully assessed.

Observation 2: For SAN Type 1-O the MU above 10 GHz is not a straightforward choice, as the 10-15 GHz interval lies in the middle of previously defined frequency ranges. Reusing either of the MUs does not seem to be a good option.

Proposal 1: Investigate what is the proper MU for 1-15 GHz and align also with BS specifications.

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT