3GPP TSG-RAN WG4 Meeting #116-bis                                                	R4-2514532

# Prague, CZ, 13th – 17th of October 2025

Agenda item:			4.1.3

Source:	Moderator (Iridium)

Title:	Topic summary for [116bis][325] NR_IoT_NTN_Maintenance_RRM

Document for:	Information

# Introduction

This document is a summary of the proposals made in the contributions submitted under AIs 4.11.2.3, 4.25.3, 4.26.2, 4.27.4 for the RAN4#116-bis meeting.

With the following pre-meeting and meeting schedule:

Topic moderator will be designated to provide the summary for a topic before the meeting

- Before 06.10.2025 (Monday): Session chairs will provide the list of topics with moderator assignments.

- 08.10.2025 (Wednesday), 17:00 UTC: Moderators provide the initial summary for a topic.

- 09.10.2025 (Thursday), 12:00 UTC: Deadline for companies review of initial summary.

- 10.10.2025 (Friday), 17:00 UTC: Moderators submit the formal tdoc of summary for a topic.

- 12.10.2025 (Sunday): Session chairs share the initial meeting notes taking moderators summary in consideration.

- In online discussions, session chairs will handle topics based on the moderator summary.

The following documents submitted into [116bis][325] NR_IoT_NTN_Maintenance_RRM under AI 4.11.2.3 are:

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2514156 | draftCR | draftCR on requirements for less than 5MHz for NTN | Huawei, HiSilicon | Endorsement | 4.11.2.3 |

The following documents submitted into [116bis][325] NR_IoT_NTN_Maintenance_RRM under AI 4.25.3 are:

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513188 | discussion | Discussion on RRM core maintenance for Rel-19 NTN phase3 | CATT | Discussion | 4.25.3 |
| R4-2513189 | draftCR | Draft CR to TS 38.133 on R19 core maintenance for NR NTN Phase 3 | CATT | Endorsement | 4.25.3 |
| R4-2513459 | discussion | (NR_NTN_Ph3-Core) Discussion on remaining issues on RRM core requirements of NTN for NR Phase 3 | Xiaomi | Discussion | 4.25.3 |
| R4-2513460 | LS out | Reply LS on SMTC enhancements | Xiaomi | Approval | 4.25.3 |
| R4-2513461 | draftCR | (NR_NTN_Ph3) draftCR on Cell Re-selection requirememts for SMTC enhancement | Xiaomi | Endorsement | 4.25.3 |
| R4-2513609 | discussion | On R19 NTN RRM core part | Apple | Discussion | 4.25.3 |
| R4-2513680 | discussion | Discussion on the RRM requirement maintenance for NTN phase3 | CMCC | Discussion | 4.25.3 |
| R4-2513681 | draftCR | Draft CR on correction of SMTC enhacement requirement for NTN phase 3 | CMCC | Endorsement | 4.25.3 |
| R4-2513907 | discussion | Discussion on core requirements of R19 NR NTN Phase 3 | vivo | Discussion | 4.25.3 |
| R4-2514062 | discussion | Discussion on RRM requirements for NTN for NR Phase 3 | Ericsson | Discussion | 4.25.3 |
| R4-2514063 | draftCR | (NR_NTN_Ph3-Core) draft CR on Handover for SAN with 6 SMTCs and different periodicities | Ericsson | Endorsement | 4.25.3 |
| R4-2514102 | other | Discussion on RRM requirements on NTN phase 3 | ZTECorporation,Sanechips | Approval | 4.25.3 |
| R4-2514170 | LS out/Discussion paper? | Discussion on RRM core requirements for Rel-19 NR NTN | Huawei, HiSilicon | Approval | 4.25.3 |
| R4-2514171 | draftCR | draftCR on RRM requirements for RedCap UE in NTN | Huawei, HiSilicon | Endorsement | 4.25.3 |
| R4-2514172 | draftCR | draftCR on DL coverage enhancement in NTN | Huawei, HiSilicon | Endorsement | 4.25.3 |
| R4-2514332 | draftCR | DraftCR to TS 38.133 on Updating Measurmement Requirements under SMTC enhancements for DL Coverage | Nokia | Endorsement | 4.25.3 |
| R4-2514333 | discussion | Impact of configuring 2 SMTC periodicities over multiple SMTC occasions in NTN | Nokia | Discussion | 4.25.3 |
| R4-2514490 | draftCR | Draft CR38.133 Correction to applicability requirements for RedCap in NTN | Nokia | Endorsement | 4.25.3 |

Papers listed above (AI 4.25.3), split by Tdoc type:

- Type #1: Discussion papers

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513188 | discussion | Discussion on RRM core maintenance for Rel-19 NTN phase3 | CATT | Discussion | 4.25.3 |
| R4-2513459 | discussion | (NR_NTN_Ph3-Core) Discussion on remaining issues on RRM core requirements of NTN for NR Phase 3 | Xiaomi | Discussion | 4.25.3 |
| R4-2513609 | discussion | On R19 NTN RRM core part | Apple | Discussion | 4.25.3 |
| R4-2513680 | discussion | Discussion on the RRM requirement maintenance for NTN phase3 | CMCC | Discussion | 4.25.3 |
| R4-2513907 | discussion | Discussion on core requirements of R19 NR NTN Phase 3 | vivo | Discussion | 4.25.3 |
| R4-2514062 | discussion | Discussion on RRM requirements for NTN for NR Phase 3 | Ericsson | Discussion | 4.25.3 |
| R4-2514102 | other | Discussion on RRM requirements on NTN phase 3 | ZTECorporation,Sanechips | Approval | 4.25.3 |
| R4-2514333 | discussion | Impact of configuring 2 SMTC periodicities over multiple SMTC occasions in NTN | Nokia | Discussion | 4.25.3 |

- Type #2: Draft CRs

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513189 | draftCR | Draft CR to TS 38.133 on R19 core maintenance for NR NTN Phase 3 | CATT | Endorsement | 4.25.3 |
| R4-2513461 | draftCR | (NR_NTN_Ph3) draftCR on Cell Re-selection requirememts for SMTC enhancement | Xiaomi | Endorsement | 4.25.3 |
| R4-2513681 | draftCR | Draft CR on correction of SMTC enhacement requirement for NTN phase 3 | CMCC | Endorsement | 4.25.3 |
| R4-2514063 | draftCR | (NR_NTN_Ph3-Core) draft CR on Handover for SAN with 6 SMTCs and different periodicities | Ericsson | Endorsement | 4.25.3 |
| R4-2514171 | draftCR | draftCR on RRM requirements for RedCap UE in NTN | Huawei, HiSilicon | Endorsement | 4.25.3 |
| R4-2514172 | draftCR | draftCR on DL coverage enhancement in NTN | Huawei, HiSilicon | Endorsement | 4.25.3 |
| R4-2514332 | draftCR | DraftCR to TS 38.133 on Updating Measurmement Requirements under SMTC enhancements for DL Coverage | Nokia | Endorsement | 4.25.3 |
| R4-2514490 | draftCR | Draft CR38.133 Correction to applicability requirements for RedCap in NTN | Nokia | Endorsement | 4.25.3 |

- Type #3: Draft LS

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513460 | LS out | Reply LS on SMTC enhancements | Xiaomi | Approval | 4.25.3 |
| R4-2514170 | Discussion paper submitted as “LS out” | Discussion on RRM core requirements for Rel-19 NR NTN | Huawei, HiSilicon | Approval | 4.25.3 |

The following documents submitted into [116bis][325] NR_IoT_NTN_Maintenance_RRM under AI 4.26.2 are:

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513636 | discussion | Discussion on RRM core requirements for IoT NTN phase 3 | MediaTek inc. | Discussion | 4.26.2 |
| R4-2513637 | draftCR | CR on channel quality report in Msg-3-EDT for anchor carrier | MediaTek inc. | Endorsement | 4.26.2 |
| R4-2513682 | discussion | Discussion on the RRM requirement maintenance for IoT NTN phase3 | CMCC | Discussion | 4.26.2 |
| R4-2513683 | draftCR | Draft CR on correction of CB-msg3 based random access requirement | CMCC | Endorsement | 4.26.2 |
| R4-2514178 | discussion | Discussion on RRM core requirements for Rel-19 IoT NTN | Huawei, HiSilicon | Discussion | 4.26.2 |
| R4-2514179 | draftCR | draftCR on RRM core requirements for Rel-19 IoT NTN | Huawei, HiSilicon | Endorsement | 4.26.2 |
| R4-2514338 | discussion | Replying to RAN2 LS on CQI reporting in MSG3 | Nokia | Discussion | 4.26.2 |
| R4-2514340 | draftCR | DraftCR to TS 36.133 on conditional HARQ-ACK feedback for CB-Msg3 Requirements | Nokia | Endorsement | 4.26.2 |
| R4-2514341 | draftCR | DraftCR to TS 36.133 on CQI reporting for CB-Msg3 Requirements | Nokia | Endorsement | 4.26.2 |

Papers listed above, split by Tdoc type:

- Type #1: Discussion papers

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513636 | discussion | Discussion on RRM core requirements for IoT NTN phase 3 | MediaTek inc. | Discussion | 4.26.2 |
| R4-2513682 | discussion | Discussion on the RRM requirement maintenance for IoT NTN phase3 | CMCC | Discussion | 4.26.2 |
| R4-2514178 | discussion | Discussion on RRM core requirements for Rel-19 IoT NTN | Huawei, HiSilicon | Discussion | 4.26.2 |
| R4-2514338 | discussion | Replying to RAN2 LS on CQI reporting in MSG3 | Nokia | Discussion | 4.26.2 |

- Type #2: Draft CRs

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513637 | draftCR | CR on channel quality report in Msg-3-EDT for anchor carrier | MediaTek inc. | Endorsement | 4.26.2 |
| R4-2513683 | draftCR | Draft CR on correction of CB-msg3 based random access requirement | CMCC | Endorsement | 4.26.2 |
| R4-2514179 | draftCR | draftCR on RRM core requirements for Rel-19 IoT NTN | Huawei, HiSilicon | Endorsement | 4.26.2 |
| R4-2514340 | draftCR | DraftCR to TS 36.133 on conditional HARQ-ACK feedback for CB-Msg3 Requirements | Nokia | Endorsement | 4.26.2 |
| R4-2514341 | draftCR | DraftCR to TS 36.133 on CQI reporting for CB-Msg3 Requirements | Nokia | Endorsement | 4.26.2 |

The following documents submitted into [116bis][325] NR_IoT_NTN_Maintenance_RRM under AI 4.27.4 are:

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513824 | discussion | Maintenance of RRM core requirements for IoT NTN TDD | Iridium, CCL, THALES | Discussion | 4.27.4 |
| R4-2514180 | discussion | Discussion on core requirements for IoT NTN TDD | Huawei, HiSilicon | Discussion | 4.27.4 |
| R4-2514181 | draftCR | draftCR on core requirements for IoT NTN TDD | Huawei, HiSilicon | Endorsement | 4.27.4 |
| R4-2514250 | discussion | On remaining issues of IoT_NTN_TDD related to RRM core | Nordic Semiconductor ASA | Decision | 4.27.4 |
| R4-2514342 | discussion | Open issues the measurement framework for the new IOT TDD Punctured frame | Nokia | Discussion | 4.27.4 |
| R4-2514343 | draftCR | DraftCR to TS 36.133 on Editorial Correction for IoT NTN TDD Requirements | Nokia | Endorsement | 4.27.4 |

Papers listed above (AI 4.27.4), split by Tdoc type:

- Type #1: Discussion papers

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2513824 | discussion | Maintenance of RRM core requirements for IoT NTN TDD | Iridium, CCL, THALES | Discussion | 4.27.4 |
| R4-2514180 | discussion | Discussion on core requirements for IoT NTN TDD | Huawei, HiSilicon | Discussion | 4.27.4 |
| R4-2514250 | discussion | On remaining issues of IoT_NTN_TDD related to RRM core | Nordic Semiconductor ASA | Decision | 4.27.4 |
| R4-2514342 | discussion | Open issues the measurement framework for the new IOT TDD Punctured frame | Nokia | Discussion | 4.27.4 |

- Type #2: Draft CRs

| TDoc Number | TDoc Type | Title | Company/Source | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- |
| R4-2514181 | draftCR | draftCR on core requirements for IoT NTN TDD | Huawei, HiSilicon | Endorsement | 4.27.4 |
| R4-2514343 | draftCR | DraftCR to TS 36.133 on Editorial Correction for IoT NTN TDD Requirements | Nokia | Endorsement | 4.27.4 |

# Recommendations for TDocs treatment

## (AI 4.11.2.3) RRM Core Req [NR_IoT_NTN_req_test_enh-Core]

| Tdoc number | Type | Recommendation | Comments |
| --- | --- | --- | --- |
| R4-2514156 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |

## (AI 4.25.3) RRM Core Req [NR_NTN_Ph3-Core]

| Tdoc number | Type | Recommendation | Comments |
| --- | --- | --- | --- |
| R4-2513188 | discussion | Discuss |  |
| R4-2513189 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2513459 | discussion | Discuss |  |
| R4-2513460 | LS out | Discuss | to be discussed during the meeting |
| R4-2513461 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2513609 | discussion | Discuss |  |
| R4-2513680 | discussion | Discuss |  |
| R4-2513681 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2513907 | discussion | Discuss |  |
| R4-2514062 | discussion | Discuss |  |
| R4-2514063 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514102 | other | Discuss |  |
| R4-2514170 | Discussion paper submitted as “LS out” | Discuss | to be discussed during the meeting |
| R4-2514171 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514172 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514332 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514333 | discussion | Discuss |  |
| R4-2514490 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |

## (AI 4.26.2) RRM Core Req [IoT_NTN_Ph3-Core]

| Tdoc number | Type | Recommendation | Comments |
| --- | --- | --- | --- |
| R4-2513636 | discussion | Discuss |  |
| R4-2513637 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2513682 | discussion | Discuss |  |
| R4-2513683 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514178 | discussion | Discuss |  |
| R4-2514179 | draftCR | Discuss |  |
| R4-2514338 | discussion | Discuss |  |
| R4-2514340 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514341 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |

## (AI 4.27.4) RRM Core Req [IoT_NTN_TDD-Core]

| Tdoc number | Type | Recommendation | Comments |
| --- | --- | --- | --- |
| R4-2513824 | discussion | Discuss |  |
| R4-2514180 | discussion | Discuss |  |
| R4-2514181 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |
| R4-2514250 | discussion | Discuss |  |
| R4-2514342 | discussion | Discuss |  |
| R4-2514343 | draftCR | Endorsement? | to be discussed during the meeting, to be endorsed if no objections |

List of topics/sub-topics/issues prior to the meeting is:

- Topic #1: [NR_IoT_NTN_req_test_enh-Core] under AI 4.11.2.3

Sub-topic 1-1: Draft CR

Topic #2: [NR_NTN_Ph3-Core] under AI 4.25.3

- Sub-topic 2-1: SMTC and TRS related

Issue 2-1-1: Decide on the value of Kmulti_SMTC,i

Issue 2-1-2: Trs in Handover requirement

Issue 2-1-3: Trs in Release with Redirection requirement

Issue 2-1-4: Clarify SMTC selection if more than 4 SMTCs are configured

Issue 2-1-5: SMTC NTN enhancements for FR1 only, or both FR1 and FR2?

Issue 2-1-6: SMTC selection rule for when in RRC-IDLE/Inactive

- Sub-topic 2-2: Draft CRs for endorsement: 8 draftCRs

- Sub-topic 2-3: Draft LS for approval: 1 or 2 LS out (one seems to be discussion paper)

- Topic #3: [IoT_NTN_Ph3-Core] under AI 4.26.2

  - Sub-topic 3-1: CQI related

    - Issue 3-1-1: Channel quality reporting in CB‑Msg3‑EDT

  - Sub-topic 3-2: Msg4 related

    - Issue 3-2-1: UE behavior on receiving / not receiving CB‑Msg4 (RAN2 LS)

    - Issue 3-2-2: HARQ‑ACK behavior linked to Msg4 configuration

    - Issue 3-2-2: HARQ‑ACK behavior linked to Msg4 configuration

  - Sub-topic 3-3: Applicability of quality reporting to eMTC

  - Sub-topic 3-4: Draft CRs for endorsement - Five submitted for 36.133

- Topic #4: [IoT_NTN_TDD-Core] under AI 4.27.4

  - Sub-topic 4-1: RCC Re-establishment

    - Issue 4-1-1: Definition/Scaling of Ksatellite

  - Sub-topic 4-2: RRC Connection Redirection to Non-Anchor Carrier

    - Issue 4-2-1: T_period_DL_bitmap

    - Issue 4-2-2: T_DL-UL Switch Time

    - Issue 4-2-3: T_RRC_procedure_delay

    - Issue 4-2-4: T_UL_grant

  - Sub-topic 4-3: Draft CRs for endorsement – Two submitted for 36.133

# Topic #1 AI 4.11.2.3 RRM Core Req [NR_IoT_NTN_req_test_enh-Core]

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2514156 | Huawei, HiSilicon | draftCR – to be discussed during the meeting |

- Topic #1: [NR_IoT_NTN_req_test_enh-Core] under AI 4.11.2.3

  - Draft CR for endorsement

Recommended WF: Endorse the DraftCR.

# Topic #2 AI 4.25.3 RRM Core Req [NR_NTN_Ph3-Core]

Agreements at the previous meeting for AI 4.25.3:

Agreement (online):

- When two different SMTC periodicities (with different offsets) in one frequency layer are configured,

  - No requirements if the SMTC periodicity is smaller than the actual SSB periodicity of the cells associated with the SMTC

  - For Trs in Handover requirement:

    - Trs is the SMTC corresponding to the physical cell ID of the target cell

    - Further discuss the proposal “or the longer SMTCconfigured in the measObjectNR having the same SSB frequency and subcarrier spacing.”

  - For Trs in RRC Connection Release with Redirection requirement:

    - If physical cell ID is provided in RAN2 signalling design, Trs is the SMTC corresponding to the physical cell ID of the target cell

    - Otherwise, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Agreement (ad-hoc):

- When two different SMTC periodicities (with different offsets) in one frequency layer are configured,

  - For Ttarget_cell_SMTC_period in paging interruption requirements:

    - Ttarget_cell_SMTC_period is the SMTC periodicity corresponding to the physical cell ID of the target cell

  - For Kmulti_SMTC, K_satellite, Kp, Klayer1_measurement, and Kgap in RRC Connected mode intra/inter-frequency measurement requirement:

    - To be confirmed in maintenance phase:

      - Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

    - Klayer1_measurement: Calculate scaling factor Klayer1_measurement based on the union of the SMTCs

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513188 | CATT | Proposal 1: For STMC enhancements, if FR2 is supported, RAN4 to consider the further scenario limitation is needed since it is so hard to support FR2 for different satellites, and limitation to single satellite is needed. Observation 1: In R15, RAN4 has already introduced two SMTC configurations, but it is not clear how to choose smtc1 or smtc2 according to the physical cell ID of the target cell. Proposal 2: For Trs in handover requirement, if the UE has not been provided with an SMTC configuration for the target cell, • If companies have the common understanding for the legacy requirements regarding the issue in the above observation, then the legacy principle can be reused and Trs is the SMTC corresponding to the physical cell ID of the target cell. • Otherwise, Trs is maximum SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. Proposal 3: For Trs in RRC Connection Release with Redirection requirement, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. Proposal 4: For TSMTC/TSMTC,i in RRC Re-establishment delay requirements, • If companies have the common understanding for the legacy requirements, then the legacy principle can be reused and TSMTC/TSMTC,i is the SMTC corresponding to the physical cell ID of the target cell. • Otherwise, the maximum SMTC can be used in the definition. Observation 2: For the case of SMTCs partially overlapping with each other, as 2 SMTC with different periodicity configuration may not always be collided for every SMTC occasion. Proposal 5: RAN4 to clarify that the definition of overlapping when configuring two different SMTC periodicities (with different offsets) in one frequency layer, such as: two SMTCs are considered as overlapping if they overlap in one or more occasions. Proposal 6: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. |
| R4-2513189 | CATT | draftCR – to be discussed during the meeting |
| R4-2513459 | Xiaomi | Proposal 1: Applicability of 7 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN. Proposal 2: If the UE supports Location based SMTC selection and related NW configuration is present, requirements in section 4.2C.2.3 and 4.2C.2.4 apply and the SMTCs referenced in the requirements refer to the SMTCs selected by the UE. Proposal 3: RAN4 to clarify that the SMTC periodicity used during PSS/SSS detection is the shortest SMTC periodicity among all the configured SMTCs, for intra-frequency measurement and inter-frequency measurement requirements in RRC_IDLE/INACTIVE mode. |
| R4-2513460 | Xiaomi | LS out – to be discussed during the meeting |
| R4-2513461 | Xiaomi | draftCR – to be discussed during the meeting |
| R4-2513609 | Apple | Proposal 1: For RRC Idle/Inactive mode RRM requirements, if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement. Proposal 2: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE. |
| R4-2513680 | CMCC | Proposal 1: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording of intra-frequency cell measurement is proposed as an example: The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, and the number of SMTC periodicity doesn’t exceed 1 if UE is not capable of twoSMTC-Periodicities-r19, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. If UE doesn’t support twoSMTC-Periodicities-r19, the selection of SMTCs to be used shall have same periodicity with smtc1. If UE support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, the selection of SMTCs to be used is based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [2], otherwise, the selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected. |
| R4-2513681 | CMCC | draftCR – to be discussed during the meeting |
| R4-2513907 | vivo | Proposal 1: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation and no update is needed for current RRC Idle/Inactive mode RRM requirements. Proposal 2: When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For Trs in Handover requirement: - Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured. - For Trs in RRC Connection Release with Redirection requirement: - Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured. Proposal 3: In response to RAN2’s reply LS on supporting SMTC enhancements, RAN4 provides the following feedback: - Regarding the increase in the maximum number of configured SMTCs for idle/inactive from 6 to 7, no update is needed for current RRC Idle/Inactive mode RRM requirements. - The applicability of RAN4 requirements for 6 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN. |
| R4-2514062 | Ericsson | Proposal 1: Confirm Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-the occasion of SMTC#i, and it is applied for idle mode and connected mode. Proposal 2: For handover, it is assumed that the SMTC of the target cell is always provided, not to consider the case where the SMTC of the target cell is not provided. |
| R4-2514063 | Ericsson | draftCR – to be discussed during the meeting |
| R4-2514102 | ZTECorporation,Sanechips | Observation 1: There is a controversial between RAN2 understanding and RAN4 specification description on SMTC selection if more than 4SMTCs are configured by network. Proposal 1: RAN4 shall clarify the principle on SMTC selection if more than 4SMTCs are configured with RAN2 understanding. Observation 2: If UE is capable of twoSMTC-Periodicities-r19, UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. If UE does not support this capability, UE will have same periodicity with smtc1. Proposal 2: RAN4 shall consider RAN4 specification description with RAN2 outcome on two different SMTC priodicities. |
| R4-2514170 | Huawei, HiSilicon | Proposal 1: SMTC enhancements are supported for both FR1-NTN and FR2-NTN. Proposal 2: Add the following texts to cell reselection requirements: For UE supporting Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if refLocList is provided, the requirements in this clause apply for the SMTCs selected by the UE based on the UE location and the reference location associated with SMTCs, subject to parallelSMTC-r17. Proposal 3: Do not define larger M1/M2/M3/M4 values in cell reselection requirements. Proposal 4: For HO requirements, Trs is defined as the SMTC corresponding to the physical cell ID of the target cell, and “longer SMTC configured for the MO” is not needed. Proposal 5: Confirm the RAN4#116 agreement on RRC re-direction. Proposal 6: To support configuring two different SMTC periodicities with different offsets, RAN4 to update the definition of scaling factor Kmulti_SMTC for measurement requirements: For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. Proposal 7: Send LS to inform RAN1 about the need for time offset for PRACH transmission considering UL beam hopping. LS out – to be discussed during the meeting |
| R4-2514171 | Huawei, HiSilicon | draftCR – to be discussed during the meeting |
| R4-2514172 | Huawei, HiSilicon | draftCR – to be discussed during the meeting |
| R4-2514332 | Nokia | draftCR – to be discussed during the meeting |
| R4-2514333 | Nokia | Observation 1: For UEs that do not support the new feature (DL Coverage enhancements), legacy requirements are still applicable. Proposal 1: In IDLE/INACTIVE mode, if the UE is configured with more SMTCs than supported by UE capability in a frequency layer, including the SMTC related to the serving cell, and location information is associated to each SMTC, the UE is expected to downselect the number of SMTCs, based on the location information, to match the maximum supported by UE capability. Proposal 2: The SMTC associated to the serving cell has to be always within the set of SMTCs selected by the UE to perform measurements. Proposal 3: In connected mode, the NW is not expected to configure a number of SMTCs that exceeds UE capability. If a UE is configured with more SMTCs than supported, the UE will only measure the first X SMTCs (where X is the number of supported SMTCs by this UE). Observation 2: In the example of Figure 1, the requirements related to SMTC1 are expanded by 2, even though only 1/8 of the SMTC occasions are colliding with other SMTCs. Figure 1. Two SMTCs (SMTC1 and SMTC2) configured with different periodicities. SMTC 1 is configured with 20 ms periodicity, while SMTC 2 is configured with 160 ms periodicity. All occasions of SMTC2 are overlapped by SMTC1 occasions. Proposal 4: When SMTCs with different periodicities collide, for measurements purposes, the SMTC with longer periodicity shall always take precedence over the SMTC with shortest periodicity. Observation 3: In the example above, the total time for the UE to have 5 measurement opportunities in SMTC2 will be enhanced from 1285 ms to 645 ms. Proposal 5: The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity. Figure 2 – Four SMTCs with different periodicities and offsets. Proposal 6: For SMTCs with shorter periodicity that overlayp with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions. Figure 3 – Example with four configured SMTCs with different periodicities and offsets. SMTC occasions which overlap with SMTCs with different periodicities will be used for SMTCs with the largest periodicity |
| R4-2514490 | Nokia | draftCR – to be discussed during the meeting |

For ONline and OFFline sessions:

Topic #2 [NR_NTN_Ph3-Core] AI 4.25.3

- Sub-topic 2-1: SMTC and TRS related

Issue 2-1-1: Decide on the value of Kmulti_SMTC,i

- Proposal 1: Confirm Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-the occasion of SMTC#i
(multiple companies)

- Proposal 2: The value of K_multi_SMTC is to be calculated only considering SMTCs with same periodicity.

  - For SMTCs with shorter periodicity that overlayp with SMTCs with larger periodicity the formula for the measurement requirements shall consider a factor L_multi_SMTC, which corresponds to the number of measurement opportunities lost due to collision with SMTCs with larger periodicity within 5 SMTC occasions.

- Moderator: Can this be applied to both idle mode and connected mode ?

Issue 2-1-2 : Trs in Handover requirement:

- Moderator: All companies agree: if the UE has been provided with an SMTC configuration for the target cell(in the handover command), then Trs is the SMTC periodicity of that target NR SAN cell.

- If the UE has NOT been provided with an SMTC:

- Proposal 1: For Trs in handover requirement, if the UE has not been provided with an SMTC configuration for the target cell, 	… then the legacy principle can be reused:
Trs is the SMTC corresponding to the physical cell ID of the target cell.

- Proposal 2: Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

Issue 2-1-3 : Trs in Release with Redirection requirement:

- Proposal 1: Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

- Proposal 2: Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection if only a single SMTC periodicity is configured, and longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing if multiple SMTC periodicities are configured.

Issue 2-1-4: Clarify SMTC selection if more than 4 SMTCs are configured

- Proposal 1: For RRC Idle/Inactive mode RRM requirements, if the NW configures more than 4 SMTCs, the UE in idle/inactive selects up to 4 SMTCs (or less, depending on the UE capability) based on the closest reference locations for measurement.

- Proposal 2: For supporting configuration of more than 4 SMTCs per frequency (i.e., 6), the SMTC selection is left to UE implementation and no update is needed for current RRC Idle/Inactive mode RRM requirements.

- Proposal 3: Do not define larger M1/M2/M3/M4 values in cell reselection requirements.

Issue 2-1-5: SMTC NTN enhancements for FR1 only, or both FR1 and FR2?

- Proposal 1: if FR2 is supported, RAN4 to consider the further scenario limitation is needed since it is so hard to support FR2 for different satellites, and limitation to single satellite is needed.

- Proposal 2: Support for both

Issue 2-1-6 SMTC selection rule for when in RRC-IDLE/Inactive

- Proposal 1: Update the SMTC selection rule, following wording of intra-frequency cell measurement is proposed as an example:

- The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, and the number of SMTC periodicity doesn’t exceed 1 if UE is not capable of twoSMTC-Periodicities-r19, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the

- SMTCs can be measured. If UE doesn’t support twoSMTC-Periodicities-r19,  the selection of SMTCs to be used shall have same periodicity with smtc1. If UE support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, the selection of SMTCs to be used is based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [2], otherwise, the selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected.

- Proposal 2: if the UE is configured with more SMTCs than supported by UE capability in a frequency layer, including the SMTC related to the serving cell, and location information is associated to each SMTC, the UE is expected to downselect the number of SMTCs, based on the location information, to match the maximum supported by UE capability.

Issue 2-1-7 Clarification for RACH transmission in case of satellite beam hopping

- Proposal 1: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

- Proposal 2: Send LS to inform RAN1 about the need for time offset for PRACH transmission considering UL beam hopping.

Sub-topic 2-2: draftCRs

Sub-topic 2-1: draftLS

# Topic #3 AI 4.26.2 RRM Core Req  [IoT_NTN_Ph3-Core]

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513636 | MediaTek inc. | Proposal 1: Add new clause on DL channel quality reporting in MSG3 for CB-Msg3-EDT, with time period T updated as follows: - T is the period before CB-Msg3 transmission used for NRSRP measurement for CB-Msg3-EDT procedure on anchor carrier. |
| R4-2513637 | MediaTek inc. | draftCR – to be discussed during the meeting |
| R4-2513682 | CMCC | Proposal 1: Include following UE behavior when receiving the CB-Msg4 in the Clause “Correct behaviour when receiving CB-Msg4” according to RAN2’s agreements: - Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH. - Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE. Proposal 2: For “Correct behaviour when not receiving CB-Msg4”, include following UE behavior when the CB-Msg3ResponseTimer expires and the UE has reached maximum number of transmissions according to RAN2’s agreements: - When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, and the UE has not received a CB-Msg4, or the UE doesn’t successfully decode the corresponding PDSCH after the CB-Msg3ResponseTimer has expired, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation. Proposal 3: For NB-IoT UE, take the legacy MSG3-based CQI report requirement in Clause 6.6A.2.6 as the reference, with removing the non-anchor carrier part, and re-define the period to derive reported NPDCCH repetition level as the period before CB-Msg3 transmission used for NRSRP measurement for enhanced coverage level estimation. Proposal 4: For eMTC CE-ModeA UE, further study how and when does the UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level. Proposal 5: If RAN4 fail to see the feasibility of introducing the CQI reporting for CB-Msg3 EDT for eMTC UE, LS is needed to inform RAN2. |
| R4-2513683 | CMCC | draftCR – to be discussed during the meeting |
| R4-2514178 | Huawei, HiSilicon | Proposal 1: In 6.6A.2.6, define new requirements for channel quality reporting in CB-Msg3 on anchor carrier for NB-IoT, where the measurement is performed in period before CB-Msg3 transmission. |
| R4-2514179 | Huawei, HiSilicon | draftCR – to be discussed during the meeting |
| R4-2514338 | Nokia | Proposal 1: For the case where CB-MSG3 EDT is applied, revise the definition to T1 to make it suitable for the CB-MSG3 EDT transmission. Proposal 2: In case of CB-Msg3 retransmission, if the back-off timer is below [2] DRX Cycles and the UE cannot complete the CQI report measurements before the next CB-Msg3 attempt, the UE is allowed to postpone the next CB-Msg3 transmission to a later CB-Msg3 transmission window, to complete the channel quality measurement. Proposal 3: In case of CB-Msg3 retransmission, if the interval between the current retransmission and the previous CB-Msg3 transmission is below 4 DRX Cycles, the UE is allowed to reuse the CQI reported in the previous CB-Msg3 transmission. Proposal 4: Capture in the specification that: the UE shall send HARQ-ACK if the contention resolution is successful only if HARQ feedback is configured in Msg4. |
| R4-2514340 | Nokia | draftCR – to be discussed during the meeting |
| R4-2514341 | Nokia | draftCR – to be discussed during the meeting |

For ONline and OFFline sessions:

- Topic #3: [IoT_NTN_Ph3-Core] under AI 4.26.2

  - Sub-topic 3-1: CQI related

    - Issue 3-1-1: Channel quality reporting in CB‑Msg3‑EDT 
(All companies agree: anchor only, and NB IoT only)

      - Proposal 1: Add new clause on DL channel quality reporting in MSG3 for CB-Msg3-EDT, with time period T updated as follows:

T is the period before CB-Msg3 transmission used for NRSRP measurement for CB-Msg3-EDT procedure on anchor carrier.

      - Proposal 2: For the case where CB-MSG3 EDT is applied, revise the definition to T1 to make it suitable for the CB-MSG3 EDT transmission.

      - Proposal 3: In 6.6A.2.6, define new requirements for channel quality reporting in CB-Msg3 on anchor carrier for NB-IoT, where the measurement is performed in period before CB-Msg3 transmission.

- Recommended WF: Proposal 1, seems to include all other proposals.

Issue 3-1-2: CQI, CB-Msg3 retransmission and DRX

      - Proposal 1: In case of CB-Msg3 retransmission, if the back-off timer is below [2] DRX Cycles and the UE cannot complete the CQI report measurements before the next CB-Msg3 attempt, the UE is allowed to postpone the next CB-Msg3 transmission to a later CB-Msg3 transmission window, to complete the channel quality measurement

      - Proposal 2: In case of CB-Msg3 retransmission, if the interval between the current retransmission and the previous CB-Msg3 transmission is below 4 DRX Cycles, the UE is allowed to reuse the CQI reported in the previous CB-Msg3 transmission.

Recommended WF: Proposal 1 and 2 go hand in hand. Companies to discuss.

  - Sub-topic 3-2: Msg4 related

    - Issue 3-2-1: UE behavior on receiving / not receiving CB‑Msg4 (RAN2 LS)

      - Proposal 1: Include following UE behavior when receiving the CB-Msg4 in the Clause “Correct behaviour when receiving CB-Msg4” according to RAN2’s agreements:

        - Upon receiving a CB-Msg4 including a matching Contention Resolution Identity and a C-RNTI without a RRC message, UE shall continue monitoring PDCCH.

        - Upon receiving a CB-Msg4 including a matching Contention Resolution Identity without both a RRC message and a C-RNTI, UE shall terminate the CB-Msg3-EDT procedure and keep in RRC_IDLE.

- Proposal 2: For “Correct behaviour when not receiving CB-Msg4”, include following UE behavior when the CB-Msg3ResponseTimer expires and the UE has reached maximum number of transmissions according to RAN2’s agreements:

        - When the CB-Msg3ResponseTimer expires and the maximum number of re-attempts has been reached, and the UE has not received a CB-Msg4, or the UE doesn’t successfully decode the corresponding PDSCH after the CB-Msg3ResponseTimer has expired, RRC procedure is re-initiated, which procedure is initiated is up to UE implementation.

- Recommended WF: Agree to both Proposals 1 and 2, since they describe 2 scenarios(when CB-msg4 is received, and when it Is not received)

    - Issue 3-2-2: HARQ‑ACK behavior linked to Msg4 configuration

      - Proposal 1: the UE shall send HARQ-ACK if the contention resolution is successful, only if HARQ feedback is configured in Msg4.

- Recommended WF: Agree to Proposal 1.

  - Sub-topic 3-3: Applicability of channel quality reporting in CB‑Msg3‑EDT to eMTC

- Proposal 1: For eMTC CE-ModeA UE, further study how and when does the UE measure the channel quality for deriving the reported MPDCCH repetition level or aggregation level.

- AND (A consequence)

- Proposal 2: If RAN4 fail to see the feasibility of introducing the CQI reporting for CB-Msg3 EDT for eMTC UE, LS is needed to inform RAN2.

- Recommended WF: Agree to both Proposals, they are sequential in nature.

- Sub-topic 3-4: Draft CRs for endorsement - Five submitted for 36.133

# Topic #4 (AI 4.27.4) RRM Core Req [IoT_NTN_TDD-Core]

| TDoc # | Company | Proposals/Observations |
| --- | --- | --- |
| R4-2514342 | Nokia | Proposal 1: For re-establishment requirements, if the UE is provided with information about the DL periods of the neighbor cells, the value of Ksatellite is equal to 2 if there are overlapping DL opportunities from cells associated to different satellites, otherwise, Ksatellite equal to 1. Proposal 2: For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TUL_grant shall be described as: “It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on the interval between the UE processing the RRC message and the next DL opportunity in the non-anchor carrier and the number of repetitions of NPDCCH used in the non-anchor carrier.” Proposal 3: For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TDL-UL switch shall be described as: “It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is equal to the DL-to-UL gap.” |
| R4-2514250 | Nordic Semi | Proposal-1: Update requirement 6.9.B2 as the following: ### 6.9B.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the DL burst containing the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 172 ms. - Tperiod_DL_bitmap: 0 ms. - TUL_grant: It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on Tperiod_DL_bitmap and the number of repetitions of NPDCCH used in the non-anchor carrier. - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 50 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110 148 ms, the overall RRC connection redirection delay is extended by multiples of 90ms may be extended.. Proposal-2: Definition of Ksatellite can be reused for R19 IoT NTN TDD. |
| R4-2514180 | Huawei | Proposal 1: Re-use the definition of Ksatellite as in existing requirements. RAN4 to revert the following agreement from RAN4#116, i.e. do not introduce the following scaling: - Scale Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC by a factor of 2, when the UE measures cells across 3 or more different patterns of DL opportunities Proposal 2: For re-direction requirements - Change TRRC_procedure_delay from 110ms to 172ms - Change TDL-UL switch from 8ms to 50ms. |
| R4-2513824 | Iridium, CCL, THALES | - Increase the value TRRC_procedure_delay from 110ms to 160ms to accommodate fixed time gap of 50ms between active downlink and active uplink time. - Existing value of 8ms for TDL-UL switch is not sufficient for IoT NTN TDD mode and should be updated to 50ms. To take into account the time delay associated with the concept of Tperiod_DL_bitmap , RAN4 could consider (at least) three options: - Option-1: Increase the value of TRRC_procedure_delay from 110ms to 250ms to accommodate the fixed time gap of 50ms between active downlink and active uplink periods, as well as the 90ms periodicity of active downlink subframes. In this case Tperiod_DL_bitmap = 0. - Option-2: Introduce a new time component Tperiod_DL_periodcity = 90ms in the place of Tperiod_DL_bitmap. - Option-3: Use the value Tperiod_DL_bitmap = 90ms. - Proposal 4: Increase the value of TRRC_procedure_delay from 110ms to 250ms to accommodate the fixed time gap of 50ms between active downlink and active uplink periods, as well as the 90ms periodicity of active downlink subframes. In this case Tperiod_DL_bitmap = 0. - Proposal 5: For RRC re-establishment delay requirement, re-use existing definition of Ksatellite,i . |

For ONline and OFFline sessions:

## Sub-topic 4-1

Issue 4-1-1:

- Proposals:

  - Proposal 1: Keep existing definition of Ksatellite

  - Proposal 2: if the UE is provided with information about the DL periods of the neighbor cells: the value of Ksatellite is equal to 2 if there are overlapping DL opportunities from cells associated to different satellites, otherwise, Ksatellite equal to 1.

- Recommended WF

  - Majority favor Proposal 1:

    - Keep existing definition of Ksatellite (3 out of 4 companies agree)

## Sub-topic 4-2

Issue 4-2-1: T_period_DL_bitmap

  - Proposal 1: Tperiod_DL_bitmap hardcoded to 0 per prior agreement.

( Reminder: Agreement, recorded in Chair Notes RAN1#120 (Malta)

Confirm the working assumption of RAN1#120b with the following modifications:

DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell.)

- Recommended WF: Proposal 1

Issue 4-2-2: T_DL-UL Switch Time

  - Proposal 1: No need for  T_DL-UL Switch Time: Remove it and only use T_RRC_procedure_delay.

  - Proposal 2: T_DL-UL Switch Time = 50ms

- Recommended WF:

  - Proposal 1: No need for T_DL-UL Switch Time: Remove it and only use T_RRC_procedure_delay.

Issue 4-2-3: T_RRC_procedure_delay (today: “shall be less than 110 ms”):

- Proposal 1: T_RRC_procedure_delay < 172 ms

- Proposal 2: With Tperiod_DL_bitmap = 0:

T_RRC_procedure_delay < 250ms

(fixed time gap = 50ms DL UL) + (90ms DL sf periodicity)

- Recommended WF

  - Proposal 2: T_RRC_procedure_delay < 250ms (then, it is <172 as well).

Issue 4-2-4: T_UL_grant – definition and wording

  - Proposal 1: For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TUL_grant shall be described as: “It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on the interval between the UE processing the RRC message and the next DL opportunity in the non-anchor carrier and the number of repetitions of NPDCCH used in the non-anchor carrier.”

- Recommended WF: Proposal 1.

## Sub-topic 4-3

- Draft CRs for endorsement – Two submitted for 36.133

Issue 4-3-1: Draft CRs R4-2514181 brings 2 points:

    - First: tied to Issue 4-2-3: T_RRC_procedure_delay

    - Secord: Remove delay of 1400ms, that still exists in RRC re-estalishement requirements, update the definition of Ksatellite based on agreement:

If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown Tsearch_NB1-NC,i = Ksatellite *2.52 s. Where Ksatellite,i is defined as the number NGSO satellites to be measured on i-th frequency for RRC re-establishment.

- Recommended WF:

- After agreement on Issue 4-2-3, update the DraftCR to reflect the agreement.

- Do companies agree on 1400ms removal, sentence removal, as above?

Issue 4-3-2: DraftCR R4-2514343

- Recommended WF:

Endorse DraftCR R4-2514343 (Editorial)