3GPP TSG RAN WG4 Meeting #116bis                  	                                   R4-2514822

Prague, CZ, 13th – 17th October 2025

Agenda item:			7.5.1

Source:	China Telecom

Title:	Ad-hoc minutes for NR_demod_Ph6_Ph5

Document for:	Approval

Introduction

The objectives of the Rel-20 Demod performance requirements WI in the approved WID RP-252957 are as follows:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered. - Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

- UE requirements for 6Rx UE with interference

- General

Issue 3-1-2: Test scope

- Proposals:

  - Proposal 1: Focus PDSCH requirements on Mapping Type A only. (Apple, Samsung, Qualcomm)

  - Proposal 2: Do not cover PDSCH absolute physical layer throughput requirements (Nokia)

Ad-hoc agreement

  - Focus PDSCH requirements on Mapping Type A only.

  - Do not cover PDSCH absolute physical layer throughput requirements

Issue 3-1-4: Receiver assumption

- Proposals:

  - Proposal 1: Only MMSE-IRC is considered (Apple, Nokia, Qualcomm)

Ad-hoc agreement

  - Only MMSE-IRC is considered

Issue 3-1-5: Duplex mode and SCS

- Agreement for basic 6Rx UE requirements in R4-2504750

- Only define test cases for FR1 TDD - 30kHz SCS - 40MHz CBW - TDD UL-DL pattern: 7DS2U, S: 6D+4G+4U

- Proposals:

  - Proposal 1: Consider 15kHz/10MHz for FDD and 30kHz/40MHz with 7D1S2U for TDD. (Apple, Ericsson, Nokia, ZTE)

  - Proposal 2: Only to define requirements for FR1 TDD with 30kHz/40MHz 7D1S2U with S: 6D+4G+4U (Samsung)

Ad-hoc agreement

  - Align with the basic 6Rx requirement: Only to define requirements for FR1 TDD with 30kHz SCS/40MHz 7D1S2U with S: 6D+4G+4U

- PDSCH requirements with inter cell interference

Issue 3-2-1: Test scenarios and interference profile

- Proposals on coverage of test scenario:

  - Option 1: Cover both HomNet and Hetnet scenarios (China Telecom, Qualcomm, Samsung, Nokia, Ericsson)

- Nokia: Introduce applicability rules such that if UE supporting both TDD and FDD with 6Rx, UE will pass test cases with 2 interference cells (HomNet scenario) with FDD mode, and pass test cases with 1 interference cell (HetNet scenario) with TDD mode.

- Proposals on interference modelling:

  - Option 1: (China Telecom, Huawei, Qualcomm, Apple, Samsung, Ericsson, Nokia, ZTE) Reuse the same interference modeling defined for Rel-17 2/4Rx and Rel-19 8Rx:

- INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet (if covered) - INR1 = 7.58dB for HetNet (if covered) - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Random 16QAM modulated symbols for the interference cell(s) - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

Ad-hoc agreement

  - Cover both HomNet and HetNet scenarios

  - Interference modelling: Reuse the same interference modeling defined for Rel-17 2/4Rx and Rel-19 8Rx:

- INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet (if covered) - INR1 = 7.58dB for HetNet (if covered) - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Random 16QAM modulated symbols for the interference cell(s) - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

Issue 3-2-2: Rank for the serving cell

- Proposals:

  - Option 1: Cover rank 2 and rank 4 (China Telecom, Nokia, ZTE, Qualcomm)

- Option 1a: rank 2 for HomNet and rank 4 for HetNet (CTC, Nokia)

  - Option 2: Rank1 as starting point (Huawei)

  - Option 3: Rank 1 and Rank 2 (Apple)

  - Option 4: Cover rank2 and FFS rank 4 based on simulation results (Samsung)

  - Option 5: Cover rank 1,2,4 for initial evaluation (Ericsson)

Ad-hoc agreement

  - For initial evaluation purpose, cover rank 1,2 and 4 And make further selection based on simulation.

Issue 3-2-3: Antenna configuration

- Proposals:

  - Option 1: 4T6R (Samsung)

  - Option 2: 2T6R for rank 1/2 and 4T6R for rank 4 (Ericsson)

  - Option 3: 2T6R for rank 2 and 4T6R for rank 4 (Nokia)

Ad-hoc agreement

  - For initial simulation purpose, 2T6R for rank 1/2 and 4T6R for rank 4

Propagation condition

- Candidate options:

  - Option 1: TDLA30-10

  - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only.

  - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

MCS

- Candidate options:

  - Option 1: MCS 17 for HomNet and MCS 13 for HetNet

  - Option 2: MCS 17

  - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

Issue 3-2-6: Other parameters

- Proposals:

  - Proposal 1: Reuse the same configuration for Rel-17 2/4Rx as the starting point. (Samsung, Ericsson, Nokia)

Ad-hoc agreement

  - Reuse the same configuration for Rel-17 2/4Rx as the starting point.

- PDSCH requirements with intra cell inter user interference

Issue 3-3-1: MU-MIMO Beamforming modeling

- Proposals:

  - Option 1: Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2 (Apple, Samsung, Ericsson, Nokia)

Ad-hoc agreement

  - Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2

Issue 3-3-3: Antenna configuration and rank for target + co-scheduled UE

- Proposals:

  - Option 1: Rank 2+2 (China Telecom, Nokia, ZTE, Qualcomm)

- Nokia: Introduce applicability rules for intra-cell inter-user interference so 6Rx UE which pass rank 2 requirements do not have to pass 4Rx rank 2 requirements.

  - Option 2: Rank 1+1 and Rank 2+2 (Samsung)

  - Option 3: Rank 1+1 (2T6R), rank 2+2, rank 2+2 and rank 1+2 (4T6R) for initial evaluation (Ericsson)

QC: Same rank for different UEs.

Samsung: We should evaluate performance for rank 1+1 and rank 2+2 but same rank for different UEs.

Ericsson: Agree Samsung.

Nokia: Our preference is rank 2+2. Fine to consider rank 1+1 in addition.

CMCC: The antenna configuration? 2T for rank 1+1 and 4T for rank 2+2.

ZTE: ok with rank 1+1 and rank 2+2 for evaluation purpose. But for requirement, not to have both.

Nokia: Can we select rank 2+2 if simulation results show feasibility?

QC: Agree with Nokia.

Ad-hoc agreement

  - For initial evaluation purpose, cover Rank 1+1 (with 2T6R) and Rank 2+2 (with 4T6R).

  - Select rank 2+2 if simulation results show feasibility

Issue 3-3-2: Signal power assumptions and SNR assumptions

- Proposals:

  - Option 1: (Nokia)

- Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

- SNR = (STargetUE+ SInterfUE)/N

Ad-hoc agreement

  - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

  - SNR = (STargetUE+ SInterfUE)/N

Issue 3-3-7: Other test parameters and simulation assumptions

- Proposals:

  - Option 1: (China Telecom, Nokia, [Huawei, Qualcomm, Apple, Samsung, Ericsson, ZTE]) Use the existing test parameters for 8Rx as a start point as shown below

- 1 Co-scheduled UE number - Precoding for the target UE: Random Single Panel Type I precoding - 2PRBs for PRB bundling size and precoding granularity - PDCCH and PDSCH allocation in time domain - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain - Full PRB

Ad-hoc agreement

- 1 Co-scheduled UE number - Precoding for the target UE: Random Single Panel Type I precoding - 2PRBs for PRB bundling size and precoding granularity - PDCCH and PDSCH allocation in time domain - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain - Full PRB

- UE requirements for FR1 SU-MIMO with spatial channel model

Issue 2-1-3: Frequency generalization for rCDL channel modeling – part of the approved objectives

Session chair:

At least the proponents for the different options to bring detailed proposals on the channel, the example frequencies (for option 2 and 3):

- Option 1: Define the requirements based on the same channel model for all different FR1 frequencies. (ZTE, E///, Nokia, MTK, QC, BT, China Telecom, Apple, CMCC, Samsung as compromise)

  - Option 1a: The maximum Doppler shift fD should be fixed as [10Hz] to avoid carrier frequency impact on doppler shift (CTC)

- Option 2: Evaluate the performance based on the same channel model for different FDD bands/frequencies, and the same channel model for different TDD bands/frequencies. (HW)

- Option 3: Evaluate the performance difference based on the channel model on the different frequiences, with limited number of example bands/frequencies for the evaluation (for example, around 1GHz, 2GHz, 3GHz). Based on the evaluation results, RAN4 to decide whether or not to define band agnostic or duplex agnostic requirement. (China Telecom, Samsung, Apple)

CMCC: Are we only consider doppler 10Hz? We have multiple ones in legacy.

Samsung: We can compromise to option 1. Also same question as CMCC.

Nokia: We only considered 10Hz in the SI.

Ad-hoc agreement

- Define the requirements based on the same channel model for all different FR1 frequencies for this WI

  - The maximum Doppler shift fD should be fixed as 10Hz

  - FFS if other  fD can be considered

- Proposal on the note to Maximum Doppler after ad-hoc:

  - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be [10]Hz (maximum Doppler).

- Test scope and test parameters for SCM SU-MIMO requirements

Issue 2-2-1-3: Duplex mode and SCS

- Proposals:

  - Option 1: Cover TDD 30kHz SCS with 40MHz CHBW and discuss whether to also cover FDD 15kHz SCS with 10MHz CHBW (China Telecom)

Ad-hoc agreement

- Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

- PDSCH demodulation requirements with SCM

Issue 2-2-2-1: Test scope for PDSCH demodulation requirements with SCM

- Proposals:

  - Proposal 1: Cover PDSCH mapping type A demodulation requirements with 8T8R 8 layer case and the 4T4R 4 layer case. (ZTE, Ericsson, Qualcomm if results are aligned)

Ad-hoc agreement

- Cover PDSCH mapping type A demodulation requirements with 8T8R 8 layer case and the 4T4R 4 layer case.

Issue 2-2-2-2: Precoding and MCS for 8T8R 8L test case

- The assumptions used in the Rel-19 SI:

MCS; 13 (64 QAM table). Codebook configuration for PDSCH and DMRS; CodebookType; Single Panel Type I; Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook.. Codebook configuration; (N1,N2,O1,O2) = (4,1,4,1).

- Proposals:

  - Option 1: Consider random precoding and different MCS levels for the two codewords. For example, choose MCS 13 for CW0 and seek for some suitable lower MCS for CW1. (MTK, Nokia, QC, Apple)

  - Option 2: Follow the assumptions used in Rel-19 SI for CDL: Random precoding and MCS 13 for both CWs (Nokia, ZTE, HW, E///)

QC: option 1 is the only way if we are going to define single SNR value.

CTC: By opt 2. We can run the test twice with different SNR to check the performance for each CW.

HW: Opt 2 is also workable with defining SNR @ max total Thput.

Samsung: How to decide the MCS for CW1?

Ad-hoc agreement

  - For 4t4r 4L test, test metric: SNR @ 70% max TP.

  - For 8T8R 8L test, Test metric: Define single SNR based on:

    - Option a: Based on per CW thput with Different MCS per CW (Nokia, QC)

    - Option b: Based on total thput with different MCS per CW (Apple, MTK, QC)

    - Option c: Based on total thput with same MCS per CW (HW, ZTE, Samsung)

    - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.