3GPP TSG-RAN4 Meeting #116bis	R4-2514876

# Prague, CZ, 13th - 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft big CR on core maintenance for NR_Mob_Ph4_RRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections for Rel-19 NR_Mob_Ph4 maintenance to TS 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | R4-2514855 Draft CR to TS 38.133 on core requirements for Rel-19 mobility enhancements - Some typos need to be revised in clauses 9.14a.3.4, 9.14a.3.5 and clause 9.14a.7. - Revise some typos in clauses 9.14a.3.4, 9.14a.3.5 and 9.14a.7. R4-2514856 Corrections on R19 mobility RRM - Correct some incorrect or redundant content. - Merges R4-2514373 implementing the following agreements: RAN4#112bis Known/unknown cell related: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition RAN4#113 Prerequisite for CSI-RS measurement: - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition. RAN4#114 Whether to use detectable or measurable? - Use measurable and specify the side condition for RS being measurable. RAN4#114bis Issue 4-1-1: Whether to use detectable or measurable? Agreement: - Confirm the RAN4#114 tentative agreement “Use measurable and specify the side condition for RS being measurable” Issue 4-1-2: Side condition for CSI-RS based L1 measurement for neighbor cell Agreement: - Use legacy side condition of SINR=-3dB. R4-2514867 DraftCR for Conditional Intra-CU LTM - Capture the agreement about CLTM measurement time requirements for target cells without active TCI states in FR2. - Do some editorial changes. R4-2514052 DraftCR for LTM event-triggered L1 reporting - Clarify the agreed applicability rule related to event-triggered reporting and on leave reporting: - Event-triggered L1 report for LTM candidate cell may be SSB or CSI-RS based, so change “SSB” to “RS” to cover both. - Clarify that the on leave report needs to be for the same RS as the event-triggered report, because without this clarification any on leave report even for a different candidate cell would be covered, which does not make sense. - Also add MAC spec reference for the event-triggered L1 reporting MAC-CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements remain faulty |  |  |  |  |  |  |  |  |
| Clauses affected: |  | R4-2514855 - 9.14a.3.4, 9.14a.3.5, 9.14a.7 R4-2514856 - 9.14a.2, 9.14a.5 R4-2514867 - 6.3.2.2, 6.3.2.2.1, 6.3.2.2.2, 6.3.2.2.3, 6.3.2.3 R4-2514052 - 6.3.1.1, 8.20.1, 8.25.1, 9.14.3.4, 9.14.3.5, 9.15.3.4, 9.15.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

#### 9.14a.3.4	Event-triggered Periodic Reporting

Reported CSI-RS based L1-RSRP measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses 10.1.19D.2 for FR1 and 10.1.20A.2 for FR2.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.14a.3.5.

#### 9.14a.3.5	Event Triggered Reporting

Reported CSI-RS based L1-RSRP measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.19D.2 for FR1 and 10.1.20A.2 for FR2.

The UE shall not send any event triggered measurement reports as long as no reporting criterion is fulfilled.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

If serving cell is involved in event evaluation, L1-RSRP measurement period of serving cell refers to TL1-RSRP_Measurement_Period_CSI-RS as defined in clause 9.5, assuming TReport = 0 and TCSI-RS is the periodicity of the serving cell CSI-RS configured for event triggered L1-RSRP measurement report.

If neighbor cell is involved in event evaluation, L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_CSI_intra defined in clause 9.14a.5, assuming TReport = 0, and TCSI-RS is the periodicity of the neighbor cell CSI-RS configured for intra-frequency event-triggered L1-RSRP measurement report.

### 9.14a.7	Scheduling availability of UE during CSI-RS based L1-RSRP measurement

Scheduling availability restrictions described in the following clauses apply when UE is performing CSI-based L1-RSRP measurement on neighbor cell(s) without measurement gap.

Unless explicitly stated, the CSI-RS resource to be measured for L1-RSRP measurement is transmitted from neighbor cell(s).

# <End of Change #1>

# <Start of Change #2>

### 9.14a.2	Requirements Applicability

The requirements in the clause 9.14a are applicable to FR1 and FR2-1 for LTM.

The requirements in clause 9.14a apply for CSI-RS L1-RSRP measurements for configured LTM candidate cell, provided the following conditions are met:

-	The cell is known, and

-	The CSI-RS L1-RSRP measurement is configured as periodic CSI-RS or semi-persistent CSI-RS measurement, and

-	at least 48 RBs of the CSI-RS configured for measurement is within the active BWP, and

-	repetition is set to “OFF” or not configured, and

-	UE observed RTD <= CP, and

-	All CSI-RS resources are configured within up to two separate windows where each window is up to 5 ms, and

-	When the configured CSI-RS based L1 RSRP measurement for LTM candidate cell does not exceed UE measurement capability, and

-	For FR1 and FR2-1 when there are scheduling restrictions according to to 9.14a.7, the requirements apply provided all CSI-RS resources within a 40 ms window on one intra-frequency layer are configured within up to two separate windows, each lasting up to 5 ms. The separation of the two 5ms windows is at least 4ms.

A CSI-RS resource configured for L1-RSRP for LTM candidate cell shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	Intra-frequency L1-RSRP related side conditions given in clause 10.1.19D.2 for FR1 and 10.1.20A.2 for FR2-1, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to annex B.2.4.2 for a corresponding band.

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 SSB measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L3 measurement remains detectable according to the cell identification requirements in clause 9.2, and

-	CSI-RS from the target cell configured for L1 measurement remains measurable.

Otherwise, the cell is unknown.

# <End of Change #2>

# <Start of Change #3>

### 9.14a.5	CSI-RS based L1-RSRP measurement requirements without measurement gaps

The requirements specified in this clause are only applicable when

-	maximum RTD between cells are within CP.

- 	At least 48 RBs of the CSI-RS configured for measurement is confined within the active BWP of the UE.

If a neighbor cell is known according to 9.14a.2, the UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS_intra.

The value of TL1-RSRP_Measurement_Period_CSI-RS_intra is defined in table 9.14a.5-1 for FR1 and in table 9.14a.5-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF

- 	N=8 if UE is capable of skippingSSBbasedL1mesurement-R19 and the CSI-RS resources shall be Type-D QCL’ed with the associated SSB for L3 measurement and the CSI-RS resource is configured with LTM-CSI-ResourceConfig; Otherwise,

- 	N=1 if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with SSB for L1-RSRP measurement.

-	P value for CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target CSI-RS resource.

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer measurement period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.14a.5-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS_intra in FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(P)*TCSI-RS_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K*P)*max(TDRX,TCSI-RS_NBC)) |
| DRX cycle > 320 ms | ceil(P)*TDRX |
| NOTE 1: TCSI-RS_NBC is the periodicity of neighbor cell CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1.5. |  |

Table 9.14a.5-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_CSI-RS_intra in FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(P*[PL1_sharing]*N)*TCSI-RS_NBC) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*P*[PL1_sharing]*N)*max(TDRX,TCSI-RS_NBC)) |
| DRX cycle > 320 ms | ceil(M*P*[PL1_sharing]*N)*TDRX |
| NOTE 1: TCSI-RS_NBC is the periodicity of neighbor cell CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

# <End of Change #3>

# <Start of Change #4>

## 6.3.2	Conditional L1/L2-Triggered Mobility

#### 6.3.2.1	Introduction

The purpose of CLTM cell switch is to switch PCell to a target cell based on configured CLTM execution condition.

The requirements in this clause are applicable to both intra-frequency and inter-frequency CLTM cell switch and subsequent CLTM cell switch.

The requirements in this clause are applicable only to SA, including the following scenarios:

PCell switch to a neighbouring CLTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to a CLTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

#### 6.3.2.2	CLTM Cell Switch delay

Procedure delays for all procedures that can command a CLTM are specified in TS 38.331 [2].

When the UE receives CLTM candidate cell configuration implying CLTM, the UE shall start to transmit the new uplink transmission to the target cell fulfilling the configured CLTM condition within DCLTM seconds from the end of the last TTI containing the CLTM candidate cell configuration.

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt

Where:

- TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CLTM configuration until a condition exists at the measurement reference point which will trigger the CLTM cell switch.

- Tmeasure is the time for the UE to measure and realize the condition for CLTM cell switch is fulfilled. Tmeasure equals to the measurement time stated in clause 6.3.2.2.1.

- TCLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the CLTM target cell stated in clause 6.3.2.2.2.

- TCLTM-interrupt is the interruption time stated in clause 6.3.2.2.3.

##### 6.3.2.2.1	Measurement time

The measurement delay is defined from the end of TEvent_DU until UE starts conditional cell switch execution to a target cell.

In FR2, when the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of at least one of these neighbor cells is in the active TCI state list, the requirements apply only to the cell(s) that have TCI state(s) in the active TCI state list.

1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell.

2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and

2> if the target cell is an intra-frequency cell:

3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, otherwise.

2> if the target cell is an inter-frequency cell:

3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.3.4 and clause 9.15.5 or 9.15.6 assuming TReport = 0, if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell, or

3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter + TSSB_time_index_inter as defined clause 9.3.4 and in clause 9.15.5 or 9.15.6 assuming TReport = 0, otherwise.

1> If SSB-based L3-RSRP measurement is used in the event and target cell is in FR1, Tmeasure in CHO stated in clause 6.1.4.2.2 is applicable.

1> If CSI-RS-based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of CSI-RS-based L1-RSRP measurement periods of the serving cell and the target cell.

2> serving cell measurement period TL1-RSRP_Measurement_Period_CSI-RS is defined in clause 9.5.4.2 assuming TReport = 0, and

2> target cell measurement period is TL1-RSRP_Measurement_Period_CSI-RS_intra defined in clause 9.14a.5 assuming TReport = 0

##### 6.3.2.2.2	CLTM RRC processing time

TCLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the CLTM target cell.

1> if the UE supports ltm-FastProcessingConfig-r18 capability, and

2> if the number of LTM and CLTM candidate cell configurations does not exceed maxNumberConfigs-r18, and

2> if the number of the configured serving cells and the cells in the LTM and CLTM configuration does not exceed maxNumberStoredConfigCells-r18

3> TCLTM-RRC-processing = 0

2> else

3> if UE has received CLTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before UE starts cell switch execution to the target cell, and/or

3> if UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before UE starts cell switch execution to the target cell, where NT,2 is defined in section 8.1 of TS 38.213 [3], and

3> if the total number of LTM and CLTM candidate cells for which TCI state(s) were activated or PDCCH order was received before UE starts cell switch execution to target cell does not exceed maxNumberConfigs-r18, and

3> if the total number of serving cells and the cells in the LTM and CLTM configuration for which TCI state(s) were activated and/or PDCCH order was received before UE starts cell switch execution to the target cell does not exceed maxNumberStoredConfigCells-r18

4> TCLTM-RRC-processing = 0

1> else

2> TCLTM-RRC-processing = 10 ms

##### 6.3.2.2.3	Interruption time

The interruption time TCLTM-interrupt is between the time the UE starts to execute cell switch towards the target cell until the time the UE starts to transmit the new uplink transmission to the target cell, excluding TCLTM-RRC-processing.

TCLTM-interrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms,

Where:

-	TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change, stated in clause 6.3.1.3.

-	Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

-	TRS-proc is the time for SSB processing.

1> if the target TCI state is in the serving cell active TCI state list or is associated to a TCI state in the serving cell active TCI state list

2> Tfirst-RS = 0 and TRS-proc= 0

1> else if the UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list or is associated to a TCI state in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

3> if the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the CLTM candidate cell TCI state activation MAC-CE is received

4> Tfirst-RS = 0 and TRS-proc= 0

1> else if the target cell is an FR1 cell, and the UE is not configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list or is associated to a TCI state in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and UE starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if not more than TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

3> if not more than TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell

4> Tfirst-RS = 0 and TRS-proc= 0

2> else if the time between the latest PDCCH ordered RACH preamble transmission on the target cell and starting to execute cell switch towards the target cell is not more than 160 ms

3> Tfirst-RS = 0 and TRS-proc= 0

1> else

2> Tfirst-RS is the time to the first SSB transmission on the target cell after TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + TLTM-processing.

2> TRS-proc = 2 ms.

-	TLTM-IU is the interruption uncertainty during CLTM cell switch.

-	For RACH-based CLTM cell switch, TCLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TCLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

-	For RACH-less CLTM cell switch, TCLTM-IU is the uncertainty on transmitting the new uplink transmission on the target cell.

#### 6.3.2.3	Subsequent CLTM Cell Switch delay

When the UE sends a RRCReconfigurationComplete message indicating completion of the previous CLTM cell switch, the UE shall start to transmit the new uplink transmission to the target cell fulfilling the configured CLTM condition within DSubsequent-CLTM seconds from the end of the last TTI containing the RRCReconfigurationComplete message.

DSubsequent-CLTM = TEvent_DU + Tmeasure + TCLTM-RRC-processing + TCLTM-interrupt

Where:

-	Tmeasure, TCLTM-RRC-processing and TCLTM-interrupt are defined in clause 6.3.2.2.

-	TEvent_DU is the delay uncertainty which is the time from when the UE successfully sends a RRCReconfigurationComplete message indicating completion of the previous CLTM cell switch until a condition exists at the measurement reference point which will trigger the subsequent CLTM cell switch.

# <End of Change #4>

# <Start of Change #5>

### 6.3.1	LTM PCell Cell Switch

#### 6.3.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to a target cell indicated in LTM cell switch command. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before the cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or

-	the target cell is an FR1 cell, and the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the cell switch command.

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

# <End of Change #5>

# <Start of Change #6>

## 8.20	LTM PSCell Cell Switch

### 8.20.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this clause are applicable to LTM PSCell switch.

The requirements in this clause are applicable to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to NR-DC for the following scenarios:

PSCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PSCell switch to an LTM candidate cell that is a serving SCell in SCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM TCI state activation command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or

-	the target cell is an FR1 cell, and the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the cell switch command.

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The definition of known LTM target cell and the definition of known target joint DL/UL TCI state or separate DL and UL TCI states are as stated in clause 6.3.1.2.

# <End of Change #6>

# <Start of Change #7>

## 8.25	TCI state activation for LTM candidate cell

### 8.25.1	Introduction

The requirements in this clause apply for a UE configured with one or more joint or separate uplink/downlink TCI state configurations on LTM candidate cell. UE shall complete the activation of the target joint or separate uplink/downlink TCI states within the delay defined in this clause after receiving a MAC CE indicating LTM candidate cell TCI state activation before cell switch command as specified in clause 6.1.3.75 of TS 38.321 [7].

The requirements in this clause apply for

-	Known downlink TCI state in FR1 and FR2 specified in clause 8.25.2, and

-	Unknown downlink TCI state in FR1 specified in clause 8.25.2, provided the following conditions are met:

-	the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the TCI state activation command, or

-	UE has reported L3 measurement result with the associated SSB index of the downlink TCI state within 5 seconds before the LTM TCI state activation command, and

-	SNR of the associated SSB is above -3 dB.

# <End of Change #7>

# <Start of Change #8>

#### 9.14.3.4	Event Triggered Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19D for FR1 and 10.1.20/10.1.20A for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

If serving cell is involved in event evaluation, L1-RSRP measurement period of serving cell refers to TL1-RSRP_Measurement_Period_SSB as defined in 9.5, assuming TReport = 0 and TSSB is the periodicity of the serving cell SSB-Index configured for event triggered L1-RSRP measurement report, which is indicated by the ssb-periodicityServingCell

If neighbor cell is involved in event evaluation, L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_intra for SSB based intra-frequency measurement, as defined in 9.14, assuming TReport = 0 and TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency event-triggered L1-RSRP measurement report.

#### 9.14.3.5	Event-triggered Periodic Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19D for FR1 and 10.1.20/10.1.20A for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.14.3.4. For the subsequent reports in the event triggered periodic report, UE follows the reportInterval-r19 and reportAmount-r19 as specified in TS 38.331.

# <End of Change #8>

# <Start of Change #9>

#### 9.15.3.4	Event Triggered Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19E for FR1 and 10.1.20/10.1.20B for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report measurement result over the air interfac. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

If serving cell is involved in event evaluation, L1-RSRP measurement period of the serving cell refers to TL1-RSRP_Measurement_Period_SSB as defined in clause 9.5, where TReport = 0 and TSSB is the periodicity of the serving cell SSB-Index configured for event triggered L1-RSRP measurement report, which is indicated by the ssb-periodicityServingCell.

If an inter-frequency neighbor cell is involved in event evaluation, L1-RSRP measurement period of the neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.15, where TReport = 0 and TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for inter-frequency event-triggered L1-RSRP measurement report.

#### 9.15.3.5	Event Triggered Periodic Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19E for FR1 and 10.1.20/10.1.20B for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.15.3.4. For the subsequent reports in the event triggered periodic report, UE follows the reportInterval-r19 and reportAmount-r19 as specified in TS 38.331.

# <End of Change #9>