# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2515062

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			6.8.1

Source:	Moderator (Huawei)

Title:	Ad-hoc meeting minutes for [116bis][305] NR_BS_RF_Part1_SE_CLTA

Document for:	Information

# Introduction

This thread covers the following topics:

- Transmitter co-existence spurious emission requirements re-evaluation in Topic #1.

- TRP estimation methods simplification in Topic #2.

The following agenda items were covered in this summary: 6.8.3.1, 6.8.4.2.2, 6.8.4.3.

Related objectives were extracted from the WID in RP-252578 are provided for reference:

| Core | Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz. |
| --- | --- |
| Perf | OTA test enhancement - TRP simplification: Simplify BS TRP estimation methods (e.g., Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods. Transmitter co-existence spurious emission requirements - Conformance testing corresponding to co-existence spurious emission requirements, if needed. |

According to the last WF in R4-2512578, the following timeline was greed for the Topic #1:

- Simulation calibration in October meeting.

- Aim for conclusions and final CRs (if any) for November meeting.

With the above, Moderator would suggest the following handling the Topic #1:

- RAN4#116bis (October 2025) meeting:

  - Conclude on simulator’s calibration.

  - Decide final set of simulation assumption for the requirement derivation.

    - BS-BS

    - BS-UE

  - Arrange work-split on the conclusions documentation (either into TR, or just a regular tdoc):

    - Introduction, motivation, and general sections,

    - Simulation assumptions,

    - Simulation results,

    - Requirement impact.

- RAN4#117 (November 2025) meeting:

  - Plan for Draft LS reply to ETSI MSG TFES.

  - Focus on the technical work and impact on TS.

- RAN4#118 (February 2026) meeting (handled under Maintenance agenda):

  - TR/tdoc inputs on agreements documentation of the work performed.

# Topic #1: TX co-existence spurious emissions requirements re-evaluation

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513816 | Nokia | Simulation results for transmitter co-existence spurious emission requirements |
| R4-2513817 | Nokia | Observation 1: The 53 dB BS to UE MCL used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency. Observation 2: The 67 dB BS to BS MCL used to derive the current -49 dBm/MHz transmitter co-existence spurious emission requirements for UL protection seems to be too optimistic at 6425 MHz carrier frequency. Proposal 1: To keep the 53 dB BS to UE MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1. Proposal 2: To further consider the 67 dB BS to BS MCL assumption for deviation of the transmitter co-existence spurious emission requirements for UL protection in the upper region of FR1. |
| R4-2513862 | Ericsson | Observation 1: A more realistic approach could be a parameterized roll-off model for ρ(f), where correlation declines progressively from 1 inside the carrier to 0 far outside. Observation 2 : The combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminate the possibility for the antenna array to achieve full array gain. Proposal 1 : Further clarification is needed on the assumption of the additional 3 dB polarization gain, which we interpret from previous studies primarily intended for the serving link and not the interfering link. Proposal 2 : RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions. Proposal 3 : RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation. Proposal 4 : RAN4 to discuss the percentile on the Coupling Loss CDF curves, instead of the minimum value. |
| R4-2514270 | ZTE Corporation, Sanechips | Proposal 1: propose to adopt the simulation assumptions in clause 2.1 for the BS2BS coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission; Proposal 2: propose to adopt the simulation assumptions in clause 2.2 for the BS2UE coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission. |
| R4-2514469 | Huawei, HiSilicon | Observation 1: We have not yet agreed how to use the CCDF curves to set an interference requirement. Observation 2: For ACIR and OTA blocking statistical analysis both the level of the interferer and the wanted signal are considered. Observation 3: Using the current set of results, we can see that using the harmonic aggressor antenna response: - For BS-BS interference we have 8.5 to 10 dB margin over the existing requirement. - For BS–UE we have >23dB margin over the existing requirement. Proposal 1: The array response should be based on the out of band harmonic array radiation pattern not the fundamental. Proposal 2: The metric used to set interferer level should be based on both the interferer power and the victim network wanted power so the effect on the victim network can be seen. Proposal 3: If interferer only CCDF analysis is used the percentage should be lower (95%) than if throughput or SNR is used which would require a higher CCDF percentage (99%). Proposal 4: Decide on the following topics for the Tx spurious co-existence limits derivation: - CDF %-tile: decide on the threshold value and the metric. - ISD value for results derivation: 500 m - Grid shift for results derivation: 100% - Antenna correlation value for the OoB antenna gain for co-ex limits results derivation: (0, or 1): TBD - Evaluation scenarios for results derivation: Same value as for calibration, i.e., 19sites, 3 sectors, wrap around (if supported by the simulation platform). - How and where to capture outcomes of this discussion: Do not generate new TR. Consider to reuse TR 38.817-2, subject to feedback from MCC. |
| R4-2513863 | Ericsson | Observation1: The only performance impact will be on the test requirements on Clause 6.6.5.5.1.3 TS 38.141-1 and Clause 6.7.5.4.5 TS 38.141-2, based on the spurious emission limits derived from the system level evaluations. |

## Open issues summary

The following sub-topics were identified based on contributions listed in clause 1.1.

### Sub-topic 1-1: Polarization gain

Open issues and candidate options before meeting:

- Proposal 1: Further clarification is needed on the assumption of the additional 3 dB polarization gain, which we interpret from previous studies primarily intended for the serving link and not the interfering link. (R4-2513862, Ericsson)

Recommended WF: Discuss online. Clarify if this would need to be reflected in the calibration results (refer to topic 1-7), or sufficient to address it only in requirement derivation simulation results.

ZTE: there is no polarization match, so the gain is no need for BS-BS

Nokia: it was already discussed in NR SI. Initially a single polarization was used, but it was questioned what happen with the other polarization. We have no strong view, as this is calibration phase.

Ericsson: Referring to NR SI, it is not clear. Even is we have polarization march; it will not matter.

Huawei: agree with issue. No polarization gain.

Nokia: first go through calibration phase. We need to clarify BS-BS and BS-UE cases.

Nokia: if there is consensus for BS-BS to go with no polarization gain, then we agree. We still need to clarify for BS-UE.

Ericsson: for BS-UE, we are not sure if it is needed either.

ZTE: if the signal is interference, we don’t have MRC combining, so there is no need for polarization gain.

Nokia: MRC is on the wanted signal.  You process received signal, not the interfering signal.

Nokia: for BS-UE, this is just calibration. We need to justify simulation assumptions if we would use 0 dB gain for polarization gain.

Ericsson: we shall not have 2 different assumptions. Ok for calibration, but may require more thinking for requirement derivation.

ZTE: based on clarification from Nokia, we understand motivation.

Agreements:

- [BS-BS: no polarization gain to be added].

- BS-UE:

  - Consider 0dB gain for calibration.

  - For requirement derivation keep 0dB and 3dB for polarization gain.

### Sub-topic 1-2: Antenna / array gain considerations

Open issues and candidate options before meeting:

- Proposal 1: The array response should be based on the out of band harmonic array radiation pattern not the fundamental. (R4-2514469, Huawei)

Recommended WF:

- Discuss online whether Proposal 1 is agreeable for the requirement derivation, in relation with sub-topic 1-3 (i.e., correlation factor for OoB).

Nokia: we need to have more clarification, on which variables to be used.

ZTE: there was related technical offline discussion. Huawei view is technically right. For antenna element patterns, there is need for more clarification on the pattern.

Ericsson: There seems to e a good point, and which lambda to use. For antenna element, with gaussian element seems good enough.

Nokia: it is more matter of changing parameters, as we use the same set of equations. To clarify whether we would use correlation as 1, if we proceed with this model.

Agreements:

[Collect more clarification on the parameter values used and the relation with the OoB antenna correlation values in sub-topic 1-3.

If Proposal 1 is agreed, aim to re-run calibration simulations.]

### Sub-topic 1-3: Correlation factor for OoB

Open issues and candidate options before meeting:

- Proposal 1: RAN4 needs to agree on an appropriate correlation factor ρ(f) that effectively balances the trade-off in modeling the coherence of unwanted emissions. (R4-2513862, Ericsson)

Moderator: There is related discussion on the LS from ECC PT1 in thread [116bis][324] LS_BDaT. While correlation factor for the out-of-band emissions is valid for the discussion on Tx spurious co-ex limits, it is suggested to handle discussion on the correlation factor for OoB region in [116bis][324] LS_BDaT. Due to limited time to conclude on Tx spurious co-existence requirement re-evaluation, suggest to rely on the simplified modelling of the correlation factor for OoB. Refer to the simulation assumptions in the WF in R4-2512578.

Recommended WF:

- Rely on the simplified modelling of the correlation factor for OoB, i.e., 0, or 1.

- Discuss whether also 0.5 correlation shall be considered.

ZTE: what is the expected way forward on the value?

Ericsson: there is related discussion on the OoB gains – there are related contribution from vendors for ITU work.

ZTE: Discussion for ITU is for the adjacent channel. We are discussing spurious range. We are trying to provide measurement results from chambers.

Nokia: it may be hard to reach consensus. We have provided results in the past as well.

Agreements:

[Postpone till sub-topic 1-2 is resolved.]

### Sub-topic 1-4: Grid shift

Open issues and candidate options before meeting:

- Proposal 1: RAN4 to keep similar assumption of Rel-99 approach between BSs of different operators - 50% grid shift for BS-to-BS propagation. (R4-2513862, Ericsson)

Moderator:

- The same Proposal 1 was submitted last meeting, with no conclusion reached.

- During previous discussion, some companies were commenting that consistency needs to be kept with the Rel-99 requirements, and grid shift shall be kept as 100%.

- Simulation assumption for the calibration results was grid shift of 100%.

Recommended WF:

- Final decision needs to be reached this meeting. Check online if we can proceed with 100%.

- Aim to clarify legacy assumptions taken from TR 25.942.

Nokia: we have discussed this already. Legacy TR 25.942 did 50% with other hexagon arrangement. Recent TR 38.922 assumed another hexagon arrangement. It is UE close to BS dominating.

ZTE: current assumptions of 288m are from legacy TR 25.942. ISD is not 500m. Probably 750m with 50% grid shift.

Nokia: current assumptions of cell radius is around 288m – no need to change. With 3 hex, the ISD is 3x cell radius.

Agreements:

Grid shift: 100%. Clarifications on the underlying cells arrangements to be captured in the final simulation assumptions documentation.

### Sub-topic 1-5: Performance metrics

Open issues and candidate options before meeting:

- Proposal 1: RAN4 to discuss the percentile on the Coupling Loss CDF curves, instead of the minimum value. (R4-2513862, Ericsson)

- Proposal 2: The metric used to set interferer level should be based on both the interferer power and the victim network wanted power so the effect on the victim network can be seen. (R4-2514469, Huawei)

- Proposal 3: If interferer only CCDF analysis is used the percentage should be lower (95%) than if throughput or SNR is used which would require a higher CCDF percentage (99%). (R4-2514469, Huawei)

Recommended WF:

- Before deciding on the CCDF %-tile value, decide on Proposal 2, i.e., whether metric used to set interferer level should be based on both the interferer power and the victim network wanted power.

- Clarify that this is for the final results derivation, and not for the calibration phase.

ZTE: this may be fundamental change. Better to use legacy approach. Technically those proposal may be right, but those are late.

Nokia: we need more details, as it is not clear what it actually means. Is this required for BS-BS as well as BS-UE? There is similar case as SBFD.

ZTE: we may need to consider both wanted and interferer to get SNR or thp degradation. This may be too late due to time pressure.

Ericsson: similar as ZTE. It may be too late to agree on this methodology.

Nokia: BS-UE: this is similar as ACIR. But for BS-BS, this is similar to co-ex.

### Sub-topic 1-6: Summary of system-level simulation assumptions; other assumptions

Open issues and candidate options before meeting:

- Proposal 1: Adopt the simulation assumptions in clause 2.1 for the BS2BS coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission. (R4-2514270, ZTE)

- Proposal 2: Adopt the simulation assumptions in clause 2.2 for the BS2UE coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission. (R4-2514270, ZTE)

- Proposal 3: Decide on the following topics for the Tx spurious co-existence limits derivation: (R4-2514469, Huawei):

  - ISD value for results derivation: 500 m

  - Grid shift for results derivation: 100%

  - Antenna correlation value for the OoB antenna gain for co-ex limits results derivation: (0, or 1): TBD

  - Evaluation scenarios for results derivation: Same value as for calibration, i.e., 19sites, 3 sectors, wrap around (if supported by the simulation platform).

Recommended WF:

- Consider Proposals 1 and 2 as baseline to be further reviewed and updated in the WF.

- Include offline agreements on propagation models.

- Assumptions for calibration (phase 1) and results derivation (phase 2) to be separated, to account for any further updates for the results derivation (phase 2).

Agreements:

- Target updated set of simulation assumptions.

- Companies to review and update.

### Sub-topic 1-7: Simulation results collection for calibration

Moderator: Simulation results using the agreed assumptions for calibration were provided in the following contributions:

  - R4-2513816, Nokia

  - R4-2514270, ZTE

  - R4-2513862, Ericsson

Recommended WF: Collect simulation results into the Excel template based on R4-1610157. Excel with the calibration results to be updated if any additional results are available.

Agreement:

- Companies to update their calibration results based on the above agreements.

### Sub-topic 1-8: Requirements impact

Open issues and candidate options before meeting (proposals and observations):

- Proposal 1: To keep the 53 dB BS to UE MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1. (R4-2513817, Nokia)

- Proposal 2: To further consider the 67 dB BS to BS MCL assumption for deviation of the transmitter co-existence spurious emission requirements for UL protection in the upper region of FR1. (R4-2513817, Nokia)

- Observation 1: Using the current set of results, we can see that using the harmonic aggressor antenna response (R4-2514469, Huawei):

  - For BS-BS interference we have 8.5 to 10 dB margin over the existing requirement.

  - For BS–UE we have >23dB margin over the existing requirement.

- Observation 2: The only performance impact will be on the test requirements on Clause 6.6.5.5.1.3 TS 38.141-1 and Clause 6.7.5.4.5 TS 38.141-2, based on the spurious emission limits derived from the system level evaluations. (R4-2513863, Ericsson)

Recommended WF: First conclude on results calibration. Discussion on requirement derivation to proceed after conclusion on calibration is reached.

Moderator: Asked Nokia for clarification on the observations.

Nokia: we have observed that some limits may require to be re-considered. If we consider MCL approach, there is worsts case.

ZTE: Nokia is somehow right. MCL is minimum. For inband blocking, usually it is 99%. 100% coupling loss would be the worst case. Probably this is why Nokia has observed that there may be need for tightening.

Ericsson: there is 40dB difference among 99 and 100%.

Nokia: For blocking, we were using 99.99%.

ZTE: for Nokia, this is right for UTRA, for EUTRA and NR it was 99%.

### Sub-topic 1-9: Documentation of findings (TR)

Open issues and candidate options before meeting:

- Proposal 1: How and where to capture outcomes of this discussion: Do not generate new TR. Consider to reuse TR 38.817-2, subject to feedback from MCC. (R4-2514469, Huawei)

Recommended WF: Check if proposal 1 is agreeable, or to proceed with capturing all agreements in a tdoc only.

# Topic #2: TRP estimation methods simplification

## Companies’ contributions summary

The following contributions were submitted to the meeting:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513818 | Nokia | Proposal 1: The ‘preferred’ terminology should be used instead of the ‘reference’ terminology in the RAN4 specifications to avoid unnecessary ambiguity in ETSI and the European Commission. Proposal 2: The definition of the ‘anechoic chamber’ should be placed within clause 3.1 ‘Definitions of Terms’ in TR 37.941 [5] as: anechoic chamber: shielded OTA measurement chamber including IAC, CATR, or PWS. Proposal 3: Annex I.1 of TS 38.141-2 [7] should be updated to define spherical equal angle grid as the TRP sampling grid for the ‘preferred test method’. Proposal 4: Use the notes in Table 6 of ERC/REC 74-01 as a reference for improvement in annex I of TS 38.141-2. |
| R4-2513819 | Nokia | Draft CR to TR 37.941 on definition of anechoic chamber |
| R4-2513820 | Nokia | Draft CR on TRP sampling grid for the preferred test method |
| R4-2514107 | Ericsson | draft CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures |
| R4-2514272 | ZTE Corporation, Sanechips | Proposal 1: For the definition of anechoic chamber, it should include the following chamber defined in TR 37.941: - Indoor Anechoic Chamber - Compact Antenna Test Range - Near Field Test Range - One Dimensional Compact Range - General Chamber - Reverberation Chamber - Plane Wave Synthesizer Proposal 2: Propose not to define the preferred anechoic chamber for different TRP based radiated RF requirements; Proposal 3: Regarding the preferred test method for TRP sampling and calculation, propose to use the Reference angular step as the preferred test method for all TRP based radiated RF requirement. Proposal 4: Regarding the Pre-scan or orthogonal cut grid, propose to continue the improvement on top of existing specification instead of creating new testing method. |
| R4-2514473 | Huawei, HiSilicon | Proposal 1: Proceed with the “preferred” wording, when referring to the TRP measurements in the anechoic chamber, unless there is another agreeable alternative. Proposal 2: Introduce generic definition of the “anechoic chamber” in TR 37.941. Proposal 3: Endorse Draft CR to TS 38.141-2 and TS 37.145-2, aiming to proceed with formal CRs during RAN4#117 meeting. |
| R4-2514474 | Huawei, HiSilicon | Draft CR to TS 38.141-2: implementation of the preferred test method solution |
| R4-2514475 | Huawei, HiSilicon | Draft CR to TS 37.145-2: implementation of the preferred test method solution |
| R4-2514476 | Huawei, HiSilicon | Draft CR to TR 37.941: Definition of anechoic chamber |

## Open issues summary

The following sub-topics were identified based on the formal Proposals, as capture in contributions listed in clause 2.1.

### Sub-topic 2-1: Terminology selection (preferred vs. reference)

Open issues and candidate options before meeting:

- Proposal 1: The ‘preferred’ terminology should be used instead of the ‘reference’ terminology in the RAN4 specifications to avoid unnecessary ambiguity in ETSI and the European Commission. (R4-2513818, Nokia)

- Proposal 2: Proceed with the “preferred” wording, when referring to the TRP measurements in the anechoic chamber, unless there is another agreeable alternative. (R4-2514473, Huawei)

- Proposal 3: Propose not to define the preferred anechoic chamber for different TRP based radiated RF requirements. (R4-2514272, ZTE)

Recommended WF:

- This topic was discussed for multiple meetings – conclude it this meeting.

- Verify whether Proposal 1 is agreeable (while Proposal 3 is the opposite). If there is no conclusion, proceed with the following:

  - Do not introduce “preferred” terminology for the anechoic chamber for TRP measurements procedures at this point of time and conclude this discussion.

  - In case European Commission blocks any of the Harmonized Standards developed by ETSI MSG TFES due to lack of “preferred” test methodology identified, then RAN4 re-open this topic and proceeds directly with formal CRs (TS 38.141-1, TS 37.145-2) to agree introduction of the “preferred” wording for the anechoic chamber for TRP measurements procedures.

### Sub-topic 2-2: Anechoic chamber definition

Open issues and candidate options before meeting:

- Proposal 1: The definition of the ‘anechoic chamber’ should be placed within clause 3.1 ‘Definitions of Terms’ in TR 37.941 [5] as (R4-2513818, Nokia):

anechoic chamber: shielded OTA measurement chamber including IAC, CATR, or PWS.

- Proposal 2: Introduce generic definition of the “anechoic chamber” in TR 37.941. (R4-2514473, Huawei)

- Proposal 2: For the definition of anechoic chamber, it should include the following chamber defined in TR 37.941 (R4-2514272, ZTE):

  - Indoor Anechoic Chamber

  - Compact Antenna Test Range

  - Near Field Test Range

  - One Dimensional Compact Range

  - General Chamber

  - Reverberation Chamber

  - Plane Wave Synthesizer

Moderator: Previous WF in R4-2512578 has captured the following: “Further work on CR to TR 37.941 to clarify definition of the anechoic chamber.”

Recommended WF: Agree to introduce a new definition of anechoic chamber into the TR 37.941. Exact wording to be refined withing the Draft CR, based on inputs from submitted contributions. See topic 2-4 for the Draft CRs handling and work-split.

### Sub-topic 2-3: TRP sampling simplifications

Open issues and candidate options before meeting:

- Proposal 1: Annex I.1 of TS 38.141-2 [7] should be updated to define spherical equal angle grid as the TRP sampling grid for the ‘preferred test method’. (R4-2513818, Nokia)

- Proposal 2: Use the notes in Table 6 of ERC/REC 74-01 as a reference for improvement in annex I of TS 38.141-2. (R4-2513818, Nokia)

- Proposal 3: Regarding the preferred test method for TRP sampling and calculation, propose to use the Reference angular step as the preferred test method for all TRP based radiated RF requirement. (R4-2514272, ZTE)

- Proposal 4: Regarding the Pre-scan or orthogonal cut grid, propose to continue the improvement on top of existing specification instead of creating new testing method. (R4-2514272, ZTE)

Recommended WF:

- Proposal 1 is subject to the discussion on the “preferred” wording in topic 2-1.

- Proposal 1-4 to be considered while drafting exact wording in the revised Draft CRs.

- See topic 2-4 for the Draft CRs handling and work-split.

### Sub-topic 2-4: Draft CRs and work-split

The following Draft CRs were submitted to the meeting:

- Draft CRs to TR 37.941:

  - R4-2513819	Draft CR to TR 37.941 on definition of anechoic chamber, Nokia

  - R4-2514476 	Draft CR to TR 37.941: Definition of anechoic chamber, Huawei, HiSilicon

- Draft CR to TS 38.141-2

  - R4-2513820	Draft CR on TRP sampling grid for the preferred test method, Nokia

  - R4-2514107	draft CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures, Ericsson

  - R4-2514474	Draft CR to TS 38.141-2: implementation of the preferred test method solution, Huawei, HiSilicon

- Draft CR to TS 37.145-2

  - R4-2514475 Huawei, HiSilicon	Draft CR to TS 37.145-2: implementation of the preferred test method solution

Recommended WF:

- Aim to Endorse agreeable wording in Draft CRs to TR 37.941, TS 38.141-2 and TS 37.145-2.

- Based on the submitted contributions, proceed with the following work-split (revised CRs):

- Draft CRs to TR 37.941: Nokia

- Draft CR to TS 38.141-2:  Ericsson

- Draft CR to TS 37.145-2: Huawei