3GPP TSG-RAN WG4 Meeting #116bis                      	R4-2515075

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Way Forward for [116bis][305] NR_BS_RF_Part1_SE_CLTA

Agenda Item:	6.8.1

Source: 	Huawei

Document for:	Approval

# Way forward for Tx spurious co-ex re-evaluation

- Polarization gain

For BS-UE case, as well as for BS-BS:

- Consider 0 dB gain for calibration.

- For requirement derivation keep both 0 dB and 3 dB for polarization gain. Final decision to be taken during RAN4#117 meeting.

- Antenna / array gain considerations

It is allowed use the array response to be based on the out of band harmonic array radiation pattern (instead of the fundamental).

Aim to re-run calibration simulations accordingly.

- Correlation factor for OoB

Proceed with the OoB correlation factor of [1].

- Grid shift

100% for both calibration and requirement derivation.

Clarifications on the underlying cells arrangements to be captured in the final simulation assumptions documentation.

- Performance metrics

CDF %-tile value to be used for the requirement derivation to be agreed at RAN4#117.

- Summary of system-level simulation assumptions; other assumptions

See annex A and B.

- Simulation results collection for calibration

Excel spreadsheet with updated calibration results:

- Requirement’s impact

Final requirement to be derived at RAN4#117 to conclude technical work.

- Documentation of findings (TR)

Aim to handle documentation of findings under Maintenance from RAN4#118. Companies to investigate workable alternatives (as TR 38.817-02 could not be re-used).

# Way forward for TRP sampling grids

The following is agreed towards simulation assumptions and their alignment.

- Anechoic chamber definition

Introduce generic anechoic chamber definition into the TR 37.941, listing only few examples of different anechoic chambers (i.e., not to have explicit list of specific chambers to avoid the need for future updates).

- “Reference” vs. “preferred” wording

Use the “preferred” wording for anechoic chamber, as well as for the reference TRP sampling grid.

- Content of modifications

Limit modifications in CRs to TS 38.141-2, TS 37.145-2 to reflect WID objective. Other corrections to be handled under Rel-15 maintenance.

Modifications to the TRP annex to be first concluded based on TS 38.141-2. Afterwards, those are to be mirrored to the related Annex in TS 37.145-2.

# Annex A: System-level simulation assumptions: BS-BS

Table 1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 500 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% | ` |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| BS2 UE Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss model for BS2BS |  | PL (dB) = 32.45+ 20 log₁₀(d in km) + 20 log₁₀(f in MHz) | Free space pathloss model |
| Pathloss model for BS2UE |  | Please use the UMa model in the TR 38.901 7.4.1 Uma channel model |  |

Table 2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | Un-coordinated operation (100% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Table 3: Antenna configurations for 2GHz and U6GHz BS in Macro Urban scenario

| U6GHz BS antenna configuration |  | 2GHz BS antenna configuration |  |
| --- | --- | --- | --- |
| Parameter | Macro urban | Parameter | Macro urban |
| Am | 30 dB | Am | 30 dB |
| SLAv | 30 dB | SLAv | 30 dB |
| j3dB | 90 deg. | j3dB | 90 deg. |
| q3dB | 65 deg. | q3dB | 65 deg. |
| GE,max | 6.4 dBi | GE,max | 6.4 dBi |
| Msub | 3 | Msub | 3 |
| dv,sub | 0.7l m | dv,sub | 0.7l m |
| qsubtilt | 3 deg. | qsubtilt | 3 deg. |
| M | 8 | M | 4 |
| N | 16 | N | 8 |
| dh | 0.5l m | dh | 0.5l m |
| dv | 2.1l m | dv | 2.1l m |
| jescan | deg. | jescan | deg. |
| r | 1 | r | 1 |
| Ptx | 46 dBm | Ptx | 46 dBm |
| qmech | 6 deg. | qmech | 6 deg. |

Table 4: Extended AAS antenna modelling

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

# Annex B: System-level simulation assumptions: BS-UE

Table 5: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell As |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. As shown in Figure 2.2-1 |
| Radius of UE dropping within a micro cell |  | < 72.17 m | Calculated based ISD. Nokia CR number: Sqrt(3)/4*ISD/3 |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi | TR 38.901 Umi channel model should be used |
| Shadowing correlation |  | Between Micro site: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD =500 m is assumed. |  |  |  |

Figure 1: Network layout for dense urban

Table 6: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

Table 7: Antenna configurations for 2GHz and U6GHz BS in Dense Urban scenario

| U6GHz BS antenna configuration |  | 2GHz BS antenna configuration |  |
| --- | --- | --- | --- |
| Parameter | Micro BS | Parameter | Micro BS |
| Am | 30 dB | Am | 30 dB |
| SLAv | 30 dB | SLAv | 30 dB |
| j3dB | 90 deg. | j3dB | 90 deg. |
| q3dB | 65 deg. | q3dB | 65 deg. |
| GE,max | 6.4 dBi | GE,max | 6.4 dBi |
| Msub | N/A | Msub | N/A |
| dv,sub | N/A | dv,sub | N/A |
| qsubtilt | N/A | qsubtilt | N/A |
| M | 8 | M | 8 |
| N | 8 | N | 8 |
| dh | 0.5l m | dh | 0.5l m |
| dv | 0.7l m | dv | 0.7l m |
| r | 1 | r | 1 |
| Ptx | 37 dBm | Ptx | 37 dBm |
| qmech | N/A | qmech | N/A |

Table 8: Composite antenna pattern for AAS antenna modelling without sub-array configuration

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |