3GPP TSG-RAN WG4 #117	R4-2520042

Dallas, US, November 17-21, 2025

Agenda item:	8.7		Demodulation

Source:	Qualcomm Incorporated

Title:	Further comments on demodulation aspects of 6GR

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. It also includes a study on the 6GR core and performance requirements. In addition, a study on the 6G scenarios and requirements is conducted in RAN plenary [2]. Its outcome is summarized in the technical report 38.914 [3].

In RAN4 #116bis the work on 6G demod was started. Meeting minutes of the discussion and the way forward are provided in [4] and [5].

In this contribution, we discuss our views on the goals of the RAN4-driven demodulation aspects of the 6G study item including enhancements for the requirement framework and the applicability rules.

# Views on the 6G Study Item for Demodulation Performance

In RAN4 #116bis the work on demod performance aspects for the 6G study item started with initial discussions on TxEVM and SNR operation points and performance testing and requirement [4]. In the following sections we provide our views on several of the discussion points in RAN4 #116bis.

## SNR Operation Point and TxEVM

Regarding TxEVM and SNR operating points, it has been pointed out by several companies that TxEVM applied in the demod performance requirements and the minimal requirement for base station RF TxEVM should not be the same since it limits the achievable SNR significantly. Since SNR values observed in the field are significantly larger than the SNR given by the minimal requirement for base station RF TxEVM, it is obvious that commercial base stations operate below their minimal TxEVM requirements. If TxEVM applied in the demod performance requirements and the minimal requirement for base station RF TxEVM are not decoupled, a large range of practicable SNR values remain untested as is currently the case in 5G NR. We provide our views on TxEVM and SNR operating points in one of the following subsections.

Observation 1: Since SNR values observed in the field are significantly larger than the SNR given by the minimal requirement for base station RF TxEVM, it is obvious that commercial base stations operate below their minimal TxEVM requirements.

Observation 2: If TxEVM applied in the demod performance requirements and the minimal requirement for base station RF TxEVM are not decoupled, a large range of practicable SNR values remain untested as is currently the case in 5G NR.

TxEVM aspects

- Proposals

  - Option 1: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

    - Option 1A: Study impact of TX EVM for higher modulation order/ MIMO layers on Demodulation requirements.

    - Option 1B: Abandon the SNR operating point limitations via fixed 20dB rule, or fixed TE TxEVM assumptions, and adopt a SNR limitation derivation based on actual TDRA/FDRA configuration.

    - Option 1C: Study whether the TxEVM requirements for the base station, at least in the simulations for deriving the demodulation requirements, could be tightened.

In our previous contribution on the 6G Demodulation topic we proposed to improve 6G demodulation testing, with SNR operating points that reflect real-world conditions observed in the field. The discussion has been captured in the WF [4] as pasted here below.

We have observed that in the history of 5G NR, TxEVM assumptions in simulations (based on Base Station Tx EVM requirements) have de-facto constrained the achievable SNR, and as such current RAN4 requirements cap SNR at 25–30 dB, while measurements in the field show SSB SNR values exceeding 35 dB per Rx antenna.

Observation 3: In RAN4 UE demod, the use of Tx EVM assumptions based on BS EVM requirements has constrained the achievable SNR.

We believe that it would be a better way forward in 6G to decouple the different discussions between the maximum operating or testable SNR and the TxEVM assumption for the simulation, and as such we share our support to Option 1B:

Proposal 1: Support Option 1B on the TxEVM aspects. RAN4 should study whether SNR testable ranges starting from 35 dB can be achieved, assuming ideal Tx EVM, and how this range depend on test parameters (such as, e.g. number of carriers, channel bandwidth, signal power level).

Proposal 2: RAN4 to study how to account for SNR degradation from realistic Test Equipment TxEVM, based on performances expected by real Test Equipment. The options could include the use of an impairment margin, or an additional noise-based TE EVM value.

## Dynamic Test Equipment Functionality

The second main discussion topic in RAN4 #116bis was performance testing and requirements. Aspects under discussion were OLLA (outer loop link adaptation) and SRS based beamforming. Both topics are closely related to dynamic TE functionality that was also being proposed for further study by several companies. In our view support for dynamic TE functionality should be discussed in detail in the 6G study item since it allows enhancing the test framework in 6G significantly and adapting the RAN4 demod performance tests much closer to real field scenarios. For example, inner loop and outer loop link adaptation testing allows testing at BLER operating points that are seen in real field deployments. Other dynamic TE functionality like SRS based beamforming like SVD beamforming or inter-codeword zero forcing beamforming allows testing the UE receiver conditions observed in many TDD deployments.

Observation 4: Dynamic TE functionality allows enhancing the test framework in 6G significantly and adapting the RAN4 demod performance tests much closer to real field scenarios.

### SRS based Beamforming

SRS-based precoding was briefly discussed in the previous meeting. The feature lead starting point and ideas for discussion are captured below.

Feature Lead starting point and ideas for discussion:

- Option A: UE sends SRS, TE receives SRS through the channel, SRS is estimated, Precoder is calculated based on SRS

  - Note, TEs may usually assume ideal uplink and this may be impractical and difficult to implement.

- Option B: TE uses ideal channel knowledge, adds AWGN on channel knowledge, calculate precoder, add processing delay to use precoding

SRS-based precoding is observed in field deployments for TDD scenarios, particularly in cell-edge scenarios where the network is less likely to rely on UE feedback for downlink precoding. As RAN4 aims to develop test cases in 6G that closely reflect real-world conditions for 6G scenarios, it is important that this functionality be considered as an integral part of the RAN4 test framework.

To obtain knowledge of downlink channel for its precoding, it is possible to utilize the estimated channel in the uplink (obtained via SRS transmission in the uplink) under the assumption of channel reciprocity. An SVD-based approach could then be used to obtain the precoding matrices for downlink transmission. However, channel estimation inherently involves estimation errors, which are commonly modeled as Gaussian uncertainty when estimated from pilot symbols [6]. Given that the channel estimation in this case is based on SRS pilots, modeling the estimation error as Gaussian noise is a reasonable approach. Therefore, instead of relying on true channel estimation from the SRS signal, a simplified approach is to use ideal (genie) SRS channel with added Gaussian noise to account for the channel estimation error. This approach can ensure robust system performance in a statistical sense. As presented in our previous contribution, we note that this method is aligned with our proposal of testing with extended TE support, i.e., TE algorithms should be utilized to assess features dependent on network-side processing.

Observation 5: SRS-based precoding is frequently utilized in TDD field deployments, especially at the cell edge where the network may not rely on UE feedback for downlink precoding.

Proposal 3: As part of broader proposal of testing with extended TE support, consider SRS-based precoding via SVD decomposition for demodulation performance requirements in RAN4.

Observation 6: Since channel estimation in the uplink will be obtained from SRS pilots, modeling the estimation error as Gaussian noise is justified.

Proposal 4: For SRS-based precoding, use ideal (genie) SRS channel and model uplink estimation errors as Gaussian noise to calculate the downlink precoding matrices (support option B).

### Outer and Inner Loop Link Adaptation

In the RAN4 #116bis adhoc minutes [4], there was an open issue for OLLA study in 6G demod.

Feature Lead starting point for discussion:

Discuss if it can be agreed to study OLLA in 6G demod

- We can check if proposed OLLA model from R4-2300703 can be assumed as a starting point (update suggestions are still welcome)

Other options are not precluded if new proposals are introduced in next meetings

In the current RAN4 requirements, all CSI reporting requirements tests are based on link adaptation wherein one or more among the parameters CQI, PMI and RI are applied by gNB as “follow parameter”. The objective was to specify requirements for a given parameter based on relative throughput gains. In section 5.6 of TS 38.101-4, link adaptation approach was used where MCS follows reported CQI to measure throughput for demod performance. All CSF and current demodulation requirements intend to specify minimum performance requirements of a single (isolated) parameter, but don’t measure spectral efficiency – the most critical measure of a link.

To measure spectral efficiency, our view is to have new tests where gNB has the flexibility to change the MCS to achieve the highest throughput. At a given set SNR where a consistent reported rank is ensured, gNB can change the scheduled PDSCH MCS to measure the maximum throughput achievable. Set outer loop first transmission BLER target such as 10%, often seen in the field, can be used for such requirements.  Moreover, for each reported rank, suitable SNR points can be chosen, and throughput requirements can be specified based on outer-loop link adaptation (OLLA). We believe this new approach comprehensively captures the capability of a receiver along with absolute physical later throughput requirements as already defined in TS 38.101-4. In the following we denote absolute physical layer throughput tests as inner loop link adaptation (ILLA).

In the ILLA mode, the CSI feedback (i.e. the RI, CQI and PMI) from the UE is used to modify the DL PDSCH configuration. The RI, CQI and PMI directly determine the number of layers, MCS and precoder, respectively.

- The Channel Quality Indicator (CQI) is translated into Spectral Efficiency (SE) value based on the tables 5.2.2.1-2 to 5.2.2.1-4 in 38.214, and a specific MCS is chosen to satisfy the SE, modulation and coding requirements based on 5.1.3.1-1 to 5.1.3.1-4.

- The Precoding Matrix Indicator (PMI) directly specifies the PDSCH precoder.

- The Rank Indicator (RI) specifies the number of layers in the PDSCH.

It can be further discussed to use the instantaneous CSI feedback or with a simple first-order filter.

Proposal 5: Study ILLA (absolute physical layer throughput) to adjust the number of layers, MCS, and precoder based on CSI feedback (i.e. the RI, CQI and PMI) from the UE report.

In R4-2300703 a simple OLLA algorithm is proposed. It applies the spectral efficiency values of the MCS tables as selection criteria, based on the adjusted CQI values. Here is a debrief of the proposal extracted from “2.1.3 Proposed simple OLLA model applied on CQI”.

MCS can be selected based on the interpolated Spectral Efficiency (SE) values

MCSi+1=SelectMCS1-α*SE0+α*SE1#  SEQ ( \* ARABIC 14

where SelectMCS() is a function that selects the MCS from the given SE to MCS table, which has the closest spectral efficiency to the input argument, and

SE0=LookupSEk#  SEQ ( \* ARABIC 12

SE1=LookupSEk+1#  SEQ ( \* ARABIC 13

where LookupSE() is the Spectral Efficiency lookup from the 3GPP RAN1 tables for the corresponding integer CQI.

Here k is the integer part of CQIadj, and α is the fractional part of CQIadj, which is based on the measured CQI

CQIadj=CQImeas-Δi = k+α #  SEQ ( \* ARABIC 11

The Δi+1 can be calculated as follows:

Stepdown[CQI units]=Stepup[CQI units]*BLERtarget1-BLERtarget#  SEQ ( \* ARABIC 9

Δi is updated iteratively based on HARQ feedback

Δi+1= Δi-Stepdownif ACKΔi+Stepupif NACK#  SEQ ( \* ARABIC 10

Reasonable numerical values and limites:

Stepup and Stepdown = 0.45 [CQI units] and 0.05 [CQI units]

Δi+1ϵ-1, +3 CQI steps

In the following we denote the OLLA scheme in R4-2300703 as OLLA (reference). We propose that RAN4 should study a new test framework where gNB has the flexibility to change the MCS based on measured SE to achieve the highest throughput. In addition to ILLA and OLLA (reference), we propose two additional variants of OLLA implementations for further discussion. We call them OLLA only and OLLA+ILLA in the following.

In the OLLA-only mode, the UE desired SE is adjusted based on a target BLER (aka, PDSCH HARQ ACK/NACK data). This adjusted SE is then used to determine the MCS. OLLA allows the gNB to account for biases or errors in UE reported CQI. In this case, the CQI feedback from the UE is ignored. The OLLA-only mode could converge slower. The initial reference SE can be configured (value TBD).

In addition to the OLLA-only mode, we want to suggest another mode that we call OLLA+ILLA. In the OLLA+ILLA mode, the OLLA is used in conjunction with ILLA. The initial reference SE is based on the UE reported CQI, and then the SE is adjusted up and down based on the BLER.

Here is a proposed formula for the k-th instance of OLLA update,

SEfinalk=SEreported- SEbackoff[k] 											(1)

where SEreported is the initial reference SE for the OLLA-only mode or it is derived from CQI of the latest CSI report for the OLLA+ILLA mode, based on 38.214 MCS table.  SEbackoff[k] is the backoff SE number iteratively calculated from previous instance (k-1).

SEbackoffk=SEbackoffk-1-SEbackoffIncrementif k>0 and PDSCH is ACKSEbackoffk-1+SEbackoffDecrementif k>0 and PDSCH is NACKinitialSeBackoffif k=0 				(2)

SEbackoffIncrement=δ∙1- PBLER										(3)

SEbackoffDecrement=δ∙PBLER											(4)

where PBLER is the target BLER rate, and δ is a configurable SE backoff step size. Since mapping SNR with SE is not always linear, a single backoff step size may be too conservative to high SE regime and too aggressive for the low SE regime.  Multiple backoff step size can be considered for multiple SE regions.

Proposal 6: RAN4 to investigate Inner-Loop (ILLA) and/or Outer-Loop Link Adaptation (OLLA) methods to specify requirements for throughput.

We propose to study the schemes OLLA (reference), OLLA-only and OLLA+ILLA as simple OLLA schemes. As already mentioned above, OLLA-only does not rely on CQI. OLLA (reference) and OLLA+ILLA differ whether the up and down correction is applied in CQI or SE domain.

Proposal 7: RAN4 should study the OLLA schemes OLLA (reference), OLLA-only and OLLA+ILLA as described in this paper.

## Advanced Receivers

Another early focus in the 6G study should be defining the baseline device receiver, especially for six or eight Rx antennas. Non-linear receivers, so-called R-ML receivers, are meanwhile state-of-the-art in 5G NR devices, although they are optional in RAN4 demod tests.

One important consideration for enhancing the test framework in RAN4 to conditions that reflect field-relevant scenarios is the receiver type for downlink MIMO. Table 2-1 lists the SU-MIMO receivers that have been defined for 5G NR over various releases.

Table 2-1: MIMO Receiver Types in 5G NR

| Receiver type | Release | Type | Release |
| --- | --- | --- | --- |
| MMSE | Rel-15 | Mandatory | Default receiver in 5G NR. MMSE without interference rejection capability |
| Enhanced receiver type I | Rel-15 | Optional | R-ML receiver with enhanced inter-stream interference suppression for SU-MIMO transmissions |
| MMSE-IRC | Rel-17 | Mandatory in Rel-17 Optional in Rel-15/16 | MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference |
| Simplified 8Rx Receiver | Rel-18 | Optional | 8Rx receiver for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors |
| Baseline 8Rx Receiver | Rel-18 | Optional | 8Rx receiver for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector |

The default receiver in 5G NR is a plain MMSE receiver without inter-cell and intra-cell inter-user interference rejection capabilities. MMSE-IRC receivers have been introduced in Rel-17 and are optional for Rel-15/16 devices. The simplified and baseline 8Rx receivers that have been introduced in Rel-18 are MMSE-IRC type of receivers. Demodulation requirements for interference have been introduced for 8Rx in Rel-19 without introducing a new receiver type. Meanwhile, MMSE-IRC receiver is state of the art for all number of Rx antennas.

Observation 7: The default receiver in 5G NR is MMSE. MMSE-IRC receivers are mandatory since Rel-17.

The enhanced receiver type I is a R-ML (reduced Maximum-Likelihood) receiver with enhanced inter-stream interference suppression for SU-MIMO transmission. In contrast to the MMSE-IRC receiver, a R-ML receiver is a non-linear receiver. This receiver type has been introduced as an optional feature in Rel-15 and provides significant performance gains over MMSE-IRC receiver in case of more severe inter-stream (inter-layer) interference. R-ML receivers are widely available in real world UE devices and should be the baseline receiver for 6GR as well.

Observation 8: Non-linear R-ML receivers are widely available in real world UE devices.

Both MMSE-IRC and R-ML receivers are important for real world scenarios. For scenarios with less inter-layer interference, a R-ML receiver does not provide performance gains over a MMSE-IRC receiver. In such a scenario, applying a R-ML receiver would not be power-efficient. Hence, it is important to define both MMSE-IRC and R-ML receiver as the baseline for 6GR to allow for an optimal performance-power trade-off.

Proposal 8: Both R-ML receiver and MMSE-IRC receiver should be baseline receivers for 6GR, independent on the number of Rx antennas.

## Channel Modelling

In the following subsections we provide our view about the channel models to be used in 6GR.

### Channel Modelling for RAN4 Demodulation Requirements in 6GR

In the history of 5G NR, Tapped Delay Line (TDL) models have served as the primary reference for performance evaluation, particularly within RAN4 testing scenarios. These models offer temporal resolution but lack spatial coherence, which limits their ability to capture the full benefits of advanced MIMO schemes. Despite these limitations, TDL remains an extremely valuable benchmark for continuity and comparative analysis, and its inclusion in future evaluations is essential. As we have seen with the recent developments in 5G NR Rel-19 that led to the publication of TR 38.753, Spatial Channel Modeling (SCM) is gathering increasing attention for the purposes of the definition of demodulation performance requirements.

Observation 9: TDL models, while foundational for 5G NR, are spatially agnostic and in some cases have been shown not to sufficiently characterize spatially dependent features for the purpose of testing certain scenarios (e.g. advanced MIMO schemes, multiple codeword scenarios).

To address these limitations, SCM models have gained prominence, and within them the Clustered Delay Line (CDL) model. CDL models introduce spatial determinism and coherence, making them more suitable for evaluating spatial processing performance. Their structured nature allows for an increased level of modeling of certain real-world propagation phenomena (e.g. distribution of SNR per layer, spatial properties influencing precoding choices), which is especially beneficial for demodulation studies that rely on spatial diversity and channel predictability.

Proposal 9: Maintain both TDL and CDL models in future 6GR performance requirements. TDL should continue to serve as a reference and comparison, while CDL provides spatial features that can be used to test advanced demodulation scenarios.

As AI/ML-based receivers become increasingly relevant in the context of 6G demodulation, the choice of spatial channel models plays a critical role in ensuring realistic and reliable performance evaluations. While CDL (Clustered Delay Line) models offer deterministic spatial behavior and structured propagation paths, this determinism introduces risks when used in conjunction with learning-based receivers.

Observation 10: The deterministic nature of CDL models can lead AI/ML-based receivers to learn the channel behavior during testing, resulting in artificial performance that does not generalize to real-world conditions.

This phenomenon arises because CDL models, based on sum of sinusoids with stable spatial directions, might allow receivers to infer and memorize channel patterns. In a test environment, this can produce misleading results, as the receiver may exploit the predictability of the model rather than demonstrating adaptability. Consequently, CDL may not be suitable for evaluating AI/ML receivers until methods are developed to mitigate this inference risk.

Proposal 10: Consideration of CDL modeling in 6GR for AI/ML receiver evaluations should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

While CDL models offer valuable spatial realism for traditional demodulation studies, their deterministic properties pose significant challenges for AI/ML-based receiver evaluations. TDL models, with their stochastic nature, may remain the safer choice for benchmarking robustness and generalization in AI/ML contexts. A similar concern applies to PMI (Precoding Matrix Indicator) prediction tasks. CDL models maintain stable spatial directions, which can be exploited by learning algorithms to predict PMI values with high accuracy in test scenarios. However, this does not reflect the variability and unpredictability of real-world channels, potentially leading to over-optimistic conclusions about receiver performance.

Observation 11: Stable spatial directions in CDL models can distort PMI prediction results, making them appear more effective than they would be in dynamic environments.

Proposal 11: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

One of the potential countermeasures for PMI unbiasing could be based on simulating UE rotation in CDL modeling, which is expected to result in PMI reporting unbiasing.

Proposal 12: CDL modeling in 6GR for PMI report evaluation could investigate the introduction of UE rotation for PMI unbiasing.

Another area of improvement lies in the antenna correlation assumptions used in TDL models. Current implementations often rely on idealized or overly simplified correlation matrices that do not reflect practical deployments. Enhancing TDL with more realistic spatial correlation values would improve its utility in scenarios where legacy compatibility is required.

Proposal 13: Extend the current TDL framework to include more realistic antenna correlation values, enabling more advanced modeling while preserving backward compatibility.

In summary, spatial channel modeling for 6G demodulation should embrace both legacy and forward-looking approaches. TDL remains essential for continuity with 5GNR, while CDL offers spatial behavior that can be exploited in testing next generation receivers. Enhancements to TDL and strategic use of CDL will ensure that demodulation performance is accurately and comprehensively evaluated across diverse deployment scenarios.

### CDL Modelling for 6GR-NTN

RAN4 has recently completed a study on spatial channel modeling, focusing on CDL channel models for TN, with conclusions documented in TR 38.753. Building on this work, we propose initiating a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the predominantly LOS nature of NTN links. Aligned with terrestrial network modeling practices, this extension would improve the realism of NTN channel modeling and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

Observation 12: RAN4 has recently concluded a study on CDL channel modeling for terrestrial networks, documented in TR 38.753.

Proposal 14: We propose initiating a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

## Demodulation Tests for the Base Station

For 6G demodulation performance for base stations, one of the limitations of the 5G base station demodulation performance requirements is the limitation by the 8Rx configuration for all demodulation requirements/ tests. Moving to 6G, it is viable also to consider more than 8Rx antennas to address massive MIMO deployments, which is a key pillar for 5G and 6G deployments. As base station demodulation requirements are typically capped at 8Rx, it would be desirable to understand the feasibility from RAN4 point of view to study more than 8Rx demodulation requirements.

Proposal 15: RAN4 to study the feasibility of considering higher than 8Rx scenarios when defining the BS demodulation requirements.

Another important aspect to be considered for base station demodulation performance in 6G, is the consideration of spatial channel modelling for base station demodulation performance. Although the work done in Rel-19 study item on spatial channel modelling focused on downlink and the impact on UE demodulation performance, spatial channel modelling and its impact are expected to be considered as well for uplink and thus, it is expected that base station demodulation performance will be studied and provided adopting CDL channel models.

Observation 13: CDL channel models are expected to be considered in RAN4 for uplink to address base station demodulation performance.

## Device Types

The introduction of device types is an ongoing discussion in the plenary and in RAN1. RAN1 studies the device types form physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN. Hence, RAN4 should wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards RAN4 should discuss how device types can be covered in the test framework of RAN4.

Observation 14: The introduction of device types is an ongoing discussion in the plenary and in RAN1. RAN1 studies the device types form physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

Proposal 16: RAN4 should wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards RAN4 should discuss how device types can be covered in the test framework of RAN4.

# Conclusions

In this paper we provided our views for the demodulation performance in the 6GR study item. Our observations and proposals are summarized as follows:

SNR Operation Point and TxEVM

Observation 1: Since SNR values observed in the field are significantly larger than the SNR given by the minimal requirement for base station RF TxEVM, it is obvious that commercial base stations operate below their minimal TxEVM requirements.

Observation 2: If TxEVM applied in the demod performance requirements and the minimal requirement for base station RF TxEVM are not decoupled, a large range of practicable SNR values remain untested as is currently the case in 5G NR.

Observation 3: In RAN4 UE demod, the use of Tx EVM assumptions based on BS EVM requirements has constrained the achievable SNR.

Proposal 1: Support Option 1B on the TxEVM aspects. RAN4 should study whether SNR testable ranges starting from 35 dB can be achieved, assuming ideal Tx EVM, and how this range depend on test parameters (such as, e.g. number of carriers, channel bandwidth, signal power level).

Proposal 2: RAN4 to study how to account for SNR degradation from realistic Test Equipment TxEVM, based on performances expected by real Test Equipment. The options could include the use of an impairment margin, or an additional noise-based TE EVM value.

Dynamic Test Equipment Functionality

Observation 4: Dynamic TE functionality allows enhancing the test framework in 6G significantly and adapting the RAN4 demod performance tests much closer to real field scenarios.

Observation 5: SRS-based precoding is frequently utilized in TDD field deployments, especially at the cell edge where the network may not rely on UE feedback for downlink precoding.

Proposal 3: As part of broader proposal of testing with extended TE support, consider SRS-based precoding via SVD decomposition for demodulation performance requirements in RAN4.

Observation 6: Since channel estimation in the uplink will be obtained from SRS pilots, modeling the estimation error as Gaussian noise is justified.

Proposal 4: For SRS-based precoding, use ideal (genie) SRS channel and model uplink estimation errors as Gaussian noise to calculate the downlink precoding matrices (support option B).

Proposal 5: Study ILLA (absolute physical layer throughput) to adjust the number of layers, MCS, and precoder based on CSI feedback (i.e. the RI, CQI and PMI) from the UE report.

Proposal 6: RAN4 to investigate Inner-Loop (ILLA) and/or Outer-Loop Link Adaptation (OLLA) methods to specify requirements for throughput.

Proposal 7: RAN4 should study the OLLA schemes OLLA (reference), OLLA-only and OLLA+ILLA as described in this paper.

Advanced Receivers

Observation 7: The default receiver in 5G NR is MMSE. MMSE-IRC receivers are mandatory since Rel-17.

Observation 8: Non-linear R-ML receivers are widely available in real world UE devices.

Proposal 8: Both R-ML receiver and MMSE-IRC receiver should be baseline receivers for 6GR, independent on the number of Rx antennas.

Channel Modelling

Observation 9: TDL models, while foundational for 5G NR, are spatially agnostic and in some cases have been shown not to sufficiently characterize spatially dependent features for the purpose of testing certain scenarios (e.g. advanced MIMO schemes, multiple codeword scenarios).

Proposal 9: Maintain both TDL and CDL models in future 6GR performance requirements. TDL should continue to serve as a reference and comparison, while CDL provides spatial features that can be used to test advanced demodulation scenarios.

Observation 10: The deterministic nature of CDL models can lead AI/ML-based receivers to learn the channel behavior during testing, resulting in artificial performance that does not generalize to real-world conditions.

Proposal 10: Consideration of CDL modeling in 6GR for AI/ML receiver evaluations should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

Observation 11: Stable spatial directions in CDL models can distort PMI prediction results, making them appear more effective than they would be in dynamic environments.

Proposal 11: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

Proposal 12: CDL modeling in 6GR for PMI report evaluation could investigate the introduction of UE rotation for PMI unbiasing.

Proposal 13: Extend the current TDL framework to include more realistic antenna correlation values, enabling more advanced modeling while preserving backward compatibility.

Observation 12: RAN4 has recently concluded a study on CDL channel modeling for terrestrial networks, documented in TR 38.753.

Proposal 14: We propose initiating a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

Demodulation Tests for the Base Station

Proposal 15: RAN4 to study the feasibility of considering higher than 8Rx scenarios when defining the BS demodulation requirements.

Observation 13: CDL channel models are expected to be considered in RAN4 for uplink to address base station demodulation performance.

Device Types

Observation 14: The introduction of device types is an ongoing discussion in the plenary and in RAN1. RAN1 studies the device types form physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

Proposal 16: RAN4 should wait for further clarification in RAN and RAN1 what device types may get defined. Afterwards RAN4 should discuss how device types can be covered in the test framework of RAN4.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025

- R4-2514589, Adhoc meeting minutes for [116bis][106] 6G Demod, RAN4 #116bis, October 2025

- R4-2514645, WF on 6G demod, RAN4 #116bis, October 2025

- G. Zheng, S. Ma, K.-K. Wong, and T.-S. Ng, Robust beamforming in cognitive radio, IEEE Trans. Wireless Commun., vol. 9, no. 2, pp. 570–576, Feb. 2010.

The simplified SU-MIMO 8Rx receiver was renamed to simplified 8Rx receiver in Rel-19.

The baseline SU-MIMO 8Rx receiver was renamed to baseline 8Rx receiver in Rel-19.