3GPP TSG-RAN WG4 #117	R4-2520046

Dallas, US, November 17-21, 2025

Agenda item:	8.12.2.3		Demod spec

Source:	Qualcomm Incorporated

Title:	Further comments on RAN4 operational efficiency for Demod

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. One important topic from a RAN4 point of view is to improve the operational efficiency in RAN4. The discussion was initiated in RAN4 #116bis and a way forward was agreed in [2]

In this contribution, we continue to discuss topics related to RAN4 operation efficiency for demod.

# Views on the Operation Efficiency for Demod

## 6G Performance Requirements Specification Structure

The relevant issues from the WF [2] related to the structure of the Performance Requirements specification are pasted in this section.

Issue 3-1-2: Band-Combination Handling and Tool Support

Conclusions:

  - Companies are encouraged to

    - Study how to incorporate this database into RAN4 specification and meeting handling like CR process, etc

Issue 1-1-5: Issues on Performance or Demodulation Specifications

- Observed issues

  - Issue 1: In the 5G BS demodulation performance specifications, applicability of certain requirements is not explicitly linked to the support of the corresponding features.

  - Issue 2: The current structure of applicability rules in TS 38.101-4 is fragmented and inconsistent across device types

Issue 3-4: Perf/Demod specs improvements

- Proposals

  - Proposal 1: Replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

  - Proposal 3: FRC table improvements considering

    - necessary configuration and avoid derived information

    - methodology to generate FRC table efficiently

In our previous contributions to the 6G discussion in RAN4, an included in the annex for information, we have highlighted several structural issues in the current demodulation performance specifications, particularly the fragmented applicability rules and the lack of explicit linkage between requirements and supported features. These gaps lead to ambiguity in test coverage and inconsistencies across device types, making compliance verification more complex and error-prone. Additionally, the broad applicability statements in TS 38.101-4 fail to account for capability-specific scenarios, such as devices without legacy TN support, which results in unnecessary dependencies and inefficiencies in the testing process.

Observation 1: Structural issues in the current Performance Requirements specification include the convoluted mapping between supported feature and applicable testing, fragmentation of the requirement configuration across multiple table containing parameters, non spec-compliant defined parameters.

Based on the experience gathered with the band combination database (which is completing its work in RAN4), we believe that a database approach offers a very practical solution to the challenges highlighted in the current version of the specifications, and we recommend for performance requirement to adopt a database format for including requirements and FRC tables in 6G.

The JSON database proposed for the maintenance of band combinations provides a good model for approaching complex conditions in a structured and scalable manner, and it also demonstrates how centralized, machine-readable data can simplify the interpretation and ensure consistency. Applying a similar methodology to requirements and FRC tables would allow for a framework that addresses the current structural issues.

Observation 2: RAN4 has worked on the implementation of a Band Combination database in JSON format as part of the RAN4 specification.

We believe that a structured database containing all demodulation requirements could help RAN4:

- Ensure test configuration parameters clarity and consistency

- Eliminates fragmentation across multiple specification tables

- Enforceing uniformity through database entry validation;

- Streamline CR handling for the implementation, update and maintenance of  requirements and FRCs;

- Centralize the mapping of requirement applicability and rules based on different capabilities

- Improve operational efficiency by supporting a more scalable and transparent framework for managing complex requirement configurations

- Allow management via version control, enabling traceability and collaborative change management.

Proposal 1: RAN4 to use a database approach for the introduction and maintenance of performance requirements in 6G specification.

Moreover, companies could automate the generation of configuration files for internal simulators or testing equipment directly from these 3GPP database files, reducing manual effort and minimizing errors.

Following the example of the band combination a practical baseline option for a database approach could be a JSON-based structure where each test is represented as a self-contained JSON file validated against a common schema. The JSON schema would define and clearly explain all spec-compliant parameters, ensuring consistency and future extensibility. Each test file would include its configuration parameters, the requirement value, and all relevant applicability flags, making it fully self-consistent without relying on external tables. By adopting this approach, RAN4 would establish a scalable, transparent, and automation-friendly framework that aligns with modern software practices.

Proposal 2: RAN4 to evaluate the use of JSON as database format for performance requirements in 6G specification.

Proposal 3: RAN4 to encourage companies to submit JSON Schema proposals with spec-compliant configuration parameters necessary for the definition of the requirements.

## Modernization and New Tools

Issue 2-2-2: Modernization and new tools

- Proposals

  - Proposal 1: Follow conclusions in 6GSM SID (SP-250802).

    - Modernization of drafting tool: markdown, version control (TR 21.802).

    - Study RAN4 impacts on new formats such as Markdown or LaTeX.

    - RAN4 needs to keep an eye on the progress of alternative tools to replace current Word-based specification, and prepare for the potential impacts.

  - Proposal 2: The general specification modernization works (6GSM SID) can be discussed and decided in RANP to identify the needs and feasibility first, before starting any trial/study in RAN WGs level.

  - Proposal 3: Adopt equivalent multi-formatted specifications with each format tailored to a specific purpose, e.g., facilitating script-based tools for CR drafting and consolidation: one format as the root specification under version control, and other formats can automatically be generated from the root format for different purposes.

  - Proposal 4: New tools to avoid cover sheet issues.

Below we paste the relevant proposals in WF [2] for the topic of modernization and new tools.

For the purposes of the Performance requirements specification 38.101-4, we support replacing Word with machine-readable formats, with the best option being Markdown-based languages.

That is because from a drafting point of view, specifically to the content of the Performance Requirements TS, the vast majority of the content is in text or tabular format, with a limited number figures, and a handful of block diagrams. The use of Markdown-based languages provides a very good solution to handle this  kind of content, and we believe that RAN4 demod should start scoping the creation of a draft specification in the Forge 3GPP version control environment, created by MCC for this purpose at https://forge.3gpp.org/rep/fs_6gspecs_new.

Proposal 4: RAN4 to scope usage of Markdown-based language for the drafting and maintenance of the performance specification.

Proposal 5: RAN4 to collect drafting file format proposals from the companies for 6G Spec , and then create drafts in the 3GPP Forge for delegates to comment and evaluate.

A starting point could be to choose one section of the existing specification, for example 38.101-4, Section 5.2.2.2 – FR1 2RX PDSCH requirements

Proposal 6: RAN4 to choose one clause or subclause in the existing specification (e.g. one or more subclauses in 5.2.2.2 in TS 38.101-4) and encourage companies to reformat the existing material in the proposed file format.

## Structure and Drafting Principles for TS 38.101-4

We recommend RAN4 to review how applicability rules are currently handled in RAN4 demodulation specifications (i.e., 38.101-4 for UE demodulation and 38.104/38.141-1/38.141-2 for BS demodulation) and to identify opportunities for improvement in 6G. This specification, while comprehensive, presents challenges in clarity and consistency that affect both interpretation and implementation.

One key observation is that the applicability of tests varies significantly across device types. Some devices are assigned dedicated sections, others are embedded within broader clauses, and some rely on tables that differ in structure and completeness. This fragmented approach makes it difficult for stakeholders to quickly determine which tests apply to which configurations, especially as new device categories emerge.

Some applicability rules are structured by antenna configuration, others by optional features, and many are buried in tables with conditional logic. This complexity underscores the need for a harmonized and streamlined approach

Observation 3: The current structure of applicability rules in TS 38.101-4 is fragmented and sometimes inconsistent across device types, making it difficult for stakeholders to interpret test requirements reliably, especially for different types of devices.

There are many examples of the inconsistencies above, but we can limit the list to a glaring example, regarding the applicability of demodulation requirements for 2RX UEs. Indeed, this is presented in a way that reveals inconsistencies that complicate understanding.

In Table 5.1.1.2-1, TS 38.101-4, the specification states that for UEs supporting only 2RX, “all tests in Clause 5.2.2” apply. This suggests a unified and complete applicability of demodulation requirements for 2RX devices.

However, Clause 5.2.2 is subdivided into multiple subclauses each addressing different requirements and configurations. Some of these subclauses include tests that are only relevant when certain features are signaled (e.g., enhanced receiver types, HST-SFN, HST-DPS, RedCap), which are not necessarily supported by all 2RX UEs and are reported via additional applicability tables or conditions.

This fragmented structure means that the reader needs to manually verify each subclause and its conditional logic. So, applicability is not centralized, and the reader needs to navigate multiple layers of specification to determine the correct test scope.

Proposal 7: RAN4 should discuss improving Demodulation spec in 6GR by replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

With the emergence of new application scenarios, novel device types are being introduced, and RAN4 is actively defining their corresponding performance requirements. It is important to recognize that while some devices may support TN functionality, others may operate independently without supporting it. Therefore, when determining test applicability across device types, it is essential to consider both the device capabilities and their intended application scenarios. Specifically, for devices that do not support legacy TN, test applicability should be structured to avoid dependence on legacy TN test cases.

Observation 4: RAN4 defines performance requirements for emerging device types, recognizing that some devices may support TN functionality while others may operate independently without supporting it.

Proposal 8: RAN4 should implement a capability-aware test applicability framework that considers device functionality. For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases.

# Conclusions

In this paper we provided our views related to the operation efficiency in RAN4. Our observations and proposals are summarized as follows:

Observation 1: Structural issues in the current Performance Requirements specification include the convoluted mapping between supported feature and applicable testing, fragmentation of the requirement configuration across multiple table containing parameters, non spec-compliant defined parameters.

Observation 2: RAN4 has worked on the implementation of a Band Combination database in JSON format as part of the RAN4 specification.

Proposal 1: RAN4 to use a database approach for the introduction and maintenance of performance requirements in 6G specification.

Proposal 2: RAN4 to evaluate the use of JSON as database format for performance requirements in 6G specification.

Proposal 3: RAN4 to encourage companies to submit JSON Schema proposals with spec-compliant configuration parameters necessary for the definition of the requirements.

Proposal 4: RAN4 to scope usage of Markdown-based language for the drafting and maintenance of the perfromance specification.

Proposal 5: RAN4 to collect drafting file format proposals from the companies for 6G Spec , and then create drafts in the 3GPP Forge for delegates to comment and evaluate.

Proposal 6: RAN4 to choose one clause or subclause in the existing specification (e.g. one or more subclauses in 5.2.2.2 in TS 38.101-4) and encourage companies to reformat the existing material in the proposed file format.

Observation 3: The current structure of applicability rules in TS 38.101-4 is fragmented and sometimes inconsistent across device types, making it difficult for stakeholders to interpret test requirements reliably, especially for different types of devices.

Proposal 7: RAN4 should discuss improving Demodulation spec in 6GR by replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

Observation 4: RAN4 defines performance requirements for emerging device types, recognizing that some devices may support TN functionality while others may operate independently without supporting it.

Proposal 8: RAN4 should implement a capability-aware test applicability framework that considers device functionality. For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- R4-2514648, WF on 6G operation efficiency, RAN4 #116bis, October 2025