3GPP TSG-RAN WG4 Meeting #117	R4-2520119

# Dallas, US, 17th – 21st Nov, 2025

Title: 	Discussion on RRM requirements for HD-FDD for Ku band with FR1 numerology

Source: 	CATT

Agenda item:	7.6.4.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for enhanced requirements for NR NTN and IoT NTN Phase 2 in Rel-20 was approved in RAN#109.The current objectives of the work item which are related to HD-FDD for NTN is copied as follows [1]:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note : Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

In the last meeting, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2 was approved [2], and this document will discuss HD-FDD impact for NTN RRM requirements for VSATs in Ku bands and give our proposals.

- Discussion

HD-FDD technology has been widely used, demonstrating the fact that cost reduction is a key driving factor for successful markets in other non-3GPP satellite communication systems. In addition, automotive-mounted VSAT (circa. 20 x 20 cm) are considered a key segment for Ku/Ka band NR-NTN market penetration due to large volumes. Considering that HD-FDD antennas are simpler to implement than FD-FDD antennas and better match the consumer expected price point, introducing HD-FDD Mobile VSAT with small antennas for Ku/Ka band NR-NTN is very meaningful [1].

In the last meeting, RAN4 has agreed the principle for defining HD-FDD VSAT with FR1 numerology RRM requirements and RRM impact list, the agreements are copied as below [2]:

Agreement - Define the HD-FDD VSAT with FR1 numerology RRM requirements based on the Rel-19 Ku-band NTN FDD requirements, and incorporate updates to address HD-FDD impacts, using the RedCap NTN approach as the baseline. - The following RRM requirements scope for HD-FDD VSAT in Ku band with FR1 numerology - Cell reselection, including paging reception - Handover and CHO - Satellite switching - Random access - UE UL timing accuracy - RLM, including scheduling availability and L1 indication - intra/inter-frequency measurements, including scheduling availability - L1-RSRP measurement, including scheduling availability

  - Handover interruption time

For handover interruption time, there was controversy about whether there is impact on NR handover, and the WayForward in the last meeting is copied as follows [2]:

WayForward - Option 1: HD-FDD has no impact on NR handover. - Option 2: FFS impact due to reduced Rx time - Further discuss the following conditions for HD-FDD - For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - One SSB is available during T∆ - One SSB is available during TIU

As we know, the same issue has been discussed in RedCap with NTN in R19. Considering that SSB is always prioritized over UL following RAN1 agreement (Case 5), if RAN4 define those applicability rules listed in option 2, it may be mis-interpreted as SSB may be dropped due to collision with UL, which is not our intention. Therefore, RAN4 eventually agreed that there is no spec impact on NR Handover due to HD-FDD in RAN4#116 meeting, and the agreement is copied as below [3]:

#### Issue 1-3-2: The impact on NR Handover due to HD-FDD Online agreement Agreement: - No spec impact

Based on the same reason clarified above, considering that RAN1 agreement for collision handling in Case 5 does not change at least at this stage, so we believe that there is also no spec impact for HO/CHO/ o	Satellite switching requirements due to HD-FDD for VSAT.

Observation 1: The same issue has been discussed in RedCap with NTN in R19, and it is agreed that there is no spec impact on NR Handover due to HD-FDD.

Proposal 1: There is also no spec impact for HO/CHO/ Satellite switching requirements due to HD-FDD for VSAT.

  - UE UL timing accuracy

Regarding UE UL timing accuracy for HD-FDD VSAT, the WayForward in the last meeting is copied as follows [2]:

WayForward - Further discuss the following options - Option 1: HD-FDD has no impact on UE UL timing accuracy. - Option 2: FFS impact due to Tx/Rx pattern.

In our view, considering that downlink reciption and uplink transmission can’t be performed at the same time for HD-FDD UE, so some companies concerned that UL timing accuracy may not be guaranteed.

However, in legacy timing requirements for NTN for Ku band, the UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms [4], and it is also useful for HD-FDD UE to guarantee uplink synchronization. In legacy RedCap related requirements defined in TN and NTN, RAN4 also didn’t specifically define other additional requirements due to HD-FDD, so we support that HD-FDD has no impact on UE UL timing accuracy for VSAT for Ku band with FR1 numerology.

Observation 2: In legacy timing requirements for NTN for Ku band, the UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms and it is also useful for HD-FDD UE to guarantee uplink synchronization.

Observation 3: In legacy RedCap related requirements defined in TN and NTN, RAN4 also didn’t specifically define other additional requirements due to HD-FDD.

Proposal 2: HD-FDD has no impact on UE UL timing accuracy for VSAT for Ku band with FR1 numerology.

  - Other RRM requirement impact due to HD-FDD

For other RRM requirement impact due to HD-FDD, the WayForward is copied as below [2]:

WayForward - RAN4 to further discuss if any RRM requirement impact due to HD-FDD for the following aspects - Maximum interruption in paging reception - initial access/random access - inter and/or intra-measurement requirement - scheduling availability

Considering that RAN1 agreement for collision handling in Case 5 does not change at least at this stage, so we believe that the legacy applicability clarification due to HD-FDD for the following RedCap with NTN requirements can be reused as baseline for VSAT for Ku band with FR1 numerology:

- Cell reselection, including paging reception

- Random access

- RLM, including scheduling availability and L1 indication

- intra/inter-frequency measurements, including scheduling availability

- L1-RSRP measurement, including scheduling availability

Proposal 3: The legacy applicability clarification due to HD-FDD for the following RedCap with NTN requirements can be reused as baseline for VSAT for Ku band with FR1 numerology:

- Cell reselection, including paging reception

- Random access

- RLM, including scheduling availability and L1 indication

- intra/inter-frequency measurements, including scheduling availability

- L1-RSRP measurement, including scheduling availability

  - Collision handling and other RAN1 impact

In our view, the legacy collision handling approach and other impact for HD-FDD UE in RAN1 is band and nurmology agnostic, and the only change for RAN1 spec will be adding some new descriptions for the new type of VSAT HD-FDD UE, which does not require new technical discussion and only to define new name of parameters for VSAT HD-FDD UE.

Observation 4: The only change for RAN1 spec will be adding some new descriptions for the new type of VSAT HD-FDD UE, which does not require new technical discussion and only to define new name of parameters for VSAT HD-FDD UE.

Proposal 4: The RAN1 impact for HD-FDD UE is band and nurmology agnostic, and the existing collision handling approach can be reused for HD-FDD VSAT for Ku band with FR1 numerology.

  - Specification structure

Since HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) is a new VSAT type, and it is operating in both Ka and Ku bands with FR1 and FR2 numerologies, in Phase 1, we propose that HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

Proposal 5: In Phase 1, HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

- Conclusion

This document discussed RRM requirements for HD-FDD VSATs in Ku bands and presented following observations and proposals:

Observation 1: The same issue has been discussed in RedCap with NTN in R19, and it is agreed that there is no spec impact on NR Handover due to HD-FDD.

Proposal 1: There is also no spec impact for HO/CHO/ Satellite switching requirements due to HD-FDD for VSAT.

Observation 2: In legacy timing requirements for NTN for Ku band, the UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms and it is also useful for HD-FDD UE to guarantee uplink synchronization.

Observation 3: In legacy RedCap related requirements defined in TN and NTN, RAN4 also didn’t specifically define other additional requirements due to HD-FDD.

Proposal 2: HD-FDD has no impact on UE UL timing accuracy for VSAT for Ku band with FR1 numerology.

Proposal 3: The legacy applicability clarification due to HD-FDD for the following RedCap with NTN requirements can be reused as baseline for VSAT for Ku band with FR1 numerology:

- Cell reselection, including paging reception

- Random access

- RLM, including scheduling availability and L1 indication

- intra/inter-frequency measurements, including scheduling availability

- L1-RSRP measurement, including scheduling availability

Observation 4: The only change for RAN1 spec will be adding some new descriptions for the new type of VSAT HD-FDD UE, which does not require new technical discussion and only to define new name of parameters for VSAT HD-FDD UE.

Proposal 4: The RAN1 impact for HD-FDD UE is band and nurmology agnostic, and the existing collision handling approach can be reused for HD-FDD VSAT for Ku band with FR1 numerology.

Proposal 5: In Phase 1, HD-FDD related requirements should be defined under the clauses defined for NTN RRM requirements for Ku band in Rel-19.

- Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MediaTek

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- 3GPP TS 38.133, NR Requirements for support of radio resource management