# 3GPP TSG-RAN WG4 Meeting #117	R4-2520230

# Dallas, USA, 17th November 2025 – 21st November 2025

Agenda item:			8.1

Source:	Feature lead (MediaTek inc.)

Title:	Running Summary of 6G demod

Document for:	Information

# Introduction

Based on the latest approved RAN working group level SID on 6G Radio [1], RAN4 study on demodulation and performance aspects, with the detailed objective provided as:

- 6GR core and performance requirements - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

In this running summary, it is intended to include the following aspects relevant to the RAN4 study on 6G demod:

- Status update for 6G RAN4 demod issues

- RAN4 key Tdocs and key agreements achieved before this meeting.

- RAN1 summary for relevant and essential agreements related to RAN4 6G demod

- RAN2 summary for relevant and essential agreements related to RAN4 6G demod

The information for feature lead is provided:

| Feature Lead | Company | Email address |
| --- | --- | --- |
| Hannu Vesala | MediaTek inc. | hannu.vesala@mediatek.com |

# Status update

The following status is provided based on RAN4 discussion till the end of RAN#116bis.

- The high-level goal of the RAN4 study is to develop a framework for minimum demodulation and test performance requirements that more accurately reflect real-world conditions than current 5G standards, while considering test coverage, alignment, feasibility, complexity, and cost.

- DPoD and DPD discussion including both AI and non-AI based will be firstly discussed in AI (AI-based) and UE RF (non-AI based) threads.

- Discussions have begun on several new test equipment (TE) functionalities.

  - Outer-Loop Link Adaptation (OLLA)

  - SRS based precoding

  - Time/Frequency/Phase offset precompensation

  - Dynamic resource scheduling

- Discussions have also started on TxEVM and SNR aspects.

# RAN4 - 6G demod agreements

## RAN4 #116bis meeting (Oct-2025)

### Key Tdocs

| R4-2514513 | Topic Summary for [116bis][106] 6G Demod |
| --- | --- |
| R4-2514589 | Ad-hoc meeting minutes on 6G demod |
| R4-2514645 | WF on 6G demod |

### Key agreements

#### Performance testing and requirement

General performance requirement aspects

- Agreement

  - In 6G SI, RAN4 will study how to strive to develop a demod/test performance requirement framework for the minimum performance to better reflect real field conditions than 5G, if necessary, by taking into consideration of the requirement/test coverage/alignmentablity and the test feasibility, complexity and cost/resources.

#### Uplink demod

Digital Pre-Distortion and Post-Distortion Techniques

- Agreement

  - DPoD and DPD discussion including both AI and non-AI based will be firstly discussed in AI (AI-based) and UE RF (non-AI based) threads. Once sufficient progress is made, it can be further discussed if the discussion should be continued in Demod thread.

# RAN1 – Demod related agreements summary

## Overview of 6GR air interface

### RAN1#122bis (Oct-2025)

No relevant and essential agreements related to RAN4 6G demod.

### RAN1#122 (Aug-2025)

No relevant and essential agreements related to RAN4 6G demod.

## Evaluation assumptions for 6GR air interface

### RAN1#122bis (Oct-2025)

Agreement

For around 700MHz, for TXRU mapping at base station, it is adopted as mandatory option for simulation campaign that a single TXRU is mapped per panel per subarray per polarization.

Note: Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization.

Agreement

- For around 700MHz, 32 for total number of antenna element at base station, 4 for total number of TXRU at base station, (8, 2, 2, 1, 1; 1, 2) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the baseline combination.

- For around 700MHz, 64 for total number of antenna element at base station, 8 for total number of TXRU at base station, (8, 4, 2, 1, 1; x, y) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the optional combination.

Note: Other values/combinations are up to company to report

Agreement

For around 2GHz carrier frequency, for BS antenna modelling (Details not included in this running summary)

Agreement

For around 4GHz carrier frequency (Details not included in this running summary)

Agreement

At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations:

- L-band (i.e., 1.5GHz)

- S-band (i.e. 2 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

### RAN1#122 (Aug-2025)

No relevant and essential agreements related to RAN4 6G demod.

## Waveform and frame structure for 6GR air interface

### RAN1#122bis (Oct-2025)

Agreement

For communication, 6GR considers NR frame structure used as a starting point for the study item,

  - Resource defined by one subcarrier and one symbol is called as resource element (RE).

  - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12

  - Radio Frame length is 10ms

  - Each radio frame is split into 10 subframes, each with a duration of 1 ms

  - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design.

  - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Agreement

6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band.

- FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1.

- Note: ISAC is separately discussed in ISAC session.

Agreement

- RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz

- Study whether and how to enable UE to support 400MHz bandwidth

### RAN1#122 (Aug-2025)

Agreement

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Agreement

6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

- FFS potential need for other CP.

## Channel coding and modulation for 6GR interface

### RAN1#122bis (Oct-2025)

Agreement

- For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH)

- For 6G LDPC

  - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported

  - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side

    - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed

- For 6G Polar code

  - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported

  - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side

    - Note: Necessity for control information beyond NR range is to be further discussed

    - Polar code maximum mother code length is kept as 1024.

  - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

### RAN1#122 (Aug-2025)

Agreement

- For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

- For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel

  - FFS: Enhancements and other modulation schemes

# RAN2 – Demod related agreements summary

No relevant and essential agreements related to RAN4 6G demod in RAN2#131bis.

# References

[1] RP-252912, ”Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone