3GPP TSG-RAN4 Meeting #117	R4-2520263

# Dallas, TX, US, Nov. 2025

Title:	On HPUE RF requirements for Rel-20

Source:	Qualcomm Incorporated

Agenda item:	7.1.4

Document for:	Discussion

# Introduction

The Rel20 UE RF enhancement WI [1] approved in RAN#109 outlines the following objectives regarding High power UE (HPUE) for NR single carrier operation:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

In this paper, we provide in this paper our preliminary views on the UE RF aspects related to the 4X26 dBm (FWA) UE in RAN4 Rel-20.

# General discussion on TDD 4x26 dBm UE

## New power class?

### Background

A separate co-existence study is being pursued to revisit the ACLR requirement for the target 4x26 dBm UE. As of this writing, the updated requirement is not known. The PAs being aggregated in the target UE are intended to be legacy PC2 PAs, which are generally subject to a 31 dB ACLR requirement. It is self-evident that the max. power level specified for the target UE can approach the aggregate power level (32 dBm) only if the ACLR requirement is not significantly higher than the same for PC2 (31 dB).

For the power class discussion, we analyze the situation that is most relevant to the discussion on need for a new power class, i.e. the target UE is expected to transmit at 32 dBm.

### Maximum output power level

Legacy PC1 is specified at 31 dBm, which is within power regulation error of the nominal aggregate power of 4x26 (32 dBm). Is the target UE worthy of a new power class from a network management perspective?

Now, the peak power difference does not matter from a network management perspective, due to power regulation loops and supplementally, PHR reporting.

The other potential difference between legacy PC1 and the target UE is ‘spectral spillage’ into adjacent channels. This is fundamentally an out-of-channel consideration and has no direct bearing on the network’s management of these UEs. i.e. If the adjacent channel spillage of the UEs is within accepted levels, the max. output level difference between PC1 and the target UEs remains transparent to the network.

Observation 2-1:  From a power control perspective, the network does not need to distinguish between a 31 dBm UE and a 32 dBm UE of equivalent configuration.

### Maximum UL ports

The legacy PC1 devices is commonly understood to be a single port device, while the target UE has 4 Tx chains. This means that the target UE has multi-port capability while the legacy device does not. Is the target UE worthy of a new power class from a network management perspective due to higher potential port count?

Note that legacy network must already manage multi-port devices, including 4x23 PC1.5. If a PC1 UE were to simply report higher port count via the IE maxNumberTxPorts, the network would organically manage it like other multi-port devices. From a network management perspective then, a 4x26 device poses no new challenge to existing networks already capable of handling multiport UEs.

Observation 2-2:  A 4x26 dBm UE poses no new UE configuration challenge compared to legacy 5G UEs.

### Feasibility of using legacy PC1 for the target 4x26 dBm UE

The observations above suggest that even if the target UE is expected to transmit at 32 dBm, there is no network management need to segregate from PC1. Consequently, there is no obvious benefit to undertaking the design effort to create a new power class.

Observation 2-3:  From a network management perspective, a 4x26 dBm does not need a new power class to be defined for it.

In other words, the 4x26 UE can be absorbed into PC1 even if it is capable of 32 dBm output power. Any new power class proposal must first be technically justified based on network benefit. For completeness, we also note that if the target UE can only transmit 31 dBm or less, it is a trivial matter of absorbing into PC1.

Proposal 2-1: No new power class is needed for the 4x26 dBm UE, PC1 can be re-used.

If no new power class is defined for the 4x26 dBm UE, it is necessary to understand how a 32 dBm output can be enabled in the standard. Now, to prevent ambiguity, the expectation that the UE must transmit at 32 dBm must be captured in a backward compatible way.

This can be achieved for example by retaining 31 dBm for the UE in single port configuration (non-suffix requirements) and enabling 32 dBm only when the UE is configured for multiple SRS ports, i.e.through requirements 6.xD.

Observation 2-4: Any potentially elevated power expectation (like 32 dBm) from the 4x26 dBm UE can be captured in D-suffix requirements.

## Ant. Sw. SRS allowance

For SRS transmissions with usage set to ‘Antenna Switching’, RAN4 uses the PPowerClass parameter to accommodate reduced power for transmission from fewer chains:

If the 4x26 dBm UE is defined with a 31 or 32 dBm output power, it is self-evident that it must transmit through all its defined ports to achieve that max. power level. In the case of the 4x26 dBm UE, only 4T transmissions can expected to be full power. Max. power expectations for 1T and 2T transmission would be respectively, 10log(4) (i.e. 6 dB) and 10log(2) (i.e. 3 dB) lower.

Observation 2-5: A 4x26 dBm UE would need to be specified with a reduced max. output expectation of 3 dB for t2rN and 6 dB for t1rn SRS transmissions.

Proposal 2-2: The target 4x26 dBm UE is specified with PPowerClass of 3 and 6 dB for SRS-TxSwitch capability of ‘t2rN’ and ‘t1rN’ respectively for SRS transmissions with usage ‘antennaSwitching’.

## TRxSRS

The standard accommodates many different UE types, including 4Tx FWA devices (PC1.5). The problem of routing losses associated with sounding the channel through its receive antennas is acknowledged and provided for via the parameter ‘TRxSRS’. It would be logical and consistent to reuse this entire framework of conditions and values for the target 4x26 dBm UE, since it uses the same topology as the 4x23 dBm PC1.5 UE.

Proposal 2-3: The Rel-19 TRxSRS framework is re-used for the target 4x26 dBm UE. Specification for additional SRS-TxSwitch capabilities is not precluded.

## Max. UL duty cycle capability

The max. UL duty cycle capability was originally instituted to ensure regulatory exposure compliance at the UE. The max. duty cycle capability is unfortunately of limited value to the network, for reasons ranging from complexity at the gNB, to the exposure counter at the UE being a multi-RAT issue. Moreover, existing PC1 UEs do not have this capability. The need for this capability is being re-evaluated for 6G.

In the case of the 4x26 target UE however, the intent is to use legacy PC2 PAs. Legacy PC2 PAs are designed to a max. duty cycle expectation and furthermore, are expected to be allowed to ‘fall back to lower power class requirements’. This behavior must be preserved for the 4x26 UE to enable the use of legacy PC2 PAs.

Observation 2-6: The target 4x26 dBm UEs must be enabled for ‘power class requirement fall back’ behavior to enable the use of legacy PC2 PAs.

Proposal 2-4: A new capability like ‘maxUplinkDutyCycle-PC2-FR1’ is defined for the 4x26 dBm UE to enable use of legacy PC2 PAs and to allow ‘power class requirement fallback’ behavior when indicated capability is exceeded by scheduling.

## CA power class

A complete treatment would include future considerations like inter-band UL CA. If the UE is capable of a certain power class in a certain band, at a minimum, the UE must be able to support the same power class in CA.

Proposal 2-5: RAN4 to request RAN2 to enhance the CA power class list to include the power class chosen for the target 4x26 dBm UE.

# General discussion on FDD PC1.5 with 2 Tx

## 2.3 NS to be evaluated

Agreement:

- Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49.

- Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed.

  - If needed, how to define UTRA ACLR for PC1.5

## 2.4 REFSENS degradation

Agreement:

Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.

The WF [3] objectives from RAN4#116-bis for PC1.5 with 2Tx in FDD bands are provided below.

From the WF objectives it can be observed that companies were requested to check if NS_03U, NS_05U and NS_100 need to be supported. While our preliminary thinking is that in the absence of an agreement stating that 2Tx PC1.5 will not be deployed in regions where EUTRA is deployed in bands n1 and n25, 2Tx PC1.5 would have to support these EUTRA cases. However, we think that companies should investigate this further and only conclude once the need for supporting these NS cases is verified as the extra workload in supporting these cases is considerable.

Observation 3-1: Currently no agreement that 2Tx PC1.5 will not be deployed in the same regions as EUTRA in bands n1 and n25.

Observation 3-2: Extra workload in supporting 2Tx PC1.5 for NS_03U and NS_05U is considerable

Proposal 3-1: Companies should further study whether support of NS_03U and NS_05U and NS_100 is required for 2Tx PC1.5.

UTRA ACLR requirement is applicable when the network signaling value NS_03U, NS_05U or NS_100 is signaled by the network in the field additionalSpectrumEmission and is as presented below in Table 3-1.

Table 3-1: UTRA ACLR requirement

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB |

Observation 3-3: UTRA ACLR requirement is applicable for NS labels NS_03U, NS_05U and NS_100.

Proposal 3-2: If it is decided that NS_03U, NS_05U and NS_100 will be supported, evaluate whether new UTRA ACLR is required for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signaled.

Maximum power reduction (MPR) for PC 1.5 with 2 Tx for a handheld UE is already defined in the standards as shown below in Table 3-2.

Table 3-2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Proposal 3-3: Analyze the applicability of existing MPR table for PC 1.5 with 2 Tx for a FDD handheld UE.

Maximum power reduction (MPR) for PC 1.5 with 2 Tx for a FWA UE is already defined in the standards as shown below in Table 3-3.

Table 3-3: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Proposal 3-4: Analyze the applicability of existing MPR table for PC1.5 with 2 Tx for a FDD FWA UE.

Multiple network signaling (NS) labels applicable to bands n1, n25 and their corresponding A-MPR details are provided in Tables 3-4 and 3-5.

Table 3-4: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |

Table 3-5: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 (NOTE 7) |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the upper channel edge is ≥ 1930 MHz,10 MHz BWChannel where the upper channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the upper channel edge is ≥ 1955 MHz and 20 MHz BWChannel where the upper channel edge is ≥ 1970 MHz. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 or txDiversity2Tx-r18 [TS 38.306], the additional relaxation of [2] dB is applicable. |  |  |  |  |  |

Observation 3-4: For a PC2 UE with Tx Diversity additional relaxation is provided in Note 7 of Table 3-5 (Table 6.2.3.1-1 in 38.101-1).

Proposal 3-5: Evaluate whether further relaxation in A-MPR for PC1.5 with 2 Tx FDD architecture is required for a UE indicating Tx Diversity.

Proposal 3-6: Update the modified MPR behavior table L.1-1 for bands n1 and n25 to support PC1.5 with 2 Tx for FDD handheld and FWA UEs.

# Conclusions

## 4x26 dBm UE

Observation 2-1:  From a power control perspective, the network does not need to distinguish between a 31 dBm UE and a 32 dBm UE of equivalent configuration.

Observation 2-2:  A 4x26 dBm UE poses no new UE configuration challenge compared to legacy 5G UEs.

Observation 2-3:  From a network management perspective, a 4x26 dBm does not need a new power class to be defined for it.

Proposal 2-1: No new power class is needed for the 4x26 dBm UE, PC1 can be re-used.

Observation 2-4: Any potentially elevated power expectation (like 32 dBm) from the 4x26 dBm UE can be captured in D-suffix requirements.

Observation 2-5: A 4x26 dBm UE would need to be specified with a reduced max. output expectation of 3 dB for t2rN and 6 dB for t1rn SRS transmissions.

Proposal 2-2: The target 4x26 dBm UE is specified with PPowerClass of 3 and 6 dB for SRS-TxSwitch capability of ‘t2rN’ and ‘t1rN’ respectively for SRS transmissions with usage ‘antennaSwitching’.

Proposal 2-3: The Rel-19 TRxSRS framework is re-used for the target 4x26 dBm UE. Specification for additional SRS-TxSwitch capabilities is not precluded.

Observation 2-6: The target 4x26 dBm UEs must be enabled for ‘power class requirement fall back’ behavior to enable the use of legacy PC2 PAs.

Proposal 2-4: A new capability like ‘maxUplinkDutyCycle-PC2-FR1’ is defined for the 4x26 dBm UE to enable use of legacy PC2 PAs and to allow ‘power class requirement fallback’ behavior when indicated capability is exceeded by scheduling.

Proposal 2-5: RAN4 to request RAN2 to enhance the CA power class list to include the power class chosen for the target 4x26 dBm UE.

## FDD PC1.5 with 2 Tx

Observation 3-1: Currently no agreement that 2Tx PC1.5 will not be deployed in the same regions as EUTRA in bands n1 and n25.

Observation 3-2: Extra workload in supporting 2Tx PC1.5 for NS_03U and NS_05U is considerable

Proposal 3-1: Companies should further study whether support of NS_03U and NS_05U and NS_100 is required for 2Tx PC1.5.

Observation 3-3: UTRA ACLR requirement is applicable for NS labels NS_03U, NS_05U and NS_100.

Proposal 3-2: If it is decided that NS_03U, NS_05U and NS_100 will be supported, evaluate whether new UTRA ACLR is required for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signaled.

Proposal 3-3: Analyze the applicability of existing MPR table for PC 1.5 with 2 Tx for a FDD handheld UE.

Proposal 3-4: Analyze the applicability of existing MPR table for PC1.5 with 2 Tx for a FDD FWA UE.

Observation 3-4: For a PC2 UE with Tx Diversity additional relaxation is provided in Note 7 of Table 3-5 (Table 6.2.3.1-1 in 38.101-1).

Proposal 3-5: Evaluate whether further relaxation in A-MPR for PC1.5 with 2 Tx FDD architecture is required for a UE indicating Tx Diversity.

Proposal 3-6: Update the modified MPR behavior table L.1-1 for bands n1 and n25 to support PC1.5 with 2 Tx for FDD handheld and FWA UEs.

# References

- RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, vivo (Moderator), RAN Meeting #109, Beijing, China.

- R4-2521896, ‘Views on the coexistence study for High Power UE’, Qualcomm, RAN4#117.

- R4-2515087, “Way Forward for [116bis][316] NR_UE_RF_Ph5”, RAN4#116-bis, 13th – 17th October, 2025.