3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2520292

Dallas, USA, Nov. 17-21, 2025

Title: 	UE Tx performance target for 6G

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.3 General RF and UE RF [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#116bis, a way forward [1] captured several guidance for the 6G UE RF study. This contribution is relevant to the topics in the way forward guidance: 2) Single carrier output power, 3) CA output power, 4) Tx requirements and partially to 8) Spectrum aggregation. It is based on further developing our input from different parts of our first inputs in previous meeting [2,3,4].

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several guidance for the 6G UE RF study, the parts relevant to the UE Tx performance are the following topics: 2) Single carrier output power, 3) CA output power, 4) Tx requirements and 8) Spectrum aggregation.

The relevant parts of the way forward are copied here in italics.

Single carrier output power 2): Agreements

- Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band

  - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO

- Study improvements on configured maximum output power requirements and associated parameters

- Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

- Study the default power class for FDD and TDD bands for 6G

- Study how to handle SAR/MPE for 6GR

CA output power 3): Agreements

- Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements

- Study how to maximize the UL transmission power capability for CA

Prioritize discussion for single carrier in RAN4#117 and RAN4#118

Tx Requirements 4): Agreements

- PA modelling is discussed in system parameter thread

- In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear

- Spurious emissions requirements, SEM and ACLR will be defined for 6GR

    - FFS for OOB boundary, potential accommodations for different frequency sub-ranges

- Encourage inputs on regulatory survey being conducted in spectrum agenda

- Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5)

In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

Spectrum aggregation 8): Agreements

- Below spectrum aggregation topics can be discussed as starting point.

  - RF switch-time performance

  - Need for intra-band support of non-contiguous UL CA and RB allocations

  - Study benefits-drawbacks of using CA vs. using wider single carrier BW

  - Study how to account for simultaneous TX/RX between the bands properly from the beginning

Discuss band combination simplification in spectrum agenda

In this contribution we will cover the topics in separate chapter and proposals to facilitate the construction of the summary but there is obviously a tight connection between a UE power capability and the requirements it has to meet. The yellow highlighted parts of the way forward correspond to the key aspects of our contribution.

## 2.2 Single carrier output power

For 5G FR1, the latest releases all focused on higher UE output power for UL coverage. For 6G, we think this issue will be further exacerbated as the cell edge performance should improve. This means that the minimum 6G UE power capability should start from the current state of the art which is based on PC2 capable PAs used in 1Tx (PC2), 2Tx (PC1.5), and 4Tx (PC1). 3Tx is also supported but is more anecdotic as it does not fully realise HPUE.

There are artificial limitations to the power that the UE can reach:

- Boosting is only allowed optionally and only for a limited set of allocations and modulation order.

- It was shown that many other boosting options may exist:

  - For lower modulation order w/wo shaping

  - For inner third allocations

  - In cases where the OOB emissions are further away (extended bandwidth cases)

UE output power capability should be defined based on its nominal maximum output power for a fully allocated DFT-s-OFDM QPSK channel. This channel bandwidth may be different at different frequency ranges to better fit with the typical channel CBW used.

As a default capability we propose a 26dBm nominal maximum output power unless limited by regulation (FDD bands in Japan, specific in-band PSD/power regulation). The tolerance for the nominal maximum output power regardless of number of Tx chains and frequency range should be +2/-2dB.

For smaller allocations (inner half, inner third), lower MCS w/wo spectrum shaping, or when OOB requirement are further away from the channel edge (BW extension concept), the UE should always be allowed to boost up to 3dB unless it is not permitted by the NW by explicit signalling.

Proposal on single carrier output power:

- Unless otherwise limited by regulation, the default maximum nominal output power is 26dBm

  - At least for smartphone form factor, smaller form factors may still use 23dBm as the default maximum nominal output power, while CPE/FWA may target higher power.

- Maximum nominal output power is defined for a fully allocated DFT-s-OFDM QPSK waveform of:

  - 20MHz for ≤ ~1GHz UL bands

  - 50MHz for ≤ ~2GHz UL bands

  - 100MHz for < ~5GHz band

  - 200MHz for > ~5GHz and < ~52GHz bands

  - FFS if frequency thresholds should be defined differently (at least potentially for NTN bands)

- The tolerance for all maximum nominal output power <29dBm regardless of #Tx, and frequency range is +2/-2dB.

- DeltaT is not specified and considered within the post PA losses while considering multi-band support and triplexing frequency sub-ranges onto one antenna.

- For any maximum nominal output power from 23dBm to 29dBm the UE is allowed to boost its output power by up to 3dB unless limited by an explicit maximum output power limit signalled by the network.

- Within the maximum nominal output power range from 23dBm to 32dBm:

  - Up to 4Tx is supported up to 16GHz, FFS for 8Tx for larger form factors

    - 4x4 UL MIMO should be considered for smartphone form factor.

  - 2x1 to 8x8 antenna arrays are supported from >10GHz to cover all mobile/transportable UE form factors. FFS for larger arrays. 2x2 UL MIMO is supported.

  - Support of circular polarisation should be studied for NTN

- For bands <10GHz, up to 200MHz UL CBW is considered to allow PA efficiency improvement techniques.

  - If APT and ET including CFR/pre-distortion aspects must be considered, it is unlikely that a common model and assumptions can be agreed in RAN4.

  - Also, if these techniques can improve PA linearity in-band and within the ACLR1 region at low backoff levels, it comes at the expense of lower linearity improvement at higher backoff and potentially worse performance at larger frequency offsets: the intrinsic PA linearity is still what counts in the most critical OOB emission cases.

For increased UL coverage and cell edge performance, when multiple antennas are available multi-stream MIMO is not the best approach as the required SNR is not available. However, single stream MIMO based on different phases on the different antennas, although not providing beamforming as in phase arrays, may prove useful to shape the radiation pattern and gain a few dB in one direction. To fully exploit this, it may be of interest to have more phase states than every 90 degrees as of today, at least for more than two UL antennas. This may prove practical for NTN or combat hand effect.

Proposal on UE digital beamforming for multiple antennas: Investigate higher phase step granularity for single stream MIMO to improve radiation pattern directivity for above two discrete UL/DL antennas.

## 2.3 UL CA output power / Spectrum aggregation (UL)

5G FR1 also has different set of power classes that do not necessarily align:

•	The single band power class

•	The intra-band CA power class

•	The inter-band CA power class

•	The per band per BC power class

This creates confusion on which output power applies in different mode of operation: 1UL band, intra-band ULCA (1Tx/2Tx, 2PA), inter-band ULCA, in different UL MIMO/TxD modes.

To streamline this, we propose the following.

Proposal for UL CA output power and Spectrum aggregation:

- Contiguous Intra-band ULCA power class is the same than the band power class

  - Non-contiguous allocation is not supported or specified

  - intra-band CA should not be specified for a band if its spectrum can be addressed by a single CC

  - it should support the same number of Tx chain than in single CC case.

- Non-contiguous Intra-band ULCA is not supported

  - If supported, it should be based on UL switching

  - If RAN4 still studies a simultaneous 2CC NC ULCA, only 2LO(2PA) case is supported

  - In all cases the ULCA power class is the same than the band power class, but it can only be reached for balanced TxBW between the two CCs.

- No power class is defined for inter-band ULCA

  - The power capability in inter-band ULCA is the power sum of the power capability in each band signalled by the UE and no inter-band power class is defined/signalled

  - The limitation of only up to two UL clusters is maintained but 2CC in each band can be studied (4CC total). However, it should not be a priority since wider CBW than in 5G should be supported in 6G thus the need for 2UL CC in each of the two bands must first be justified.

  - There should be no limitation on the #Tx per band but it may need a per band per BC signalling when PAs are shared.

  - DeltaT is not specified and considered within the post PA losses considering multi-band support and triplexing on one antenna.

## 2.4 Tx Requirements

As already proposed in the last meeting, like it was done in 5G vs 4G, we support that the 6G UE supports a higher default UL QAM which has an impact on transmitter impairments and in return, its OOB emissions.

For bands ≤5GHz it should be feasible to support 256QAM UL by default and thus assume >34dB image and carrier rejections. At higher frequencies, maintaining good impairments may prove more difficult and 31dB image and carrier rejection may be more reasonable. While we have a good understanding of the RF front-end and the RF transceiver capabilities, we are not legitimate to comment on the CIM3 and CIM5 performance which relates to the BB which will also need to support wider BW.

For the SEM requirements, as discussed last meeting, it is essential for the SU and edge RB MPR that the measurement BW for the first MHz of the SEM mask is increased, in 5G it is 1% of CBW up to 50MHz CBW and 30kHz at 50MHz and above (which is detrimental).

Since 1% of 50MHz is 500KHz we already allow the 1% up to 45MHz we can change the 1st MHz SEM MBW requirement to 1% of CBW up to 50MHz included and 500kHz above.

For spurious emissions, it is important to check the regulation for different frequency ranges and use the worst case worldwide as a first step. However, if a local regulation is significantly different it may be useful to consider this as a band specific requirement under NS.

For ACLR we currently have 30/31/31dB ACLR for PC3/PC2/PC1.5 except band n104 that is now looking for a PC3 ACLR of 26dB. It seems that we should look into at least two separate values for ≤5GHz and >5GHz. To harmonize across a wide range of output power up to 29dBm, we suggest using 31dB ACLR for bands ≤5GHz and 27dB ACLR for bands >5GHz.

Proposal on Tx Requirements:

- Default supported UL QAM:

  - UL 256QAM for band ≤5GHz

  - UL 64QAM for band >5GHz and <16GHz

  - FFS for >16GHz

  - For NTN bands it may not be needed to specify A-MPR for the highest QAM

  - UL 1024QAM is considered optionally

    - The Tx impairments should be further improved accordingly, and Tx EVM should be based on assuming better Rx EVM on the BS side.

- ACLR for up to 29dBm nominal output power:

  - 31dB for band ≤5GHz

  - 27dB for band >5GHz and <16GHz

  - FFS for >16GHz

  - FFS if frequency thresholds should be chosen differently

- Tx impairments up to 29dBm nominal output power:

  - 34dB image and carrier for band ≤5GHz

  - 31dB image and carrier for band >5GHz and <16GHz

  - FFS for >16GHz

  - FFS if frequency thresholds should be chosen differently

- Measurement BW for the first SEM segment (first OOB MHz):

  - 1%CBW ≤50MHz CBW

  - 500kHz for >50MHz CBW

  - FFS if frequency threshold should be chosen differently (within 40MHz to 50MHz)

- Spurious emissions should be based on the worst case worldwide regulation per relevant frequency ranges

  - If a local regulation is significantly more restrictive/permissive, it may be useful to consider this as a band specific requirement under NS.

- This may result into different MPR/A-MPR in different frequency ranges. In any case, the number of MPR/A-MPR requirements should be reduced by covering single and multiple Tx by the same requirements.

  - To simplify MPR specification across #Tx, antenna coupling assumptions could be varied across frequency ranges and #Tx to have a single requirement at least up to 4Tx.

  - To simplify A-MPR, it could be defined only for the CBW that are relevant to the deployment in the region where the additional emissions apply and only tested for those. With wider channel bandwidths, the -50dBm/MHz band protection may become more difficult and be subject to A-MPR.

- Since 6G is striving to offer flexibility in UL/DL BW support, the Tx requirements should target flexible BW and verification done only for a limited set of CBW. For A-MPR it may still be necessary to test for specific CBW.

# Conclusion

In this contribution, we provide our view on 6G UE Tx performance targets and make the following proposals.

Proposal on single carrier output power:

- Unless otherwise limited by regulation, the default maximum nominal output power is 26dBm

  - At least for smartphone form factor, smaller form factors may still use 23dBm as the default maximum nominal output power, while CPE/FWA may target higher power.

- Maximum nominal output power is defined for a fully allocated DFT-s-OFDM QPSK waveform of:

  - 20MHz for ≤ ~1GHz UL bands

  - 50MHz for ≤ ~2GHz UL bands

  - 100MHz for < ~5GHz band

  - 200MHz for > ~5GHz and < ~52GHz bands

  - FFS if frequency thresholds should be defined differently (at least potentially for NTN bands)

- The tolerance for all maximum nominal output power <29dBm regardless of #Tx, and frequency range is +2/-2dB.

- DeltaT is not specified and considered within the post PA losses while considering multi-band support and triplexing frequency sub-ranges onto one antenna.

- For any maximum nominal output power from 23dBm to 29dBm the UE is allowed to boost its output power by up to 3dB unless limited by an explicit maximum output power limit signalled by the network.

- Within the maximum nominal output power range from 23dBm to 32dBm:

  - Up to 4Tx is supported up to 16GHz, FFS for 8Tx for larger form factors

    - 4x4 UL MIMO should be considered for smartphone form factor.

  - 2x1 to 8x8 antenna arrays are supported from >10GHz to cover all mobile/transportable UE form factors. FFS for larger arrays. 2x2 UL MIMO is supported.

  - Support of circular polarisation should be studied for NTN

- For bands <10GHz, up to 200MHz UL CBW is considered to allow PA efficiency improvement techniques.

  - If APT and ET including CFR/pre-distortion aspects must be considered, it is unlikely that a common model and assumptions can be agreed in RAN4.

  - Also, if these techniques can improve PA linearity in-band and within the ACLR1 region at low backoff levels, it comes at the expense of lower linearity improvement at higher backoff and potentially worse performance at larger frequency offsets: the intrinsic PA linearity is still what counts in the most critical OOB emission cases.

Proposal on UE digital beamforming for multiple antennas: Investigate higher phase step granularity for single stream MIMO to improve radiation pattern directivity for above two discrete UL/DL antennas.

Proposal for UL CA output power and Spectrum aggregation:

- Contiguous Intra-band ULCA power class is the same than the band power class

  - Non-contiguous allocation is not supported or specified

  - intra-band CA should not be specified for a band if its spectrum can be addressed by a single CC

  - it should support the same number of Tx chain than in single CC case.

- Non-contiguous Intra-band ULCA is not supported

  - If supported, it should be based on UL switching

  - If RAN4 still studies a simultaneous 2CC NC ULCA, only 2LO(2PA) case is supported

  - In all cases the ULCA power class is the same than the band power class, but it can only be reached for balanced TxBW between the two CCs.

- No power class is defined for inter-band ULCA

  - The power capability in inter-band ULCA is the power sum of the power capability in each band signalled by the UE and no inter-band power class is defined/signalled

  - The limitation of only up to two UL clusters is maintained but 2CC in each band can be studied (4CC total). However, it should not be a priority since wider CBW than in 5G should be supported in 6G thus the need for 2UL CC in each of the two bands must first be justified.

  - There should be no limitation on the #Tx per band but it may need a per band per BC signalling when PAs are shared.

  - DeltaT is not specified and considered within the post PA losses considering multi-band support and triplexing on one antenna.

Proposal on Tx Requirements:

- Default supported UL QAM:

  - UL 256QAM for band ≤5GHz

  - UL 64QAM for band >5GHz and <16GHz

  - FFS for >16GHz

  - For NTN bands it may not be needed to specify A-MPR for the highest QAM

  - UL 1024QAM is considered optionally

    - The Tx impairments should be further improved accordingly, and Tx EVM should be based on assuming better Rx EVM on the BS side.

- ACLR for up to 29dBm nominal output power:

  - 31dB for band ≤5GHz

  - 27dB for band >5GHz and <16GHz

  - FFS for >16GHz

  - FFS if frequency thresholds should be chosen differently

- Tx impairments up to 29dBm nominal output power:

  - 34dB image and carrier for band ≤5GHz

  - 31dB image and carrier for band >5GHz and <16GHz

  - FFS for >16GHz

  - FFS if frequency thresholds should be chosen differently

- Measurement BW for the first SEM segment (first OOB MHz):

  - 1%CBW ≤50MHz CBW

  - 500kHz for >50MHz CBW

  - FFS if frequency threshold should be chosen differently (within 40MHz to 50MHz)

- Spurious emissions should be based on the worst case worldwide regulation per relevant frequency ranges

  - If a local regulation is significantly more restrictive/permissive, it may be useful to consider this as a band specific requirement under NS.

- This may result into different MPR/A-MPR in different frequency ranges. In any case, the number of MPR/A-MPR requirements should be reduced by covering single and multiple Tx by the same requirements.

  - To simplify MPR specification across #Tx, antenna coupling assumptions could be varied across frequency ranges and #Tx to have a single requirement at least up to 4Tx.

  - To simplify A-MPR, it could be defined only for the CBW that are relevant to the deployment in the region where the additional emissions apply and only tested for those. With wider channel bandwidths, the -50dBm/MHz band protection may become more difficult and be subject to A-MPR.

- Since 6G is striving to offer flexibility in UL/DL BW support, the Tx requirements should target flexible BW and verification done only for a limited set of CBW. For A-MPR it may still be necessary to test for specific CBW.

# References

[1] R4-2514642 WF on [116bis][102] 6G general RF and UE RF, Qualcomm, RAN4#116b

[2] R4-2513115 Inputs to RAN4 6G study on general RF and UERF, Skyworks Solutions Inc., RAN4#116b

[3] R4-2513114 Inputs to RAN4 6G study on system parameters, Skyworks Solutions Inc., RAN4#116b

[4] R4-2513062 Inputs to RAN4 6G study on general aspects, Skyworks Solutions Inc., RAN4#116b