3GPP TSG-RAN4 Meeting #117	R4-2520340

# Dallas, USA, 17th Nov 2025 - 21st Nov 2025

# Agenda item:			7.3.3

# Source:					Eutelsat Group

# Title:						VSAT parameters for testability study

# Document for:		Information

# Overview

This paper provides additional details from [1] on VSAT physical and parameters and other aspects for the study on test methods for VSAT agreed at RAN #109 in [2].

# Discussion

## Antenna aperture

The WF in [2] agreed an initial QZ target of [63] cm based on a 45 x 45 device size. This can incorporate a variety of antenna apertures without the need to specify antenna phase centres (white box).

Other common antenna apertures are 20 x 20, 35 x 35, 65 x 65 and 78 x 78 requiring QZ of 28.3, 49.5, 91.9 and 110.3 cm. It is proposed to focus on the larger 65 x 65 VSAT with a [91.9] cm QZ at this time as a vehicle for addressing the wider issues of VSAT testability. Larger QZ can be considered as required.

Proposal 1: It is proposed to focus on the [91.9] cm QZ at this time as a vehicle for addressing the wider issues of VSAT testability. Larger QZ can be considered as required.

## Device size and weight

Unlike FR2 handsets, the range of VSAT sizes and weights covers a very wide range of possibilities. Based on proposal 1, it is proposed to focus initially on a VSAT with external dimensions of 65 cm x 65 cm, with 10 cm height and a weight around 12 kg.

Larger and heavier VSAT are not precluded but it is expected the market will be dominated by type 6 devices of these proportions.

Significantly larger and heavier devices may require bespoke test systems similar to those required for BS testing.

Proposal 2: It is proposed to focus on VSAT with dimensions 65 cm x 65 cm x 10 cm with a weight around 12 kg.  Larger VSAT can be considered as required.

## Polarization

Currently, 38.101-5 and 38.108 do not define the relationship between minimum requirements and polarization. In practice, polarization is an operational choice not governed by regulation. Traditionally, Ka band uses left-hand circular polarization (LHCP) and/or right-hand circular polarization (RHCP) for both the downlink and uplink. Ku band also uses CP but additionally, GSO can use linear polarization which can be X (also known as “H” or “Horizontal”) or Y (also known as “V” or “Vertical). Note that the reference plane for X is not the terrestrial horizon but the equatorial plane.

The need to signal the difference between X and Y linear polarization (LP) is currently missing in the RRC specifications and is being addressed in this meeting cycle with CRs in RAN2 [3], [4] & [5].

The minimum requirements defined for the VSAT and SAN apply for the deployed polarization. This is not explicitly defined in 38.101-5 or 38.108. Without this clarification it would be possible to define conformance tests that do not align the Tx and Rx signals of the test system with the declared capabilities of the VSAT or SAN. To solve this, CRs to 38.101-5 and 38.108 in [6] and [7] will clarify the applicability of polarization to the minimum requirements.

For the VSAT and SAN RF requirements, the CRs in [6] and [7] add the necessary applicability statements for how the minimum requirements apply to specific manufacturer declared polarization capabilities.

Proposal 3: The CRs in [6] and [7] add the necessary applicability statements for how the VSAT and SAN minimum requirements apply to specific manufacturer declared polarization capabilities.

## Beam steering in the test system

In normal operation the VSAT will steer its beam towards the satellite based on VSAT GNSS location, the ephemeris data broadcast on the downlink and the VSAT propagator and tilt sensor. In an OTA test system, it is not expected that these conditions will be fully emulated as the SAN probe antenna will be fixed in space which is not a real scenario. The consequence is that for test purposes, the VSAT is likely to be initially locked on boresight and manually positioned to point towards the SAN probe antenna. Automatic SAN location features would therefore be disabled. It is proposed that the initial setup will require the VSAT to be mounted on its side such that its boresight direction, which is normally perpendicular to the horizon, will point horizontally along the test chamber towards the measurement probe. For this to occur, it will be necessary for the VSAT to disable operational beam pointing algorithms which may include GNSS acquisition, ephemeris reading, VSAT propagator tracking of the SAN position and VSAT tilt sensors.

The desired initial test state is likely to be a boresight configuration with the VSAT nominally orientated towards the test probe, which is likely to be in a horizontal, not vertical plane. From this initial setup, measurements associated with the beam peak direction such as EIRP and EIS will require location of the true beam peak. It would be possible to use existing very fine beam pointing mechanisms in the VSAT to point the VSAT exactly at the probe antenna. Alternatively, the VSAT could remain fixed at nominal boresight and the test system positioner could be used to find the beam peak as is the case with FR2 handsets.

Using operational VSAT antenna steering mechanisms would require special handling given that the test system does not fully represent a real network with a moving SAN and real locations. The alternative of locking the VSAT on boresight and using a positioner would seem to be less complex assuming the positioner has the necessary resolution. To enable such a configuration a special conformance test function would need to be defined in 38.509.

Proposal 4: For initial test setup, the VSAT shall be locked on boresight and nominally positioned to point towards the SAN probe antenna.

Proposal 5: Beak peak search shall be carried out using the test system positioner, not the VSAT’s fine beam pointing capability.

Proposal 6: A special conformance test function to configure the VSAT to boresight, disabling GNSS, ephemeris, propagator and tilt sensor capabilities needs to be defined.

For other requirements at alternative VSAT pointing angles, the simplest mechanism to steer the VSAT beam across the range of declared pointing angles would be to define an additional special conformance test function to set the elevation and azimuth of the Tx an Rx antenna pointing angles.

Proposal 7: A special conformance test function to configure the VSAT to a specific azimuth and elevation within the manufacturer’s declared range is required.

# Summary

Proposal 1: It is proposed to focus on the [91.9] cm QZ at this time as a vehicle for addressing the wider issues of VSAT testability. Larger QZ can be considered as required.

Proposal 2: It is proposed to focus on VSAT with dimensions 65 cm x 65 cm x 10 cm with a weight around 12 kg.  Larger VSAT can be considered as required.

Proposal 3: The CRs in [6] and [7] add the necessary applicability statements for how the VSAT and SAN minimum requirements apply to specific manufacturer declared polarization capabilities.

Proposal 4: For initial test setup, the VSAT shall be locked on boresight and nominally positioned to point towards the SAN probe antenna.

Proposal 5: Beak peak search shall be carried out using the test system positioner, not the VSAT’s fine beam pointing capability.

Proposal 6: A special conformance test function to configure the VSAT to boresight, disabling GNSS, ephemeris, propagator and tilt sensor capabilities needs to be defined.

Proposal 7: A special conformance test function to configure the VSAT to a specific azimuth and elevation within the manufacturer’s declared range is required.

# References

[1]	R4-2514266 “Scoping of VSAT testability study”, Eutelsat Group, RAN4 #116bis Oct 2025

[2]	R4-2514820 “WF on NR_UE_OTA_Enh”, Eutelsat Group, Eutelsat Group, RAN4 #116bis Oct 2025

[3]	R2-2508786 “Linear polarization orientation RRC signalling for NR NTN”, Eutelsat Group, Thales, Hispasat, Airbus RAN2 #132 Oct 2025

[4]	R2-2508788 “Linear polarization orientation RRC signalling for NR NTN”, Eutelsat Group, Thales, Hispasat, Airbus RAN2 #132 Oct 2025

[5]	R2-2508789 “Linear polarization orientation RRC signalling for NR NTN”, Eutelsat Group, Thales, Hispasat, Airbus RAN2 #132 Oct 2025

[6]	R4-2514355 “Applicability of minimum requirements related to polarization”, Eutelsat Group, RAN4 #117 Nov 2025

[7]	R4-2514368 “Applicability of minimum requirements related to polarization”, Eutelsat Group, RAN4 #117 Nov 2025