3GPP TSG-RAN WG4 Meeting #117	R4-2520522
Dallas, United States, 17th – 21th Nov, 2025

Agenda item:	6.9.2.2

Source:	CAICT

Title:	Analysis for FR1 MIMO OTA measurement campaign

Document for:	Approval

1	Introduction

In 3GPP Rel-19 FR1 MIMO OTA measurement campaign, volunteer labs provided measurement data for specifying FR1 MIMO OTA performance requirements [1]-[4]. This contribution presents the final analysis of all measurement data submitted by volunteer labs based on the approved Framework for FR1 MIMO OTA requirements development [5]. The detailed analysis data is attached in the Excel worksheet of this contribution.

2	Discussion

According to the test plan in TS38.151, DUTs shall meet the additional criteria in azimuthal orientations during MIMO OTA measurement, otherwise, the DUTs fail the FR1 MIMO OTA test:

The additional criterion in azimuthal orientations shall be met: - The EUT must meet 70% throughput in 11 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test. - The EUT must meet 90% throughput in 10 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

Table 1 summarizes the number of DUTs tested and those that failed the additional criteria per band. It can be observed from the measurement results that no DUT failed the additional criteria at bands n3, n7, n77, and n8.

Table 1. Summary of the amount of DUTs

| Band | n3 | n7 | n77 | n8 |
| --- | --- | --- | --- | --- |
| Total amount of DUTs | 19 | 14 | 30 | 25 |
| Amount of DUTs fail 90% criteria but pass 70% | 0 | 0 | 0 | 0 |
| Amount of DUTs fail 90% and 70% criteria | 0 | 0 | 0 | 0 |

The CDF curves of the TRMS70 for bands n3, n7, n77, and n8 are plotted in Figs. 1~4, and Fig. 5 includes the CDF curves of all four measurement data for comparison.

Figure 1. CDF of measurement data at band n3

Figure 2. CDF of measurement data at band n7

Figure 3. CDF of measurement data at band n77

Figure 4. CDF of measurement data at band n8

Figure 5. The CDF of the summarized measurement data for bands n3, n7, n77, and n8

The TRMS values at 80%, 85%, 90%, 95% percentiles of the CDF curves are summarized in Table 2.

Table 2. Summary of CDF analysis results [dBm/15 or 30 kHz]

| Percentile (pass rate) | n3 TRMS70 | n7 TRMS70 | n77 TRMS70 | n8 TRMS70 |
| --- | --- | --- | --- | --- |
| 80%-tile | -96.51 | -97.67 | -96.00 | -89.32 |
| 85%-tile | -96.10 | -97.15 | -95.27 | -89.14 |
| 90%-tile | -95.85 | -96.50 | -94.45 | -87.95 |
| 95%-tile | -95.64 | -96.12 | -93.77 | -86.58 |
| Total amount of samples (Threshold: 15) | 19 | 14 | 30 | 25 |

In the Framework [5], it was agreed that the value at [80%] percentile of the CDF curve can be selected as the starting point for requirement discussion. Based on the CDF analysis, we have the following proposal accordingly:

Proposal 1: Adopt the values at 80% percentile of the CDF curves as starting point for requirements discussion. (rounded to a precision of 0.5 dB for simplification)

- For band n3, define the performance requirement as -96.5 dBm/15 kHz.

- For band n7, define the performance requirement as -97.5 dBm/15 kHz.

- For band n77, define the performance requirement as -96.0 dBm/30 kHz.

- For band n8, define the performance requirement as -89.0 dBm/15 kHz.

3	Conclusion

This contribution provides the final analysis of FR1 MIMO OTA measurement campaign and proposals on performance requirements.

Proposal 1: Adopt the values at 80% percentile of the CDF curves as starting point for requirements discussion. (rounded to a precision of 0.5 dB for simplification)

- For band n3, define the performance requirement as -96.5 dBm/15 kHz.

- For band n7, define the performance requirement as -97.5 dBm/15 kHz.

- For band n77, define the performance requirement as -96.0 dBm/30 kHz.

- For band n8, define the performance requirement as -89.0 dBm/15 kHz.

References

R4-2520521, CAICT Rel-19 measurement results for FR1 MIMO OTA Measurement Campaign, CAICT,  3GPP RAN4#117, Nov 2025.

R4-2521535, CMCC&BUPT joint lab measurement data for 3GPP Rel-19 FR1 MIMO OTA Measurement Campaign , CMCC, BUPT, 3GPP RAN4#117, Nov 2025.

R4-2520494, 3GPP Rel-19 FR1 MIMO OTA Measurement Campaign Results, Xiaomi, 3GPP RAN4#117, Nov 2025.

R4-2520599, 3GPP Rel-19 FR1 MIMO OTA Measurement Campaign results, Apple, 3GPP RAN4#117, Nov 2025.

R4-2506286, Framework for Rel-19 MIMO OTA requirements (May 2025), CAICT, 3GPP RAN4#115, May. 2025.