3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2520545

Dallas, USA, Nov. 17-21, 2025

Title: 	UE Rx performance target for 6G and RFFE characteristics up to 15GHz

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.3 General RF and UE RF [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#116bis, a way forward [1] captured several guidelines for the 6G UE RF study. This contribution is relevant to the topics in the way forward: 5) Rx requirements and 7) Frequency range between FR1 and FR2-1. It is based on further developing our inputs from different contributions of in last meeting [2,3,4].

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several guidelines for the 6G UE RF study, the parts relevant to the UE Rx performance are the following topics: 5) Rx requirements.

The relevant parts of the way forward are copied here in italics.

Rx requirements 5): Agreements

- Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s

- Study concept for RX requirements for new frequencies, especially those above 7.125GHz

- Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases

- Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well

  - Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference

- Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements

- Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable

Frequency range between FR1 and FR2-1 7): Agreements

- Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz.

In accordance with the guidelines, each topic is treated in separate chapters in this contribution. However, different frequency ranges and implementation options call for different Rx requirements and thus there is some redundancy in the two chapters.

## 2.2 Rx requirements

Like what we propose in Tx in another contribution, Rx requirements need to be improved for 6G.

In the same way that we propose that DeltaT is not specified and have post PA losses to account for it, assuming switching and antenna multiplexing losses, we suggest ignoring DeltaR and absorb it within the REFSENS requirement.

To support full flexibility for the supported DL BW, REFSENS should be fully scalable with the number of DL RBs in FDD bands as is currently the case for TDD bands. This requires that the UL configuration or power is chosen in a way that UL interference (except the transmitter noise floor which de-sense can also be scaled as its contribution is flat in frequency) will not cause minimum degradation. If needed, additional de-sense indications beyond REFSENS can be given in terms of potential UL related issue (for example which IMD order(s) may affect the DL). To verify REFSENS, only a small number of channel BW may be verified (for example the relevant CBW for a band out of 5/10/20/40/50/80/100MHz).

In order to achieve the above, a number of improvements can be accounted for:

Proposal on improved REFSENS parameters allowing REFSENS BW scaling for FDD and TDD bands:

- Assuming 55dB T/R isolation for duplexers <2.7GHz, FFS if higher value can be agreed vs for lower frequency range.

- Improved IMD2 in the receiver

- Improved linearity of the PA and improved image and carrier rejection to 34dB for FDD bands

- Assume systematically a 12dB UL interference imbalance between main and diversity

- Use a limited number of UL RBs (using a maximum of 20-25RBs depending on the channel CBW)

- Place the UL RB in the most favourable position where the IMDs related to its image and carrier leakage do not affect the DL. An indication may still be given on what the worst-case configuration is, FFS if a desense number is provided.

- DeltaR is not specified and is absorbed within REFSENS while considering multi-band support and triplexing frequency sub-ranges onto one antenna.

- Re-consider implementation margin and using MRC combining as the baseline.

While scaling is needed for flexible BW support, it is also important that REFSENS is easily scaled for the number of Rx from 1Rx to 8Rx (for example by having an agreed main to diversity imbalance to derive 1Rx from 2Rx reference, and say 2.5dB REFSENS improvement per doubling #Rx, i.e. scaling with 10*log(#Rx/2)-#Rx/4*0.5dB).

Finally, we also have preliminary inputs to DL MIMO and DLCA operation.

Proposal on Rx requirements:

- 2Rx is still the baseline for REFSENS and scalable by RB including for FDD bands.

- 1Rx to 8Rx scalability is granted by using:

  - 12dB UL interference imbalance between main and diversity to derive 1Rx REFSENS from 2Rx reference

  - scaling with 10*log(#Rx/2)-#Rx/4*0.5dB for #Rx>2

  - FFS the exact values and equations

- Default supported DL QAM:

  - DL 1024QAM for band ≤5GHz

  - UL 256QAM for band >5GHz and <16GHz

  - FFS for >16GHz

  - For NTN bands it may not be needed to specify the highest QAM

  - DL 4096QAM is considered optionally

    - The Rx impairments like IMD2, image and dynamic range should be further improved accordingly, and Rx EVM should be based on assuming better Tx EVM on the BS side.

- Rx requirement should be scalable in BW for:

  - 3 to 100MHz for FDD band < 2.7GHz

  - 10 to 200MHz for TDD bands <5GHz

  - 20 to 400MHz for TDD bands >5GHz and <16GHz

  - 20 to 1600MHz for bands >16GHz

  - FFS if the frequency thresholds should be defined differently:

    - At least potentially for NTN bands

    - Depending on SCS and FFT size support per band.

- Antenna scalability:

  - Up to 8Rx is supported up to 16GHz, FFS for >#Rx for larger form factors

    - 8x8 DL MIMO should be considered for smartphone form factor.

  - 2x1 to 8x8 antenna arrays are supported from >10GHz to cover all mobile/transportable UE form factors. FFS for larger arrays. 2x2 DL MIMO is supported

  - Support of circular polarisation should be studied for NTN

- DL CA operation:

  - DeltaR is not specified and is absorbed within REFSENS while considering multi-band support and triplexing frequency sub-ranges onto one antenna.

  - Intra-band contiguous and non-contiguous is supported with up to 4CC but the #Rx may be scaled back from the single CC operation depending on the total BW and number of DL clusters to be supported

    - Intra-band contiguous DL is only supported if the BW cannot be supported with a single CC

    - For FDD bands no specific REFSENS requirement is specified

  - Inter-band DL CA with 1UL or 2UL bands is automatically supported without MSD specified if the bands are in separate band groups. FFS if the bands are part of the same band group.

    - In any case if MSD cases are specified, an automatic calculation based on band group assumption are studied.

## 2.3 Frequency range between FR1 and FR2-1

As we have discussed in our paper on frequency range in the previous meeting, we are not in favour of discussing this range in isolation as we believe that from a requirement perspective, there is only two types: conducted/separate antenna or OTA/beamforming.

Given that 6G UE will cover applications ranging in size from small IoT devices and dongles to vehicles and even planes, the implementation may vary depending on the UE size and thus conducted/separate antenna and OTA/beamforming may need to be supported in the entire FR1 to FR2-1 range. Also note that NTN should be accounted for, thus we see UE implementations with:

- OTA/beamforming/antenna arrays at least from10GHz and above. We also suggest that parabolic antennas are covered here.

- Conducted/separate antennas at least up to 16GHz

In this contribution, we will not discuss the RFFE technology details or performance yet, but would first like to establish a common understanding on the UE front end architecture. Depending on the two implementations, filtering and antenna multiplexing requirements will differ.

Proposal on RFFE characteristics covering the entire 0.4GHz to 52GHz frequency range:

- In the separate antenna range up to 16GHz

  - Requirements tend to be related to conducted measurements and the radiation pattern is mostly omnidirectional

  - Multiplexing multiple frequency sub-ranges is a must and

  - Many bands are multiplexed with a frequency sub-range

    - non-3GPP systems/bands are also sharing cellular antennas (for example: GNSS, WiFi and other connectivity systems)

  - TN FDD bands are <2.7GHz requiring duplexers with small UL/DL gaps and TDD bands > 1.4GHz which, outside exceptions where a band needs to be protected in close proximity in frequency, the filtering requirement is more relaxed and co-banding

  - NTN FDD bands go up to 16GHz with some tight duplex separation for n247/248 or n508/n509 bands

  - To optimize the multiplexing, switching and hardware, co-banding is a critical aspect of the RFFE performance/size and cost.

- In the antenna array range from 10GHz and above

  - Requirements tend to be related to OTA measurements and the radiation pattern is mostly directional or with beamforming

  - Multiplexing multiple bands or frequency sub-ranges to the same array is limited as the array dimensioning is related to the lambda at the operating frequency

  - TN bands are TDD and when feasible within a small frequency range, co-banding is used.

  - NTN bands are FDD with large UL/DL frequency separation thus using separate arrays but the for the n247/248 or n508/n509 bands the array or antennas could be shared raising the need for duplexers.

- At least up to 16GHz, the technology set used in today’s T/R modules today can be re-used with small enhancements. This covers switching, filtering, multiplexing, LNA and PAs.

- Above 20GHz silicon level integration is preferred for antenna arrays with only limited filtering capability for UEs with size constraints.

# Conclusion

In this contribution, we provide our view on 6G UE Rx performance targets and make the following proposals.

Proposal on improved REFSENS parameters allowing REFSENS BW scaling for FDD and TDD bands:

- Assuming 55dB T/R isolation for duplexers <2.7GHz, FFS if higher value can be agreed vs for lower frequency range.

- Improved IMD2 in the receiver

- Improved linearity of the PA and improved image and carrier rejection to 34dB for FDD bands

- Assume systematically a 12dB UL interference imbalance between main and diversity

- Use a limited number of UL RBs (using a maximum of 20-25RBs depending on the channel CBW)

- Place the UL RB in the most favourable position where the IMDs related to its image and carrier leakage do not affect the DL. An indication may still be given on what the worst-case configuration is, FFS if a desense number is provided.

- DeltaR is not specified and is absorbed within REFSENS while considering multi-band support and triplexing frequency sub-ranges onto one antenna.

- Re-consider implementation margin and using MRC combining as the baseline.

Proposal on Rx requirements:

- 2Rx is still the baseline for REFSENS and scalable by RB including for FDD bands.

- 1Rx to 8Rx scalability is granted by using:

  - 12dB UL interference imbalance between main and diversity to derive 1Rx REFSENS from 2Rx reference

  - scaling with 10*log(#Rx/2)-#Rx/4*0.5dB for #Rx>2

  - FFS the exact values and equations

- Default supported DL QAM:

  - DL 1024QAM for band ≤5GHz

  - UL 256QAM for band >5GHz and <16GHz

  - FFS for >16GHz

  - For NTN bands it may not be needed to specify the highest QAM

  - DL 4096QAM is considered optionally

    - The Rx impairments like IMD2, image and dynamic range should be further improved accordingly, and Rx EVM should be based on assuming better Tx EVM on the BS side.

- Rx requirement should be scalable in BW for:

  - 3 to 100MHz for FDD band < 2.7GHz

  - 10 to 200MHz for TDD bands <5GHz

  - 20 to 400MHz for TDD bands >5GHz and <16GHz

  - 20 to 1600MHz for bands >16GHz

  - FFS if the frequency thresholds should be defined differently:

    - At least potentially for NTN bands

    - Depending on SCS and FFT size support per band.

- Antenna scalability:

  - Up to 8Rx is supported up to 16GHz, FFS for >#Rx for larger form factors

    - 8x8 DL MIMO should be considered for smartphone form factor.

  - 2x1 to 8x8 antenna arrays are supported from >10GHz to cover all mobile/transportable UE form factors. FFS for larger arrays. 2x2 DL MIMO is supported

  - Support of circular polarisation should be studied for NTN

- DL CA operation:

  - DeltaR is not specified and is absorbed within REFSENS while considering multi-band support and triplexing frequency sub-ranges onto one antenna.

  - Intra-band contiguous and non-contiguous is supported with up to 4CC but the #Rx may be scaled back from the single CC operation depending on the total BW and number of DL clusters to be supported

    - Intra-band contiguous DL is only supported if the BW cannot be supported with a single CC

    - For FDD bands no specific REFSENS requirement is specified

  - Inter-band DL CA with 1UL or 2UL bands is automatically supported without MSD specified if the bands are in separate band groups. FFS if the bands are part of the same band group.

    - In any case if MSD cases are specified, an automatic calculation based on band group assumption are studied.

Proposal on RFFE characteristics covering the entire 0.4GHz to 52GHz frequency range:

- In the separate antenna range up to 16GHz

  - Requirements tend to be related to conducted measurements and the radiation pattern is mostly omnidirectional

  - Multiplexing multiple frequency sub-ranges is a must and

  - Many bands are multiplexed with a frequency sub-range

    - non-3GPP systems/bands are also sharing cellular antennas (for example: GNSS, WiFi and other connectivity systems)

  - TN FDD bands are <2.7GHz requiring duplexers with small UL/DL gaps and TDD bands > 1.4GHz which, outside exceptions where a band needs to be protected in close proximity in frequency, the filtering requirement is more relaxed and co-banding

  - NTN FDD bands go up to 16GHz with some tight duplex separation for n247/248 or n508/n509 bands

  - To optimize the multiplexing, switching and hardware, co-banding is a critical aspect of the RFFE performance/size and cost.

- In the antenna array range from 10GHz and above

  - Requirements tend to be related to OTA measurements and the radiation pattern is mostly directional or with beamforming

  - Multiplexing multiple bands or frequency sub-ranges to the same array is limited as the array dimensioning is related to the lambda at the operating frequency

  - TN bands are TDD and when feasible within a small frequency range, co-banding is used.

  - NTN bands are FDD with large UL/DL frequency separation thus using separate arrays but the for the n247/248 or n508/n509 bands the array or antennas could be shared raising the need for duplexers.

- At least up to 16GHz, the technology set used in today’s T/R modules today can be re-used with small enhancements. This covers switching, filtering, multiplexing, LNA and PAs.

- Above 20GHz silicon level integration is preferred for antenna arrays with only limited filtering capability for UEs with size constraints.

# References

[1] R4-2514642 WF on [116bis][102] 6G general RF and UE RF, Qualcomm, RAN4#116b

[2] R4-2513115 Inputs to RAN4 6G study on general RF and UERF, Skyworks Solutions Inc., RAN4#116b

[3] R4-2513114 Inputs to RAN4 6G study on system parameters, Skyworks Solutions Inc., RAN4#116b

[4] R4-2513062 Inputs to RAN4 6G study on general aspects, Skyworks Solutions Inc., RAN4#116b