3GPP RAN WG4 Meeting #117		R4-2520558

Dallas, USA, 17th – 21st November 2025

Agenda item:	10.7

Source:	Apple

Title:	Power back-off analysis for ETSI regulations in L-band

WI/SI:	NR_NTN_solutions-Core

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

RAN#86 meeting approved the Rel-17 WI that aimed to enable the 5G/NR radio access technology for non-terrestrial satellite deployments [1], whereupon one of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of the Rel-17 discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. While most of the satellite MSS bands have global ITU allocations and can be enabled in different regions, there exist regional and country specific requirements. As an example, US FCC defines additional out-of-band emission masks for all MSS bands that can be operated in US, and so ETSI does for the satellite MSS bands that can be activated in the ETSI regulatory domain. 3GPP did not capture all ETSI regulations for all applicable MSS bands, which makes it impossible to ensure compliant UE behaviour in countries following the ETSI standards.

Thus, in this discussion paper we present an overview of the existing ETSI requirements for the MSS L-bands with associated power back-off analysis.

# 2	ETSI standards for the MSS L-band

During the Rel-19 specification work for so-called combined L-bands, 3GPP already analysed applicable ETSI standards and performed the power back-off analysis for the 5G/NR based NTN technologies. The outcome of that work was new NS flags that were defined for the NTN NR bands. However, there was no analysis performed for the IOT bands 255 (ETSI L-band sub-rage 1) and 253 (ETSI L-band sub-range 2).

## 2.1	ETSI in-band emissions

The L-band in-band and out-of-band emission requirements are defined in ETSI EN 301681. The in-band emission mask can be found in Annex A, whereupon Figure A-1 shows the super-position of the ETSI in-band mask (pink) and the 3GPP NB-IOT mask (blue). In Figure A-1 all emission levels are scaled to the same measurement unit expressed in dBm/MHz, whereupon we indicate both original and scaled values.

As can be seen from the figure, the 3GPP NB-IOT mask is more stringent except a small frequency range at offset of 100-150kHz, where the ETSI mask has more stringent emission levels. Nevertheless, we conducted a set of additional power back-off simulations, presented in Figure 2.1-1, from which we can see that irrespective of how many tones are allocated – 1 tone or 12 tones – all emissions stay within the ETSI limits.

It is also worth noting that according to ETSI EN 301681 same in-band mask is assumed for ETSI sub-range 1 and sub-range 2, i.e. IOT bands 255 and 253, respectively. Thus, the existing ETSI in-band mask is not a limiting factor for the IOT bands 255 and 253.

Observation 1a:	Same ETSI in-band mask from EN 301681 is assumed for sub-range 1 and sub-range 2, i.e. IOT bands 255 and 253.

Observation 1b:	While the ETSI in-band mask is more stringent for the offset at 100-150kHz, actual power back-off simulations indicate that all emissions stay within the ETSI limits.

Figure 2.1-1: Power back-off simulations for PC3 and the ETSI in-band mask (1, 3, 6 and 12 tones).

## 2.2	ETSI out-of-band emissions

ETSI EN 301681 follows different approaches while defining out-of-band emission limits for the sub-range 1 and 2:

-	Sub-range 1 (band 255): The lower edge range out-of-band emission limits start with a stringent limit of -10dBm/30kHz, i.e. 5dBm/MHz, which will cause power back-off for the NB-IOT channel if placed at the lower edge of the band. To solve that issue 3GPP has already decided to consider a "guard band" of 190kHz. The upper edge emission limits are based on the same in-band mask presented in section 2.1, i.e. there should be no additional constraints.

-	Sub-range 2 (band 253): Both lower and the upper edge out-of-band emission limits are based on the ETSI in-band emission mask and thus would not trigger any additional constraints.

Observation 2a:	Sub-range 1 (band 255) lower edge strict emission requirements are handled via additional guard band. Upper edge emission limits are based on the in-band mask and thus would not require any power back-off.

Observation 2b:	Sub-range 2 (band 253) lower and upper edge emission requirements are based on the in-band mask and thus would not require any power back-off.

# 3	Conclusions

In this discussion paper we presented our considerations on the ETSI requirements for the L-band(s) and whether they can be met by the NB-IOT. Based on the presented analysis and additional power back-off simulations conducted for the ETSI in-band mask, our conclusion is that no additional power back-off should be defined for the IOT bands 255 and 253.

Observation 1a:	Same ETSI in-band mask from EN 301681 is assumed for sub-range 1 and sub-range 2, i.e. IOT bands 255 and 253.

Observation 1b:	While the ETSI in-band mask is more stringent for the offset at 100-150kHz, actual power back-off simulations indicate that all emissions stay within the ETSI limits.

Observation 2a:	Sub-range 1 (band 255) lower edge strict emission requirements are handled via additional guard band. Upper edge emission limits are based on the in-band mask and thus would not require any power back-off.

Observation 2b:	Sub-range 2 (band 253) lower and upper edge emission requirements are based on the in-band mask and thus would not require any power back-off.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

Annex A:	ETSI EN 301681 in-band emission mask

Table A-1: ETSI in-band mask for sub-range 1 and sub-range 2.

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |

Figure A-1: Superposition of the ETSI in-band mask (pink) and 3GPP NB-IOT mask (blue).