3GPP RAN WG4 Meeting #117		R4-2520559

Dallas, USA, 17th – 21st November 2025

Agenda item:	10.7

Source:	Apple

Title:	Power back-off analysis for ETSI regulations in S-band

WI/SI:	NR_NTN_solutions-Core

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

RAN#86 meeting approved the Rel-17 WI that aimed to enable the 5G/NR radio access technology for non-terrestrial satellite deployments [1], whereupon one of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of the Rel-17 discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. While most of the satellite MSS bands have global ITU allocations and can be enabled in different regions, there exist regional and country specific requirements. As an example, US FCC defines additional out-of-band emission masks for all MSS bands that can be operated in US, and so ETSI does for the satellite MSS bands that can be activated in the ETSI regulatory domain. 3GPP did not capture all ETSI regulations for all applicable MSS bands, which makes it impossible to ensure compliant UE behaviour in countries following the ETSI standards.

Thus, in this discussion paper we present an overview of the existing ETSI requirements for the MSS S-band.

# 2	ETSI standards for the MSS S-band

The situation for the ETSI requirements and the S-band is a bit different compared to other MSS standards because it is regulated by three different ETSI standards depending on the type of the system and its parameters, such as wide- narrow-band channel. Table 2-1 below makes a small summary, from one can see that EN 301442 regulates non-GEO systems irrespective of the channel size, whereas EN 302574-2/3 are for the GEO systems with further differentiation between narrow and wide-band systems.

Table 2-1: Summary of ETSI S-band standards.

| System bandwidth | non-GEO system | GEO system |
| --- | --- | --- |
| wide-band (>1MHz) | EN 301 442 | EN 302 574-2 |
| narrow-band (<1MHz) |  | EN 302 574-3 |

## 2.1	ETSI EN 301 442

EN 301442 in-band mask is similar to the ETSI L-band mask but still has different emission levels.  Nevertheless, as one can see from Figure 2.1-1 showing a superposition of the ETSI 301442 mask and the 3GPP NB-IOT mask, the ETSI mask should not cause any power limitations for the NB-IOT technology.

Figure 2.1-1: Superposition of the ETSI in-band mask (pink) and 3GPP NB-IOT mask (blue).

As for the 3GPP NR/5G based technology, Figure 2.1-2 below shows the NR 5MHz mask combined with the ETSI mask. While the ETSI mask should not be a limiting factor for the 5G/NR NTN, there are certain frequency ranges where the ETSI mask has more stringent limits, which requires further power back-off analysis.

Figure 2.1-2: Superposition of the ETSI in-band mask (pink) and 3GPP NR 5MHz mask (blue).

As for the out-of-band unwanted emission requirements, EN 301442 defines same mask for the lower and upper edges, which comprises logically two parts: the 2MHz offset from the band edge is the same in-band mask as presented earlier, and the remaining offsets are emission levels similar to other ETSI standards for the MSS bands. For the sake of clarity Figure 2.1-3 below shows resulting out-of-band mask for the lower edge, i.e. below 1980MHz, combined with the 3GPP NR 5MHz mask. As can be seen from the figure, we are not anticipating any extra power back-off for the NB-IOT technology, but NR NTN may require extra power back-off if the channel is right at the band edge.

Figure 2.1-3: Superposition of the ETSI out-of-band mask (pink) and 3GPP NR 5MHz mask (blue).

Observation 1a:	For NB-IOT NTN, ETSI 301442 in-band or out-of-band emission requirements should not cause any extra power back-off.

Observation 1b:	For NR NTN, further power back-off analysis is needed because ETSI mask has more stringent requirements for certain frequency ranges and offsets.

## 2.2	ETSI EN 302 574-2 and EN 302 574-3

As mentioned earlier the S-band is regulated by several ETSI standards, whereupon EN 302574-2 and 302574-3 define additional requirements for the GEO systems further differentiating between narrow and wide-band systems. However, it is worth checking whether 3GPP needs to account for EN 302574-2 and 302574-3 because these specifications do not necessarily define more stringent requirements compared to EN 301 442. As an example, Figure 2.2-1 below presents the in-band mask for the narrow-band system from 302574-3. And as can be seen, the ETSI mask in Figure 2.2-1 is even relaxed compared to the mask in Figure 2.1-1, i.e. if NB-IOT can meet a more stringent without any extra power back-off then it is not clear whether 3GPP even needs to capture requirements from EN 302 574-3.

Thus, 3GPP should check further EN 302 574-2 and EN 302 574-3 because it might be sufficient to capture only more stringent requirements from EN 301 442 making them applicable to both GEO and non-GEO systems.

Observation 2:	If ETSI EN 301442 contains more stringent requirements compared to EN 302 574-2 and EN 302 574-3, it might be sufficient to capture only one set as a common worst-case baseline.

# 3	Conclusions

In this discussion paper we presented our considerations on the ETSI requirements for the S-band and whether they can be met by the 3GPP NB-IOT and NR. Based on the presented analysis, no addition power back-off is anticipated for the NB-IOT, but NR requires further power back-off analysis.

Observation 1a:	For NB-IOT NTN, ETSI 301442 in-band or out-of-band emission requirements should not cause any extra power back-off.

Observation 1b:	For NR NTN, further power back-off analysis is needed because ETSI mask has more stringent requirements for certain frequency ranges and offsets.

Observation 2:	If ETSI EN 301442 contains more stringent requirements compared to EN 302 574-2 and EN 302 574-3, it might be sufficient to capture only one set as a common worst-case baseline.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales